About One Touch Soft Keys

There are 14 softkeys on the ScanFX , those can be defined to achieve a series jobs ( like a Micro key). For example, you can define  one of the softkeys for OCR with your Word Processor. Then you can OCR the documents by just touching the pre-defined softkey on the ScanFX panel. The system will runs the OCR, and launch your Word Processor  with the OCRed text file on the screen  ready to go!

Customizing  the  ScanFX soft key :

1. If the [ONE-TOUCH], ScanFX key Manager, is on the screen then go to step3.

2. Otherwise press [Alt-Tab] to turn the [ONE-TOUCH]  into the foreground.

3. Click the [ Key Define ] under [ONE-TOUCH].

4. Click the key number under the [ KEY DEFINE] menu. There are 14 softkeys

available on the ScanFX. Note:  Key-14 may be pre-installed for OCR and

output to Notepad or windows write.

5. Click on the number you want to program and then click the [SETUP] option.

6. Now select your application from the process application list (other application

software is available Ex. Scan&Type by GDI, CardScan by Cypher Tech).

Then make a selection from the [setting file] options:

| File name | page orientation | paper size | export file format | support word processor |
| --- | --- | --- | --- | --- |
| WP-P-11 | Portrait | letter size | WordPerfect | WordPerfect |
| AMI-P-11 | Portrait | letter size | AmiPro (*.sam) | AmiPro |
| WW-p-11 | Portrait | letter size | Win word (*.Doc) | Win word |
| RTF-P-11 | Portrait | letter size | *.RTF | Most Word Processor |
| Default | Portrait | letter size | *.TXT | Most Word Processor |

If these setting files do not fit your needs please refer to next page (Define a setting file for one touch OCR).

7. If the application is OCR related, you may need to define the [export

application]. If the export application you are looking for is not listed , please

select [User Define] in the lists and Click on [Browse]  to select the export

application.

8. After you select the export application, the command  box  will show the

executing command. For example to export to WordPerfect it should show

wp.exe.  Note if the application is not in your autoexec.bat path statement add

the full path to the command line.

9, Click [OK] to close the [Setup] window and the [keys define] windows.

Defining a WordScan Setting File for ONE-TOUCH OCR.

1. Run WordScan by double Clicking the WordScan icon.

2. Setup the WordScan export file format:

a). Click the arrow sign of the file format box, on the top-right of the 	   	     screen.

b). Click the proper file format. The default file format is ASCII.

3. Setup the WordScan Working environment:

a). Paper Size: click [Settings], then Click [Scanner Setting],  then click 	  	     the proper paper size ( letter, legal or A4).

b). Page orientation: Please click the proper Icon, on the Setup windows 	    	     (on the right of the screen ), or the setup Bar.

4. Save the settings: when  the definitions are done,  Click the File, then Click

the [save settings], then  Name the setting file.

One-Touch Trouble shooting

Message 				Solution

Application not found:   		The Application ‘s execution file was not found 						or the application setting file was not found:  						Check the path statement in your exec.bat file 						and be sure the application is included.

Export application not found	The export application execution command 						was not found:  Check to make sure the the 						.exe file was spelled correctly.  Check to make 						sure the path statement in your autoexec.bat 						file includes the application.  Include the 						complete path statement in the 								command line.