MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

���������������������������������������������������������ͻ

�                                                         �

�                Mr. Machinist User Manual                �

�                =========================                �

�                   by Allan J. Bremer                    �

�                                                         �

�                                                         �

�           Copyright (c) 1994 by Allan J. Bremer         �

�                                                         �

�                                                         �

���������������������������������������������������������ͼ

Customer Service

Customer Service Phone Number: (716)434-7823

Compuserve User Number 73113,2206

Mr. Machinist is always uploaded to these popular BBS's:

Mr. Machinist BBS - 716-434-1448             14400 - 24hrs

Deep South BBS    - 716-433-5524             14400 - 24hrs

SME BBS           - 313-271-3424             14400 - 24hrs

Exec. PC          - 414-789-4390             14400 - 24hrs

Turbo City        - 209-599-7435             14400 - 24hrs

July 26, 1994                    Page 1

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

TABLE OF CONTENTS               Page

A) PREFACE . . . . . . . . . . . . . . . . . . . .  5

B) LEGAL NOTICE  . . . . . . . . . . . . . . . . .  6

1. INTRODUCTION  . . . . . . . . . . . . . . . . .  6

1.1  What is Mr. Machinist? . . . . . . . . .  6

1.2  Why use Mr. Machinist? . . . . . . . . .  6

1.3  History of Mr. Machinist . . . . . . . .  7

1.4  How to use this manual . . . . . . . . .  8

1.5  Definition of Shareware  . . . . . . . .  9

1.6  Disclaimer Agreement . . . . . . . . . . 10

2. GETTING STARTED . . . . . . . . . . . . . . . . 11

2.1  Backing Up Your Disk and Data  . . . . . 11

2.2  Hardware requirements  . . . . . . . . . 11

2.3  Files Needed . . . . . . . . . . . . . . 12

2.4  File Created On Your Drive . . . . . . . 12

2.5  Installation . . . . . . . . . . . . . . 13

2.6  Quick Start  . . . . . . . . . . . . . . 14

2.7  Additional Materials . . . . . . . . . . 15

2.8  Registration . . . . . . . . . . . . . . 16

2.9  Customization. . . . . . . . . . . . . . 17

3. SUPPORT POLICIES  . . . . . . . . . . . . . . . 17

3.1  Technical Support BBS  . . . . . . . . . 17

3.2  Voice Calls  . . . . . . . . . . . . . . 18

3.3  Mail . . . . . . . . . . . . . . . . . . 18

4. REFERENCE . . . . . . . . . . . . . . . . . . . 18

4.1  Introduction . . . . . . . . . . . . . . 18

4.2  Bringing Up Menus  . . . . . . . . . . . 18

4.3  Moving Around  . . . . . . . . . . . . . 19

4.4  Entering Data  . . . . . . . . . . . . . 19

4.5  The Title Bar  . . . . . . . . . . . . . 20

4.6  Systems Date . . . . . . . . . . . . . . 20

4.7  Exiting Mr. Machinist  . . . . . . . . . 20

4.8  Manipulating Windows . . . . . . . . . . 21

4.8.1   Mouse. . . . . . . . . . . . . . 21

4.8.2   Keyboard . . . . . . . . . . . . 21

4.9  The Help File  . . . . . . . . . . . . . 23

4.10 Date, Time and Screen Saver  . . . . . . 23

July 26, 1994                    Page 2

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

TABLE OF CONTENTS               Page

4.11 File Menu  . . . . . . . . . . . . . . . 24

4.11.1  Open . . . . . . . . . . . . . . 24

4.11.2  New  . . . . . . . . . . . . . . 26

4.11.3  Save . . . . . . . . . . . . . . 27

4.11.4  Save As  . . . . . . . . . . . . 27

4.11.5  Save Done. . . . . . . . . . . . 27

4.11.6  Print  . . . . . . . . . . . . . 27

4.11.7  Print Answer File. . . . . . . . 29

4.11.8  Show Answer File . . . . . . . . 30

4.11.9  Change Directory . . . . . . . . 30

4.11.10 Dos Shell  . . . . . . . . . . . 30

4.11.11 Print Registration Invoice . . . 31

4.11.12 Sequence a CNC File. . . . . . . 31

4.11.13 Un-Sequence a CNC File . . . . . 33

4.11.14 Split a CNC File . . . . . . . . 34

4.11.15 Exit Mr. Machinist . . . . . . . 35

4.12 Edit Menu  . . . . . . . . . . . . . . . 36

4.12.1  Undo . . . . . . . . . . . . . . 36

4.12.2  Cut  . . . . . . . . . . . . . . 36

4.12.3  Copy . . . . . . . . . . . . . . 37

4.12.4  Paste  . . . . . . . . . . . . . 37

4.12.5  Show Clipboard . . . . . . . . . 37

4.12.6  Clear  . . . . . . . . . . . . . 37

4.13 Windows Menu . . . . . . . . . . . . . . 38

4.13.1  Resize/Move  . . . . . . . . . . 38

4.13.2  Zoom . . . . . . . . . . . . . . 39

4.13.3  Next . . . . . . . . . . . . . . 39

4.13.4  Close  . . . . . . . . . . . . . 39

4.13.5  Tile . . . . . . . . . . . . . . 40

4.13.6  Cascade  . . . . . . . . . . . . 40

4.14 Search Menu  . . . . . . . . . . . . . . 40

4.14.1  Find . . . . . . . . . . . . . . 41

4.14.2  Replace  . . . . . . . . . . . . 41

4.14.3  Search Again . . . . . . . . . . 42

4.15 Circle Menu  . . . . . . . . . . . . . . 42

4.15.1  Circle Segment . . . . . . . . . 42

4.15.2  Circle Area  . . . . . . . . . . 43

4.15.3  Circular Ring Area . . . . . . . 43

4.15.4  Circular Ring Sector Area  . . . 43

4.15.5  Circular Sector Area . . . . . . 43

4.15.6  Circumference  . . . . . . . . . 44

4.15.7  Three Points on a Circle . . . . 44

4.15.8  Pin in a Diamond . . . . . . . . 44

4.16 Math Menu  . . . . . . . . . . . . . . . 44

4.16.1  Trig.  . . . . . . . . . . . . . 45

4.16.2  Conversions  . . . . . . . . . . 46

4.16.3  Polar Coordinates  . . . . . . . 47

4.16.4  Distance Across Square Corners . 47

4.16.5  Distance Across Hex Corners  . . 47

4.16.6  Distance Between Two Points  . . 47

July 26, 1994                    Page 3

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

TABLE OF CONTENTS               Page

4.16.7  Middle of Two Points . . . . . . 47

4.16.8  Three Points, Shift and Rotate . 47

4.16.9  Bolt Circle. . . . . . . . . . . 48

4.16.10 Calculator . . . . . . . . . . . 48

4.17 Areas and Volumes Menu . . . . . . . . . 50

4.17.1  Acute Angle  . . . . . . . . . . 50

4.17.2  Barrel . . . . . . . . . . . . . 51

4.17.3  Cone . . . . . . . . . . . . . . 51

4.17.4  Cube . . . . . . . . . . . . . . 51

4.17.5  Cycloid  . . . . . . . . . . . . 51

4.17.6  Cylinder . . . . . . . . . . . . 51

4.17.7  Ellipse  . . . . . . . . . . . . 52

4.17.8  Ellipsoid  . . . . . . . . . . . 52

4.17.9  Frustum of a Cone  . . . . . . . 52

4.17.10 Frustum of a Pyramid . . . . . . 52

4.17.11 Hexagon  . . . . . . . . . . . . 52

4.17.12 Hollow Cylinder  . . . . . . . . 53

4.17.13 Hollow Sphere. . . . . . . . . . 53

4.17.14 Obtuse Angle . . . . . . . . . . 53

4.17.15 Octagon  . . . . . . . . . . . . 53

4.17.16 Parallelogram  . . . . . . . . . 53

4.17.17 Polygon  . . . . . . . . . . . . 53

4.17.18 Portion of a Cylinder  . . . . . 53

4.17.19 Prism  . . . . . . . . . . . . . 54

4.17.20 Pyramid  . . . . . . . . . . . . 54

4.17.21 Rectangle  . . . . . . . . . . . 54

4.17.22 Right Triangle . . . . . . . . . 54

4.17.23 Spandrel/Fillet  . . . . . . . . 54

4.17.24 Square . . . . . . . . . . . . . 54

4.17.25 Square Prism . . . . . . . . . . 55

4.17.26 Sphere . . . . . . . . . . . . . 55

4.17.27 Spherical Sector . . . . . . . . 55

4.17.28 Spherical Segment  . . . . . . . 55

4.17.29 Spherical Wedge  . . . . . . . . 56

4.17.30 Spherical Zone . . . . . . . . . 56

4.17.31 Torus  . . . . . . . . . . . . . 56

4.17.32 Trapezium  . . . . . . . . . . . 56

4.17.33 Trapezoid  . . . . . . . . . . . 57

4.17.34 Wedge  . . . . . . . . . . . . . 57

4.18 Rates Menu . . . . . . . . . . . . . . . 57

4.18.1  Speed and Feed for Drilling. . . 58

4.18.2  Speed and Feed for Milling . . . 62

4.18.3  Speed and Feed for Tapping . . . 63

4.19 Options Menu . . . . . . . . . . . . . . 64

4.19.1  Mouse. . . . . . . . . . . . . . 64

4.19.2  Save Desktop . . . . . . . . . . 64

4.19.3  Retrieve Desktop . . . . . . . . 65

4.19.4  Use .BAK File. . . . . . . . . . 65

July 26, 1994                    Page 4

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

TABLE OF CONTENTS               Page

5. EDITOR HELP . . . . . . . . . . . . . . . . . . 65

( Many editing procedures are discussed here. )

6. QUICK-KEY CHART . . . . . . . . . . . . . . . . 77

7. SPECIAL THANKS. . . . . . . . . . . . . . . . . 80

A) PREFACE

Welcome to Mr. Machinist.

Mr. Machinist is a program for the machinist, the engineer,

and any one else who is involved in the process of design and

engineering. It was developed for the machinist who is new at

computers and especially revolves around the CNC milling and

turning machinist and operator. The program is a useful tool

and time saver for the CAD/CAM programmer also. In fact, Mr.

Machinist can give you an answer to many problems faster than

the shops CAM software can start up!

This easy to use menu driven program is designed to solve the

every day geometry and trigonometry problems that machinist

and industrial engineers encounter.

Get the chord of a circular sector, convert decimal degrees to

hours, minutes, and seconds, find the proper speed and feed

for a machining center, view a decimal chart, use the

scientific calculator for formulas, edit multiple files that

were created manually or by your CAM software, insert or

delete N sequence numbers from CNC text files, 'split' a large

CNC text file into their appropriate program numbers instead

of cutting and pasting.  You also have the option to strike F1

for instant help on hundreds of subjects. These are just some

of the many possibilities of Mr. Machinist.

One major advantage of Mr. Machinist is its ability to

continually keep an answer file of all the functions you

choose.  This Answer file is constantly being updated as you

require answers to functions.  You may view the answer file

or print it at will.  This answer file, called MRMSOLVE.TXT,

exists on your drive only during your current session.  The

answer file is deleted upon exiting but you have the choice

of editing it or renaming it for future reference.  There are

two other files that are created if certain functions are

preformed and they are the Drill Speed and Feed, and the Bolt

Circle function.  The reason behind this is that these

functions, at times, require a large file or separate piece of

information.  You may read about these functions in more

detail in their appropriate sections of this manual.

The Mr. Machinist program is intended for any machine shop

that wants to save time and money. Some operators of machine

tools can spend a lot of time trying to figure out problems

July 26, 1994                    Page 5

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

like trigonometry or getting the chord of a circle. Some of

these functions require more than a few entries in a

calculator and can cause a machinist to sit down and scratch

his head for an hour causing all kinds of lost time.

Ultimately, the best way to use the Mr. Machinist program is to

put a computer in the center of the shop and let everyone who

needs it use it. Buy an old used computer with a mouse and run

Mr. Machinist from it. The program can run on any DOS based

computer and needs no extra memory or speed requirements.

B) LEGAL NOTICE

Mr. Machinist is distributed as is. The author disclaims all

warranties, expressed or implied. The author will assume no

liability for damages either from the direct use of this

product or as a consequence of the use of this product.

1 INTRODUCTION

1.1 WHAT IS THE MR. MACHINIST PROGRAM?

Mr. Machinist is a program that was developed by a CNC

machinist for other machinist, programmers and engineers.

It is a menu driven, text based environment with the novice

computer user in mind.

Mr. Machinist's main purpose is to save time and money and

increase production in a shop environment by solving math

calculations with the least amount of data input.

The Mr. Machinist program is not a CAD/CAM program nor is

it a drawing program. It is meant to be used along with CAM

software to further the programmers capabilities and speed

up production time. One example is finding the chord of a

circle. If you wanted to find the chord of a circle in a

major CAM software package, nine times out of ten you will

have to draw the circle and check the distance. It takes

about ten seconds to find a chord in Mr. Machinist!

Mr. Machinist can also find the solutions to many other shop

math orientated calculations like trigonometry, geometry,

conversions, areas and volumes, and the program can also

help the machinist in calculating speeds and feeds, chip

load and surface feet per minute.

1.2 WHY USE THE MR. MACHINIST PROGRAM?

Use Mr. Machinist to increase productivity, decrease down

time, and increase machining knowledge. Mr. Machinist is

also a great program to introduce to machinist who want to

get into using a computer for the first time.  90% of the

July 26, 1994                    Page 6

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

people in the average machine shop have never had the

opportunity to acquire knowledge of computers. Here is a

program that will do that and at the same time make there

job a lot easier!

1.3 HISTORY OF THE MR. MACHINIST PROGRAM

1985 - A passing thought...

1990 - A trig problem on a Vic 20...

1991 - Mr. Machinist Beta Version in Pascal...

1992 - Started developing program in October of 1992 and began

to release the Beta versions to major bulletin boards

in January of 1993.

1993 - Version 19.0 Beta, the last Beta version. ( 10-01-93 )

1993 - Version 1.00 - Released 10-31-93

1993 - Mr. Machinist 1.01  ( 12-15-93 )

Tried a mouse arrow instead of a block cursor but their

was reports of screen bugs so it's back to the block

cursor for now.

There was also a bug found in the Split CNC file function.

Program would lock up if a non-existing file was chosen.

1993 - Mr. Machinist 1.02  ( 12-21-93 )

Back to the block cursor.

CNC file split function fixed.

1994 - Mr. Machinist 1.03  ( 01-05-94 )

Started to use an overlay file called MRM.OVR.  This allo

fewer memory problems and allows the program to upgrade to

a much larger size.  The single MRM.EXE file was just to

big.

Also added an Oblique Triangle function.

1994 - Mr. Machinist 1.04  ( 02-26-94 )

Re-compiled under Turbo Pascal Version 7.0.

1994 - Mr. Machinist 1.05  ( 03-23-94 )

Found bugs in last version.  System would lock-up if a fo

place decimal was input into the 'Chip Load' input line in

the 'Milling' feed function.

July 26, 1994                    Page 7

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

System would also lock-up if you tried to sequence/un-seq

a file.

Added a 'Bugs' help screen to main help menu to display b

from previous versions.

1994 - Mr. Machinist 1.06  ( 03-30-94 )

Input line bug found in 1.05.  Also added information out

to the answer file called MRMSOLVE.TXT upon executing the

sequence and un-sequence functions.

1994 - Mr. Machinist 1.07  ( 04-06-94 )

Added extra 'Spaces' function to sequence and un-sequence

functions.  Also changed 'N' numbers to four digit leading

zeros.

1994 - Mr. Machinist 1.08  ( 05-05-94 )

Made a few minor changes in dialog box positions and then

uploaded the program to a few places.  After a few days

a decision was made to up-grade many functions and then

call it 1.09.

1994 - Mr. Machinist 1.09  ( 05-18-94 )

Moved dialog boxes out of the way of menus, caused execut

of 'OK' button by striking 'Enter' on last input line,

removed the bug that caused system lock-up in 3 points on

circle if a user input coordinates that rest on a straigh

added a function in the circle menu called, 'Pin in a Dia

that calculates a diameter within a four sided symmetrica

added a printhelp choice in the helpfile, revised the man

fixed the bug in the bolt circle function where if a user

the function once then 'Escaped' out and then used the fu

a second time right away it would cause a system lock-up,

the views in 'bolt circle' and 'drill details' to 'editab

1.4 HOW TO USE THIS MANUAL

We suggest you read the first four chapters to acquaint

yourself with the basics of shareware, the requirements and

options of the program, and the installation procedure.  Then

browse the remainder to get an idea of its content.  Once the

program is installed and running, refer to the references for

the specific area you that you may have a question about.

A HINT:  This file is plain ASCII text.  If you save the

original and make a copy, you could insert any specific key

for a text search to any topic you use frequently.  You could

also delete portions and modify the table of contents.  Then

call up the file in a second ( or third ) window in the editor

and you would have an instant reference.  This process would

July 26, 1994                    Page 8

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

give a searchable, scrollable supplement to the on-line help

from the F1 key.  *< NOTE >*  The editor in this program has

a capacity of about 64K which is smaller than this file.  You

may have to break the file into pieces, or use some type of

file viewer like LIST.COM possibly in a task swapper like the

DOS shell.

To get the most out of the Mr. Machinist program, you must

read this manual carefully. It will answer any questions you

might have about Mr. Machinist's operation.  You are able to

search through this document by line numbers if you are

viewing it from a computer program.  There are many programs

on the market that can do this but I believe the LIST program

by Vernon D. Buerg to be the best.

It is assumed that you have access to a DOS manual and you

will try your best to get acquainted with the different

commands that DOS has to offer. Even though Mr. Machinist is

an easy to use menu driven program it is still a good idea

get an understanding of the basics of DOS.

Instead of using the phrase, 'Mr. Machinist' in this document

you will see the letters 'MRM' which mean the same thing.

This will save a lot of time and space.

1.5 DEFINITION OF SHAREWARE

Shareware distribution gives users a chance to try software

before buying it. If you try a Shareware program and

continue using it, you are expected to register. Individual

programs differ on details -- some request registration

while others require it, some specify a maximum trial

period. With registration, you get anything from the simple

right to continue using the software to an updated program

with printed manual.

Copyright laws apply to both Shareware and commercial

software and the copyright holder retains all rights, with a

few specific exceptions as stated below. Shareware authors

are accomplished programmers, just like commercial authors,

and the programs are of comparable quality. (In both cases,

there are good programs and bad ones!) The main difference

is in the method of distribution. The author specifically

grants the right to copy and distribute the software, either

to all and sundry or to a specific group. For example, some

authors require written permission before a commercial disk

vendor may copy their Shareware.

Shareware is a distribution method, not a type of software.

You should find software that suits your needs and

pocketbook, whether it's commercial or Shareware. The

Shareware system makes fitting your needs easier, because

you can try before you buy. And because the overhead is low,

prices are low also. Shareware has the ultimate money-back

July 26, 1994                    Page 9

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

guarantee -- if you don't use the product, you don't pay for

it.

1.6 DISCLAIMER - Agreement

Users of Mr. Machinist must accept this disclaimer of

warranty:  "Mr. Machinist is supplied as is.  The author

disclaims all warranties, expressed or implied, including,

without limitation, the warranties of merchantability and of

fitness for any purpose.  The author assumes no liability for

damages, direct or consequential, which may result from the

use of Mr. Machinist."

Mr. Machinist is a "shareware program" and is provided at no

charge to the user for evaluation.  Feel free to share it

with your friends, but please do not give it away altered or

as part of another system.  The essence of "user-supported"

software is to provide personal computer users with quality

software without high prices, and yet to provide incentive

for programmers to continue to develop new products.  If you

find this program useful and find that you are using

Mr. Machinist and continue to use Mr. Machinist after a

reasonable trial period, you must make a registration payment

of $49.00 to F1 COMPUTING.  The $49.00 registration fee

will license one copy for use on any one computer at any one

time.  You must treat this software just like a book.  An

example is that this software may be used by any number of

people and may be freely moved from one computer location to

another, so long as there is no possibility of it being used

at one location while it's being used at another.  Just as a

book cannot be read by two different persons at the same

time.

Commercial users of Mr. Machinist must register and pay for

their copies of Mr. Machinist within 30 days of first use or

their license is withdrawn.  Site-License arrangements may be

made by contacting Allan J. Bremer.   The MRMORDER.DOC file is

provided for registration.

Anyone distributing Mr. Machinist for any kind of

remuneration must first contact Allan J. Bremer at the

address below for authorization.

You are encouraged to pass a copy of Mr. Machinist along to

your friends for evaluation.  Please encourage them to

register their copy if they find that they can use it.  All

registered users will receive a copy of the latest version of

the Mr. Machinist system.

July 26, 1994                   Page 10

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

2. GETTING STARTED

2.1 BACKING UP YOUR DISK AND DATA

The first thing you must do is to make a working copy of

your original disk. Because disks are susceptible to damage,

it is unwise to use the original program disk for regular

use. If it is ever lost or damaged you would have to

purchase a new disk or spend more time on the phone lines

downloading the program.

If you should lose or damage the working copy, you can

always use the original program disk to make another working

copy.

In order to make a working copy of your original disk or

file you will need to use one of the DOS commands that can

transfer or copy the contents of your original disk onto

another disk.

Here is one example that will copy ALL of the contents of

one disk onto another:

Let us assume that you have the Mr. Machinist program on a

high density 5 1/4 disk and you want to make a backup copy

on another 5 1/4 high density disk. You could use the

command called DISKCOPY. One thing to keep in mind about the

DISKCOPY command is that if you use it on a 5 1/4 high

density disk then you have to have a 5 1/4 high density disk

to copy it to. Both disk must be the same format!

Insert the original disk into your drive, get onto that

drive by typing its letter and a colon, ( A: ).

Now type the command, ( DISKCOPY ). At this point your

system will ask you to insert the SOURCE disk. The source

disk is your original disk. After it reads a sufficient

amount of data into your RAM memory it will ask you to

insert your TARGET disk, that's your backup or working copy

disk. Be careful not to get your source and target disks

mixed up. If your using high density disk and there is a

lot of data on it then it can take a few times of inserting

the target and source.

The only real way to get efficient at using DOS commands is

to use them and read a book that explains DOS in a way that

you can understand. Some people can learn it well from the

DOS manual and others may want to go out and purchase a

beginners guide to DOS. Either way, you will do yourself a

favor by getting good at DOS!

2.2 HARDWARE REQUIREMENTS

Any IBM or IBM compatible XT computer can operate the Mr.

Machinist program. XT, 286, 386, 486, it does not matter

what the computer is, it will run!  It does not matter what

July 26, 1994                   Page 11

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

type of video adapter you have either, it will run in color,

in mono, in Hercules, CGA, EGA, VGA, SVGA, it's all

automatic.  The program does not use extended or expanded

memory but it needs all the standard memory you can give it.

It also has no use for a math co-processor.  A mouse is an

excellent way to run the Mr. Machinist program but it is not

required.  The Mr. Machinist program must be executed from

a hard drive.  It is possible for it to run on a floppy disk

but it is very slow in execution and very non-productive.

DOS version 3.3 or above is required to run the program. It

is very advisable to run your system under DOS version 5.0

or above.  This version is a much better version than the

older versions and it also has a little help file that you

can use to get help on almost any DOS command.  Many people

never read their DOS manual and therefore they are lacking in

the knowledge of operating system commands.  If you have DOS

version 5 or above a good command to remember is the help

command.  Let's say that you need help on the COPY command.

You would type, 'COPY /?' and then on-line help will pop up.

Mr. Machinist can utilize a printer also and one is highly

recommended.  There may be times when a CNC programmer may

want to take a copy of the program to there machine for a

reference.

One thing that is not needed to run Mr. Machinist is an

expensive computer.

2.3 FILES

There are two files that are needed to run Mr. Machinist:

MRM.EXE and MRM.OVR. There are other files that come

with the program disk but are not required to execute the

program.  The third most important file is MRMHELP.HLP, the

help file.

Here is a list of the files that you should have on the

distribution disk:

FILE_ID.DIZ  =  The description file used by BBS programs.

HISTORY.TXT  =  The history of Mr. Machinist.

INSTALL.EXE  =  Mr. Machinist's installation program.

MRMORDER.DOC =  The order form to register the program.

K0203.TXT    =  A sample CNC milling program.

MRM.EXE      =  The main executable file.

MRM.OVR      =  The overlay file.

MRMANUAL.DOC =  The Mr. Machinist Manual. (Your reading it!)

MRMHELP.HLP  =  The help file.

README.TXT   =  First time information about Mr. Machinist.

WHATSNEW.TXT =  New functions in this version.

2.4 FILES CREATED ON YOUR DRIVE

July 26, 1994                   Page 12

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

Mr. Machinist Text Files

========================

The Mr. Machinist program can create, edit, and delete

three different files on your hard drive upon each execution.

MRMSOLVE.TXT,  MRMDRILL.TXT,  and MRMBOLT.TXT.

MRMSOLVE.TXT

============

This is a file that is created and deleted each

time the program is executed.  It holds all of the answers

to each function except drill speeds and feeds and bolt

circle calculations.  This file can be viewed and printed at

any time.  Holding down the 'Ctrl' key and then striking 'F1'

will let you view this file and holding down the 'Ctrl' key

and then striking 'P' will print this file.

MRMDRILL.TXT

============

This is a file that is created only when you choose the drill

speed and feed function.  It holds all of the answers

to the drill speed and feed calculations.  This file can be

viewed and printed at any time.  Holding down the 'Ctrl' key

and then striking 'F4' will let you view this file and holding

down the 'Ctrl' key and then striking 'F6' will print this

file.

MRMBOLT.TXT

============

This is a file that is created only when you choose the bolt

circle calculation function.  It holds all of the answers

to the bolt circle calculations.  This file can be

viewed and printed at any time.  Holding down the 'Ctrl' key

and then striking 'F8' will let you view this file and holding

down the 'Ctrl' key and then striking 'F9' will print this

file.

===============================================================

All of these files get  deleted or 'cleaned up' after you exit

the program.  If you would like to save these files in order

to edit them later or have them available for information then

you must rename them before you exit the program.  You would

do this by bringing them up in the editor and choosing the

'Save As' function.  If for some reason the files do exist on

your drive when you first execute MRM (Mr. Machinist) the

program will detect this and ask you if you want to delete

them or leave them alone.  If you do not delete them then the

new information will be appended or 'added' to the already

existing files.

2.5 INSTALLATION   (Read README.TXT file for install updates!)

The installation program makes it easy for a user to transfer

July 26, 1994                   Page 13

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

the proper files from a disk to their hard drive.  The

installation program is only meant to be executed from a

disk drive and not the hard drive.

The files that get copied in the installation process are as

follows:

FILE_ID.DIZ  =  The description file used by BBS programs.

HISTORY.TXT  =  The history of Mr. Machinist.

MRMORDER.DOC =  The order form to register the program.

K0203.TXT    =  A sample CNC milling program for tutorial.

MRM.EXE      =  The main executable file.

MRM.OVR      =  The Overlay file.

MRMANUAL.DOC =  The Mr. Machinist Manual. (Your reading it!)

MRMHELP.HLP  =  The help file.

README.TXT   =  First time information about Mr. Machinist.

WHATSNEW.TXT =  New functions in this version.

2.6 QUICK START

If you have been following this manual then you should

have a working copy of the Mr. Machinist program and your

original disk is put away some where. If your not running

the program from a disk, which is true 99% of the time, then

you should have two working copies of the Mr. Machinist

program and also your original disk.

To start-up Mr. Machinist simply enter, 'MRM'. To preform a

function, hold down the 'Alt' key and strike the highlighted

letter of your choice from the upper menu. If you wanted to

edit a file you would hold down the 'Alt' key and then

strike 'F', this will bring down the file menu. From there

you could choose from a new file to one that already exists.

At this time you can see all the other menus by simply

pressing your right or left arrow keys. Press the 'Enter'

key to view the choices under the menu item. Always remember

that help is always near by simply striking the 'F1' key.

Keep in mind that there will always be a file called

MRMSOLVE.TXT created and deleted each time you run the

program.  This file holds all of your answers for viewing

or printing at any time.  At the end of the session, upon

exit of the program, MRM will delete this file.  There is

a possibility of two other files being created and deleted

the same way.  They are MRMDRILL.TXT and MRMBOLT.TXT.  The

reason behind these two files being separate from the main

answer file called MRMSOLVE.TXT is that they could, at times,

hold a lot of information.  If you wanted to calculate a bolt

circle pattern with 237 holes then this file will have all of

those X and Y coordinates for you to print.  You also may, at

times, want to know some drilling speeds and feeds and only

that.  MRMDRILL.TXT holds all of the hardness, speed and feed,

information for you to view or print at any time.

July 26, 1994                   Page 14

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

2.7 ADDITIONAL MATERIALS

It is very advisable to own your own Machinery's Handbook.

The latest one at this writing is the 24th Edition. This

book is considered to be the Machinist's 2nd Bible, the 1st

Bible should be the real one!

There are many books about formulas and feed rates and

engineering calculations available at your public library.

There is one excellent publisher that is considered to be

the main supplier of industrial books and that is the

Industrial Press, Inc. Write or call them for a catalog

of all their books.

INDUSTRIAL PRESS, INC.

200 MADISON AVENUE

NEW YORK, NY 10016

PHONE (212) 889-6330

2.8 REGISTRATION

MR. MACHINIST (TM) for MS-DOS

INDIVIDUAL USE

* Diskette with programs and documentation ... $49.00

SITE LICENSE

============

The following information describes the Mr. Machinist

registration fees for use on more than one computer:

Site licenses are available for Mr. Machinist.  A site

license is needed if Mr. Machinist will be used on more

than one computer.  The rates of these licenses are as

follows:

2  to     9 computers ..... at $39 each

10  to    24 computers ..... at $30 each

25  to    49 computers ..... at $24 each

50  to    99 computers ..... at $18 each

100  to   200 computers ..... at $14 each

200  to   500 computers ..... at $12 each

Write or call for pricing on quantities over 500.

FUTURE ENHANCEMENTS

Job log database

Points/Tangent math

Larger Edit Buffer

Windows version in C++

News letters (Registered users)

July 26, 1994                   Page 15

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

Allan Bremer

134 West Ave. #2

Lockport, NY 14094

(716)434-7823 --> Voice  ( Please, Not after 10:00pm )

(716)434-1448 --> Mr. Machinist BBS (14400 24hrs)

(716)433-5524 --> Deep South BBS (14400 24hrs)

See the file called MRMORDER.DOC for more registration

information.

Here is an example of the file MRMORDER.DOC:

============================================

TO:                     FROM:

F1 COMPUTING                   Name: _________________________

134 WEST AVE. #2

LOCKPORT, NY 14094        (Company): _________________________

716-434-7823 Voice          Address: _________________________

716-434-1448 MRM BBS

716-433-5524 Deep South BBS     City: _________________________

Today's

date: ___________   State, ZIP Code: _________________________

(Country): _________________________

Daytime

Phone #: _________________________  FAX #: ___________________

MR. MACHINIST (TM) for MS-DOS

INDIVIDUAL USE

* Diskette with programs and documentation ..... $49   _____

MULTIPLE USE

* SITE LICENSE for the use of MR. MACHINIST.

(INCLUDES ONE DISKETTE WITH PROGRAMS AND DOCUMENTATION.)

2 to   9 computers .. at $39 each # computers ___x 39   ____

10 to  24 computers .. at $30 each # computers ___x 30   ____

25 to  49 computers .. at $24 each # computers ___x 24   ____

50 to  99 computers .. at $18 each # computers ___x 18   ____

100 to 199 computers .. at $14 each # computers ___x 14   ____

200 to 500 computers .. at $12 each # computers ___x 12   ____

WRITE OR CALL FOR PRICING ON QUANTITIES OVER 500.

Extra program disk & documentation with purchase of

SITE LICENSES OF 2 OR MORE available at $8.00 each.  ___x 5 __

Shipping & Handling (Postal) US & Canada $5.00/item

July 26, 1994                   Page 16

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

Outside US & Canada $11.25/item                          _____

NEW YORK RESIDENTS add applicable State & County Sales Tax____

TOTAL ENCLOSED US FUNDS                             US$________

Registered users will receive upgrades for 50% of the current

registration fee and registered site licenses will receive

upgrades for 25% of current registration fees for life.

MS-DOS is a registered trademark of Microsoft Corporation.

2.9 CUSTOMIZATION

If there are certain formulas or functions and/or procedures

that your company or you would like to see in MRM let us know

on the registration order form.  You or your place of

employment may use a standard formula that requires a lot of

input and time to get the result.  Well, with Mr. Machinist

you only need input the known values and all other formulas

are done instantly!  Still figuring out formula upon formula

using your little calculator?  Get it into Mr. Machinist and

get productive!

Customization can be done for registered users.  The cost

depends upon the time and size of the procedure but most

formulas can be implemented at the standard cost of 50% of

the registration fee.

For further information on customization, contact the author

at the address given.

3. SUPPORT POLICIES

3.1 TECHNICAL SUPPORT BBS

There is no support BBS for the Mr. Machinist program at this

time, however you may call the BBS's listed here:

Mr. Machinist BBS: 716-434-1448                14400 - 24hrs

Deep South BBS:    716-433-5524                14400 - 24hrs

SME BBS:           313-271-3424                14400 - 24hrs

Exec PC:           414-789-4390                14400 - 24hrs

Turbo City:        209-599-7435                14400 - 24hrs

If you have trouble getting a copy of the latest version please

leave a message on Compuserve for user number 73113,2206.  You

may also send $5.00 to the author and you will be sent the

latest version on a disk size that you specify.  All mail is

sent 2-day air.  The easiest way to receive the Mr. Machinist

program is to call the Mr. Machinist BBS, the only time the line

would be busy is if someone else is on or the author is using

the computer.

July 26, 1994                   Page 17

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

3.2 VOICE CALLS

There is a customer service number for registered users to

call.  If unregistered users wish to speak to the author to

confirm his existence or to give comments and suggestions

then they are free to do so.  We cannot, however, accept

collect calls.

Customer service phone number: (716) 434-7823 (Voice)

3.3 MAIL

If you have comments or suggestions or bug reports then

you can certainly send them in the mail.  When you register

you will print and fill out the included file called

MRMORDER.DOC and send it to:

F1 COMPUTING

134 WEST AVE. #2

LOCKPORT, NY 14094

4. REFERENCE

4.1 INTRODUCTION

The reference section of this manual is designed to give

you the information you need to understand the functions

of the Mr. Machinist program and at the same time to

explain the reasoning behind some of the methods.  It

takes you through all of the functions of the program and

gives you many hints on how to operate the program in such

a way as to give you peak performance and productivity.

Remember, if your viewing this file from your computer you

may use a program like LIST by Vernon D. Buerg (c) and use

the line number functions to search the document.  The

line number functions for the list program are the '+',

'-', and the '#' keys.  If you hold Shift and strike the

'#' key you will be prompted for the line number to go to.

If you strike the '+' key you will be prompted for the

line number to go to forward from the point you are at.

If you strike the '-' key you will be prompted for the

line number to go to backward from the point you are at.

This method makes it very simple to get to where you want

to be in the document at any given time.

4.2 BRINGING UP MENUS

In this type of standard interface you will find there are

many different ways to do the same thing.  You can strike

the 'F10' key and the cursor will enter into the top menu

words.  From there you can move your arrow keys to the

left or right to land on the menu of your choice.  Once

July 26, 1994                   Page 18

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

your on the menu title you can then strike 'Enter' and the

menu will be 'pulled down' for your viewing.  Using your

arrow keys once again you can go up or down in the menu

to highlight your choice.  When your menu choice is

highlighted you may strike 'Enter' to 'pop up' your menu.

The fastest method of menuing in Mr. Machinist is by using

your mouse to point and shoot.  To bring up a menu using

this method you would double click on the menu title to

pull down the menu and then double click on your menu

choice to execute the function.  For right handed people

the left mouse button is used to double click.  If your

left handed you can use the mouse option in Mr. Machinist

to switch buttons so that the right mouse button is the

execute switch.  Keep in mind that there is a time limit

to your double clicking. If you double click too slow your

choice will not be executed.  Mr. Machinist does have an

option that you can choose in order to slow down or speed

up your mouse double click.

4.3 MOVING AROUND

In no time at all you will begin to feel at home with this

type of interface or 'look'.  As you get familiar with the

program you will learn to 'drag' the windows around, edit

files, print, and do math functions without ever getting

rid of a window.  This type of interface is used very

widely in the computer environment.  The things you will

learn while using this program will be used in many other

programs that you come across.

4.4 ENTERING DATA

In order to get answers you must give questions and the

way to give questions in MRM is to input data in input

lines.  The two most important keys that have to do with

an input line is the 'Tab' key and the 'Shift' key.

These are the keys that cycle you through the dialog boxes.

In other words you can move to the different choices in a

dialog box by using these keys.  Using a mouse you would

have to click once in the input line and you will be ready

for input.  You may also use your arrow keys to cycle through

input lines.

Some input lines will only take certain types of data.  In

most input lines you will be expected to insert numerical

data and not letters.  If you happen to input the wrong

type of data into an input line, you will be displayed an

error message telling you what you did wrong and then you

will be taken back to the dialog box so you can start

over again.

July 26, 1994                   Page 19

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

4.5 THE TITLE BAR

Every window has a title bar.  The title bar is simply the

name of the file that is associated with this particular

window.  The title bar of a dialog box us usually an

indicator as to the function of that dialog.

��[�]����� Title �����[ ]ͻ <--Title Bar

�                         �

�                         �

�                         �

�                         �

�                         �

��������������������������� <--Resize corner

4.6 SYSTEMS DATE

If your computer system has the proper date set then

MRM will display today's date at the top right of your

screen.  Today's date will also be inserted automatically

into the answer file called MRMSOLVE.TXT which is created

on your hard drive each time the program is executed or

run.  You will also observe the date that the program and

help file was compiled or created.  From the main program

you can do an 'Alt-F1' and the 'about' information box

will be displayed.  An 'Alt-F1' is done by holding down

the 'Alt' key and then tapping the F1 key.  After you tap

'F1' you then let up on the 'Alt' key.

4.7 EXITING MR. MACHINIST

You exit MRM by 'Alt-X', clicking on the bottom portion of

the screen over the letters 'Alt-X', or pulling down the

file menu and then selecting 'Alt-X'.  You will find that

through out MRM there will be many ways to do the same

thing.  This gives you the chance to do what you like to

do best.

In the shareware version of the program you will

be asked if you would like to print a registration invoice

so you can be a registered user.  If you choose 'Yes' you

will print a copy of the file MRMORDER.DOC on your

printer.  If your printer is not working properly for some

reason then you will be displayed an Error box telling you

that there is a problem.  If you choose 'No' from the

print invoice message you will exit with a information

message just before DOS takes over.

These 'nag' screens will not appear in a registered

version.  An unregistered shareware version of MRM is a

little slow due to these start and finish nag screens.  To

be productive with the program you must become a registered

user.

July 26, 1994                   Page 20

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

4.8 MANIPULATING WINDOWS

Many windows can be open at one time while your using MRM.

At times there may be windows that are blocking the view

of other windows, this is when a user needs to know how to

manipulate windows quickly.

Windows are manipulated by the mouse, the keyboard, or

both mouse and keyboard.  Manipulation is done by resizing

and/or moving.

4.8.1 USING A MOUSE TO MOVE WINDOWS

You can move a window using the mouse by

aiming for the 'header' of the window and then 'dragging'

the mouse.  The header is the top double line of a window

and it usually has a file name at the top or a caption.

To 'drag' the mouse means to click the mouse and hold the

button down while moving the mouse around.  Once you get

to where you want to be you release the button.  You can

resize the window using the mouse by 'dragging' the lower

right hand corner of the window.  The resize corner looks

different than any other corner of the window.

To 'zoom' a window using the mouse you must double click

anywhere on the top line of a window.  When this is done

the window will take up all of the screen except the

program headers and footers.  If you double click the top

of the window again it will go back to it's original size.

You can also cycle through window sizes the same way by

striking the 'F5' key.

You can activate a window by clicking anywhere on it or in

it.  The active window is shown by a double outlined box

and all the other windows are single lined.  Always

remember, some windows may be hidden by other windows and

therefore you can't find them to click on them!

Well, no problem, you could either move your windows

around to see if there are other windows behind them or

you can strike the 'F6' key which is the 'next' key.  Using

the 'next' function you will be cycled through the windows

in the order that they were brought up.

To close a window using the mouse you would go to the

upper left of any window and there you will see a small

box that is enclosed in brackets, that is the 'close box'.

Clicking on the close box will close the window for good.

4.8.2 USING THE KEYBOARD TO MOVE WINDOWS

To fastest way to access the windows menu is to use

July 26, 1994                   Page 21

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

'Alt-W'.  This will bring up the windows menu.  You will

notice six different choices:

Resize/move Ctrl-F5

Zoom             F5

Next             F6

Close        Alt-F3

Tile

Cascade

To resize a window using the keyboard you would highlight

the Resize/move menu choice and then strike 'Enter'.  Of

course for this function to work you have to have a window

open in the first place.  After you strike 'Enter' you

will notice that the window border goes from a double line

to a single line.  This indicates that the computer is in

the process of manipulating that window.  You will not be

able to strike other keys or do other functions until your

move is complete.  When the window has a single line

border it is ready and waiting to be moved.  You now must

use the arrow keys to move the window in the size that it

is now at.  If you want to make the window smaller or

larger you hold down the 'Shift' key while you use the

arrow keys.  Using these two methods you can place a

window anywhere on the screen and make it any size you

want.  After you have completed the move or resize you

must do one last important thing, strike the 'Enter' key

to complete the process!  If you forget to do this you

might think that you locked up your computer when all

you really need to do is to strike 'Enter'.

To 'Zoom' a window to full size and then back to its

previous state you would toggle the 'F5' key or highlight

the menu choice and strike 'Enter'.  To toggle means to go

off and on, back and forth, etc.

The 'F6' key is the 'Next' key which cycles you through

your open windows.  This function is activated by actually

striking the 'F6' key or highlighting the 'Next' key using

your arrow keys and then striking 'Enter'.

To 'Close' a window use 'Alt-F3' or highlight the 'Close'

menu choice using your arrow keys and then strike 'Enter'.

To 'Tile' a window means to place all windows that are

currently open, next to each other.  The meaning of the

word lies behind floor tile.  When you look at a floor

that has tile on it you will notice that all the squares

are next to each other.  This is just how the 'Tile'

function works.  To activate this function you would

highlight the 'Tile' menu choice and then strike 'Enter'.

To 'Cascade' a window means to place one window on top of

another while letting the previous windows header be

exposed.  To activate this function you would highlight

July 26, 1994                   Page 22

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

the 'Cascade' function and then strike 'Enter'.

4.9 THE HELP FILE  ( F5 will Resize it )

The help file is one of the major functions of MRM.  It is

basically a huge book with many cross references.  When you

see words that are highlighted in yellow you can 'click' on

them for there cross reference.  If you have a monochrome

monitor you will notice the difference of these words by

a brighter color.  If you do not have a mouse you can use

the 'Tab' key to access the cross references.

You will also notice that you can go to an index by striking

'Enter' at any time while your in the help file.  The

index is actually a dictionary of manufacturing terms

geared toward a Machinist or Industrial Engineer.

The help file is actually the manual on-line!  There is a

help function for everything in MRM!  If at any time you

you have a question about what you are doing or your stuck

at comprehending a certain function then you should go

to the help file.  This help file is growing rapidly as

new versions of MRM come out.  There basically is no limit

on the size of the help file so maybe it will eventually

be a machinist handbook on line!

There is much more information in the help file than one

may expect.  Decimal charts, formulas, G and M code

definitions, manufacturing history, machining hints, and

much more.

4.10 DATE, TIME, AND SCREEN SAVER

The date that your system has accepted will be shown in

the upper right hand corner at the top of the screen.

This date should be today's date.  If it is not then you

should re-set your systems date by using the DATE command

through DOS.

If you have the wrong date on your system then all the files

that your system creates will have the wrong creation date

and time stamped in them.

The time is shown in the lower right hand corner of the

screen.  This is the system time and it should be set the

same way as the date.  If it is wrong then you should use

the TIME command through DOS.

There is a screen saver in the Mr. Machinist program that

helps computer monitors from burn-in.  This especially

helps monochrome monitors because they are easily done in

by burn-in.  There is no option for a user to set the time

of the screen saver activation at this time.  Possibly in

July 26, 1994                   Page 23

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

future versions this option will be installed.  For the

time being, the screen saver is set to begin after five

minutes of nothing happening.  If you touch a key or move

the mouse then the count down will begin all over again.

The message that the screen saver will display can be

customized by registered users when they specify on the

registration form.  The shareware version shows:

'Strike Esc key to return to Mr. Machinist'

4.11 FILE MENU

The file menu brings down a menu full of choices that have

to do with editing and printing files.  This menu also has

the change directory function which helps you search

certain directories without typing in full file names.

There is also a Dos Shell function within this menu that

'shells' you out to DOS.  To 'shell' out means to leave

your program temporarily and go into DOS so you can

execute other programs.  Keep in mind that when you do

this, Mr. Machinist still takes up a little memory so do

not try to execute a program that uses a lot of memory or

you will not be able to execute it.  To get back into your

program after shelling out type 'EXIT' at the DOS command

line.  One thing you can also do to make DOS shell easier

is to write a small batch file so that when you strike E

a batch file called E.BAT executes and gives the command

EXIT.

The last command in the file menu is the 'Alt-X' or 'Esc'

command.  This command will exit Mr. Machinist.  Strike it

any time to exit the program.  If your editing a file and

you have not saved it, the program will ask you to save it

before exiting.

4.11.1 File Menu - Open   [F3]

To access the 'Open' menu choice using the keyboard

you must use 'Alt-F' to first bring down the file

menu and then strike 'Enter'.  You do not need to

use the arrow keys to highlight the menu choice if

it is your first time into the 'File Menu' because

the 'Open' menu choice will already be highlighted

by default seeing that it is the first choice on the

list.

Using the mouse you would click on the 'File' menu and

then click on the 'Open' line.

The ultimate way to get to the 'Open  a File' dialog

box is to strike the 'F3' key!

July 26, 1994                   Page 24

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

This will open a file selection dialog box that will

display all the files in your current directory.

There is a limit on the number of files in a directory

that this file dialog box will seek.  Let's say you have

400+ files in one directory, well when you strike 'F3'

to bring up the file dialog box it may say, 'To many

files'.  This means that you have to many files in the

current directory for this function to handle.  This

limitation is being worked on so in the near future

this problem will not exist.

At the top is an input line under the 'Name'

designation.  You may type in the file name of your

choice and then strike 'Enter'.  You do have the

option of using wild card characters.  If you do not

know the name of the file you want to edit or the

file is in another directory then you have two

choices in the method of retrieval.

1) From the file menu select, 'Change Dir'.  A

directory selection box will appear and you can use

the keyboard or the mouse to highlight your choice,

then strike 'Enter' or double click with the mouse to

get to that directory.  Then the 'Change Dir' box

will display the files and sub-directories of that

particular choice.  At that point you can highlight

the file name of your choice and strike 'Enter' or

double click.

2) If you do not know the name of the file you want

to edit then you will have to search for it.  If you

remember the first few characters of the file name,

you could use wild cards to cause the selection box

to display only those files that meet your criteria.

Example: RA??.* will show you all files that start

with RA and have one or two characters after it and

any extension or no extension.

RAT.TXT

RA.DOC

RA

RA55

If you type a file name in the input line that does

not exist and then strike 'Enter', the editor will

create and open an empty file for you by that name.

You will notice a down arrow symbol to the right of

the input line, this is a history list, a list of

names you have already used.  In order to view

certain files that you have already viewed using

this dialog then you simply strike the down arrow

July 26, 1994                   Page 25

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

and a list of previous viewed files will appear.

You have the option to choose file names from it by

highlighting the file and striking enter or double

clicking it with your mouse.

Choosing the 'Open' box will load the file you have

chosen into a new edit window.

The file list box is the largest box of the file

selection dialog box.  This box will show you every

file that matches up to your specifications in the

input line above it.  It will also display the child

and parent directories of the current directory.

(You can further your understanding of the principles

of directories and sub-directories by referring to

your DOS manual.)

There are some helpful functions you can use while

your in the file list box.  One way to get into the

file list box is to strike 'Enter' after you have

chosen 'Open' from the 'File' menu.  Another way is

to click on any file name in the box with your mouse.

Your 'Tab' key on the key board will cycle you

through the many choices of your file selection box

in one direction, while holding down the 'Shift' key

and then striking 'Tab' will cycle you through in the

other direction.  The Page Up and Down keys will let

you view more files if there are more than can fit in

one windows view port.  The arrow keys will cycle you

through individually.

When your in the file list box you can also type a

lower case letter to search for a file name that

starts with that letter.  If you type a second

letter, it will search for the file name with the

first letter you typed and the second letter you

typed.  You can continue this for all file letters

or number characters.  If you type a capital letter

you will search for a directory name in the same

manner.

At the bottom of the file dialog box is an

information panel.  This will tell you all of the

information of the file that is currently

highlighted.

4.11.2 File Menu - New

The 'New' command will bring up an edit window with

no title.  At that time you can start editing a new

file.  If you want to save it you can bring up the

file menu and choose 'Save as' or 'Save'.

Either way you will be asked to give the file a name

because it does not have a name and it needs one.  If

July 26, 1994                   Page 26

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

you try to 'Exit' the program without saving it you

will be asked if you want to save it or not.  It's

always a good idea to save the file your working on

about every 5 minutes or so.

4.11.3 File Menu - Save   [F2]

The 'Save' menu will save a named file to your disk.

You will be asked to give the file a name if you try

to save an un-named file.  You must be editing a file

before this can be active.  If your editing a file

that is already named, the 'Save' command will save

the file without showing you a thing.  You'll see

your hard drive light come on if your saving it to

the hard drive or you will see your floppy light

light up.

The fastest way to save a named file is to strike

the F2 key!

4.11.4 File Menu - Save as

Save text in the current window to disk under a new

file name.  You are asked to which file you would

like to save the text.  The file may be an existing

disk file, or a new file.

Once your text is saved, you can continue using the

editor on the current edit window.  Note that the

window title will change to reflect your new file

name.

This command is disabled until you open an edit

window.

4.11.5 File Menu - Save Done   [Ctrl]-[K]-[D]

Saves text in the current window to disk and closes the

edit window.  If your window is titled UNTITLED,  you

will be asked to supply a file name.  Otherwise, the

text is saved to disk using the current window title.

This command is disabled until you open an edit

window.

4.11.6 File Menu - Print

You will be able to search through a dialog box

and choose the file that you would like to print.

If you are editing a file or files and you would

like to print one of them, then strike 'F8'.

July 26, 1994                   Page 27

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

You may print the current file that is being edited

by striking the 'F8' key.  A window will pop up

asking you if you really want to print the current

file.  Using the mouse you may click on yes, strike

'spacebar' or 'Enter' and your file will print.

Strike the 'Esc' key to exit and say no, or you may

click on the 'no' button.

This will open a file selection dialog box that will

display all the files in your current directory.  At

the top is an input line under the 'Name'

designation.  You may type in the file name of your

choice and then strike 'Enter'.  You do have the

option of using wild card characters.  If you do not

know the name of the file you want to print or the

file is in another directory then you have two

choices in the method of retrieval.

1) From the file menu select, 'Change Dir'.  A

directory selection box will appear and you can use

the key board or the mouse to highlight your choice,

then strike enter or double click with the mouse to

get to that directory.  Then the 'Change Dir' box

will display the files and sub-directories of that

particular choice.  At that point you can highlight

the file name of your choice and strike 'Enter' or

double click to print that particular file.

2) If you do not know the name of the file you want

to print then you will have to search for it.  If you

remember the first few characters of the file name,

you could use wild cards to cause the selection box

to display only those files that meet your criteria.

Example: RA??.* will show you all files that start

with RA and have one or two characters after it and

any extension or no extension.

RAT.TXT, RA.DOC, RA, RA55

If you type a file name in the input line that does

not exist and then strike 'Enter', an error will

occur and you will be told that there is no such file

in that path, by that name.  You will notice a down

arrow symbol to the right of the input line, this is

a history list, a list of names you have already

used.  Strike the down arrow while your in the input

line to activate it.

You have the option to choose file names from it by

highlighting the file and striking enter or double

clicking it with your mouse.

Choosing the 'Open' box will print the file you have

July 26, 1994                   Page 28

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

chosen.

The file list box is the largest box of the file

selection dialog box.  This box will show you every

file that matches up to your specifications in the

input line above it.  It will also display the child

and parent directories of the current directory.

(You can further your understanding of the principles

of directories and sub-directories by referring to

your DOS manual.)

There are some helpful functions you can use while

your in the file list box.  One way to get into the

file list box is to strike 'Enter' after you have

chosen 'Open' from the 'File' menu.  Another way is

to click on any file name in the box with your mouse.

Your 'Tab' key on the key board will cycle you

through the many choices of your file selection box

in one direction, while holding down the 'Shift' key

and then striking 'Tab' will cycle you through in the

other direction.  The Page Up and Down keys will let

you view more files if there are more than can fit in

one windows view port.  The arrow keys will cycle you

through individually.

When your in the file list box you can also type a

lower case letter to search for a file name that

starts with that letter.  If you type a second

letter, it will search for the file name with the

first letter you typed and the second letter you

typed.  You can continue this for all file letters or

number characters.  If you type a capital letter you

will search for a directory name in the same manner.

At the bottom of the file dialog box is an information

panel.  This will tell you all of the information of

the file that is currently highlighted.

4.11.7 File Menu - Print Answer File   [Ctrl]-[P]

There is a file that is created on your drive each

and every time you execute MRM called MRMSOLVE.TXT.

This file holds the answers to every function that is

executed in the program.  It gives the date and time

of file creation and neatly formats your answers so

that you may choose to print them out at any time by

holding down the 'Ctrl' key and then tapping the 'F1'

key.

MRMSOLVE.TXT is not something that is in memory, it

is a file that gets written to every time you do a

math function in the program and it gets deleted upon

program exit.  If you choose to keep the file for

later reference you must rename the file.

July 26, 1994                   Page 29

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

4.11.8 File Menu - Show Answer File   [Ctrl] [F1]

'Ctrl-F1' will bring up the answer file called

MRMSOLVE.TXT in the editor for you to view or edit.

This is the file that contains all of your answers to

the current session.

4.11.9 File Menu - Change Directory

From the 'File' menu select, 'Change Dir'.  The

'Change Directory' box will appear and you can use

the key board or double click the mouse to highlight

your choice, then strike 'Enter' or double click to

enter that directory into the input line.  At that

point you can click the mouse on the 'OK' button or

strike 'K' to make the choice valid and make this

directory the default one.  If you choose the 'Chdir'

button you will make the input lines directory your

choice also but it will not continue to be the

default.

Your 'Tab' key on the key board will cycle you

through the many choices of the 'Change Directory'

box in one direction, while holding down the 'Shift'

key and then striking 'Tab' will cycle you through in

the other direction.  The Page Up and Down keys will

let you view more directories in the directory tree

list box if there are more than can fit in one

windows view port.  The arrow keys will cycle you

through individually.

If you type a directory name in the input line that

does not exist and then strike 'Enter', or click on

'OK' you will get an error message that says,

"Invalid drive or directory".

You will notice a down arrow symbol to the right of

the input line, this is a history list, a list of

directories that you have already used.  You have the

option to choose from it by highlighting the

directory and striking 'Enter' or double clicking it.

Choosing the 'Revert' button makes your previous

choice the current directory.

4.11.10 File Menu - Dos Shell

To 'shell' out means to leave your program

temporarily and go into DOS so you can execute other

programs.  Keep in mind that when you do this,

Mr. Machinist still takes up a little memory so do

not try to execute a program that uses a lot of

memory or you will not be able to execute it.  To get

back into your program after shelling out type 'EXIT'

July 26, 1994                   Page 30

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

at the DOS command line.  One thing you can also do

to make DOS shell easier is to write a small batch

file so that when you strike 'E' a batch file called

E.BAT executes and gives the command EXIT.  If you

need more information on creating BATCH files you can

look into a DOS manual.

Remember that the library has rows of computer books

to!

4.11.11 File Menu - Print Registration Invoice   [F9]

This option will check to see if your printer is

online and then it will print an invoice so that you

can register this software.  If you have registered

the Mr. Machinist program than this menu choice is

here for you to give order forms to other companies

and friends who may want the program.

For a $49.00 registration fee you will become a

registered user of the Mr. Machinist program with

a license for use on one computer only.

For SITE LICENSE REGISTRATION and other information

see the enclosed file called MRMORDER.DOC.

To view the order form before you print it, tap your

'tab' key.  That will cause the cursor to shoot down

to the first help word which in this case is

MRMORDER.DOC.  Once the phrase is highlighted you can

strike the 'Enter' key to view the document.

4.11.12 File Menu - Sequence a CNC File   [Ctrl]-[F3]

A sequence number is usually the first word that

appears in a block or sentence of a text file for CNC

machines.  There are two main reasons behind a

sequence number.

1) Lets the operator know which line the machine is

currently processing.

2) Provides the operator a way to search or find a

particular line.

These sequence numbers usually go to four digits and

begin with an N.

The very first line of a CNC program or 'text file' alw

begins with an O on the CNC machine but not always in a

file on a personal computer.

Here is an example of a G and M code file with

sequence numbers:

July 26, 1994                   Page 31

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

:0203 (05-07-93)

N0005 (ARM-ROCKER)

N0010 (123456 JOB#1234)

N0015 G00 G17 G40 G49 G80

N0020 G30 G91 Z0

N0025 T01M6

N0030 (.625 EMILL)

N0035 G55 G90 G00 X-.65 Y0.0

N0040 S1800 M03

N0045 G43 Z1. H12

N0050 M08

N0055 Z.1

N0060 G01 Z-.031 F20.0

N0065 X.65 F5.0

The Mr. Machinist program has a function in it that

can insert N sequence numbers into an un-sequenced

G and M code file on the personal computer.

If there are already scrambled sequence numbers in

the file it would be wise to remove the sequence

numbers using the remove function and then

re-sequence the file using this function.

There is also the capability to insert or remove

spaces between letters and numbers in a 'G' coded

file for the sequence and un-sequence function. Some

shops like to get rid of the spaces for more disk

space.  The CNC controler can handle and except a

file whether it has spaces or not.

To Insert Sequence Numbers into a File

======================================

A Sequence dialog box will first display and it

will ask you for the 'source' file, the file that

needs to be sequenced.  At this input line you

insert the file that you would like to sequence.

The next input line is the destination file.  The

Mr. Machinist program will not over write the old

file but it will create a new file so that you will

always have your original to go back to.

The next input line needs to know the 'N' start

number.  All 'N' numbers will have leading zeros and

four digits but you do not need to type in the leading

zeros.  If you want your first number to be 'N0010'

you would type '10' in this input line, you do not

have to insert the 'N', input only the number.

Next you will be asked for the increment number.

This number can not be greater than 10 because if you

incremented at a very large number your file could go

July 26, 1994                   Page 32

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

over the CNC control's alloted amount of 4 digits.

Lastly you will be asked to input 'y/n/a'.  This

input line has to do with spaces.  Here is the

definition of what 'y/n/a' means:

y = (y)es, add one space between every letter and number

in the file, except for the first letter 'N'.  If

there is already a space between each letter and

number in the file then after this function there

will be two spaces between them, you probably do

not want that.  Some 'CAM' programs generate 'G'

coded files that have no spaces between letters and

numbers.  Spaces create what is called, 'white spac

White space can make a file much more readable!

n = (n)o spaces.  If you want a single space removed

between each letter and number then type 'n'.  If t

file does not have a space between a letter and

number then nothing will happen.  If the file has t

spaces between a letter and a number then it will

remove one space and leave the other after this

function goes to work.  This 'remove space' function

can save disk space on a computer but with today's

hard drives...what's a space?

a = (a)lone or you could say, 'leave it alone'.  This

choice will sequence a file but will leave it the

way the original was, except for the sequenced 'N'

numbers ofcourse.

4.11.13 File Menu - Un-Sequence a CNC File   [Ctrl]-[U]

The Mr. Machinist program has a function in it that

can remove 'N' sequence numbers from a CNC text file

on your computer.  In the first input line you will

be asked for the source file that you want the 'N'

numbers removed from.  In the next input line you will

insert the name of the new destination file.  If the

new file exists on your drive you will be shown an

error box and then you will start the process over.

The removal function will un-sequence every line

except lines that do not have 'N' sequence numbers in

them.

Lastly you will be asked to input 'y/n/a'.  This

input line has to do with spaces.  Here is the

definition of what 'y/n/a' means:

y = (y)es, add one space between every letter and number

in the file, except for the first letter 'N'.  If

there is already a space between each letter and

number in the file then after this function there

July 26, 1994                   Page 33

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

will be two spaces between them, you probably do

not want that.  Some 'CAM' programs generate 'G'

coded files that have no spaces between letters and

numbers.  Spaces create what is called, 'white spac

White space can make a file much more readable!

n = (n)o spaces.  If you want a single space removed

between each letter and number then type 'n'.  If t

file does not have a space between a letter and

number then nothing will happen.  If the file has t

spaces between a letter and a number then it will

remove one space and leave the other after this

function goes to work.  This 'remove space' function

can save disk space on a computer but with today's

hard drives...what's a space?

a = (a)lone or you could say, 'leave it alone'.  This

choice will sequence a file but will leave it the

way the original was, except for the sequenced 'N'

numbers ofcourse.

4.11.14 File Menu - Split a CNC File   [Ctrl] [F10]

Before we get into 'Split a CNC File' let's begin

with clearing a CNC's storage capacity:

Clear Your CNC's Storage Capacity

=================================

Some machine shops have to download there CNC files

to a personal computer in an office some where.  Many

shops now are supplying there workers with computers

right on there bench near there machine.  As a CNC's

control gets filled with programs it can become full

and cause the operator to have to download one or

more programs to make room enough for the current

program to load.

Many CNC controls offer a function that lets them

download ALL files in the CNC's storage to ONE file on

the personal computer.  This is the fastest method of

clearing the CNC storage.  There is just one big

problem that people face when they have this huge

file on there computer with many programs all in one

file, they have to manually cut and paste to get

even one program back to the CNC.

Mr. Machinist has a one of a kind function in it that

can 'split' a large file that has been downloaded

from a CNC.  The purpose of this function is to

rewrite ALL of the individual programs to separate

files on the personal computer and they are named by

there four digit program number just as they were on

the CNC with an additional extension of .TXT.

July 26, 1994                   Page 34

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

The main purpose of the 'Split' function is to split a

large text file that was downloaded to a personal

computer by a CNC machine into all of the original

separate programs.  You would have to manually cut

and paste using an editor to do this if you did not

have this function to work with and that would take

a lot of time and money!

Mr. Machinist will ask you for the file name of the

large text file and then it will look for the first

occurrence of a line that starts with a ':'.  Once it

finds the colon Mr. Machinist will get the next four

digits and add an extension of .TXT to it and give

the new file the proper program number.  It will then

begin to copy every line after the colon to the new

file UNTIL it reaches a blank line.

Once a blank line is encountered the program will

look for the next occurrence of a ':' which should be

the next line.  At that time the same process occurs

until every file is written to the specified

directory.

The split function will show the MRMSOLVE.TXT answer

file after the process is complete.  If a file was

found to already exist on your drive and the split

function could not create the file then you will

be shown an error message stating that the file

already exist.  These error messages will go to the

answer file also in order that you can view the

results at the end of the function.  If a file exists

you can either rename it or delete it out of your

system and then run the split function again.  The

files that were already created will not be created

again.

After this process is complete, the original large

file will still remain for you to decide what you

want to do with it.

One little note: A good CNC programmer should know a

lot about DOS.  Read manuals carefully and study to

show thyself approved!

4.11.15 File Menu - Exit Mr. Machinist   [Alt]-[X]

If you hold down the 'Alt' key with one finger and

strike the 'X' key with another finger you will exit

the program.  The only reason that you may not be

able to exit is that you may have some active windows

on your desk top that you need to close or your

editing a file that has been modified and not saved.

July 26, 1994                   Page 35

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

4.12 EDIT MENU

This menu allows you to cut, copy, and paste text to and

from the clipboard.  You can also undelete your last text

operation, clear selected text from your document without

putting it into the clipboard, or view current clipboard

contents.

The Edit menu gives you six choices that have to do with

editing a file.

1. Undo  - Undeletes the last line deleted or changed.

2. Cut   - Removes selected text from file to clipboard.

3. Copy  - Copies selected text to clipboard.

4. Paste - Copies text from clipboard to file.

5. Show  - Shows context of clipboard.

6. Clear - Removes selected text without going to

clipboard.

4.12.1 Edit Menu - Undo  [Ctrl]-[Q]-[L]

Undo your last text operation.  Whether you entered

text that you don't really want, or accidentally

deleted some text you would like to restore, UNDO

will restore the text to its original context.

However, you must select UNDO before you use any

cursor key.  The UNDO buffer is based on cursor

movement.  As long as you are entering or deleting

text you can UNDO it.  This also applies to marking

blocks of text and deleting them.  The deletion can

be undone, as long as you don't move the cursor with

a cursor key (i.e., [PgUp], [Right-Arrow], [Home],

etc.).  Once the cursor is moved, the UNDO buffer

flushes itself and the original text context is

forever gone.

This does not apply to using any of the text

reformatting keys.  Wordwrap can not be undone.  Nor

can paragraph or document reformatting.

This command is disabled until you enter or delete

text in an edit window.

4.12.2 Edit Menu - Cut    [Shift]-[Del]

[Ctrl]-[K]-[Y]

Cut selected text out of your document to the

clipboard.  Using CUT indicates that you want to

"cut" your selected text, removing the text from

your document.

This command is disabled until you place text on the

clipboard.

July 26, 1994                   Page 36

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

4.12.3 Edit Menu - Copy  [Ctrl]-[Ins]

[Ctrl]-[K]-[K]

Copy selected text to the clipboard.  Using COPY

indicates that you want to take a "copy" of your

selected text, leaving the text in your document

intact.

This command is disabled until you place text on the

clipboard.

4.12.3 Edit Menu - Paste  [Shift]-[Ins]

[Ctrl]-[K]-[C]

Paste text into your document from the clipboard.

In order to use this option, text must have been

placed on the clipboard previously using COPY or CUT

commands.

This command is disabled until you place text on the

clipboard.

4.12.4 Edit Menu - Show Clipboard

Show Clipboard Contents      Alt-E-S

View the contents of the clipboard.  The clipboard

will be blank unless you previously put something on

it using the COPY or CUT commands.

To exit the clipboard, press [Alt]-[F3] and you will

return to your current edit window.

4.12.5 Edit Menu - Clear

Clear - Removes selected text without going to clipboar

Selected text is that text which you highlight in your

document.  Once text has been selected, you may COPY,

CUT, or CLEAR the text.  Selected text has a

different color than regular text.

Clear selected text from your document.  This is much

like the CUT option, but the selected text is not put

on the clipboard.

Use this command only if you want to delete selected

text for good.  This command is disabled until you

select text.

July 26, 1994                   Page 37

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

4.13 WINDOWS MENU

This menu allows you to navigate amongst the different

windows open on the desktop, resize or move a window, or

arrange your windows in a tile or cascade pattern on the

desktop.

The Windows menu gives you six window manipulation

choices.

1. Resize/move - Use arrow keys or shift-arrow keys to

move or resize.

2. Zoom - F5 will toggle off and on the 'zoom' window functi

3. Next - F6 will toggle between active windows.

4. Close - Alt-F3 will close the current window.

5. Tile - View all edit windows.

6. Cascade - Stacks all edit windows.

4.13.1 Windows Menu - Resize/Move

There are two windows that can be resized, the edit

window and the help window.  All windows can be

moved.  Learning to resize and move windows at a good

pace can really speed up the job of multiple editing

and/or viewing multiple formula answers.

Using the mouse is the quickest way to resize and/or

move a window.  To resize a window using the mouse,

'drag' the mouse on the lower right corner of the

window.  To 'drag' the mouse means to hold down the

left mouse button while you have the mouse cursor

positioned on a certain item.  As you hold down the

left mouse button on the lower right corner, begin to

move the mouse around.  You will now see that the

window resizes from the lower right position.  The

window will stay at its current position as soon as

you take your finger off of the mouse.

To resize a window using the keyboard, you hold down

the 'Alt' key and then strike 'W'. This will pop down

the 'Window menu'.  Strike the 'R' key to resize a

window.  After striking the 'R' key you can use the

arrow keys to resize the window.  When you have the

window where you want it, strike the 'Enter' key.

This will take you out of the resize mode.

To move a window using the mouse you drag the mouse

on the top heading of the window.  The top heading

of a window is the wording that is centered on the

top line of the current window.

July 26, 1994                   Page 38

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

To move a window using the keyboard you should hold

down the 'Alt' key and then strike the 'W' key and

the 'Window menu' will pop down.  Then strike the 'M'

key and you will be able to move the window using the

arrow keys.

4.13.2 Windows Menu - Zoom

The zoom icon is located in the upper right corner of

most windows.

It means that this window can be zoomed in on to fill

the whole desktop or can be oomed out of to resume its

normal size.  Not all windows can be zoomed.  Dialog

boxes are usually not "zoomable."

Zoom Icon [#]

��[�]����� Title �����[ ]ͻ <--Title Bar

�                         �

�                         �

�                         �

�                         �

�                         �

��������������������������� <--Resize corner

4.13.3 Windows Menu - Next  [F6]

Go to the next window on the desktop, if you have

multiple windows on the desktop.

This command is disabled unless there is at least one

window on the desktop.

4.13.4 Windows Menu - Close   [Alt]-[F3] or [Esc]

The close icon appears in the upper left corner of

almost every window.  By clicking on it with your

mouse, you can quickly close a window.  If you are

not using a mouse, press [Alt]-[F3] or [Esc].

Close Icon:[�]

��[�]����� Title �����[ ]ͻ <--Title Bar

�                         �

�                         �

�                         �

�                         �

�                         �

��������������������������� <--Resize corner

July 26, 1994                   Page 39

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

4.13.4 Windows Menu - Tile

Arrange all windows on the desktop in a tile pattern.

An example of four windows tiled on the desktop would

look like this:

������������������������������������Ŀ

� �[�]�������1�[ ]��������������2�Ŀ �

� �               ��               � �

� �               ��               � �

� �               ��               � �

� ���������������ͼ����������������� �

� �������������3�Ŀ�������������4�Ŀ �

� �               ��               � �

� �               ��               � �

� �               ��               � �

� ���������������������������������� �

��������������������������������������

Tiled Windows

This command is disabled unless there is at least one

window on the desktop.

4.13.5 Windows Menu - Cascade

Arrange all windows on the desktop in a cascade

pattern.  An example of four windows cascaded on

the desktop would look like this:

���������������������������1Ŀ

���������������������������2Ŀ

���������������������������3Ŀ

����[�]�����������������4�[ ]�

����                         �

����                         �

����                         �

����                         �

����                         �

����������������������������ͼ

Cascaded Windows

This command is disabled unless there is at least one

window on the desktop.

4.14 SEARCH MENU

The Search menu is used to find certain text in a file and

this function can also search and replace chosen text with

new text.

This menu has three functions to choose from.

1. Find - Finds the text of your choice.

2. Replace - Finds text and then replaces it with new text.

July 26, 1994                   Page 40

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

3. Search again - Do another search using the same choice.

4.14.1 Search Menu - Find  [Ctrl]-[Q]-[F]

Type in the text that you wish to find.  You can also

press the down arrow key to bring up a history list

of the last text you looked for.  Clicking the mouse

on the history list icon to the right of the input

line will do the same thing.

You have two options available when trying to find

text.

You can do a case sensitive search, which means text

must match the case of the text you entered before a

match is found.

You may search for whole words only, which means that

no match will be found if the text you input is part

of a word (i.e, TIME -- "sometime" is NOT a match).

You use the mouse or up/down arrow keys to highlight

the option you want, and click on it or press the

spacebar to toggle it on/off.

Find a particular bit of text in your document.  A

dialog box will appear asking you what text to find,

and gives you the option of finding text that is case

sensitive or whole words only.

[Ctrl-L] will let you do a find action or a search and

replace action again.

This command is disabled until you open an edit

window.

4.14.1 Search Menu - Replace

Type in the replacement text you want.  You can also

press the down arrow key to bring up a history list

of the last replacement text.  Clicking the mouse on

the history list icon to the right of the input line

will do the same thing.

The first two options are for finding text.

You can do a case sensitive search, which means text

must match the case of the text you entered before a

match is found.

You may search for whole words only, which means that

no match will be found if the text you input is part

of a word (i.e, TIME -- "sometime" is NOT a match).

July 26, 1994                   Page 41

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

You use the mouse or up/down arrow keys to highlight

the option you want, and click on it or press the

spacebar to toggle it on/off.

4.14.2 Search Menu - Search Again [Ctrl-L]

This function will let you continue a search or a

search and replace function without starting all over

again.

4.15 CIRCLE MENU

The Circle menu is filled with math functions that have

everything to do with circles.  There are seven main

functions.

1. Circ. Segment -Find the angle, chord, height, or

Radius.

2. Circle Area - Find the surface area of a circle.

3. Circ. Ring - Find the surface area of a circular ring.

4. Circ. Ring Sector Area - Same as 3 but only a section.

5. Circ. Sector Area - Find a circular sector area. Only

two inputs needed.

6. Circumference - Find circle circumference.

7. Three Points - Find X and Y center and radius of a

circle.

8. Pin in a Diamond - Find the pin size/circle diameter

inscribed within a four sided symmetrical shape.

4.15.1 Circle Menu - Circle Segment

This function allows a user to find the Chord,

Height, Radius, Total Segment Angle, and Arc Length

from two knowns.

There are two instances where there can be no

solution, knowing just the Chord and Arc Length or

the Height and Arc Length.

The first step is to input the two known pieces of

information.  Certain types of input are not

feasible.  Take for example a Height input that is

greater than or equal to the Radius.  If the Height

is greater than the Radius, subtract the Radius from

July 26, 1994                   Page 42

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

the Height.  Another example of improper input is a

Chord that is greater than the Diameter of the

Circle.

Remember that your answers not only go to a dialog

box but also to a file that exists on your hard drive

called MRMSOLVE.TXT.  This file can be viewed or

printed at any time.

4.15.2 Circle Menu - Circle Area

This choice simply finds the area of a given circle

diameter.

Input the Diameter of the Circle you would like to

find the Area for.

Area = Pie x Radius Squared

4.15.3 Circle Menu - Circular Ring Area

This function will find the Area of a Circular Ring.

Input the Outside Diameter and then the Inside

Diameter.

Area = Pie x (OD� - ID�)

4.15.4 Circle Menu - Circular Ring Sector Area

This function is able to give you the Area of the

Sector of a Circular Ring.  You need to know three

things: the Angle of the Sector, the OD and the ID.

Area of a Circular Ring Sector Formula

======================================

( OR = Outer Radius, IR = Inner Radius )

((Sector Angle x Pie) / 360)) x (OR� - IR�)

4.15.5 Circle Menu - Circular Sector Area

This function gives you a choice of entering the two

knowns and then calculates the answer.  You may

either enter the Radius and Angle, Radius and Arc

Length, Radius and Area, Angle and Arc Length, Angle

and Area, or Area and Arc Length...any combination is

possible!

July 26, 1994                   Page 43

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

4.15.6 Circle Menu - Circumference

This function will find the Circumference of a

Circle.

The circumference of a circle is the total length of

a circle, as if you were to 'cut' the circular line

and lay it flat and then measure it.

Circumference = Pie x Diameter

4.15.7 Circle Menu - Three Points On A Circle

A geometrical law says that if three point are in a

plane, in this case X and Y, and not in a straight

line or under one another, then it's possible to have

a circle pass through those three points.  You will

find X and Y coordinates and radiuses of large

amounts if you input three points that are almost a

straight line.

Formula

=============================================

(X3�-X1�)+(Y3�-Y1�)   (X2�-X1�)+(Y2�-Y1�)

------------------- - -------------------

2(X3-X1)              2(X2-X1)

Y =  ----------------------------------------

Y3-Y1                Y2-Y1

-----        -       -----

X3-X1                X2-X1

(X3�-X1�) + (Y3-Y1)((Y3+Y1)-2Y)

X = -------------------------------

2(X3-X1)

R = Sqrt( (X3-X)� + (Y3-Y)� )

4.15.8 Circle Menu - Pin in a Diamond

This function will calculate a pin or circle diameter

inscribed within a four sided symmetrical shape.  You

input the length of two sides to calculate the answer.

One side is the 'X' axis corner to corner and the other

side is the 'Y' axis corner to corner.  It does not

matter which side you decide to call 'X' or 'Y'.

One important note to remember about this function is

that the four sided figure must have all square corners.

4.16 MATH MENU

The Math menu has nine functions to do with math.

July 26, 1994                   Page 44

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

1. Trig. - Right angle trig. (Give the two knowns)

2. Conversions: a) Millimeters to Inches

b) Inches to Millimeters

c) Degrees to Degrees, Minutes, Seconds

d) Degrees, Minutes, Seconds to Degrees

e) Celsius to Fahrenheit

f) Fahrenheit to Celsius

3. Polar Coordinates - Gives an X and Y from a radius and an

4. Dist. Across Square Corners - Input height for answer.

5. Dist. Across Hex Corners - Input height for answer.

6. Dist. Between Two Points - Give two X and Y's.

7. Middle of Two Points - Give two X and Y's.

8. Three Points Shift/Rotate - Give three X and Y's and new

X and Y origins and the degree of rotation, (if any).

9. Calculator - Do formulas and calculations.

4.16.1 Math Menu - Trig.

Choosing this function will give you a screen that

will ask you for the two knowns of a right triangle.

The function will then give you all the angles and

side dimensions of that right triangle and also the

perimeter, inner circle diameter, outer circle

diameter, and perpendicular line that has to do with

that particular right triangle.

A triangle with a 90 degree angle in it.  With two

known pieces of information, the solution to all

other sides and angles can be obtained as long as

one of those pieces of information is a side of the

triangle.  The three angles added up must equal 180

degrees.

When you hold down the 'Alt' key and strike the 'T',

the Trig. screen will appear.  The best way to cycle

through the input lines is to strike 'Enter'.  If you

happen to put the maximum amount of digits (12) into

the input line then you will be automatically cycled

into the next input line.  When you have your two

pieces of information entered you may then hold down

the 'Alt' key and strike 'O' for OK or you may strike

'Enter' until you have reached the OK button and then

strike 'Enter'.  You may also use the up and down

arrows to move up and down the dialog box.

The 'Tab' key cycles you through the dialog box in

one direction.  If you hold down the 'Shift' key and

then strike 'Tab' you will cycle through the dialog

box in the other direction.

The Perpendicular Line calculation that is given in

the answer box is the distance of the line that

extends from the intersection of the side opposite

and the side adjacent perpendicular to the

July 26, 1994                   Page 45

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

hypotenuse.  This distance is sometimes needed when a

milling machinist must put a part up on an angle in a

vise in order to machine a chamfer on it.

Theorem of Pythagoras:

a� + b� = c�

Where c = Length of hypotenuse,

a and b = the other sides.

Formula

�

� �

�   �

a �     �  c

�       �

�         �

�        ->A �   (A = The acute angle.)

� � � � � � � �

b

a   Side Opposite

Sin A = - = -------------

c   Hypotenuse

b   Side Adjacent

Cos A = - =   -------------

c   Hypotenuse

a   Side Opposite

Tan A = - = -------------

b   Side Adjacent

b   Side Adjacent

Cot A = - = -------------

a   Side Opposite

c   Hypotenuse

Sec A = - = -------------

b   Side Adjacent

c   Hypotenuse

Csc A = - = -------------

a   Side Opposite

4.16.2 Math Menu - Conversions

1) Millimeters to Inches

2) Inches to Millimeters

3) Degrees in Decimal to D.M.S.

4) Degrees in D.M.S. to Decimal

5) Celsius to Fahrenheit

6) Fahrenheit to Celsius

July 26, 1994                   Page 46

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

Use your up and down arrow keys to choose the

conversion you would like, then strike the 'Tab' key

to move into the input line.  Enter the number that

you would like to convert and then strike 'Enter'.

Degrees, Minutes and Seconds input

**********************************

Input the Degrees to the left of the decimal place

and minutes two places to the right of the decimal

place, and the seconds after the minutes, (two places

also).

INPUT EXAMPLE:   45.5434 = 45 Degrees

54 Minutes

34 Seconds

4.16.3 Math Menu - Polar Coordinates

This function will ask you for a radius and an angle

and then it will give you the X and Y coordinate of

the point where that angle line meets that radius

circle from center X/Y zero.

4.16.4 Math Menu - Distance Across Square Corners

This function will ask you for the height of the

square and then it will give you the distance from

corner to corner.

4.16.5 Math Menu - Distance Across Hex Corners

This function will ask you for the height of the hex

from top to bottom, (From flat to flat) and then give

you the distance across hex corners.

4.16.6 Math Menu - Distance Between Two Points

This function will ask you for two X and Y

coordinates and then give you the distance between

these two points.

4.16.7 Math Menu - Middle Of Two Points

This function will ask you for the two X and Y

coordinate locations.  You will then be shown the X

and Y coordinate of the point that lies exactly in

the middle.

4.16.8 Math Menu - Three Points, Shifted And Rotated

July 26, 1994                   Page 47

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

This function will ask you to input three X and Y

coordinates, a shift location, and a rotation angle.

The X and Y Datum position is what is being shifted

here or in other words the reference or datum point.

The rotation begins at the 3 O'clock position and

swings around in a counter clock wise direction.

4.16.9 Math Menu - Bolt Circle

This function will ask you for the diameter of the

Bolt Circle and then the number of holes and then the

angle of the first hole.  Just as in most CAM

software, the angle starts at the right side of X

zero on the horizontal plane and swings its way

counter clock wise until it goes the full 360

degrees.  You may insert a negative angle for the

first hole if you like.

When the Bolt Circle function is executed, it writes

its answers to a file on your drive called

MRMBOLT.TXT.  After Mr. Machinist writes the answers

to the file it will display it.  You can print the

MRMBOLT.TXT file at any time by striking, 'Ctrl-F9'.

You can view the MRMBOLT.TXT file at any time by

striking, 'Ctrl-F8'.

Keep in mind that every time you strike 'Ctrl-F8' and

the answer file 'pops' up, it shows you the current

state of MRMBOLT.TXT.  When your done viewing the

file you should close the window by clicking on the

close icon or striking 'Alt-F3' or striking 'Enter'.

4.16.10 Math Menu - Calculator

You can use the keyboard or the mouse to operate the

calculator.  'F4' brings up the calculator at any

time and 'Alt-F3' or 'Esc' will close the calculator

window.

Left arrow erases the last character typed.

The � key toggles the number from + to -.

( � on keyboard is the 'Underscore' key )

The C key clears the input line.

Once the calculator window is on screen you will be

ready to enter your calculation or formula.

You can strike the 'Tab' key to move around the

calculator dialog box.  Striking the 'Tab' key will

July 26, 1994                   Page 48

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

take you to the 'Ok' button, once more will take you

to the 'Cancel' button and once more will take you

back to the input line.  If you hold down the 'Shift'

key and then strike 'Tab' you will move around in the

opposite direction.  You can also use the mouse to

click on one of these three items and activate them.

Once your in the input line you can start to enter

your data.  When numerical entry is complete you may

strike 'Enter' or click on 'Ok' with your mouse or

strike 'O' for 'Ok'.

Example: 4+4*87/456*tan(56) then 'Enter'.

The order of precedence is at work here.  The

calculator evaluates the parts of an expression

inside the most deeply nested parentheses before

anything else.  In the example, the tangent of 56 is

calculated and then 87/456 is calculated, then the

two are multiplied and 4 is added.

Upper and lower case input of functions are treated

the same.  You may also append an O for octal or an H

for hex after a number.  If you input 55o, your

answer will be 45 because 55 in octal is 45.  If you

input 640H, your answer will be 1600 because 640 in

hex is 1600.  When you type a hex constant you must

be sure to type a number before it.  If you type FF

for a hex number then you will get an error, but if

you type 0ff then you will get 255.

You may enter assignment statements such as, a:=3,

and b:=3, and also c:=sqrt(a*b).  Always use a single

character as your variable, A to Z.  If you use

assignment statements within expressions always be

sure they are predeclared.  Here is an example using

the formula for RPM (R) of a spindle:

12 x F (Feet per Minute)

R = --------------------------

Pi x D (Diameter of Endmill)

You would enter f:=200 and then strike 'Enter',

d:=0.5 and 'Enter', and for the end result,

r:=(12*f)/(pi*d).  If you were to leave out the

parentheses your result would be in error due to the

fact that the calculation would divide first and

then do its multiplication.

Standard Keys:

Tab       = Activate next item

Shift-Tab = Activate in opposite direction.

F4        = Bring up calculator window

Alt-F3    = Close window. (Any time)

Esc       = Close window. (Any time)

July 26, 1994                   Page 49

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

Enter     = Calculate answer. (Valid input)

Calculator Functions

====================

Be sure to inclose functions with more than one

character within parentheses.

Example: 5*tan(45)*sqrt(7)

+         = Addition

-         = Subtraction

/         = Division

*         = Multiplicational

^         = Exponentiation

O         = Octal (Precede Number)

H         = Hex (Precede Number)

Pi        = 3.1416.........

Frac      = Fractional Value

Factor    = Factorial (1 x 2 x 3 x 4....)

Exp       = Natural Exponent

Int       = Integer Value

Abs       = Absolute Value

Log       = Log Base 10

Sqrt      = Square Root

Ln        = Natural Log

ArcCos    = ArcCosine

ArcCsc    = ArcCosecant

ArcSin    = ArcSine

Cos       = Cosine

Csc       = CoSecant

Sin       = Sine

ArcCot    = ArcCotangent

ArcSec    = ArcSecant

ArcTan    = ArcTangent

Cot       = CoTangent

Sec       = Secant

Tan       = Tangent

4.17 AREAS AND VOLUMES MENU

4.17.1  Areas/Volume Menu - Acute Angle

An acute angle is an angle that is less than 90

degrees.  An acute angled triangle is one with all

the angles less than 90 degrees.

a and b and c are the sides of the acute triangle.

Formula

=======

S = 1/2 x (a + b + c)

Area = Sqrt(S x (S - a)) x (S - b) x (S - c)

July 26, 1994                   Page 50

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

4.17.2  Areas/Volume Menu - Barrel

A cylindrical object where the middle diameter is

greater than the end diameter.  The sides of a barrel

shape can either be a circular arc or a parabola arc.

In the parabola arc the radius from the center of the

arc shape is constantly changing until it's peak at

the top of center.

The formula used here in Mr.  Machinist is for the

circular arc.

Formula

=======

V = Approximate Volume

D = Middle Diameter

d = End Diameter

h = Height of Barrel Shape

Arc of a Circle (Barrel)

V = 0.262 x h x (2 x Sqr(D) + Sqr(d))

Arc of a Parabola (Barrel)

V = 0.209 x h x (2xSqr(D+Dxd+3/4xSqr(d))

4.17.3  Areas/Volume Menu - Cone

This function will ask you for a radius and a height

of a cone and then it will give you the area and the

volume of that cone in inches or millimeters.

4.17.4  Areas/Volume Menu - Cube

This function will ask you for a side dimension of a

cube and then will give you the volume of the cube in

inches or millimeters.

4.17.5  Areas/Volume Menu - Cycloid

This function will ask you for the radius of the

generating circle of a cyclode and then give you the

area.

Cycloid

=======

Location of a point on the circumference of a circle

that rolls along a straight line.

4.17.6  Areas/Volume Menu - Cylinder

July 26, 1994                   Page 51

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

This function will ask you for the radius and height

of a cylinder and then give you the area, volume, and

the total area of cylindrical surface and end

surfaces.

4.17.7  Areas/Volume Menu - Ellipse

This function will ask you for the major axis

dimension and the minor axis dimension and then give

you the area and circumference of the ellipse.

4.17.8  Areas/Volume Menu - Ellipsoid

This function will ask you for the major axis

dimension and the minor axis dimension and then give

you the volume of the ellipsoid.

4.17.9  Areas/Volume Menu - Frustum of a Cone

This function will ask you for the base diameter and

the top diameter and the height of the cone and then

give you the volume.

4.17.10 Areas/Volume Menu - Frustum of a Pyramid

This function will ask you for the top area and the

bottom area and the height of the pyramid and then

give you the volume.

4.17.11 Areas/Volume Menu - Hexagon

This function will ask you for the outside radius,

the inside radius, and the length of a side but you

do not have to answer all of the input lines!  You

only need to answer one of the questions and all

other answers will be given!

By inputting just the length of a side you get the

area, outer radius, inner radius and distance across

flats.

By inputting just the outer radius you get the area,

inner radius and distance across flats.

By inputting just the inner radius you get the area,

outer radius and distance across flats.

4.17.12 Areas/Volume Menu - Hollow Cylinder

This function will ask you for the outside diameter,

the inside diameter and the height of the cylinder

July 26, 1994                   Page 52

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

and will give you the volume.

4.17.13 Areas/Volume Menu - Hollow Sphere

This function will ask you for the outside diameter

and the inside diameter of the hollow sphere and then

will give you the volume.

4.17.14 Areas/Volume Menu - Obtuse Angle

This function will ask you for the length of 3 sides

and then will give you the area of an obtuse angled

triangle.

(An angle that is between 90 degrees and 180 degrees.)

4.17.15 Areas/Volume Menu - Octagon

This function will ask you for the length of a side,

the outside radius, and the inside radius.

You only need to input one piece of information in

order to get your answers.  The answers that are

given are the side length, the outside radius, the

inside radius, and the area of the octagon.

Octagon = A polygon with eight angles and eight

sides.

4.17.16 Areas/Volume Menu - Parallelogram

This function will ask you to input the length of

side A and the length of side B.  You will then be

displayed an answer box giving you the 'area' of the

parallelogram.

4.17.17 Areas/Volume Menu - Polygon

This function will ask you to input four pieces of

data.  You need not input all four.

You may input the number of sides, either ID or OD,

and the length of a side.  You will then be displayed

an answer box giving you the 'area' of the parallelogra

4.17.18 Areas/Volume Menu - Portion of a Cylinder

A cylinder that is cut at the top or bottom on an

angle.  If the cylinder was level at both ends then

it would be a cylinder, not a portion of a cylinder.

July 26, 1994                   Page 53

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

A portion of a cylinder has 2 heights, the upper most

portion of the angle cut and the lower most portion.

Insert these heights as Height1 and Height2.

4.17.19 Areas/Volume Menu - Prism

This function will ask you to input two pieces of

data, the area of the prism and the height.  You

would first have to find the area of the

'end surface' of the prism which could be of

different shapes.  Once you know the area of the end

surface you can then input the height which is

measured perpendicular to the end surface.  You will

then be displayed an answer box giving you the

'Volume' or the prism.

4.17.20 Areas/Volume Menu - Pyramid

This function will ask you to input two pieces of

data, the area of the pyramid's base and the height

of it.  The base of the pyramid could be different

shapes so you would have to first find the area of

the base.  After you have input the proper data you

will be displayed an answer box giving you the

'Volume' of the pyramid.

4.17.21 Areas/Volume Menu - Rectangle

This function will ask you for two pieces of data,

the length of one side and the length of the other.

You will then be displayed an answer box giving you

the 'Area' of the rectangle.

4.17.22 Areas/Volume Menu - Right Triangle

This function will ask you for two pieces of data,

the length of the side adjacent and the length of the

side opposite.  You will then be displayed an answer

box giving you the 'Area' of the right triangle.

4.17.23 Areas/Volume Menu - Spandrel/Fillet

This function will have two input lines requiring you

to input data into one of them.  Either input the

radius of the spandrel or fillet or input the chord.

You will then displayed an answer box giving you the

'Area' of the spandrel or fillet.

4.17.24 Areas/Volume Menu - Square

July 26, 1994                   Page 54

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

This function will ask you for the length of a side

only.  With that information you will be given the

diagonal length and the 'Area' of the square.  The

diagonal length is the distance from one corner to

the opposite corner.

4.17.25 Areas/Volume Menu - Square Prism

This function will ask you for the length, width, and

height of the square prism or 'square box'.  You will

then be shown an answer box giving you the volume of

the 'box'.

4.17.26 Areas/Volume Menu - Sphere

This function will ask you to input the radius of the

sphere.  You will then be shown an answer box giving

you the 'Area' and the 'Volume' of the sphere.

4.17.27 Areas/Volume Menu - Spherical Sector

Chord, Volume, and Area of a Spherical Sector

=============================================

In the input box you must enter the Radius and Height

while in the spherical sector menu.  While in the

spherical 'segment' menu you must enter the height

and one other input, either the radius or the chord.

The height can not be greater than 2 x Radius!  You

will then be displayed an answer box giving you the

chord, volume, and area of the spherical sector.

A sphere is a closed surface where all points are the

same from a given distance.  Another simple name for

a sphere is a ball.  A spherical sector is a section

of a 'ball' or sphere that is cut out from the whole

part.  It consist of a full revolution and not just a

segment.  A spherical segment is a cut section from

the 'ball' or sphere but it is cut at two parallel

lines.

Formula   (Spherical Sector)

============================

V = Volume, A = Area, C = Chord:

V = (2 x pie x Sqr(radius) x height) / 3

A = pie x radius x (2 x height + .5 x chord)

C = 2 x Sqrt(height(2 x radius - height))

4.17.28 Areas/Volume Menu - Spherical Segment

July 26, 1994                   Page 55

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

See 4.17.27 (Spherical Sector)

Formula   (Spherical Segment)

=============================

V= pie x Sqr(height) x (radius - height / 3)

A= 2 x pie x radius x height

C= 2 x Sqrt(height(2 x radius - height))

R= (Sqr(chord + 4 x Sqr(height))/(8 x height)

4.17.29 Areas/Volume Menu - Spherical Wedge

This function will ask you for the angle of the wedge

and the radius. You will then be shown an answer box

giving you the 'Area' and the 'Volume' of the

Spherical Wedge.

4.17.30 Areas/Volume Menu - Spherical Zone

This function will ask you for various input.  You

must input the height of the spherical zone and in

addition you must either input the radius or both

chord 1 and chord 2.  If you input the radius and the

height you will be given the 'Area' of the zone.  If

you input the height and chord 1 and chord 2 you will

be given the radius and volume.

4.17.31 Areas/Volume Menu - Torus

A shape that looks like an anchor ring or a doughnut.

If you were to take a ball and orbit it around a

center distance, the shape that it would leave is a

Torus.  There are two radius involved, the radius of

the ball that is orbited and the orbit radius.

Formula:

========

Radius1 = The orbiting ball.

Radius2 = The orbit radius.

Area   = 4 x sqr(pi) x Radius2 x Radius1

Volume = 2 x sqr(pi) x Radius2 x Radius1

4.17.32 Areas/Volume Menu - Trapezium

A four sided figure in which no two sides are

parallel.

Help on Trapezium input

=======================

Let's assume that the longest side of the four sided

figure is down.

July 26, 1994                   Page 56

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

�

�   �

�       � �

�           �       ( A Trapezium )

�              H1   �

��               �

� H2              �     �

�  �               �

� � � � � � � � � � � � � � �

�L2������� L1 ��������� L3 Ĵ

H1 = Tall height of the figure.  (Right)

H2 = Short height of the figure.  (Left)

L1 = Distance between heights.

L2 = Dist. from left edge to left height.

L3 = Dist. from rt. edge to rt. height.

(H1 + H2) x L1 + (L2 x H2) + (L3 x H1)

Area = --------------------------------------

2

4.17.33 Areas/Volume Menu - Trapezoid

A four sided figure in which two sides are parallel.

Let's assume that the longest side of the four sided

figure is down.

Side 1 = One parallel line.  Side 2 = The other

parallel line.  Height = Height of the figure.

(Side1 + Side2) x Height

Area = ------------------------

2

4.17.34 Areas/Volume Menu - Wedge

A three dimensional shape with a base that has length

and width.  The length of the base travels up to a

sharp point which is the height of the wedge.  The

top and the bottom of the wedge are parallel lines.

Side 1 = The base length.

Side 2 = The base width.

Side 3 = Top of the wedge, parallel to Side1.

Height = Height of base to top of wedge.

Formula

=======

(2 x Side1 + Side3) x Height

Volume = ----------------------------

6

4.18 RATES MENU

This menu choice provides you with speed and feed

information.  You input the required date and you end up

with a general speed and feed to begin to work with.

July 26, 1994                   Page 57

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

4.18.1 Rates Menu - Speed and Feed For Drilling

This choice will cause a menu to pop up that asks you

what material and drill diameter you are using.

Strike the F1 help key for a list of the type of

materials that you can input into this function.

Insert the type of material that you are drilling and

the size of the drill in decimal form.  It will then

display a dialog box that will give you the Rpm and

the minimum and maximum feed rate in inches per

minute for each material condition.

This information will also go to the MRMDRILL.TXT

file on your drive so you may print it out at any

time and/or view it.  To view the drill file with

complete information on all Brinell hardness numbers

click on the 'Details' button or hold down the 'Ctrl'

key while striking the 'F4' key.

To print the drill file with complete drill

information hold down the 'Ctrl' key while striking

the 'F6' key.

Remember that the drill file called MRMDRILL.TXT gets

created each time you start a new session with MRM.

This MRMDRILL.TXT file will automatically be deleted

upon program completion.  If you decide that you want

to keep it for later reference you will have to

rename it by using the editor and the 'Save as'

command.

Material List

=============

This is a list of materials that can be input into

the Mr. Machinist Drill Speed and Feed input line.

See the Machinery's Handbook for comparison.

Plain Carbon and Alloy Steels

=============================

Free Machining Plain Carbon Steels

(Resulphurized)

1212, 1213, 1214, 1108, 1109, 1115, 1117, 1118, 1120,

1126, 1211, 1132, 1137, 1139, 1140, 1144, 1146, 1151.

Free Machining Plain Carbon Steels

(Leaded)

11L17, 11L18, 12L13, 12L14

Plain Carbon Steels

1006, 1008, 1009, 1010, 1012, 1015, 1016, 1017, 1018,

1019, 1020, 1021, 1022, 1023, 1024, 1025, 1026, 1513,

1514, 1027, 1030, 1033, 1035, 1036, 1037, 1038, 1039,

July 26, 1994                   Page 58

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

1040, 1041, 1042, 1043, 1045, 1046, 1049, 1050, 1052,

1524, 1526, 1527, 1541, 1055, 1060, 1064, 1065, 1070,

1074, 1078, 1080, 1084, 1086, 1090, 1095, 1548, 1551,

1552, 1561, 1566

Free Machining Alloy Steels

(Resulphurized)

R4140, R4150

Free Machining Alloy Steels

(Leaded)

41L30, 41L40, 41L47, 41L50, 43L47, 51L32, 52L100,

86L20, 86L40

Alloy Steels

4012, 4023, 4024, 4028, 4118, 4120, 4419, 4422, 4427,

4615, 4620, 4621, 4626, 4718, 4720, 4815, 4817, 4820,

5015, 5017, 5020, 6118, 8115, 8615, 8617, 8620, 8622,

8625, 8627, 8620, 8822, 94B17, 1330, 1335, 1340,

1345, 4032, 4037, 4042, 4047, 4130, 4135, 4137, 4140,

4142, 4145, 4147, 4150, 4160, 7, 4340, 50B44,

50B46, 50B50, 50B60, 5130, 5132, 5140, 5145, 5147,

5150, 5160, 51B60, 6150, 81B45, 8630, 8635, 8637,

8640, 8642, 8645, 8650, 8655, 8660, 8740, 9254, 9255,

9260, 9262, 94B30

Alloy Steels

E51100, E52100

Ultra High Strength Steels (Not AISI)

AMS6424, AMS6421, 98B37, AMS6422, 98BV40, AMS6427,

AMS6428, AMS6430, AMS6432, AMS6, AMS6434, AMS6436,

AMS6442, 300M, D6AC

Maraging Steels (Not AISI)

(NG means Nickel Grade)

NG200, NG250, NG300, NG350

Nitriding Steels (N means Nitralloy)

N125, N135, N135M, N225, N230, NN, NEZ, NIT1

============================================

Tool Steels

===========

Water Hardening

W1, W2, W5

Shock Resisting

S1, S2, S5, S6, S7

Cold Work, Oil Hardening

O1, O2, O6, O7

Cold Work, High Carbon High Chromium

D2, D3, D4, D5, D7

July 26, 1994                   Page 59

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

Cold Work, Air Hardening

A2, A3, A4, A6, A7, A8, A9, A10

Hot Work, Chromium Type

H10, H11, H12, H13, H14, H19

Hot Work Tungsten Type

H21, H22, H23, H24, H25, H26

Hot Work Molybdenum Type

H41, H42, H43

Special Purpose Low Alloy

L2, L3, L6

Mold

P2, P3, P4, P5, P6, P20, P21

High-Speed Steel

M1, M2, M4, M6, M7, M10, M30, M33, M34, M36, M41,

M42, M43, M44, M46, M47, M3-1, M3-2, T1, T2, T5, T6,

T8, T15

Stainless Steels

================

Free Machining Stainless Steels

(Ferritic)

430F, 430FSE

(Austenitic)

203EZ, 303, 303SE, 303MA, 303PB, 303C11,

303PX (303 Plus X)

(Martensitic)

416, 416SE, 416PX (416 Plus X), 420FSE, 440F, 440FSE

Stainless Steels

(Ferritic)

405, 409, 429, 430, 434, 436, 442, 446, 502

(Austenitic)

201, 202, 301, 302, 304, 304L, 305, 308, 321, 347,

348, 302B, 309, 309S, 310, 310S, 314, 316, 316L, 317,

330

(Martensitic)

403, 410, 420, 501, 414, 431, GA (Greek Ascoloy),

440A, 440B, 440C

(Precipitation Hardening)

15-5PH, 17-4PH, 17-7PH, 17-14CM (17-14 Cu Mo), AF-71,

AFC-77, AM-350, AM-355, AM-362, C455 (Custom 455),

July 26, 1994                   Page 60

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

HNM, PH13-8, PH14-8M, PH15-7M, SW (Stainless W)

Ferrous Cast Metals

===================

Gray Cast Iron

20, 25, 30, 35, 40, 45, 50, 55, 60, 1, 1B, 2, 2B, 3,

4NR (4 Ni-Resist), 5NR, 6NR

Malleable Iron

(Ferritic)

32510, 35018

(Pearlitic)

40010, 43010, 45006, 45008, 48005, 50005

(Martensitic)

53004, 60003, 60004, 70002, 70003, 80002, 90001

Nodular (Ductile) Iron

(Ferritic)

604018 (60-40-18), 654512 (65-45-12)

(Ferritic-Pearlitic)

805506 (80-55-06),

(Pearlitic-Martensitic)

1007003 (100-70-03)

(Martensitic)

1209002 (120-90-02)

Cast Steels

(Low Carbon)

C1010, C1020

(Medium Carbon)

C1030, C1040, C1050

(Low Carbon Alloy)

C1320, C2315, C2320, C4110, C4120, C4320, C8020, C8620

(Medium Carbon Alloy)

C1330, C1340, C2325, C2330, C4125, C4130, C4140,

C0, C4340, C8030, C80B30, C8040, C8430, C8440,

C8630, C8640, C9525, C9530, C9535

Light Metals

============

(All Wrought Aluminum Alloys)

ALUM (Type 'ALUM' in the input line)

(All Alum. Sand and Permanent Mold Casting Alloys)

ALUMC (Type 'ALUMC' in the input line)

July 26, 1994                   Page 61

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

(All Alum. Die Casting Alloys)

ALUMD, A390, A392

(All Wrought Magnesium Alloys)

MAGW

(All Cast Magnesium Alloys)

MAGC

Copper Alloys

=============

CA314, CA332, CA340, CA342, CA353, CA356, CA360,

CA370, CA377, CA385, CA485, CA544, CA226, CA230,

CA240, CA260, CA268, CA280, CA335, CA365, CA368,

CA443, CA445, CA651, CA655, CA675, CA687, CA770,

CA796, CA102, CA110, CA122, CA170, CA172, CA175,

CA210, CA220, CA502, CA510, CA521, CA524, CA614,

CA706, CA715, CA745, CA752, CA754, CA757

4.18.2 Rates Menu - Speed and Feed For Milling

This function will ask you for the surface feet per

minute, the work piece or cutter diameter, the chip

load, and the number of teeth on the cutter.  A

general chart is positioned on the left of the screen

for handy reference of surface feet and chip loads.

Determine the Feet Per Minute from the chart on

the left and input the data into the cutting

speed input line.  Insert the chip load data by

referring to the bottom of the chart on the

left.  Keep in mind that these are very general

speeds and feeds.

Start slow and work your way up when using feeds/speeds

The three main factors that determine the rate of

metal removal are cutting speed, feedrate and

depth of cut.  First, take into consideration, the

depth of cut, then feed rate and then the cutting

speed.

Formula

=======

IPT = Inches Per Tooth

T   = Number of Teeth

FPR = Feed Per Revolution

RPM x IPT x T

Feed Rate = -------------

FPR

July 26, 1994                   Page 62

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

RPM x IPT x T

FPR = -------------

Feed rate

Feet Per Minute Chart  (Page Down for More)

===========================================

Material            HSS     Carb.

********            ***     *****

Stainless Steel      50       150

Cast Steel           50       150

Cast Iron            75       225

High-Carbon Steel    50       150

Medium-Carbon Steel  75       200

Low_Carbon Steel     125      375

Brass                300      800

Magnesium            600     1000

Aluminum             600     1000

Copper               300     1000

Inches Per Tooth

Cutter                        IPT

******                        ***

Metal Cutting Saws           .001 +

Form Cutters                 .002

End Mills ( less than 1/2" ) .001 +

End Mills ( more than 1/2" ) .003

Plain Cutters                .005

Side Cutters                 .007

Face Mills                   .010

====================================

Use these calculations as a starting

point only.

====================================

4.18.3 Rates Menu - Speed and Feed For Tapping

This menu function in Mr. Machinist will give you

the speed and feed of a tap used in a CNC machine

that utilizes ridgid tapping.  High speed tapping

is used widely in the machining industry and it is

used with high accuracy.  A hole can be bottom

tapped within 0.0010 using ridgid tapping.

Input the tap size and you will be given the speeds

and feeds for the three main categories of materials.

Acceptable tap sizes

====================

4-40, 5-40, 6-32, 8-32, 10-24, 10-32, 12-24

1/4-20, 1/4-28, 5/16-18, 5/16-24, 3/8-16

3/8-24, 7/16-14, 7/16-20, 1/2-13, 1/2-20

These are some general speeds and feeds for tapping.

July 26, 1994                   Page 63

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

This information is general and you can differ from

these given speeds and feeds.  If you are tapping

with a machine that can ridgid tap you can increase

the speeds and feeds.  Rigid tapping is an excellent

method of tapping that can be very accurate.  You may

also want to multiply your feed by .9 for spring

loaded taps to achieve some 'give' in the

synchronization of the spindle speed and the machine

feed.

S = Speed     F = Feed     T = Threads Per Inch.

================================================

T = S / F    S = TPI x F    = S / TPI

================================================

3.82

R.P.M. = S.F.P.M. x -----

O.D.

================================================

Material   Feet Per Minute  Lubrication

========   ===============  ===========

Alum.         90 - 100      Kerosene & Lard Oil

Mach.         40 - 60       Compound, Sulfur-base

or Kerosene & Paraffin

Tool          25 - 35       Sulfur-base or Kerosene

and Lard.

4.19 OPTIONS MENU

This is the user options menu.  Using the functions of

this menu, the user can change the way things look and

react.

4.19.1 Options Menu - Mouse

Using this function can adjust the speed of a double

click on your mouse.  It can also change the active

button from left to right.  (For left handed people)

4.19.2 Options Menu - Save Desktop

The 'DeskTop' is the way your screen looks at any

given time.  You may have two or three files up on

your screen that you are editing and you may have a

few answer boxes and calculations up also.  You can

'save' the things that your screen is showing you to

a file called, 'MRM.DSK'.  The DSK extension stands

for DISK.  Then at a later time you may 'retrieve' or

'restore' your desktop by choosing the

'Retrieve DeskTop' menu choice.

By choosing 'Save DeskTop' you will create a file

July 26, 1994                   Page 64

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

called, 'MRM.DSK' in your current directory.  This

file will hold all of your current views so that you

can 'Retrieve' them at a later time.

Be aware that when you 'Change Directory' in the file

menu you will then 'save' the desktop in the

directory that you changed to.  So if you start the

program at a later time and expect to find your last

desktop but you can not, then you may have saved your

desktop in another directory.

4.19.3 Options Menu - Retrieve Desktop

The 'DeskTop' is the way your screen looks at any

given time.  You may have two or three files up on

your screen that you are editing and you may have a

few answer boxes and calculations up also.  You can

'save' the things that your screen is showing you to

a file called, 'MRM.DSK'.  The DSK extension stands

for DISK.  Then at a later time you may 'retrieve' or

'restore' your desktop by choosing the

'Retrieve DeskTop' menu choice.  This option will

search your current directory for the file called

'MRM.DSK'.

Be aware that when you 'Change Directory' in the file

menu you will then 'save' the desktop in the

directory that you changed to.

So if you start the program at a later time and expect

to find your last desktop but you can not, then you may

have saved yourdesktop in another directory.

4.19.4 Options Menu - Use .BAK File

Using this option you can choose to create or not to

create backup files. Backup files are created with

the file extension .BAK by default.

When you edit a file that exists on your drive, that

file is actually in two places: On your disk or

drive, and in the RAM of your computer.  When you

first create a file, it only exists in RAM until you

save it to disk.  When you save a file to disk or

drive for the first time it exists with the name that

you chose to give it.  Mr. Machinist will create a

backup file by default when you save a file that

already exists on your disk or drive.  You can choose

to shut this feature off therefore saving on disk

space but loosing the chance to have a file that has

not been over written.

5.0 EDITOR HELP

July 26, 1994                   Page 65

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

Movement                     Command

========                     =======

Character Left               Arrow

Character Right              Arrow

Word Left                    Ctrl-Left Arrow

Word Right                   Ctrl-Right Arrow

Line Up                      Arrow

Line Down                    Arrow

Page Up                      PgUp

Page Down                    PgDn

Beginning of a Line          Home Key

End of a Line                End Key

Beginning of a File          Ctrl-PgUp

End of a File                Ctrl-PgDn

Delete Line                  Ctrl-Y

Delete Word Right            Ctrl-T

Start Marking a Block        Shift-(arrow keys)

Copy Block to Clipboard      Ctrl-Insert

Paste Block from Clipboard   Shift-Insert

Delete Block or Clipboard    Ctrl-Del

Hide Block Marker            Ctrl-K-H

Cut into Clipboard           Shift-Del

Show Clipboard Contents      Alt-E-S

Go to Menu Bar               F10

Return to Editor from Menus  Esc

Save current Workspace       F2

Save File as Another Name    Alt-F-A

Search                       Ctrl-Q-F or Alt-S-F

Search Again                 Alt-S-S

Search/Replace               Ctrl-Q-A or Alt-S-R

Replace Last Modification    Alt-E-U

Move Edit Window             Ctrl-F5

(Arrow Keys and Shift-Arrow)

Zoom Resized Edit Window     F5 or Alt-W-Z

Next Window                  F6 or Alt-W-N

Tile (One on top of another) Alt-W-T

Cascade (Headings vertical)  Alt-W-C

Close Current Window         Alt-F3

Start a New File             Alt-F-N

Open an Existing File        F3 or Alt-F-O

Quit Mr. Machinist           Alt-X

You may open and manipulate many edit windows at the

same time, cut and paste text to and from any window.

If you like, you can open the same file more than

once for reference purposes.  If you want to

reference some code from another CNC program you can

bring the file up into an edit window, resize it and

cut and paste from it to your current CNC program.

In doing this you could create a type of 'database'

for your machine files.

Let's say you have a machine file, (CNC program)

July 26, 1994                   Page 66

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

called K0154.TXT.  You could create a file called

K0154N.TXT for the 'notes' to this file.  In

K0154N.TXT you could put the job number, cycle time,

tools used, set up notes and what ever you like! Then

when it comes time to look at that CNC program you

could call up these two files, resize them to a

smaller size and look at both of them!

When it comes time to save your changes, strike F2

and it's done.  You'll know that you have made a

change to the edit window when you see a little

star '*' just to the right of the lower left edit

window.  If you strike Alt-X to quit when the star is

there (you have not saved your changes) you will be

asked if you want to save your changes.  At this time

you can strike 'Y' for yes or 'N' for no or 'C' for

cancel.  Whenever you save a file, Mr. Machinist will

keep your old one and put a BAK extension to it.

Let's say your editing a file called README.TXT and

you make a few changes to it and then you save it.

You will now have 2 files on your system by that file

name but with different extensions, README.BAK and

README.TXT.

Do remember this though, if you make a save and a

README.BAK exist, the old README.BAK file will be

replaced by the newer README.BAK file.

Note: README.BAK is used here just as an example!

Using a Mouse

=============

It's faster for a user to use the keyboard instead of

the mouse, but when it comes to resizing and moving

windows, the mouse is the preferred method.  The left

mouse button is the one you'll always use.  Just

point and shoot! Pick a menu, click the left button

and down comes a menu.  Chose from the menu and click

for results.  To move around edit windows, 'drag' the

mouse on the edit window heading.  Dragging the mouse

means to hold down the left mouse button while moving

the mouse.  When you want to resize the current

window, drag the mouse on the lower right corner of

the edit window.  To highlight a block of data while

editing drag the mouse over the text you desire to

mark and that text will become highlighted.

The editor incorporated into Mr. Machinist can

manipulate a file of 65 K or under.  That is the

limit at this time.  If you want to edit a very large

file you will have to use a more powerful editor or

'split' your file and then 'paste' it back together.

The editor in Mr. Machinist has the capabilities to

July 26, 1994                   Page 67

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

edit almost all CNC programs.  If the CNC program is

larger than this limit then maybe you can create a

smaller program by using sub-programs.  A small CNC

program that does a lot is the sign of a good program!!!

This editor is a standard editor that uses commands

that are universal in most programs.  A small book

can be created telling you how to be proficient in

using an editor but here are some basics.

Features

========

Full mouse support

Move, resize, overlap, tile, or cascade

Open and view several files at once

Search and replace text

Cut, copy and paste using the clipboard

Paste from one file into another

Use the editor while you use the math functions

View the clipboard contents

Restore deleted text

Do block operations, large deletes or inserts

Jump to Line    [Ctrl]-[J]-[L]

==============================

Quickly jump to any line in your document.  A dialog box

will appear that requests the line number you wish to jump

to.  Valid entries are 1 through 9999.

Your cursor will remain on the current line if you enter

an invalid line number.

This command is disabled until you open an edit window.

Cancel   |   No

===============

Clicking on these buttons, or pressing [Esc], says "No,

go back to what I was doing -- don't change or do

anything!".

Drag Mode

=========

The current window is being moved or resized.  Press

[Enter] to keep the current position, or [Esc] to restore

the former position and/or size.

Line Number

===========

Enter a line number from 1 to 9999.  You may press the

down arrow to select from a history list of previous

entries, or use the mouse to click on the history icon to

the right of the input line.

Reformat Options

================

July 26, 1994                   Page 68

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

Select one of the buttons to tell the editor where you

want to start searching.  You may click on the appropriate

button with your mouse, or use the up/down arrow keys to

highlight the proper button.

Replace Text

============

Type in the replacement text you want.  You can also press

the down arrow key to bring up a history list of the last

replacement text.  Clicking the mouse on the history list

icon to the right of the input line will do the same thing.

Right Margin

============

Enter a right margin value from 10 to 255.  Press the down

arrow or click on the history icon to the right of the

input line to bring up a history list of previous entries.

Tab Stops

=========

Use your right/left cursor keys to move the cursor to the

tab stop you want to delete/set.  Press the spacebar to

delete tabs, or any other character key (like [x]) to set

a tab stop.

IMPORTANT:  If you press spacebar immediately after you

bring this dialog up, all tab stops will be erased!

You can push the down arrow or click on the history icon

between the OK and CANCEL buttons to bring up a history

list of previous tab stop settings.

History Icon

============

The history icon is a little box with the down arrow in

it.  It is usually situated near an input line within a

dialog box.

The history icon is selected by pressing the down arrow,

or clicking on it with the mouse.  It brings up a new

window that contains a list of all previous commands that

may have been input.  The list is usually 10 items deep.

You can select from this list by using the mouse to scroll

through it, or the up/down arrow keys.  Pressing [Enter]

accepts the input and places it on the input line.  This

feature allows you to quickly pick previous entries.

July 26, 1994                   Page 69

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

Input Line

==========

An input line is always found within a dialog box.  It

allows you to enter data to allow some further action

to take place.  Input lines will usually have a history

icon associated with them, thereby allowing you to

quickly select from amongst a list of previous entries.

Resize Corner

=============

The "resize corner" of a window is the bottom right corner

that appears a bit different from the rest of the window

border.  By clicking on it with a mouse, you can resize a

window to the dimensions you want.  Not all windows have

this corner, for not all windows can be resized.

��[�]����� Title �����[ ]ͻ <--Title Bar

�                         �

�                         �

�                         �

�                         �

�                         �

��������������������������� <--Resize corner

Window Border

=============

All windows have a border around them, be it an edit

window or a dialog box.  The border tells you several

things about a window.  If the border is highlighted, it

means that window is the active window, or the one you are

using.  If the border is not highlighted, it means the

window is not active.  If the border changes to a color

that is not normal for active or inactive windows, it

means you are resizing and/or moving it.

Del Char Right  [BackSpace]

==============  [Ctrl]-[H]

Move the cursor to the left one space, deleting the

character before it.

Char Left       [Left Arrow]

=========       [Ctrl]-[S]

Move the cursor to the left one space.  No characters are

deleted.

Char Right      [Right Arrow]

==========      [Ctrl]-[D]

Move the cursor to the right one space.  No characters are

deleted.

July 26, 1994                   Page 70

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

Del Char Left   [Del]

=============   [Ctrl]-[G]

Delete the character to the right of the cursor.  All

other characters on the line are moved to the left to fill

the vacant space.

Del To End Line [Ctrl]-[Q]-[Y]

===============

Delete all characters from the current cursor position to

the end of the current line.

Delete Line     [Ctrl]-[Y]

===========

Delete the current line.  All lines below it are moved up

by one line.

Del To Start Line [Ctrl]-[Q]-[H]

================= [Ctrl]-[Backspace]

Delete all characters from the current cursor position to

the start of the line.

Delete Word     [Ctrl]-[T]

===========

Delete characters from the current cursor position to the

last character in the word, if the cursor is sitting in a

word.  If the cursor is sitting in spaces, all spaces to

the beginning of the next word will be deleted.

Page Bottom     [Ctrl]-[End]

===========

Move the cursor to the bottom of the current page.  A page

is defined as all text that is within the current window

frame.

Hide Selection  [Ctrl]-[K]-[H]

==============

Unhighlight any text you may have selected.  You press

this key sequence if you decide you don't need to perform

any action on the selected text.

Pressing the down arrow without the [Shift] key will do

the same thing.

Page Top        [Ctrl]-[Home]

========

Move the cursor to the top of the current page.  A page is

defined as all text that is within the current window

frame.

July 26, 1994                   Page 71

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

Auto-Indent     [Ctrl]-[Q]-[I]

===========

Toggle the editor auto-indent feature on/off.

Auto-indent will automatically indent the cursor to the

same position on the next line as the first character on

the current line when [Enter] is pressed.

If Auto-Indent is on, an "I" will appear on the edit

window indicator line.

This command is disabled until you open an edit window.

Insert Line     [Ctrl]-[N]

===========

Insert a carriage return and line feed pair at the current

cursor position.

All characters at the cursor position and to the end of the

line will be moved down to the next line.

Unlike the [Enter] key, the cursor stays at its current

position and does NOT move down to the next line.

Insert Mode     [Ins]

===========     [Ctrl]-[V]

Toggle the cursor between insert and overstrike mode.  In

insert mode, characters are inserted into the text at the

current cursor position and all characters after the

cursor are moved to the left.  In overstrike mode,

characters at the cursor position are replaced with the

new character you type.

Line Down       [Down Arrow]

=========       [Ctrl-[X]

Move the cursor down to the next line.

Line End        [End]

========        [Ctrl]-[Q]-[D]

Move the cursor from its current position to the end of

the line.

Line Start      [Home]

==========      [Ctrl]-[Q]-[S]

Move the cursor from its current position to the start of

the line.

July 26, 1994                   Page 72

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

Line Up         [Up Arrow]

=======         [Ctrl]-[E]

Move the cursor up one line.

New Line        [Enter]

========        [Ctrl]-[M]

Insert a carriage return at the current cursor position.

The cursor will move down to the next line.  If

Auto-Indent is on, the cursor will indent to the same

position as the first character of the line the cursor was

on when [Enter] was pressed.  Otherwise the cursor goes to

the first position of the new line.

Page Down       [PgDn]

=========      [Ctrl]-[C]

Move the cursor down a page at a time through your text.

Note that a page is defined as all text that is within the

current window frame.

This is not a "true" page down feature, in that the first

time you press the key the cursor simply goes to the

bottom of the current page.  You'll probably have to press

it twice to get to the next page.  Thereafter, it works as

a true page down feature.

Page Up         [PgUp]

=======         [Ctrl]-[R]

Move the cursor up a page at a time through your text.

Note that a page is defined as all text that is within the

current window frame.

This is not a "true" page up feature, in that the first

time you press the key the cursor simply goes to the top

of the current page.  You'll probably have to press it

twice to get to the previous page.  Thereafter, it works

as a true page up feature.

Document Reform [Ctrl]-[Q]-[U]

===============

Quickly reformat your document text.  A dialog box will

appear that allows you to select whether to begin

reformatting at the current cursor position or from the

beginning of the document.

Reformatting is based on the position of the first

character in the first line of a paragraph and the current

right margin setting.  You must be VERY careful when using

this feature for you can easily ruin your document's

appearance using the reformatting feature.

You can NOT undo the reformatting process.  It is

permanent.  Therefore, you are advised to save your

July 26, 1994                   Page 73

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

document BEFORE you use this feature.

This command is disabled until you open an edit window.

Paragraph Reform  [Ctrl]-[B]

================

Quickly reformat a paragraph.

Reformatting is based on the position of the first

character of the current line the cursor is on and the

current right margin setting.  Reformatting takes place

from the current line to the end of the paragraph.

You can NOT undo the reformatting process.  It is

permanent.

This command is disabled until you open an edit window.

Right Margin    [Ctrl]-[O]-[R]  {EDITOR Cmds:Editor_Commands}

============

Set a right margin value for the editor.  A dialog box

will appear that asks you to enter a right margin value.

Right margin values may range from 10 to 255.  The default

right margin is 76.

The right margin is used by the editor wordwrap,

reformatting, and centering features to determine where to

wrap a line of text.

This command is disabled until you open an edit window.

Scroll Down     [Ctrl]-[Z]

===========

Scrolls the screen up while maintaining the cursor

position.  If the original cursor position scrolls off the

screen, the cursor scrolls down a line.  This forces the

cursor to stay in the upper left corner of the screen

should the original cursor position scroll away.

Scroll Up       [Ctrl]-[W]

=========

Scrolls the screen down while maintaining the cursor

position.  If the original cursor position scrolls off the

screen, the cursor scrolls up a line.  This forces the

cursor to stay in the lower left corner of the screen

should the original cursor position scroll away.

Select Word     [Ctrl]-[K]-[T]

===========

Highlight the current word the cursor is sitting on.  The

July 26, 1994                   Page 74

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

word is marked from the current cursor position to the

next space or end of line, whichever comes first.  The

marked word is put into the clipboard without further

effort on your part.  Marking works in the normal manner,

i.e., it disappears the moment you move the cursor.

Note that NOTHING will happen if you try to use this

command when the cursor is sitting on a space or the end

of a line.

Tab Stops       [Ctrl]-[O]-[I]

=========

Enter custom tab stop preferences in a dialog box.  These

tab stops are used to allow you to quickly insert spaces,

or jump over text to a designated tab stop.

This command is disabled until you open an edit window.

Start Select [Ctrl]-[K]-[B]

============ [Shift]-[Down Arrow]

Start selecting text.  Selected text is that text which is

not the same color as normal text.  You select text when

you want to perform a particular action on that text and

no other text, such as COPY, CUT, CLEAR, etc.

Tab Key         [Tab]

=======         [Ctrl]-[I]

Move the cursor to the next tab stop.  Tab stops default

to every fifth character on a line.  You may reset the tab

stops by using [Ctrl]-[O]-[I].

When in INSERT mode, the cursor will attempt to go to the

next tab stop, happily INSERTING SPACES as it looks for

that next stopping point.  If it is at the last tab stop,

it stops inserting spaces and takes a big jump to the

first character of the next line.  Try to keep tab use to

a minimum when the cursor is in insert mode.  You eat up

valuable buffer space real quick!

When in OVERSTRIKE mode, the cursor will attempt to go to

the next tab stop, happily skipping over characters as it

looks for that next stopping point.  If it is at the last

tab stop, OR the line is too short to allow it to reach

the next tab stop, it takes a big jump to the first

character of the next line.

Document End    [Ctrl]-[PgDn]

============    [Ctrl]-[Q]-[C]

Move the cursor from its current position to the end of

the document.

July 26, 1994                   Page 75

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

Document Start  [Ctrl]-[PgUp]

==============  [Ctrl]-[Q]-[R]

Move the cursor from its current position to the start of

the document.

Word Left  [Ctrl]-[F]

=========  [Ctrl]-[Right-Arrow]

Move the cursor to the beginning of the previous word.

Use this feature when you want to quickly jump from word

to word.

Word Right  [Ctrl]-[A]

==========  [Ctrl]-[Left Arrow]

Move the cursor to the beginning of the next word.  Use

this feature when you want to quickly jump from word to

word.

Wordwrap        [Ctrl]-[O]-[W]

========

Toggle the editor wordwrap feature on/off.

Wordwrap allows you to type in text and automatically wrap

to the next line when the cursor reaches the current right

margin setting.

If wordwrap is on, a "W" will appear on the edit window

indicator line.

This command is disabled until you open an edit window.

Goto Marker     [Ctrl]-[Q]-[#]

===========

Using the WordStar [Ctrl]-[Q]-[#] key sequence, where "#"

is a number 1,2,3,4,5,6,7,8,9, or 0, will allow you to

access up to 10 place markers you have previously set in

your text.  Your cursor will go nowhere if you try to jump

to a non-existent marker.

Hold down the 'Ctrl' key and then strike 'q', let up on

both keys and then strike a previously set number key and

if this number key has been set you will go to the marked

line #.

This command is disabled until you open an edit window.

Set Marker      [Ctrl]-[K]-[#]

==========

Using the WordStar [Ctrl]-[K]-[#] key sequence, where "#"

July 26, 1994                   Page 76

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

is a number 1,2,3,4,5,6,7,8,9, or 0, will allow you to set

up to 10 place markers in your text.

While holding down the 'Ctrl' key strike 'k', then let up

on both keys and strike a number key.  Nothing will appear

to have happened but in actuality you have set a place

marker to the number key you have struck.

This command is disabled until you open an edit window.

6. QUICK-KEY CHART

F1  = Help File

F2  = Save Current File Being Edited

F3  = Open a File to Edit

F4  = Calculator

F5  = Zoom any Window

F6  = Change Window Views

F7  = Rates (Speeds and Feeds)

F8  = Print Current File Being Edited

F9  = Print Unregistered Version Invoice

F10 = Activate Menu Choices

Ctrl-F1  = View  MRMSOLVE.TXT (Answers)

Ctrl-F2  = Circular Segment Menu

Ctrl-F3  = Sequence a CNC File

Ctrl-F4  = View  MRMDRILL.TXT (Drill Speeds/Feeds)

Ctrl-F5  = Move Edit Window

Ctrl-F6  = Print MRMDRILL.TXT (Drill Speeds/Feeds)

Ctrl-F7  = Drill Speeds/Feeds Dialog

Ctrl-F8  = View  MRMBOLT.TXT (Bolt Circle Answers)

Ctrl-F9  = Print MRMBOLT.TXT (Bolt Circle Answers)

Ctrl-F10 = Split a CNC File

Ctrl-A   = Word Left

Ctrl-B   = Paragraph Reformat

Ctrl-C   = PageDown

Ctrl-D   = Character Right

Ctrl-E   = Line Up

Ctrl-F   = Word Right

Ctrl-G   = Delete Character Right

Ctrl-H   = Delete Character Left

Ctrl-I   = Tab Key

Ctrl-J-L = Jump to Line Number

Ctrl-#   = Set Marker

Ctrl-K-B = Start Select Text

Ctrl-K-C = Paste

Ctrl-K-D = Save/Done

Ctrl-K-F = Save As

Ctrl-K-H = Hide Block Marker

Ctrl-K-K = Copy Selected Text

Ctrl-K-T = Select Word

Ctrl-K-Y = Cut Selected Text

Ctrl-L   = Search Again

Ctrl-M   = New Line

July 26, 1994                   Page 77

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

Ctrl-N   = Insert Line

Ctrl-O-I = Tab Stops Set

Ctrl-O-R = Right Margin

Ctrl-O-W = Word Wrap

Ctrl-P   = Print MRMSOLVE.TXT (Answer File)

Ctrl-Q-# = Go to Marker

Ctrl-Q-A = Search/Replace

Ctrl-Q-C = Bottom of File

Ctrl-Q-D = Line End

Ctrl-Q-F = Search

Ctrl-Q-H = Delete to Start of Line

Ctrl-Q-I = Auto Indent

Ctrl-Q-L = Undo

Ctrl-Q-R = Document Start

Ctrl-Q-S = Line Start

Ctrl-Q-U = Reformat Document

Ctrl-Q-Y = Delete to End of Line

Ctrl-R   = Page Up

Ctrl-S   = Character Left

Ctrl-T   = Delete Word Right

Ctrl-U   = Un-Sequence a CNC File

Ctrl-V   = Insert Mode

Ctrl-W   = Scroll Up

Ctrl-X   = Down a Line

Ctrl-Y   = Delete Line

Ctrl-Z   = Scroll Down

Ctrl-Left Arrow  = Word Left

Ctrl-Right Arrow = Word Right

Ctrl-PgUp        = Beg. of a File

Ctrl-PgDn        = End of a File

Ctrl-Insert      = Copy Block to Clipboard

Ctrl-Delete      = Delete Block or Clipboard

Ctrl-BackSpace   = Delete to Start of Line

Ctrl-End         = Page Bottom

Ctrl-Home        = Page Top

Alt-A = Area Menu

Alt-C = Circle Menu

Alt-E = Edit Menu

Alt-F = File Menu

Alt-M = Math Menu

Alt-N = Replace Text

Alt-O = Options Menu

Alt-R = Rates Menu

Alt-S = Search/Replace Menu

Alt-T = Trig. Menu

Alt-V = Conversions Menu

Alt-W = Window Menu

Alt-X = Exit Mr. Machinist

Alt-F1 = About Mr. Machinist (Version Date)

July 26, 1994                   Page 78

MR. MACHINIST (tm) COPYRIGHT (c) 1994 BY ALLAN J. BREMER

7. SPECIAL THANKS

I must conclude this by giving Thanks to:

1) God  -  I thank Him for His Son Jesus Christ and for His

Word.  The Machinery's Handbook is the 2nd best

book in the world ... the Bible is the first!

2) Wife -  I thank my wife for sticking up for the Mr.

Machinist program and telling me to get to work

when I thought she would give me a hard time for

working so much.

3) Kids -  I thank my 5 year old twins for putting up with

Dad when he's sitting in the closet for hours at

a time, it's not over yet!  Oh, by the way, the

walk in closet was converted into an office!

4) Job  -  I also thank the management at my place of

employment for putting up with the weird hours

I've been keeping.  They also beta tested the

program and I have received much help from my

foreman and fellow workers.

5) Church- I thank the people who have prayed for me.

6) Bruce Bickle - Helped with the writting of this manual.

John Conway  - A master machinist, helped me beta test.

Jim Thiel    - Gave many thoughtful ideas.

Greg Armenia - Gave me many business tips, love ya Greg.

Greg Kraft   - Other support BBS and great friend.

Ed Hoffman   - Excellent Author and Engineer.

Gene Wood    - Many helpful business tips.

7) All Registered Users:

Thanks go to all who have decided to register the program

giving me the chance to send out disks, purchase postage,

implement ideas of theirs, upload the program to BBS's, and

much more!

Thank you and God bless you and all your family members.

Jesus answered, "I am the way and the truth and the life.

No one comes to the Father except through me.  John 14:6

+

+AL+

TO:                     FROM:

F1 COMPUTING                   Name: ________________________________

125 ELMWOOD AVE. #1

LOCKPORT, NY 14094        (Company): ________________________________

716-434-7823 Voice          Address: ________________________________

716-434-1448 BBS

City: ________________________________

Today's

date: ___________   State, ZIP Code: ________________________________

(Country): ________________________________

Daytime

Phone #: _________________________  FAX #: __________________________

MR. MACHINIST (TM) for MS-DOS

INDIVIDUAL USE

* Diskette with programs and documentation .............. $49   ___

MULTIPLE USE

* SITE LICENSE for the use of MR. MACHINIST.

(INCLUDES ONE DISKETTE WITH PROGRAMS AND DOCUMENTATION.)

===================================================================

2 to   9 computers ..... at $39 each    # computers ___x 39   ___

10 to  24 computers ..... at $30 each    # computers ___x 30   ___

25 to  49 computers ..... at $24 each    # computers ___x 24   ___

50 to  99 computers ..... at $18 each    # computers ___x 18   ___

100 to 199 computers ..... at $14 each    # computers ___x 14   ___

200 to 500 computers ..... at $12 each    # computers ___x 12   ___

WRITE OR CALL FOR PRICING ON QUANTITIES OVER 500.

Extra program disk & documentation with purchase of

SITE LICENSES OF 2 OR MORE available at $8.00 each.     ___x  5   ___

Shipping & Handling (Postal) US & Canada $5.00/item

Outside US & Canada $11.25/item                                   ___

I use 5 1/4 ______    3 1/2 ______   disks.

NEW YORK RESIDENTS add applicable State & County Sales Tax         __

TOTAL ENCLOSED US FUNDS                                      US$_____

All registered users with a single users license will receive life

time upgrades for 50% of the current registration fee.  Registered

site-licensed users will receive upgrades for 25% of the current

registration site-license fees.

MS-DOS is a registered trademark of Microsoft Corporation.

NAME TO INCLUDE ON TOP HEADER OF PROGRAM: ___________________________

NOTES and COMMENTS: