PowerCAD User Manual

Copyright (c) 1994 Paul Guimond, All Rights Reserved

1.      Table of Contents

1.      Table of Contents               1

2.      Welcome to PowerCAD             3

2.1.    How PowerCAD Works              3

3.      Before You Begin                4

3.1.    What You Need to Run PowerCAD   4

3.2.    The Licence Agreement           4

3.3.    Backing Up the Master Disk      4

4.      Installation                    5

4.1.    Before You Start                5

4.2.    Installing PowerCAD             5

4.3.    Starting PowerCAD               5

5.      Starting Work                   6

5.1.    Using the Mouse                 6

5.2.    The PowerCAD Window             6

5.2.1.  The Status Line                 7

5.2.2.  The Menu                        7

5.2.3.  The Work Area                   7

5.3.    Getting Started                 8

5.3.1.  Opening an Existing Drawing     8

5.3.2.  Starting a New Drawing          9

5.4.    Zooming and Panning             10

5.5.    Saving a Drawing                11

5.6.    Quitting PowerCAD               12

6.      Adding and Editing Components   13

6.1.    Adding                          13

6.1.1.  COMPONENTS                      13

6.1.2.  TEXT                            14

6.1.3.  LINE                            14

6.2.    Editing                         16

6.2.1.  MOVE                            16

6.2.2.  FLIP                            17

6.2.3.  ROTATE                          17

6.2.4.  SCALE                           18

6.2.5.  COLOR                           19

6.2.6.  DELETE                          19

7.      Using blocks                    20

7.1.    Loading a Block                 20

7.2.    Saving a Block                  20

7.3.    Moving a Block                  21

8.      The Function Key Commands       22

8.1.    Grid                            22

8.2.    Loop                            22

8.3.    Part Number                     22

8.4.    DXF Transfer                    22

1

Powercad User Manual

9.      New Symbols                     23

9.1.    Designing Custom Symbols        23

9.1.1.  The SYMBOL Screen               23

9.1.1.1.        Circle                  24

9.1.1.2.        Arc                     24

9.1.1.3.        Line                    24

9.1.1.4.        Arrow                   25

9.1.1.5.        Fill                    25

9.1.1.6.        Selection               25

9.1.1.7.        Line Type               26

9.1.1.8.        Base Point and Connection Points        26

9.1.1.9.        Delete                  27

9.1.1.10.       Quit                    27

9.2.    Importing DXF Files             27

9.3.    Using Custom Symbols            27

10.     Printing                        29

10.1.   Choosing a Printer              29

10.2.   Printing a Drawing              29

10.3.   Printing Documentation          30

10.4.   Adding Information to the Drawing       30

11.     Graphic Symbols                 31

11.1.   Pictures of Symbols             31

11.2.   Name of Symbols                 35

2

Powercad User Manual

2.      Welcome to PowerCAD

Welcome to PowerCAD.  PowerCAD gives you the power to design schematics

starting from the initial idea to the final drawing. These are just a small

description of the kind of schematics you can do with PowerCAD:

*       hydraulics,

*       pneumatics,

*       ladder,

*       electrical,

*       grafcet,

*       HVAC.

2.1.    How PowerCAD Works

When you start working on a drawing,  you'll usually begin by deciding on the

components you're going to place on the drawing.  To place components on the

drawing you can either choose them from the symbol menu or import them from

an already defined block.

PowerCAD is versatile.  You can define components that are not supplied in the

standard menu, and with these components you can define as many blocks as

you want.

As you build the diagram, you'll have to connect symbols together.  The ROUTER

will help you to connect the pieces together.  The ROUTER is an intelligent

piece of software that will permit you to optimize the flow of lines between

CONNECTION POINTS.

You'll have the possibility to move, rotate, flip, delete components from the

drawing.  These operations will trigger the ROUTER to optimize again the flow

of lines.

You'll have the possibility to see your drawings at any scale you want with

the ZOOM commands.

When you want to have a copy of your drawing, just select the command PRINT

and you'll obtain a print of the drawing and/or the PART LIST.

3

Powercad User Manual

3.      Before You Begin

This section explains what you need to do before you can start using PowerCAD,

and how you can begin to learn to use it.

3.1.    What You Need to Run PowerCAD

To run PowerCAD you need the following hardware and software configuration:

*       an 80x86 computer with a hard disk and a floppy disk drive,

*       at least 2 Mbytes of free space on your hard disk,

*       EGA or VGA screen and card,

*       640K of memory,

*       a MicroSoft compatible mouse,

*       a printer (9 pin, 24 pin or Laser),

*       DOS.

Note:   You don't need a math coprocessor to run PowerCAD, but if you have

one the performance of the software will improve greatly.

(4 to 7 times faster)

3.2.    The Licence Agreement

By using this program you indicate your acceptance of the following conditions:

You agree to use PowerCAD entirely at your own risk, and will indemnify and

hold harmless its author and distributors against any action resulting from

the use of PowerCAD.  The author and distributors of PowerCAD do not warrant

it fit or suitable for any particular purpose.

You agree that PowerCAD is protected by Canadian, U.S. and international

copyright law, and is the property of Paul Guimond and

Simulation Soft Expert inc.

3.3.    Backing Up the Master Disk

Before you install PowerCAD, you should make a backup copy of the master

disk.  We suggest that you use the Microsoft Windows File Manager to help you

back up the master disk.

4

Powercad User Manual

4.      Installation

You must install PowerCAD on your computer before you can use the software.

This section describes how to install and run PowerCAD.

4.1.    Before You Start

Before you start the installation procedure, you should:

*       back up the PowerCAD master disk,

*       read this section,

*       insure that your computer meets the requirements for PowerCAD.

4.2.    Installing PowerCAD

PowerCAD does not have a complicated installation process.  Setting up to run

on your system simply consists of running the setup program, which unpacks and

installs the files.  INSTALL.EXE will transfer the necessary files from the

master disk to the working directory of your hard disk.

To install the program:

1.      insert the master disk in the disk drive,

2.      call the program from the disk.                    INSTALL [ENTER]

Note:   PowerCAD will be installed on a hard disk, it needs 2 MBytes of

space to run.

4.3.    Starting PowerCAD

To start PowerCAD, type POWERCAD [ENTER]

The first thing you see is the introductory screen.  Press [ENTER].

5

Powercad User Manual

5.      Starting Work

This section covers the information you need to start drawing with PowerCAD,

from finding your way in the menu, to loading and saving drawings.

5.1.    Using the Mouse

The following terms are used in this User's manual to indicate to you what you

should do with the mouse:

Click      Press and release the left mouse button.

Cancel     Press and release the right mouse button.

Drag       Hold down the left mouse button and move the mouse.

Select     Position the mouse pointer and click the left mouse button.

5.2.    The PowerCAD Window

The PowerCAD window contains all the tools and commands you need to create

professional looking schematics.

[Figure 1 The PowerCAD Window]

6

Powercad User Manual

5.2.1.  The Status Line

On the status line, you will find information about:

*       the free memory,                        (100% when you start)

*       the operation you are doing,

*       the position of the mouse.              (X,Y)

If you look at the free memory index during the construction of a

schematic, you will notice it decreasing.  The maximum number of

components you can put on a drawing is 1000 (if all the memory is

availiable for the schematic).

5.2.2.  The Menu

In the menu you will find the command that you can select to produce a

drawing.  To select a command, just move the mouse to a menu item,

you will see a yellow box around the command you are pointing at, and

click on the mouse.

You can't select the dashes in the menu.  We have put dashes in the

menu to make logical groups of commands.

5.2.3.  The Work Area

The whole screen is the work area.  When you select a command from the

menu, the menu disappear.  You can use the whole screen to make your

schematics.

7

Powercad User Manual

5.3.    Getting Started

The first step to creating a schematic is to load an existing drawing, and

make modification on it, or create a new one.

5.3.1.  Opening an Existing Drawing

If you want to load an existing drawing you must:

1.      select LOAD in the menu,

2.      select the name of the file in the file selector,

3.      click on the name of the file you want.

Note:   When you scroll in the file selector, the drawing you are pointing

to with the mouse is shown in the preview box at the right of the

screen.

At any time, you can cancel the LOAD operation by pressing the right

mouse button.

[Figure 2 Loading a File]

8

Powercad User Manual

5.3.2.  Starting a New Drawing

If you want to start with a new page you must:

1.      select LOAD in the menu,

2.      select the NEW.#00 in the file selector,

3.      click on.

Note:   NEW.#00 is a drawing file with nothing inside.  When you load it,

you are erasing all the working memory.  If you don't have this

file, you can create it by saving an empty drawing.

9

Powercad User Manual

5.4.    Zooming and Panning

Zoom is like a magnifying window you move on the drawing.  To see more of the

drawing, but with less detail, you must reduce the zoom scale.  To zoom on a

specific detail, you will have to increase the zoom scale.

When PowerCAD starts, the zoom factor is equal to 1 and the coordinate of the

lower left corner of the screen is set to 0,0 and is called the origin.  You

can modify the zoom factor and the coordinate of the lower left corner with

the four zoom commands.

The four zoom commands are:

*       PAN

*       ZOOM ALL

*       ZOOM SCALE

*       ZOOM WINDOW

PAN

The PAN command will change the position of the viewing window on the current

display. You will have to designate two points with the mouse.  The first

point is the starting point for the displacement.  The second point is the

destination point.  When you have selected the two points, PowerCAD will

regenerate the drawing.

Note:   The selection of the second point is made by dragging the mouse to

the destination point.

ZOOM ALL

When you select the ZOOM ALL command, PowerCAD will regenerate the

drawing to display the entire drawing in the largest scale possible while

still fitting it on the screen.

ZOOM WINDOW

When you select the ZOOM WINDOW command, you will have to designate two

points with the mouse.  The first point is the first corner of the window.

The second point is the other corner of the window.  When you have selected

the two points, PowerCAD regenerates the drawing by fitting the window you

have selected inside the screen.

Note:   The selection of the second point is made by dragging the mouse to

the second corner of the window.  You will see a dotted box enclosing

the view area to zoom.

10

Powercad User Manual

ZOOM SCALE

With the ZOOM SCALE command, you can directly input the zoom factor for the

drawing.  After you have selected the command, you will see a dialog box where

you will input the new zoom factor.  The value you will enter is a relative

value.  If you enter 0.6 you will reduce the zoom factor to 60% of the

previous value.  If you enter 2.3 you will multiply the zoom factor by 2.3.

[Figure 3 ZOOM SCALE]

5.5.    Saving a Drawing

If you want to save the drawing you are working on, you must:

1.      select SAVE in the menu,

2.      type the name of the file in the dialog box.

The name of the file can have up to eight characters.  The extension is

automaticaly given by PowerCAD.  As an example when you first save a drawing,

you give it the name "ABCD".  PowerCAD will append to the drawing the

extension "#00".  The second time you give a drawing the same name,

PowerCAD will append the extension "#01".  The third time the extension will

be "#02"...

11

Powercad User Manual

5.6.    Quitting PowerCAD

If You want to quit PowerCAD, select Quit in the menu.  When you quit,

PowerCAD will save a copy of the drawing you are working on.  The name of the

backup copy is "QUIT.###".  This file is always overwritten by a new one when

you quit PowerCAD.

12

Powercad User Manual

6.      Adding and Editing Components

In this section are the commands you can use to make your schematics.  These

commands work with the intelligent ROUTER to give you the highest performan-

ce in your CAD work.

6.1.    Adding

Adding components to the drawing is like adding real components in a control

pannel or an assembly.  PowerCAD see the components as components and not

as a group of lines.  If you select the single acting cylinder, PowerCAD will

know it is drawing a single acting cylinder and also where the insertion point

and the connection point are in the drawing.

6.1.1.  COMPONENTS

When you want to insert a new component in the drawing, you must select

COMPONENTS or CUSTOM SYMB.  We give you a standard library of

200 components you can use.  If you need components who are not there,

you can create them with the program SYMBOL.  You can create 200

symbols and use them in your schematics.

After you have choosen the command, a graphic menu will display 25

symbols on the screen.  To select one of them, just select the symbol with

the mouse.  The menu will dissappear and you will see the component

drawn in shadow on the screen.  Move the component to its insertion,

point and click the mouse.  After you have inserted the component you

will see another component in shadow.  If you want to insert it, just move

it to its insertion point and click the mouse.  If you don't want it, just

cancel with the mouse.

If the component you want is not on the graphic menu, you can scan the

pages of the menu with the number keys 1-8.  When you press on 1,

you will see what is on page 1.

As you add components to the drawing, you will note a number that is

inserted near the component.  This number shows the uniqueness of the

component.  Each component has a number and it cannot be repeated.

Two components can't have the same number.

Special note:   If you pick a connection point (the symbol made with a

filled circle on pages 2 and 6 of the menu) and you place it

over a line, the line will be broken into two parts and the

two parts will be connected to the new connection point that

you have just inserted.

13

Powercad User Manual

6.1.2.  TEXT

You can insert text to add information to your drawing.  A string of text is

treated like a component of the schematic.  To add text to the drawing,

select the TEXT command.

The first thing to do is to click the mouse where you want to insert text.

Afterwards you will see a dialog box in which you will input the text you

want to display along with the size.  The maximum length of the text is 20

characters and you can input letters, numbers and special characters.

There is no restriction on the size.

You can cancel the command with the right mouse button.

[Figure 4 TEXT command]

6.1.3.  LINE

This command shows you the power of PowerCAD.  The LINE command

uses one of the most powerful piece of drawing software you have seen

until now, we call it the ROUTER.  To select, go on the LINE command

and click the mouse.

14

Powercad User Manual

To connect two components, just point the mouse near the first

connection point and click.  You don't need to be on the point, the

ROUTER will find the nearest point.  You will see a shadow line between

the first connection point and the mouse position.  Go near the second

connection point and click the mouse.

The ROUTER draws two lines.  The first line goes horizontally from the

first connection point to the horizontal position of the second connection

point.  The second line goes vertically from the second connection point

to the vertical position of the first connection point.  If you invert the

order of selection of the connection points, you obtain a different path for

the line.

The ROUTER selects the kind of line it must draw.  If the two connection

points are of the power type, the ROUTER draws a full line.  If one of the

point is of the pilot type, the ROUTER draws a dashed line.  The ROUTER will

not connect mixed types of connection points, like electrical type and power

type.

The ROUTER will generate all the loop for lines passing over other lines.

[Figure 5 LINE command]

15

Powercad User Manual

6.2.    Editing

The editing commands act upon components allowing them to be moved,

deleted, rotated, flipped, scaled, colored or combinations thereof.  These

commands work in combination with the ROUTER so you can quickly see the

results of you action on the lines and the network.

6.2.1.  MOVE

This command translates the selected component into a new position.  To

move a component:

1.      select MOVE in the menu,

2.      go onto the component you want to move and click the mouse,

3.      move the component to the new location and click the mouse,

4.      go onto another component you want to move or cancel with the mouse.

Shadow moving occurs with the movement of the selected component.

when you click the mouse on the new location of the component, you will

see a regeneration of the drawing to update the new positions of the

connecting lines.  You can't move lines alone.

[Figure 6 Moving a component]

16

Powercad User Manual

[Figure 7 Regenerating the drawing after MOVE]

6.2.2.  FLIP

This command will cause the selected component to be flipped across the

vertical axis.  To flip a component:

1.      select FLIP in the menu,

2.      go onto the component you want to flip and click the mouse,

3.      go onto another component you want to flip or cancel with the mouse.

You can't FLIP TEXT.

6.2.3.  ROTATE

This command will cause the selected component to be rotated 90

degrees counter-clockwise.  To rotate a component:

1.      select ROTATE in the menu,

2.      go onto the component you want to rotate and click the mouse,

3.      go onto another component you want to rotate or cancel with the

mouse.

17

Powercad User Manual

6.2.4.  SCALE

This command will cause the selected component to be scaled to one of

the seven standard scale factor { 8, 4, 2, 1, 1/2, 1/4, 1/8 }.  To scale a

component:

1.      select SCALE in the menu,

2.      go onto the component you want to scale and click the mouse,

3.      go onto another component you want to scale or cancel with the

mouse.

When you SCALE a component, PowerCAD will double the size of the

component.  When the component is eight times bigger than the standard

size and you try to scale it, the component will be sized at 1/8 of standard

size.  Part number is shown only when the scale of the components is

equal to 1.

If you want to SCALE TEXT, see the command EDIT.

[Figure 8 Edit commands]

18

Powercad User Manual

6.2.5.  COLOR

This command will cause the selected component to be colored with one

of the sixteen standard colors { Black, Blue, Green, Cyan, Red, Magenta,

Brown, Light Gray, Dark Gray, Light Blue, Light Green, Light Cyan, Light

Red, Light  Magenta, Yellow, White }.  To color a component:

1.      select COLOR in the menu,

2.      go onto the component you want to color and click the mouse,

3.      go onto another component you want to color or cancel with the

mouse.

When you COLOR a component, PowerCAD will scroll through the sixteen

colors to assign the color to the component.

6.2.6.  DELETE

This command will cause the selected component to be deleted from the

schematic. To delete a component :

Be sure of what you are doing, because there is no UNDO

command.

1.      select DELETE in the menu,

2.      go onto the component you want to delete and click the mouse,

3.      go onto another component you want to delete or cancel with the

mouse.

Lines who connect to the component you DELETE will also be deleted.

If you want to delete a LINE, go onto the point where the line bends and

click the mouse.

19

Powercad User Manual

7.      Using blocks

Blocks are standard circuits that enclose symbols and connecting lines.  With

blocks you can design complex schematics from standard circuits.  Blocks are

easy to create and manipulate and they give you the power to customize your

working environment.

7.1.    Loading a Block

Loading a block is like loading a file ,except that you don't erase the

working memory.  It is a merge of the block and the schematic in the working

memory.

If you want to load a block you must:

1.      select BLOCK LOAD in the menu,

2.      select the name of the block in the file selector,

3.      click on the name of the file you want,

4.      select the insertion point of the block.

Notes:  When you scroll in the file selector, the block you are pointing to

with the mouse is shown in the preview box at the right of the screen.

At any time, you can cancel the BLOCK LOAD operation by pressing

the right mouse button.

7.2.    Saving a Block

Saving a block gives you the opportunity to save standard circuits as building

blocks for bigger schematics.  It increase your design speed.

If you want to save a block, you must:

1.      select BLOCK SAVE in the menu,

2.      type the name of the file in the dialog box,

3.      you select the block you want to save by dragging the mouse over it,

4.      you put the mouse where you want the insertion point and click on the

mouse.

The name of the file can have up to 8 characters.  The extension is

automaticaly given by PowerCAD.  The extension is "CIR".

20

Powercad User Manual

7.3.    Moving a Block

This command translates the selected components into a new position.  To move

a block of components:

1.      select BLOCK MOVE in the menu,

2.      you select the block you want to move by dragging the mouse over it,

3.      you click on the starting point of displacement,

4.      you drag the mouse to the ending point of displacement.

21

Powercad User Manual

8.      The Function Key Commands

We have included four commands you can access through the function keys.

These commands have been put in the function keys because they are used less

often than the commands in the menu.

8.1.    Grid

The grid is visibly represented by a dot at each 1/8" in the horizontal and

vertical directions.  The grid can be turned on and off by hitting the [F7]

key when the menu is on the screen.  When the zoom is too small, PowerCAD

doesn't generate the grid.

8.2.    Loop

The ROUTER put loops automatically over wires and lines who cross each other

but are not connected.  The loops can be turned on and off by hitting the [F8]

key when the menu is on the screen.

8.3.    Part Number

Part number are automatically generated by PowerCAD.  The part number is

shown with the component on the schematics. You may turned this representa-

tion on and off by hitting the [F9] key when the menu is on the screen.

8.4.    DXF Transfer

This command allows you to transfer the schematic you work on in a format

other CAD can read.  The standard we use for the transfer is the Data Exchange

Format or DXF.  To transfer the schematic you are working on:

1.      press the [F10] key,

2.      enter the name of the file in the dialog box.

Notes:

*      The extension ".DXF" is automatically appended to the name of the file.

*      You can export DXF file but you can't import a DXF file in PowerCAD.

If you want to import a DXF file in PowerCAD, you must use SYMBOL to

convert the DXF file to a symbol you can use with PowerCAD.

22

Powercad User Manual

9.      New Symbols

PowerCAD comes with 200 standard symbols.  Some of the symbols are for

electrics, ladder or GRAFCET, but the majority of them are for fluid power.

We could not include all the symbols you might encounter in your field, so we

give you the opportunity to create your own custom symbols.  You can create

200 symbols, and use them in your schematics.

9.1.    Designing Custom Symbols

To design custom symbols you need the symbol editor named SYMBOL.

SYMBOL is a small CAD with only the basic drawing functions.  You can draw

circle, arc, line, arrow.  You can also import the entities part of DXF files you have

made with AutoCAD or any other CAD which produces DXF files.

9.1.1.  The SYMBOL Screen

The working area of SYMBOL is 3.25"x2.5".  It give you a lot of space to

design complex symbols.  At the upper right corner of the screen, you will

see the coordinates of the position where the mouse is pointing.  You also

have a grid to help you do the design of your symbols.

[Figure 9 The SYMBOL Window]

23

Powercad User Manual

9.1.1.1.        Circle

The first tool we will see is the circle.  With this tool, you can produce

circles on the screen.  You can have up to 50 circles in a symbol.

To draw a circle:

1.      select the circle symbol in the menu,

2.      click the mouse to indicate the center of the circle,

3.      drag the mouse to show the radius of the circle,

4.      when you have finished drawing circles, cancel with the mouse.

9.1.1.2.        Arc

The second tool is the arc.  With the arc tool, you can produce part of a

circle on the screen.  You can have up to 50 arcs in a symbol.

To draw an arc:

1.      select the arc symbol in the menu,

2.      click the mouse on the first point of the arc,

3.      click the mouse on the middle point of the arc,

4.      click the mouse on the last point of the arc,

5.      when you have finished drawing arcs, cancel with the mouse.

9.1.1.3.        Line

The third tool is the line.  With the line tool, you will produce straight

lines at any angle on the screen.  You can have up to 150 lines in a symbol.

To draw a line:

1.      select the line symbol in the menu,

2.      click the mouse on the first point of the line,

3.      drag the mouse to the second point of the line,

4.      when you have finished drawing lines, cancel with the mouse.

24

Powercad User Manual

9.1.1.4.        Arrow

The fourth tool is the arrow.  With the arrow, you will produce straight lines

at any angle on the screen, and on one side of the line you will get an

arrow.  You can have up to 50 arrows in a symbol.

To draw an arrow:

1.      select the arrow symbol in the menu,

2.      click the mouse on the first point of the arrow,

3.      drag the mouse to the second point of the arrow (on this side you will

see the tip of the arrow),

4.      when you have finished drawing arrows, cancel with the mouse.

9.1.1.5.        Fill

The fifth tool is the fill.  The fill gives you the opportunity to put a plain

fill in a part of the symbol.  The part you put the fill in must be closed.

If it is not closed, your are going to fill the whole symbol and the schematic.

You can have up to 50 fills in a symbol.

To fill a part of the symbol:

1.      select the fill symbol in the menu (the box filled in white),

2.      click the mouse in the region you want to fill (it must be a closed

region),

3.      when you have finished filling, cancel with the mouse.

9.1.1.6.        Selection

The selection box (the box with the two lines inside) must be used at two

different times when you edit a symbol.  The first time you use it is to

select the symbol you want to edit.  Symbols are shown in group of 25.

If the symbol you want to edit is not in the graphic page, you can scan the

pages of the menu with the number keys 1-8.  When you press on 1,

you will see what is on page 1.  If the symbol you want to edit is on the

page, select it with the mouse.

The second time you will use the selection box is to save the symbol you

are editing.  Just click on the selection box and your symbol will be

automatically saved in the symbol library.

25

Powercad User Manual

9.1.1.7.        Line Type

When you have lines in the symbol, you can change their types.  SYMBOL

gives you nine line types to choose from.  To change a type of line:

1.      click on the line type you want,

2.      click on the line you want to change,

3.      when you have finished, cancel with the mouse.

You can only change the type of lines.  You can't change circles, arcs or

arrows.

9.1.1.8.        Base Point and Connection Points

Base point and connection points are the most important part of a

symbol.  They give information to PowerCAD about where and how the

component must be drawn and to the ROUTER about where the lines

must connect.

Each symbol has a base point and it can't have more than one.  The

base point tells PowerCAD where to insert the component in the

schematic.  When you insert a new component or you move one on the

drawing, it is the base point who tells PowerCAD where to draw the

component in the drawing.  If you want to move the base point of the

symbol:

1.      click on BASE POINT,

2.      click on the symbol where you want to put the base point.

A symbol can have up to 50 connection points.  Connections points

are used by the ROUTER to connect lines between components.  You

must assign a type to a connection point.  Types are:

*       power (pwr),

*       fluid control (plt),

*       electrical (ele).

If you don't assign connection points to a component, it will be

impossible to connect lines to it.  If you want to assign a connection point

to a symbol:

1.      click on the kind of connection point you want,

2.      click on the symbol where you want to put the connection point,

3.      when you have finished, cancel with the mouse

26

Powercad User Manual

9.1.1.9.        Delete

This command will cause the selected object to be deleted from the

symbol. To delete an object :

Be sure of what you are doing, because there is no UNDO

command.

1.      select DELETE in the menu,

2.      go onto the object you want to delete and click the mouse,

3.      go onto another object you want to delete or cancel with the mouse.

9.1.1.10.       Quit

When you have finished designing with SYMBOL, select QUIT in the

menu.  When you quit, SYMBOL will update the symbol library, so it is

important to leave the editor by the QUIT command.

9.2.    Importing DXF Files

If you want to import a symbol you made with AutoCAD, or any other CAD, you

must save it with the DXF function of  the CAD.  Try to limit the dimension of

the symbol to 2 drawing units.

When you are in SYMBOL, press on [F10] and select the name of the file in the

file selector.

9.3.    Using Custom Symbols

SYMBOL gives you a lot of power, you can design your own symbols and add it

to  your schematics.  If you don't like a symbol in your drawings, you can

change its representation with the SYMBOL editor and it will be updated in all

your schematics and blocks.  If you combine the custom symbols capacity with

the block features you will see a noticeable increase in your productivity.

It is very important that everybody, who uses PowerCAD, uses the same custom

library.  If you don't, you will not be able to share your drawings.  The

custom library is part of the schematics you make, if you send your drawings

to others, and you use custom symbols in drawings, you must also send your

library :  "PIECEA.HYD" and "PIECEB.HYD".  If you don't, it will be impossible

to regenerate the schematics.

27

Powercad User Manual

As powerful as the software is, we recommend that you make a backup copy of

your library before you make any changes on custom symbols.  The name of the

files you must backup up are "PIECEA.HYD" and "PIECEB.HYD".

28

Powercad User Manual

10.     Printing

Printing drawings is very important.  PowerCAD gives you the opportunity to

print drawings and documentations that goes with them.

The drawing documentations are:

*       Parts List,

*       Purchase Order List.

10.1.   Choosing a Printer

PowerCAD support 3 kinds of printer; 9 pins, 24 pins and HP LASERJET.  If you

have a 9 pins or 24 pins, it must be in EPSON or IBM PROPRINTER emulation.

PowerCAD default printer operation is on 9 pin printer.  If you want to use

PowerCAD with another kind of printer you must add a software switch when you

call it.

If you are using a 24 pins printer start PowerCAD with this line :

POWERCAD  P:24

If you are using a laser printer start PowerCAD with this line :

POWERCAD  P:L

10.2.   Printing a Drawing

To print a drawing:

1.      put what you want to print on the screen (with the zoom commands),

2.      select PRINT in the menu,

3.      fill the dialog box with the informations of the drawing

(at PART LIST enter N).

Note:   If you want to cancel the PRINT command, just press the right button

of the mouse and continue to press until it stop.

29

Powercad User Manual

10.3.   Printing Documentation

To print the documentation of the schematic:

1.      select PRINT in the menu,

2.      fill the dialog box with information about the drawing

(at PART LIST enter Y).

10.4.   Adding Information to the Drawing

If you want to print a good documentation of your schematic, you will have to

add information on the components you want to put on the documentation.

To add information to a component:

1.      select EDIT in the menu,

2.      go onto the object you want to edit and click the mouse,

3.      enter information in the dialog box,

4.      go onto another object you want to edit or cancel with the mouse.

You must also add information to the file "PART.DAT".  This file is an ASCII

file and you can edit it and add information to it.  The information you will

find in "PART.DAT" are:

1.      manufacturer,

2.      part number,

3.      description,

4.      cost of the component,

5.      the sell price of the component.

The manufacturer can have up to eight characters and the part number can

have up to twenty characters.  There are no restrictions on the other fields.

All the fileds must be seperated by a comma ",".

30

Powercad User Manual

11.     Graphic Symbols

11.1.   Pictures of Symbols

[Figure 10 Page 1 Cylinders, Motors and Power Sources]

[Figure 11 Page 2 Conditionners, Regulators and Flow Controls]

31

Powercad User Manual

[Figure 12 Page 3 Directional Valves]

[Figure 13 Page 4 Directional Valves]

32

Powercad User Manual

[Figure 14 Page 5 Directional Valves, Gage and Connections]

[Figure 15 Page 6 GRAFCET and Logic Symbols]

33

Powercad User Manual

[Figure 16 Page 7 LADDER Symbols]

[Figure 17 Page 8 Electrical Symbols]

34

Powercad User Manual

11.2.   Name of Symbols

1.      Single acting cylinder

2.      Double acting cylinder

3.      Pressure intensifier

4.      Double end rod cylinder

5.      Telescopic cylinder

6.      Unidirectional fixed displacement hydraulic pump

7.      Bidirectional fixed displacement hydraulic pump

8.      Unidirectional variable displacement hydraulic pump

9.      Bidirectional variable displacement hydraulic pump

10.     Hydraulic power source

11.     Unidirectional fixed displacement hydraulic motor

12.     Bidirectional fixed displacement hydraulic motor

13.     Unidirectional variable displacement hydraulic motor

14.     Bidirectional variable displacement hydraulic motor

15.     Hydraulic oscillating motor

16.     Compressor

17.     Vacuum pump

18.     Pneumatic accumulator

19.     Gas charged hydraulic accumulator

20.     Pneumatic power source

21.     Unidirectional fixed displacement pneumatic motor

22.     Bidirectional fixed displacement pneumatic motor

23.     Unidirectional variable displacement pneumatic motor

24.     Bidirectional variable displacement pneumatic motor

25.     Pneumatic oscillating motor

26.     Power connection point

27.     Power connection point

28.     Pneumatic exhaust

29.     Hydraulic reservoir

30.     FRL

31.     Filter

32.     Water trap

33.     Cooler

34.     Heater

35.     Lubricator

36.     Pressure control  valve normally open

37.     Pressure regulator

38.     Pressure control  valve normally closed

39.     Sequence valve

40.     Differential pressure regulator

41.     Fixed restriction

42.     Throttle valve

43.     Non-return valve with restriction

44.     Non-return valve with restriction, fixed output

45.     Shuttle valve

46.     Non-return valve

47.     Non-return valve, spring loaded

48.     Non-return valve with pilot on closing

49.     Non-return valve with pilot on opening

50.     AND valve

35

Powercad User Manual

51.     2/2 normally closed, mechanical action and spring return

52.     2/2 normally closed, pneumatic pressure and spring return

53.     2/2 normally closed, hydraulic pressure and spring return

54.     2/2 normally closed, pneumatic pressure

55.     2/2 normally closed, hydraulic pressure

56.     2/2 normally open, mechanical action and spring return

57.     2/2 normally open, pneumatic pressure and spring return

58.     2/2 normally open, hydraulic pressure and spring return

59.     2/2 normally closed, roller and spring return

60.     2/2 normally open, roller and spring return

61.     3/2 normally closed, mechanical action and spring return

62.     3/2 normally closed, pneumatic pressure and spring return

63.     3/2 normally closed, hydraulic pressure and spring return

64.     3/2 normally closed, pneumatic pressure

65.     3/2 normally closed, hydraulic pressure

66.     3/2 normally open, mechanical action and spring return

67.     3/2 normally open, pneumatic pressure and spring return

68.     3/2 normally open, hydraulic pressure and spring return

69.     3/2 normally closed, roller and spring return

70.     3/2 normally open, roller and spring return

71.     3/3 closed center, pneumatic controls

72.     3/3 closed center, hydraulic controls

73.     3/3 open center, pneumatic controls

74.     3/3 open center, hydraulic controls

75.     3/3 tandem center, hydraulic controls

76.     4/2, mechanical action and spring return

77.     4/2, hydraulic pressure and spring return

78.     4/2, hydraulic pressures

79.     4/3 closed center, hydraulic controls

80.     4/3 open center, hydraulic controls

81.     5/2, mechanical action and spring return

82.     5/2, pneumatic pressure and spring return

83.     5/2, pneumatic pressures

84.     4/3 tandem center, hydraulic controls

85.     4/3 regenerative center, hydraulic controls

86.     2/2 normally closed, solenoid and spring return

87.     2/2 normally open, solenoid and spring return

88.     5/3 pressure center, pneumatic controls

89.     5/3 exhaust center, pneumatic controls

90.     5/3 closed center, pneumatic controls

91.     2/2, solenoids

92.     3/2, solenoids

93.     4/2, solenoid and spring return

94.     5/2, solenoid and spring return

95.     Magnetic switch

96.     3/2 normally open, solenoid and spring return

97.     3/2 normally closed, solenoid and spring return

98.     4/2, solenoids

99.     5/2, solenoids

100.    Pressure switch

36

Powercad User Manual

101.    3/3 closed center, solenoids

102.    3/3 open center, solenoids

103.    3/3 tandem center, solenoids

104.    4/3 closed center, solenoids

105.    4/3 open center, solenoids

106.    5/3 closed center, solenoids

107.    5/3 exhaust center, solenoids

108.    5/3 pressure center, solenoids

109.    4/3 tandem center, solenoids

110.    4/3 regenerative center, solenoids

111.    Quick exhaust valve

112.    Quick acting coupling with check valves

113.    Pressure gage

114.    Pressure transducer

115.    Visual indicator

116.    Manual shut off valve

117.    Quick acting coupling

118.    Flow meter

119.    Flow transducer

120.    Mufflers

121.    Plug

122.    Differential pressure gage

123.    Thermometer

124.    Thermal transducer

125.

126.    Electrical connection point

127.    GRAFCET

128.    GRAFCET

129.    AND pneumatic

130.    AND

131.    Electrical connection point

132.    GRAFCET

133.    GRAFCET

134.    OR pneumatic

135.    OR

136.

137.    GRAFCET

138.    GRAFCET

139.    NOT pnematic

140.    NOT

141.

142.    GRAFCET

143.    Memory pneumatic

144.    Off delay timer pneumatic

145.    NAND

146.

147.

148.    Single shot

149.    On delay timer pneumatic

150.    NOR

37

Powercad User Manual

151.    Normally open contact

152.    Energize coil

153.    Time delay energize (ON)

154.    Up counter

155.    Compare equal

156.    Normally closed contact

157.    De-energize coil

158.    Time delay de-energize (OFF)

159.    Down counter

160.    Compare greater than

161.    OFF-ON transitional contact

162.    Latch coil

163.    Retentive ON-DELAY timer

164.    Counter reset

165.    Compare less than

166.    ON-OFF transitional contact

167.    Unlatch coil

168.    Retentive timer reset

169.    Jump to label

170.    Return coil

171.    Electrical power source

172.    Electrical ground

173.

174.

175.    Label

176.    Push button NO

177.    Internal coil

178.    Input reference

179.    Capacitor

180.    Air core

181.    Push button NC

182.    Pilot light

183.    Output reference

184.    Resistance

185.    Diode

186.    Limit switch NO

187.    Solenoid

188.    Fuse

189.    Transformer

190.    DC motor armature

191.    Limit switch NC

192.    Selector switch

193.

194.    Resistance

195.    3 phase motor

196.    Pressure switch NO

197.    Pressure switch NC

198.    Thermal overload element

199.

200.

38