������������       �����   �����     �������     ������  ������

��� ���� ���       ������ ������    ���������     ����    ����

����           ���� ��� ����   ����   ����    ����    ����

����    ������ ����  �  ����  �������������   ����    ����

����           ����     ����  �������������   ����    ����

����           ����     ����  ����     ����   ����    ����   ���

����           ����     ����  ����     ����   ����    ����   ���

������          ����     ����  ����     ����  ������  �����������

������������������������������������������������������Ŀ

T-Mail v.24xx                        �

FTN-compatible mailer                   �

Copyright (c) 1991-94 by Andy Elkin (2:5030/15)     �

All rights reserved                   �

��

C O N T E N T S

�������������������

How to use this manual

Section 1. The operator's manual

����������������������������������������������������������������������

1. Main principles of operation and functions of T-Mail

2. Brief description of the organization of connections within

a FidoNet Technology Network (FTN).

Addressing Scheme.

Description of the addressing scheme used in T-Mail

� Structure of an FTN address

� Standards for FTN addresses used in T-Mail

� Short form addressing

� Address groups

� Address lists

� Summary

3. Technical requirements

� Hardware requirements

� Software requirements

� Additional requirements

4. FOSSIL, communications driver, setup

5. Installation and operation of T-Mail

6. Nodelist and nodelist compiler.

� T-Mail Nodelist Compiler, TNC

� Running T-Mail without a nodelist

7. Configuration files

� Metacommands: Define and Include

� Task number

� Task with specific time limits

� Use of environment variables

� The main configuration file     (t-mail.ctl)

� Modem set-up parameter file     (modem.ctl)

� Event set-up parameter file     (events.ctl)

� Index of periodic events

� Alias file list                 (subst.lst)

� Task password list              (password.lst)

8. The principles of Schedule organization.  ArcmailAttach and

BinkleyStyle

� Analysis of schedules and the formation of target calls

� Schedule management and display.  Status codes.

9. Outgoing call processing

� Incoming call response.  Session start-up

� Action following call answer

� External connection handlers

� FTS-0001 session

� External mail handler by key word

� BBS operation

10. Session handling

� Start of session, EMSI handshake, password processing

� Mailer session

� Exit to BBS at the end of the mail session

� On-line chat during the session

� Keys used during the session

11. Actions at the completion of the mail session.

12. File requests (FREQ)

� Restrictions and requirements for processing

File requests (Freq)

� List of directories for file requests

� Alias names

� Using password control to override restrictions

� In-transit and remote file requests

� Notification of file requests

13. MailBox (Postal boxes)

� Personal Mailbox for netmail

� File boxes, MailBoxes for files

14. Processes and flags (Process_Flag)

15. Modem management

� COM-port setup

� modem initialization

� Going Off hook when starting and external program

� Delay at loss of carrier (carrier monitor)

� Statistics log on connection termination

16. Terminal emulator

� Enter terminal mode

� External protocols

� Exit terminal mode

17. BinkleyTerm compatibility

18. T-Mail functions

� Crash and Immediate Mail

� Remote control: T-Mail server

� Service lines in messages and processing

� confirmation receipts

19. T-Mail management from the local keyboard

20. Additional set-up

� Definition of "hot" keys

� Sound effects

� Screen colors

21. Multi-line and Multi-tasking features.

� Number of tasks.  The Master/Slave(s) concept.

� Use of dedicated packer

� Flags and semaphores for multi-line operation

22. Feature specific to the OS/2 version

23. Command line parameters for T-Mail

24. Errorlevels returned by T-Mail

Section 2. Additional capabilities of the commercial version

����������������������������������������������������������������������

25. Operation with FAX messages

� Additional hardware supported

� Fax-gate

� Sending Fax messages

� Receiving Fax messages

� format conversion TIF v.1.1

26. Voice answer-back facilities

27. Operation in a local area network  (external nodes)

Section 3. Additional functionality

����������������������������������������������������������������������

28. Zmodem protocol set-up

29. Connection of key generators

30. Additional parameters in t-mail.ctl

31. Additional command-line parameters for T-Mail

Section 4. Examples and additional help information

����������������������������������������������������������������������

I. Example of a command file for T-Mail

II. Tools and libraries used to creat T-Mail

III. Standards and documents

IV. Thanks

V. Trademarks

VI. Conclusion

����ͻ

� How to use this manual

���������������������������������������������������������������������������

The first section of this document describes, in detail, the principles

of operation of T-Mail, the structure of a FidoNet Technology Network (FTN),

how to install, configure and start as well as giving help on commands and

functions within T-Mail.

In the second section will be found a description of the additional

functionality available with the commercial version of T-Mail.  If you have

not purchased the commercial version, this section may be skipped.

The third section is devoted to a description of additional

functionality aimed specifically at the advanced user, while the fourth

section is devoted to examples and help.

If you are a new user of T-Mail then it is suggested that the first

section be studied very carefully, after which you should have no fear of

your ability to modify the configuration files and start the program.

Chapter 7, section 1, describes the complete configuration of T-Mail and

offers significant help in creating a working system.  We suggest that you

do not use the advanced facilities described in section 3 until you are

familiar with T-Mail and it's capabilities.

This manual is useful as a reference document for the experienced user.

�����������������������������������������������������������������������������

���Section I������ operator manual �����������������������������

�����������������������������������������������������������������������������

����ͻ

� 1. Main principles of operation and functions of T-Mail

���������������������������������������������������������������������������

T-Mail is an electronic mail program, designed for use in FidoNet

Technology Networks, that is networks which are based upon the FidoNet

standards.  Nodes of these networks exchange information by means of the

switched telephone network using modems.

T-Mail is designed to handle the automatic exchange of information

between FTN-nodes.  T-Mail implements developed and accepted FidoNet

Technical Standards (FTS) such as EMSI, ZedZap and WaZoo.  T-Mail will

handle the receipt and transmission of mail, the preparation of mail,

and the execution of external programs by remote command.  Fax operations

(with a fax capable modem) and a voice answer-back (voice capable modem) are

possible with the commercial version.

One of the most useful features of T-Mail is that users of a connected

Local Area Network (LAN) may act as nodes of the system to read and write

messages.  (Commercial version only)

The advanced configuration capabilities of T-Mail allow almost

unlimited opportunities for maximum flexibility of operational modes,

supporting Hayes-compatible and fax modems of almost any type.  T-Mail is

fully multi-line capable provided there is some form of multi-tasking

environment or LAN.

����ͻ

� 2. Brief description of the organization of connections within

�    a FidoNet Technology Network (FTN).

���������������������������������������������������������������������������

Addressing Scheme.

Description of the addressing scheme used in T-Mail

T-Mail is intended to be used by nodes, operating within a FidoNet

Technology Network.  The simplicity of organization, minimum hardware

and software requirements necessary for the creation and operation of such

networks as well as low cost have made FTNs extremely popular all over the

world.  The largest FTN in the world at the present time and the progenitor

of all other FTNs, is FidoNet itself, with over 33000 nodes (1994).  This is

not to say that small FTNs may not be created, far from it, in fact T-Mail

facilitates the creation of FTNs quickly and easily according to your

individual needs.  These may be divisions or branches of large enterprises

or a network intended for the exchange of information between developers and

users.  Any system which involves communication between people can be served

by a FTN.

Consider an example of the organization and structure of a FTN.

A FTN has an hierarchical structure:

������������������������������������������������������>

�                              �

Zone 1                         Zone 2

�                              �

�����������������Ŀ            �����������������Ŀ

�        �        �            �        �        �

Region  region   region       region   region   region ����Ŀ

�        �        �            �        �        �        �

�����Ŀ     �     �����Ŀ     �������Ŀ    �     �����Ŀ     �

�     �     �     �     �     �   �   �    �     �     �     �

Net   net   net   net   net   net net net  net   net   net    �

�                                          �

�����������Ŀ                                    �

�     �     �                               Independent

Node  node  node                                node

- - - - - - - � - - - - - - - - - - - - - - - - - - - - - - - - - - -

�������Ŀ

�       �

Point   point

The smallest unit of this structure is the node.  All information

defining the details of the zones, regions, nets and nodes are collected

into a single document, called the NODELIST.  Nets are organised, in FidoNet

at least, on a geographic basis which is intended to reduce calling costs

for the delivery and receipt of mail.

Nodes are grouped, according to local geographic area, usually

determined by least call cost, into units called NETs.  Nets are grouped,

for administration purposes into units called REGIONs.  A region is

generally a grouping of systems which have something in common, such as

language or custom and in FidoNet, regions are often defined by the borders

of countries.  It is possible, although not recommended that a node be

placed outside of a net but inside of a region.  This node is then called an

"independent" node.

Regions are grouped together into ZONEs which are generally defined as

large countries or continents, again for administration purposes.

Thus an entire network is defined as a number of ZONEs, containing a

number of REGIONs, in turn containing a number of NETs, consisting of a

number of NODEs.  Each node represents a telephone line and a computer

system, with mailer software operated by the System Operator (SysOp).

Additionally, the structure supports automatic users, called POINTs.

The POINTs are not described in the nodelist, but are connected to the

network via an individual NODE.  The NODE, to which a POINT is connected, is

referred to as that point's BOSS node and all communication (via the

network, for the point) is handled by the BOSS node.

When creating a new FTN and selecting zone numbers, consideration

should be given to the possibility of communicating with existing FTNs.  It

is not possible for duplicate zone numbers to exist within the same FTN and

thus, for example, if communication with Fidonet is ever envisage, the zone

numbers 1 - 9 should not be used as these are reserved, exclusively for the

use of Fidonet.

Structure of an FTN address

���������������������������

Each NODE and POINT in the network has a unique address.  Addresses

within a FTN-net are formed as follows:

Zone:Net/node.Point

Where:

Zone   - The zone number (for example, 2 = FidoNet, Europe);

Net    - The network number.  As a rule, the first two digits of

the network number are the first two digits of the region

number.  e.g. 5030

Node   - The node number.  e.g. 100

Point  - The point number.  e.g. 15  A node always has a default

point number of zero.

Thus, the POINT which has been allocated number 15 and whose BOSS node

is 100 of net 5030 in zone 2 will have an address of 2:5030/100.15

The BOSS node address will be 2:5030/100.0 or more usually, 2:5030/100

Standards for FTN addresses used in T-Mail

������������������������������������������

As T-Mail is intended for operations with FTN-Networks, all addressing

is of the form acceptable for such networks.  This means that each node or

point within the network must be described by unique combinations of 3 or 4

integers:-

Zone:Net/Node.Point   This is called a 4-Dimensional address or 4D

addressing

or

Zone:Net/node         This is called a 3-dimensional address or 3D

addressing

The restrictions for creating such addresses, within T-Mail as as

follows:-

Zone - not greater than 1023       Net - not greater than 32767

Node - not greater than 32767    Point - not greater than 1023

Short form addressing

���������������������

In most cases, unless specifically stated, T-Mail will accept a short

form address. That is, T-Mail will use the default information of the

current node to fill in unsupplied information.

For example, if your address is 2:5030/15.0, then your default Zone =

2, your default Net = 5030 and your default Node is 15.  If you wish to

address one of your points, say 1, then you may address:-

2:5030/15.1   Zone = 2, Net = 5030, Node = 15, Point = 1

5030/15.1   Zone = default (2)

15.1   Zone = default (2), net = default (5030)

.1   Zone = default (2), net = default (5030),

node = default (15)

To address another node in the same net, say 2:5030/5:-

2:5030/5   Zone = 2, Net = 5030, Node = 5, Point = 1

5030/5   Zone = default (2)

5   Zone = default (2), net = default (5030)

To address a point off another node in the same net, say point 601 off

node 6 in net 5030:-

2:5030/6.601   Zone = 2, Net = 5030, Node = 6, Point = 601

5030/6.601   Zone = default (2)

6.601   Zone = default (2), net = default (5030)

This will be referred to in future as <address>

Address groups

��������������

In the majority of cases, T-Mail permits the use of address groups to

signify ALL addresses which are classified in the same way.  To signify a

group of addresses, the asterisk (*) character is used.  Assuming your

address to be 2:5030/5, then:-

All           All addresses.  i.e. all zones, all nets, all nodes and

all points.

*:*/*.*       All addresses as above

2:*/*.*       All addresses within zone 2

2:50*/*.*     All points of all nodes within any net whose first 2

digits ar "50" within zone 2.

2*:50*/10*.*  All points of any node whose number begins with "10" in

any net whose number begins with "50" in any zone whose

number begins with "2".  e.g. 21:5024/103.5

5030/*.*      All nodes in network 5030

This will be referred to in future as an <address group>

Address lists

�������������

It is possible to create groups of addresses in which not all the

addresses are contiguous.  In other words it may be desired to create a list

of addresses which both includes and excludes certain numbers.  For this

purpose the "!" symbol is used to indicate "except", thus:-

5030/* 5040/* !5040/2    All nodes in net 5030 and all nodes in net

5040 EXCEPT node 5040/2

237:*/* !237:50/*        All nodes in zone 237 EXCEPT all nodes in

net 50 of zone 237

T-Mail has a few reserved words which are used in address lists and

groups for special purposes:-

Reserved word    Meaning in T-Mail

-------------    ------------------------------------------------

All              Equivalent to *:*/*.* meaning all zones, all

nets, all nodes and all points.

Boss             Useful only for a POINT SysOp and means

Myzone: mynet/node.0

Listed           " All known systems ".  Systems which are "known"

to this system.  That is systems which are

described in local nodelists as well as in the

substitution list (subst.lst), see below for more

information on subst.lst

Protected        All systems with whom this system has establish a

session password.

It is also possible to make use of nodelist flags such as:-

CM, PVT, HST (including HST, H14 and H16), MNP, V42B, V32, V32B,

V32T, PEP, ZYX (implies V42, V42B, V32 and V32B) and ISDN (implies

ISDN*)

For example:

~~~~~~~~~~~

5020/CM! MNP     All nodes in net 5020 which are using the "CM"

flag, except those which are using te "MNP" flag.

The exclamation point symbol "!" is used to exclude defined addresses

from the specified list.  The exception is only effective on the group of

addresses immediately preceding the exclamation symbol.

The tilde symbol "~" is used to designate an inversion (logical not)

of the following nodelist flag.

For example:

~~~~~~~~~~~

5030/*.* !CM     All nodes of net 5030, except those using the

"CM" flag

5030/*.* !~CM    All nodes of net 5030, except those NOT using the

"CM" flag

CM !~ZYX         All "CM"  nodes except those NOT using the "ZYX"

flag

~CM              Non-"CM" nodes

As well, as the nodelist flags, up to 4 user-defined flags may be

defined, using the variable 'UserDef_Flag'

For example:

~~~~~~~~~~~

UserDef_Flag  ISKRA

UserDef_Flag  BEER

The user defined flags will be searched for in the same places as

normal flags, i.e. in nodelists following the 'baudrate' field and in

subst.lst.  User defined flags may be used in address lists, routing files,

resource access rights, dial_prefixes etc.  User defined flags ARE case

sensitive and therefore 'BEER' is not the same as 'beer'.  User defined

flags must be described in T-MAIL.CTL before they are used in the same file.

The specified list of addresses may also be supplied in a separate

file.  In this case, instead of supplying a list of addresses, it is only

necessary to supply the name of the file containing the list.  This is

handled by preceding the file name with the 'commercial at' character "@".

Delimiters used within the specified file may be either space (hex 20) or

new line (hex D)

For example:

~~~~~~~~~~~

@nodes.lst

Or

@c:\t-mail\dummy\nodes.lst

This will be referred to in future as an <address list>

Summary

�������

Addresses may thus be specified in many different ways:-

Absolutely, as in 2:5030/55

In groups, as in 2:5030/*

As a list, as in 5030/*.* !CM !ZYX

This mechanism is extremely flexible and allows the user to establish

exact control of which nodes, or group of node, are addressed at any one

time.

����ͻ

� 3. Technical requirements

���������������������������������������������������������������������������

Hardware requirements

���������������������

For T-Mail to function correctly, the minimum hardware requirement is:

� An IBM personal computer or 100% "compatible".

� At least 300 Kb of free memory (350 Kb for the commercial version);

� At least 600 Kb of disc space for program installation (800 Kb for the

commercial version);

� At least 32 Kb of free disc space for program operation

� Serial port for the use of an external modem

� Hayes-compatible modem (external or internal).

Software requirements

���������������������

For T-Mail to function correctly, the minimum software requirement is:

� MS-DOS or PS-DOS version 3.30 or later, or:-

Novell-Dos, version 3.41 or later, or:-

IBM  OS/2 version 2.0 or later

� FOSSIL communication driver, X00, BNU or similar for DOS, or the

driver SIO.SYS (recommended) or IBM's COM.SYS for OS/2

Additional requirements

�����������������������

The following hardware and software items, while not being mandatory

for the operations of T-Mail will enhance it's performance and capabilities

considerably and are highly recommended:

� Additional memory (EMS or XMS), which can be used for the rapid

swapping of T-Mail when caling external programs and commands.

� Disk caching software;

� A multitasking environment such as DesqView, VM/86, PC-MOS/8 etc, to

allow multiple copies of T-Mail to operate with several lines

simultaneously.

T-Mail also operates smoothly with most of the popular Bulletin Board

Systems (BBS), such as Remote Access, QuickBBS, Maximus-CBCS, SuperBBS, OPus

and others.

����ͻ

� 4. FOSSIL, communications driver, setup

���������������������������������������������������������������������������

To allow T-Mail to communicate with an internally or externally

connected modem, it is necessary to install a communications driver.  This

program is called a FOSSIL driver which stands for Fido/Opus/Seadog Standard

Interface Layer and is basically an INT 14 driver for an RS-232 serial

communications port.  We will discuss installation of two of the more common

FOSSIL drivers, X00 and BNU:

X00.SYS and X00.EXE (by Ray Gwinn).  This driver may be installed as a

resident device driver in the initialization file CONFIG.SYS:-

Device = X00.SYS E 1 T=1024 R=1024

Where:    E  ..  Inhibit display of driver initialization text

1  ..  Number of COM ports to support

T= ..  Transmit buffer size in bytes

R= ..  Receive buffer size in bytes

It is also possible to start X00 from the command line, or in

AUTOEXEC.BAT:-

X00.EXE E 1 T=1024 R=1024

BNU.COM (by David Nugent).  This driver may be started from the

command line, or in AUTOEXEC.BAT and will automatically make itself memory

resident:

BNU.EXE /P=1 /R=1024 /T=1024 /M-

Where:

P= ..  Number of COM ports to support

T= ..  Transmit buffer size in bytes

R= ..  Receive buffer size in bytes

M- ..  Inhibit display of driver initialization text

Recommendations for FOSSIL drivers:

1)  If using the command line start-up, place this command line in the

AUTOEXEC.BAT file, so the FOSSIL driver is loaded automatically at

startup.

2)  The transmission buffer should be minimum of 1024 bytes, making

this smaller may result in slow transmissions.

3)  The receive buffer should be a minimum of 1024 bytes, making this

smaller may result in lost data.

4)  To achieve maximum throughput using high-speed, error-correcting

modems it is necessary to "lock" the communications port.  The

reason for this requirement is that the data flow between the

computer (DTE) and the modem (DCE) is often significantly higher

than the data flow between the local modem (DCE) and the remote

modem (DCE).

5)  To ensure the continuous flow of data to and from the modem, some

form of "flow control" needs to be established.  For modern, high-

speed modems this needs to be handled by the computer and modem

hardware.  This is configured by the appropriate modem

initialization commands in MODEM.CTL, the command "HardWare_Flow

Yes" in TMAIL.CTL and the appropriate information being passed to

the FOSSIL driver.

In order to "lock" the ports in the FOSSIL driver the following

parameters may be given: (Assumes COM1 being locked at 19200bps)

When starting X00 as a driver in config.sys:

Device = X00.SYS E 1 T=1024 R=1024 B,0,19200

When starting X00 from the command line or AUTOEXEC.BAT:

X00.EXE E 1 T=1024 R=1024 B,0,19200

When starting BNU:

BNU.EXE /R=1024 /T=1024 /M- /L:0=19200,8N1

For the majority of V.22Bis modems it is possible to lock the port

speed at 9600 bps, for V.32 modems - 19200 bps and for V.32Bis modems 38400

or above.  It is advisable to always lock the communications port and the

FOSSIL driver to the maximum speed which can be handled by the modem.  When

using a modem capable of 9600 bps or faster, it is recommended to use a

buffered UART chip, such as an NS 16550 AFN.

For a more complete description of setting up your FOSSIL driver,

please read the documentation which accompanies that program.

����ͻ

� 5. Installation and operation of T-Mail

���������������������������������������������������������������������������

In order to install T-Mail on your system, you must execute the program

INSTALL.EXE from the distribution diskettes.  The installation process is

an interactive question and answer dialogue, to which the following will be

a useful guide.

The first question asked by the installation program is the pathname of

directory in which T-Mail should be installed.  Upon successfully verifying

that enough free disc space is available, all the necessary files will be

copied from the diskette(s) to the destination directory.  Following this, a

series of questions will be asked to determine the correct configuration to

be used (T-MAIL.CTL).

The following list will help determine the correct answers to the

configuration questions:

Net Address      -  The full address of your node as:

Zone:Net/Node.point  e.g  2:5020/15.0

Domain           -  The domain name of your network, if known.  If not

known, this may be omitted.

SysOp Name       -  The name of the person operating this system

BBS Name         -  The name of the system

Netmail path     -  Full pathname to a directory where NetMail

messages are to be stored

Inbound path     -  Full pathname to a directory where files coming in,

from other systems, are to be placed

Packets path     -  Full pathname to a directory for the temporary

storage of packets

Nodelist path    -  Full pathname to a directory where the nodelist is

to be stored

(Default pathnames are offered for the last 4 items above)

The MODEM is connected to: COM - The number COM-port, to which

Your modem is connected;

Maximal baudrate               - The maximum speed of your modem;

Locked baudrate                - The speed, at which your COM-port

is fixed.  If the COM-port is not

fixed, leave blank

Following this, the installation program will modify the default

configuration file (T-MAIL.CTL) according to your answers and create the

necessary directories.  This completes the basic installation procedure.

Following this, you should examine the configuration files in detail

and make any changes necessary to meet your needs.  All the relevant

information is given in this manual.

When you have completed any modifications to the T-Mail configuration

files you can start the T-Mail program by giving the command:-

T-MAIL

This command may be given from the command line or from within a batch

file, an example of which may be found in chapter 4-II.  It is also possible

to issue one or more parameters to T-Mail, a list of which may be found in

chapters 23 and 31.

On start-up, the screen will display information on files being loaded

and error messages should a fault be located.  Upon successful loading, the

initial screen will be replaced by the T-Mail main display, which consists

of 3 windows:

�����������������������Ŀ

�               �       �

�      3        �   2   �

�               �       �

�����������������������Ĵ

�                       �

�          1            �

�������������������������

Window #1, at the lower part of the screen, is used to display

information concerning the mail queue, about which more details may be found

in chapter 8.

Window #2, at the upper right of the screen, is used to display modem

response and file transmission status information.

Window #3, at the upper left of the screen, is used to display the

progress log and gives a brief indication of T-Mail's status.  At the very

top of the screen will be displayed your main address and the current time.

At the bottom of the screen will be a modem "status light" panel, indicating

the activity of the modem and session duration.  During a mail session, the

remote system's time will be displayed between windows #1 and #2.

����ͻ

� 6. Nodelist and nodelist compiler

���������������������������������������������������������������������������

The details of the nodes in a particular FTN are contained in a file

called a nodelist.  The format of a nodelist is defined in a FidoNet

Technical Standard (FTS), FTS-0005 and each line in a nodelist contains the

information concerning a single node.

Example:

,100,T-Mail_support,St.Petersburg,Andy_Elkin,7-812-271-0755,9600,V32,CM,XX

The above example line contains the main information of a network node:

Node number, name, location, SysOp name, telephone number and other flags

which give information about the type and speed of modem in use, operational

hours etc.

The data format for each nodelist entry, as shown above, is called the

St. Louis nodelist format and, while reasonable for humans to read, is not

the most efficient for a computer program to search and perform lookup.

In order to make efficient use of the nodelist, it needs to be

"compiled" into a special binary form with index files and fixed length

fields.  This task is performed by a nodelist "compiler" and T-Mail comes

with such a program.

T-Mail Nodelist Compiler, TNC

�����������������������������

The program TNC (T-Mail nodelist compiler), which is included in the

complete T-Mail package, is intended for use with the T-Mail program.  TNC

creates the necessary index files and tables for efficient nodelist

processing and fast lookup.  TNC can handle up to 32 different nodelists

simultaneously and can create a truncated nodelist if called upon to do so.

TNC uses it's own configuration file (TNC.CTL) which is a plain ASCII

file consisting of instructions and parameters, which must exist in the

current working directory.  Comments may be freely used by placing a semi-

colon ";" in column 1 and these are ignored by the compiler.

An example of a TNC control file:-

; --------------- Nodelist compiler TNC. Control file. --------------------

;

;

Address 2:5030/15.1          ; Full address of node/Point

;

; The full address, Zone:Net/Node.Point, of the system.

; For use with T-mail, this address must be exactly the

; same as the full system address supplied in TMAIL.CTL

;

Path c:\T-Mail\nodelist\     ; Path to nodelist

;

; The full path, Drive:\Directory\, to the directory to

; be used to store nodelists and pointlists

;

List nodelist.*              ; All NODELIST.??? files

List Contact.dat             ; Specific file

List net5030.D??             ; Net 5030 difference files

List abclist.999             ; The highest "nnn" of abclist.nnn

;

; THe nodelists and pointlists to be processed.  Each

; entry should be on a separate line and wildcards may

; be used according to MS-Dos conventions.

;

; If you are using wildcards, none of the following file

; extensions should be used, as they are ignored by TNC:-

;      ZIP   ARJ   LZH   ZOO   ARC   PAK

;      RED   IDX   NDX   INX   V8

;

; You may specify that TNC should use the highest numeric

; 3-digit extension, by specifiying an extension of 999

;

; -ZONE 1                    ; Exclude Zone 1

;

; To exclude an entire zone, use minus (hyphen) before the ZONE n

;

; +ZONE 2                    ; Include zone 2

; +ZONE 66                   ; as well as zone 66

;

; Include all zones specified by +Zone nn

;

; ReducedList Region 46 47 50

;

; This will create a reduced nodelist with the original

; nodelist name and a file extension of .RED  In other

; words if a Region tag is encountered, in NODELIST.266,

; for regions 46, 47 or 50 then a file called

; NODELIST.RED will be created containing only those

; region's information

;

; Examples:

; ~~~~~~~~

; ReducedList Zone 2 3

; ReducedList Net 5030 5040 5050

;

; ---------------------------- End of tnc.ctl------------------------------

The T-Mail nodelist compiler is initiated with the command TNC and one

or more of the parameters shown below:-

-C - Compile nodelists, if not up to date

-F - Forced nodelist compilation, always make compilation

-R - Create the specified files Rxx.RED containing the list

of networks under the "ReducedList" statement(s)

-S - Silent mode, no sound or screen output.  TNC will run

slightly faster in "silent" mode

-? - help.

Running T-Mail without a nodelist

���������������������������������

T-Mail may be run without using a nodelist, but in this case the

details of each node, with which transactions will be made, must be defined

in the configuration file SUBST.LST  In order to allow T-Mail to operate in

this mode, TNC should be run with the -F option to create an empty nodelist.

This mode of operation would be most likely in the event that T-Mail was

used as a Point mailer, due to the fact that Point systems generally

communicate only with their BOSS node or only with a very small number of

known systems.  It is unnecessary in these cases to hold information

concerning the entire network.

����ͻ

� 7. Configuration files

���������������������������������������������������������������������������

T-Mail uses a number of configuration files which are read at start-up

time.  These configuration files may be accessed during the session if

required for command handling.

All configuration files are in "plain ASCII" format and may be edited

with your favorite text editor.  Each file is described separately below

with a discussion of each function and it's use.  Please bear in mind that

this is written as reference material and it may be useful to return to this

information at a later date.

The main configuration file is called, by default, T-MAIL.CTL but

another name may be used if given on the command line by using the "-c"

parameter.  This file is used to define the main parameters of the system as

well as sound, names of other files used, directories, in-built terminal

emulator setup, fileboxes, flag definitions, compiled telephone lists, modem

management parameters, telephone answering system, external mail handlers,

EMSI procedures and transfer protocols.

Additionally, for the commercial version, the fax manager parameters,

answer-back and local area network information.

T-MAIL.CTL is the first file which is read when T-Mail starts up and

must exist in the same physical directory as T-MAIL.EXE, unless the '-c'

command-line switch was used.

The modem configuration file is called, by default, MODEM.CTL but

another name may be used if so defined in T-MAIL.CTL (variable Modem).

This file is used to define the modem type, initialization sequence,

prefixes and suffixes of telephone dialing codes, answer control and other

command strings.  This file must reside in the same directory as T-MAIL.CTL.

The event control file is called, by default, EVENTS.CTL and is used to

define the timing and behavior of events in the system as well as message

and file routing information.  This file must reside in the same directory

as TMAIL.CFG.

The substitution file is, by default, called SUBST.LST but another name

may be used if so defined in T-MAIL.CTL (variable SubstList).  This file may

be used to make changes to almost any item of variable data concerning

particular nodes.  This file must reside in the same directory as TMAIL.CFG.

The password control file is, by default, called PASSWORD.CTL, but

another name may be used if defined in T-MAIL.CTL (variable Security).  This

file is used to define file substitutions, session passwords for defined

nodes and points.  definitions described in SUBST.LST have priority over

those described in PASSWORD.CTL.

There are other files used for system initialization and session

maintainance, but these will be described at the appropriate sections in this

document.

For brevity, the default names (described above) will be used.

Comments may be freely interspersed with commands.  A comment is

defined as text which begins with a semi-colon ";" and terminates at the end

of the current line.  Comments are ignored by T-Mail.

Metacommands Define and Include

�������������������������������

The Metacommand DEFINE.  This metacommand may be used in T-MAIL.CTL,

EVENTS.CTL and SUBST.LST.  The syntax of the metacommand is:

Define <substring> <newstring>

Using a predefined Macro can greatly simplify the configuration

process and greatly simplify maintainance tasks, as can be seen from the

following examples:-

Example (T-MAIL.CTL)

~~~~~~~~~~~~~~~~~~~~

Define mynet 5030

Define mynode 15

....

Address 2: mynet/mynode.0

....

Freq_Nodes mynet/mynode.*

....

Example (EVENTS.CTL)

~~~~~~~~~~~~~~~~~~~~

Define hubs /2 /6 /35 /27 /108

....

Direct hubs

Priority hubs 11:00-13:00

Macro definitions made in T-MAIL.CTL, EVENTS.CTL and SUBST.LST are

completely independent and the maximum number of such definitions which may

be made in each file is 50.  Nested macros are not permitted.  Hexadecimal

values may be used for substitution, in the replacement field, by preceding

the hexadecimal value with a backslash "\" character:

Define <cr> \0D

Define semicolon \3B

Macro substitution is made ONLY in the 'value' part of a defined

macro and not in the 'variable_name' portion of the macro.

Example (T-MAIL.CTL):

variable         value

Address          2:5030/15

BBSname          name's personal BBS

If our macro is:-

Define    name   Andy

This will change the logical definition, in T-MAIL.CTL, to:-

Address          2:5030/15

BBSname          Andy's personal BBS

Please note that the 'variable' part is not changed at all, only the

'value' fields are affected.

The metacommand INCLUDE.  This metacommand may be used in T-MAIL.CTL.

The syntax of the metacommand is:

Include <filename>

This metacommand is used to logically pull together a number of file

fragments into one complete file.  The included files must conform to the

syntax rules of T-MAIL.CTL.  The "filename" supplied may be either a simple

file name of a path and file name.  If only a file name is given, then it is

expected to be in the same directory as T-MAIL.CTL.  The maximum depth of

inclusion is 10, but the size of the "included" file is not limited.

Examples:

~~~~~~~~

Include emsi.ctl

Include modem.ctl

Include answer.ctl

Include c:\t-mail\my_ctls\hmm.ctl

Task number

�����������

T-Mail is fully Multi-line capable when using a multi-tasking

environment or a Local Area network (for details see Chapter 21).  Each

individual line (or task) must be assigned a unique number when T-Mail is

started.  The task number is given on the T-Mail command line with the -N

<task_number> parameter, where <task_number> is in the range 0 to 9.  Thus

it is possible to utilize up to 10 lines simultaneously by running up to 10

simultaneous tasks.

Task numbers may be utilized in the configuration files (T-MAIL.CTL,

MODEM.CTL, EVENTS.CTL and SUBST.LST) to define that specific commands apply

only to a certain task or to all tasks.  This is indicated by specifying the

task number, in square brackets, before the command itself:

[N] <command and parameters>

The task number may be specified as:-

..  A number       The command applies to specified task ONLY

..  Letter   "M"   The command applies to the Master task ONY

..  Letter   "S"   The command applies to all tasks defined as SLAVE

..  Asterisk "*"   The command applies to ALL tasks

If the task number [N] is omitted, the command will apply to ALL

tasks.  Please note that task numbers are global and take on the same meaning

across all configuration files.

Example (T-MAIL.CTL):

~~~~~~~~~~~~~~~~~~~~

[1] Task_Mode Master

[2] Task_Mode Slave

[3] Task_Mode Slave

...

[M] Port 1         ; port for task #1

[2] Port 3         ; port for task #2

[3] Port 4         ; port for task #3

Example (MODEM.CTL):

~~~~~~~~~~~~~~~~~~~

[M] Init1 AT\N7|   ; modem initialization for task #1

[S] Init1 AT\N6|   ; modem initialization for tasks #2 and #3

...

[*] Init_Terminal ATE1M1|  ; global parameter for all tasks

...

Tasks with specific time limits

�������������������������������

It is often required, in a communications environment, to modify the

behavior of software based upon time considerations.  To this end T-Mail

uses time period definitions which may be applied to various commands as

parameters:

15:00-23:45

This defines the interval between 1500 hrs (3 O'clock in the afternoon)

until 2345 hrs (15 minutes before midnight).  Please note that the time is

defined in the 24 hour format and that spaces are not permitted within the

field.

It is also possible to categorize times by the day of the week:

1.01:00-5.07:00

This defines the time interval from 0100 hrs on day #1, until 0700 hrs

on day number 5.  The days of the week are numbered from 0 to 7 with 0 and 7

representing sunday, 1 .. monday, 2 .. tuesday etc.  Using these facilities

it is possible to create quite complex time definitions:

1.23:00-5.07:00,6.00:00-7.24:00

This defines the same period as shown above but also includes the

entire weekend from saturday at 0000 hrs until sunday at 2400 hrs.

2.09:00-2.17:00,3.09:00-5.15:00,4.12:00-6.18:00,5.21:00-5.23:30

This defines the periods:-

0900 - 1700 on tuesday and

0900 on wednesday until 1500 on friday and

1200 on thursday until 1800 on saturday and

2100 - 2330 on friday

The use environment variables

�����������������������������

When creating configuration files (T-MAIL.CTL, EVENTS.CTL and

SUBST.LST), environment variables may be used, both for Dos and OS/2.  The

convention used is to indicate the use of an environment variable by

enclosing the variable within percent "%" characters.

Example:

~~~~~~~

(AUTOEXEC.BAT)

SET Name=John Smith

SET Addr=2:5030/999

SET ans1=Answer

SET rin1=Ring

(T-MAIL.CTL)

SysOp %Name%          ; (equivalent to: SysOp John Smith)

Address %Addr%        ; (equivalent to: Address 2:5030/999)

(EVENTS.CTL)

Set %ans1%_%rin1%=4; (equivalent to: Answer_Ring=4)

Main configuration file (T-MAIL.CTL)

������������������������������������

The main configuration file for T-Mail is T-MAIL.CTL, a heavily

commented example is given below and this should be studied carefully.

The format of each line of this file is:

[[<task ident>]] <variable> <value> [;<comment>]

Where:

<task ident> - Task identifier (see "task number")

<variable>   - see the descriptions which follow

<value>      - variable value

<comment>    - comment

; == T-MAIL 2400 ======= T-Mail's main control file =======================

;

;

; ============================= System section=============================

;

; This section contains the main parameters of your system

;

;--------------------------------------------------------------------------

;

Address         2:5030/15.99@fidonet

;

; Main address of your node as Zone:Net/Node or

;                              Zone:Net/Node.Point or

;                              Zone:Net/Node.Point@domain

;

; AKA            5030/2.10@fidonet; Also Known As...

; AKA            237:9190/5.1@banet

;

; You may also include a number "AKA" (Also Known As) addresses, conforming

; the standard addressing scheme.  Up to 16 AKAs may be listed for THIS node

;

SysOp            John Smith  ; SysOp name

;

; The name of the system operator (SysOp) of the system.

;

BBSname          Smith-node  ; BBS Name

;

; The name of the system, which will be used for EMSI handshaking.

;

Phone            7-812-Pri-Vate; My Phone no.

;

; The telephone number of the system.  This is used in EMSI handshaking and

; for private system, this can be -Unpublished- or something similar.

;

Country          7           ; International country telephone code;

;

; The international dialing code for your country.  Russia - 7, USA - 1,

; UK - 44, Finland - 358 etc.

;

;

Place           St.Petersburg, Russia; Location

;

; Physical location of the system, used in EMSI handshaking only

;

; Synchronize 5030/15        ; Synchronize clock with this node

;

; The address of a node which will act as a "reference" system for the

; purposes of synchronizing your system's real-time clock.  Whenever your

; system connects with the system defined by this line, your real-time

; clock will be set to the same time as the remote system.

;

Mail_Hour        5:30-6:30   ; Mail hour (for non-CM nodes)

;

; A predefined "Mail hour" is a period during which it is guaranteed that

; systems which are non-CM (not operational 24 hrs/day), will be

; operational.

;

UTC +4                       ; Time Zone

;

; Establishes the difference (in hours) between UTC and local time.  For

; Moscow time this is +3 hrs in winter and +4 hrs in summer.

;

; Task_Mode Master           ; Define Master/Slave relationship

;

; In a multi-tasking environment it is necessary to define one system as

; the system which will handle the packing of mail, this system is called

; the Master system and all others are Slave systems.  Busy flags are

; created by both Master and Slave systems when they need to lock an address

; against other lines.  For Bink_mode these are -BSY flags and for

; Arcmial_attach are compatible with FrontDoor (CRC-32 calculated).

;

; ============================ Miscellaneous info ==========================

;

; This section defines the system's various additional parameters

;

;--------------------------------------------------------------------------

;

; T-Password my_password_no_5; Service request access control password

;

; Password which will be used to permit the execution of "service requests"

; from remote systems.  For more details on service requests, please see

; Chapter 18.

;

Output       Memory          ; Output by BIOS routines (default) or

;                              Directly to video memory

;

; Defines the methos of writing to the terminal screen, either as BIOS (via

; BIOS calls) or Memory (directly to the video memory).

;

Highlight ZR-16 2e           ; Highlight screen item, max 29 items

Highlight ZS-16 4e

Highlight Human caller 1f

;

; Set screen attributes for specific fields.  The attribute is defined by

; 2 hexadecimal numbers, the first number defines the background color

; and the second the foreground color.  The colors are identical to

; BIOS attributes and a maximu of 29 such attributes may be set.

;

Saver          <<Press any key>>

;

; Defines the text which is displayed to the user when the screen saver

; is in effect.

;

Saver_time 10

;

; Defines the inactivity time, since the last key depression, before the

; screen saver is activated.  Setting this value to zero, disables the

; screen saver.  The screen saver consists of the text defined by the

; command "Saver" being moved around the screen periodically.

;

; ============================ Key Definitions ==============================

;

;                         Definition of "hot" keys

;

;-----------------------------------------------------------------------------

;

; Syntax:

;

; Key <hex scancode> [<flag>] <command> [<parameters>]

;

;

Key 1200 c:\T-Mail\tm-ed.exe ; Alt-E, execute mail editor

Key 5900! C:\nc\nc.exe       ; Shift-F6, execute the program "nc" without

;                              a swap to disk

Key 2600 c:\nc\wpview.exe c:\T-Mail\T-Mail.log  ; Alt-L, execute command

;                                                 with the supplied parameter

;

; Definitions of keys, by scan code, and the actions which will be invoked.

; Up to 20 "hot-keys" may be defined here, for a more detailed description

; see chapter 20.

;

; Note:  This method, although supported, is obsolete in 24xx.  A better

; method is to define macros such as <Alt-F5> and <Alt-L>.  These are fully

; described in the READ.ME file

;

; ============================ T-Mail sound control ===========================

;

;                          Definition of sound effects

;

;-----------------------------------------------------------------------------

;

Sound            310         ; Sound yes/no (hexadecimal mask)

;

; Enables or disables sounds for each of the defined functions.  The

; hexadecimal mask is used to define which sounds are to be enabled (1)

; or disabled (0).  For more information on the meaning of each bit,

; see chapter 20.

;

; Syntax: SoundOf <event> <occurrences> <sample>

;

; Each sample contains one or some groups of 4 numbers, which are

; interpreted:-

;

; 1st number of group - starting tone (Hz) (-1 = silence)

; 2nd                   tone increment

; 3rd                   increment duration (ms)

; 4th                   number of increments

;

;

SoundOf Connect 1 100 10 100 4 100 -3 100 4

;                                                mask  Definition

;                                               ; 001  Modem connection

SoundOf Begin   1 37 1 10 64

;                                               ; 002  Begin session

SoundOf End    20 5700 1500 2 7

;                                               ; 004  End of session

SoundOf Abort   1 900 1 5 100

;                                               ; 040  Transfer aborted

SoundOf Human  10 300 150 6 10

;                                               ; 020  Human Caller

SoundOf Error   1 200 1 2 800 1000 -1 2 800

;                                               ; 080  Error (modem, fossil)

SoundOf Chat    4 3500 0 50 1 3000 0 50 1 4000 0 5 1

;                                               ; 010  Chat

SoundOf Unpack  1 12000 -100 6 100

;                                               ; 080  Unpacking mail

SoundOf Ring    4 3500 0 50 1 3000 0 50 1 4000 0 5 1

;                                               ; 200  Ring

SoundOf Tick    1 500 0 2 1

;                                               ; 100  Simple 'tick'

;

; ============================= Directory Settings ============================

;

;                           File names and pathnames

;

;-----------------------------------------------------------------------------

;

Files         C:\T-Mail\files\ ; Inbound files directory

;

; The directory where files, coming in from remote systems are stored, also

; the default directory where files (bound for other systems) may be stored.

;

; Files_UnSecured c:\t-mail\uncheck\; Inbound for non session-password

;                                     protected systems

;

; The directory where files, coming in from remote systems with whom a

; session password has NOT been established, will be stored.  If this

; directive is ommitted, the path defined by <Files> will be used.

;

; File_Boxes c:\t-mail\files\boxes\; Fileboxes path

;

; The directory where the file-boxes will be created.  If not specified,

; this will default to <Files>\boxes\.  Directories will be created, in

; this directory, for the purpose of of storing files, according to the

; address from which the files arrive.

;

; Log C:\t-mail\my.log       ; Log name

;

; The full pathname to the log file to be used by T-Mail.

; Default is T-Mail.log

;

; Chat_Log chat.log          ; Chat log name.

;

; The full pathname to the log file to be used for recording CHAT

;

NodelistPath c:\T-Mail\nodelist\; Path to nodelist (s)

;

; The full pathname to the directory used to store the nodelists and

; pointlists.

;

Security password.lst        ; Password security list

;

; The name of the file containing session passwords to be used in mail

; sessions

;

SubstList subst.lst          ; Substitution list

;

; The name of the file containing substitution information

;

; BroadCast 5030/*.* !5030/2 read.me     ; Broadcast list and file

;

; An <address list>, followed by a file pathname.  The specified file

; will be automatically delivered to any qualifying node who calls in

;

; Swap_Order XED             ; swap order XMS, EMS, Disk

;

; The priority order for swapping T-Mail when running an external program.

; X = XMS memory, E = EMS memory and D = disk

;

Swap          C:\; Swap directory

;

; The directory where T-Mail will create the necessary temporary and swap

; files, such as DOBBS.BAT, ANYFILE.BAT etc.  Default is the system

; directory and the swap file will be around 350Kb.  You must ensure that

; enough disk space is available for these files, failing which external

; programs will not execute.

;

; ============================== Terminal setup ===============================

;

;                  Set-up parameters for the terminal emulator

;

;-----------------------------------------------------------------------------

;

;

; Terminal          C:\tm\tm.bat /O ; Terminal program

;

; The full pathname of the executable for or batch file to be executed

; Whenever the terminal emulator is called (Alt-T and some other cases).

; If this variable is not defined, then an internal emulator will be

; called instead.

;

;- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

;

; If the external terminal emulator is not defined, then the following

; parameters can be used as long as the programs are available:

;

; Term_Download c:\gsz\gsz.exe port *P speed *B rz -rr

;

; External protocol which the emulator will use for receiving files.

; If this is not defined, an internal Zmodem implementation will be used.

;

; Term_Upload       C:\gsz\gsz.exe port *P speed *B sz *F

;

; External protocol which the emulator will use for transmitting files.

; If this is not defined, an internal Zmodem implementation will be used.

;

; Macro definitions which may be used as parameters of external protocols:

;

; *B - connection speed (DCE baudrate);

; *L - locked port (1 or 0);

; *P - COM-port number;

; *T - task number;

; *O - handle of open COM port (OS/2 only);

; *F - for download - path to the download directory (Term_DownDir)

;      for upload   - name of file to be transmitted

;

; Term_UpDir        C:\t-mail\files\

;

; Default directory for files to be uploaded.

;

;

; Term_DownDir c:\t-mail\files\

;

; Directory for downloaded files.

;

Term_Log           T-term.log

;

; The name of the log file, which will hold information concerning

; the session, uploads and downloads.

;

Term_Inactivity 60; Terminal inactivity timeout (minutes)

;

; The maximum operator inactivity time before the session is terminated.

; If a key is not depressed within this time, or if not transferring a

; file, the connection with the remote system will be aborted.  Setting

; this value to zero signifies an infinite timeout.

; ============================ Flags definitions ==============================

;

;                       Definition of flags (semaphores)

;

;-----------------------------------------------------------------------------

;

Flag_Dir c:\t-mail\flags\    ; Semaphores/Flags directory

;

; The full pathname of the directory in which inter and intra-task flags

; are stored.  Default is the T-Mail system directory.

;

Flag_Session busy.t-m        ; Global busy flag

;

; This flag is created by T-Mail whenever a mailer session is initiated.

; At the completion of the mailer session, the flag is removed.  It is

; created, by default, in the directory defined by Flag_Dir.

;

;Flag_Noscan noscan.t-m      ; Inhibit scanning

;

; This flag inhibits the scanning of netMail, Mailboxes and Fileboxes.

;

;- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

;

; Process flags may be defined for use by other programs to control the

; execution of T-Mail.

;

; Syntax:

;

; Process_Flag [!]<flag_name> <KeyWord> | <[!] command> | <dvpname>

;

; where

; <flag_name> -

;                The name of the flag.  This may be defined as a file

;                name or a full path and file name.  If only a file

;                name is given, the Flag_Dir directory is the path.

;

;                The exclamation mark "!" may be used to signify a NOT

;                function applied to the flag.  This is equal to inverting

;                the meaning of the flag and whatever action is defined

;                will occur if the flag does NOT exist, rather than the

;                usual case where actions occurs if the flag DOES exist.

;                The single exception to this is the keyword Sleep, in

;                which the task waits for the flag to be removed or created.

;

; <KeyWord> -    One of key words, which determines the action to occur

;                when the flag is created/removed:

;

;                  Exit <errorlevel>   - exit from T-Mail with specified

;                                        Errorlevel, errorlevel - number in

;                                        range 1-255.

;

;                  Sleep               - wait for the flag to be removed or

;                                        wait for it to be created if

;                                        inverted (!).

;

;                  Set <var> = <value> - Set a new value to a variable

;                                        defined in T-mail.ctl

;                  Poll <addr group>   - Generate a Poll to the address or

;                                        group of addresses, given in <addr

;                                        group>.

:

;                  Control             - Re-read configuration files

;

;                  Repack <addr group> - repack Netmail (rescan outbound).

;                                        If an <addr group> is given, then

;                                        mail repacking occurs for those

;                                        nodes only.

;

;                  Create <filename>   - create file <filename>

;

;                  Kill <filename>     - erase file <filename>

;

;                  Nothing             - do nothing.  This may be used to

;                                        remove or create a flag.

;

; <command> -    The name of the program or batch file (.exe, .com, .bat..)

;                which handles the appropriate processing, and the required

;                parameters.  Under Dos, an exclamation mark preceding the

;                command signifies NO MEMORY SWAP (fast execution).  Under

;                OS/2, a preceding exclamation mark signifies that the

;                communications port needs to be closed before the <command>

;                is executed and in this case the macro, *O, has no

;                meaning.

;

; <dvpname> -    The name of the DesqView program, i.e. the name of a

;                .dvp-file, defining the window to be opened.

;

; Examples:

; ~~~~~~~~

; Process_Flag anyarc.t-m c:\squish\squish In Out

; Process_Flag c:\anytic.000 anytic.bat -Debug

; Process_Flag doswap.t-m doswap.bat

; Process_Flag sleep.t-m Sleep                    ; Force sleep

; Process_Flag exit.t-m Exit 99                   ; Exit with errorlevel 99

; Process_Flag repack.t-m Repack                  ; Repack netmail

; Process_Flag alive.t-m Kill flags\alive.t-m     ; Erase file

; Process_Flag !Alive1.T-m Nothing                ; Create flagfile

; Process_Flag poll.t-m Poll /2 /100 /6           ; create polls

; Process_Flag set1.T-m Set Freq_Size=1000; set parameter

; Process_Flag test.t-m test.bat *A *P *B -debug  ; execute

;

; Macros mey be freely used to define variable parameters to <command>.

;

; *A - Current address in queue

; *B - connection speed (DCE baudrate);

; *L - locked port speed (or 0);

; *P - COM-port number;

; *T - task number;

; *O - (only for OS/2 versions) - handle of open COM port;

;

; The maximum number of Process_Flags - 32.

;

; =============================== Packer options =============================

;

;              Parameters defining the packing and processing of mail

;

;-----------------------------------------------------------------------------

;

Packets c:\T-Mail\packets\   ; Outbound packets directory

;

; The full pathname to the directory into which temporary packets and

; control files are stored for outbound mail.  This may be defined, in order

; to improve performance, as a directory on a ram-disk.  In BinkMode, this

; directory is the top of a tree-structure of mail directories, see chapter

; 17 for more details.

;

; Repack_On_Start Yes

;

; Used to define whether mail should be repacked whenever T-Mail is

; started.  Default is No, in which case the outbound mail status is

; preserved upon exit from T-Mail.

;

; Bink_Mode No               ; BinkleyTerm mode (no-ArcMail-attach)

;

; Defines the mode of operation of T-Mail.  Default is No, which signifies

; that the operational mode is ArcMial-Attach as used in FrontDoor, SEAdog

; and Dutchie.  Yes, signifies that BinkleyTerm style of mail processing

; is to be used.

;

; Scan_Circle 300            ; Bink Outbound rescan (for Bink mode)

;                                or netmail scan period in seconds

;

; For bink mode:

; Time interval between Bink-style outbound directory rescans.

; Default is 300 seconds = 5 minutes.

;

; For arcmail-attach mode:

; Time interval between outbound directory (Mail_Out) rescans and potential

; repacking of mail.  By default this is executed at the end of each cycle

; of operation.  For more details, see the definition of the Circle variable.

;

Mail_In c:\T-Mail\mail\      ; Inbound mail path

Mail_Out c:\T-Mail\mail\     ; Outbound mail path

;

; Source and target directories for messages, usually these are the same.

;

Direct          ArcMail      ; Never route messages from this robot

Direct          EchoMan      ; Max to 16 robot names

Direct          Allfix

Direct          T-Fix

Direct          TIC

;

; Messages "From" any of the above listed sources, will always be sent in

; Direct mode, i.e. never routed.  Up to 16 such sources may be defined.

;

; ================================ MailBoxes ===============================

;

;                       Definition of personal post boxes

;

;--------------------------------------------------------------------------

;

; This section describes mail boxes, or "personal post boxes" -

; additional directories for mail, for specific users.

;

; Syntax:

;

; MailBox <owner> = <directory> [<address>] [| <command>]

;

; where:

; <owner>          - The name of user of the post box (as it appears in

;                    the 'TO: ' field of messages

;

; <directory>      - The full pathname of the directory to hold messages

;                    addressed to <owner>

;

; <address>        - Address to which messages should be forwarded

;

; <command>        - The name of a program, which will be executed upon

;                    the arrival of mail to this post box.

;

; If a mailbox 'user' name is defined as UUCP, any messages addressed

; directly to UUCP or with a 'To:' field containing a commercial at symbol

; '@' will be directed to this mailbox for processing.

;

; Maximum number of mailboxes - 100.

;

; Examples:

; ~~~~~~~~

; MailBox John Smith = c:\t-mail\mail\johnmail

; MailBox SysOp = c:\t-mail\mail .1 | beep 10

; MailBox NewsMaster = c:\t-mail\mail\nsm | run-ed.bat

; MailBox UUCP = c:\t-mail\mail 5020/23

; MailBox UUCP = c:\t-mail\uumail | gate.bat

; ========================== 'After-session' Control ==========================

;

; This section defines the behavior of T-Mail at the end of session,

; i.e. sets the conditions for the processing received mail and files.

;

;-----------------------------------------------------------------------------

;

; Unpack       No            ; Do not unpack .pkt files

;

; Defines whether received mail bundles should be unpacked immediately upon

; receipt.  Default is Yes

;

Unpack_Size 2048             ; Max size of .pkt to be unpacked if "Unpack"

;                              set to Yes.

;

; This defines the maximum size of an inbound .PKT file to be unpacked if

; unpacking is to take place while the session is in progress.  The size

; should be defined according to the performance of your system and it

; should be borne in mind that that this unpacking takes place while the

; session is active.  This could cause a time-out on a slow system.  The

; default size is 8000 bytes.

;

Kill_Empty Yes               ; Kill empty messages

;

; Defines whether empty messages should be removed once unpacking has

; occurred.  Empty messages are those which contain no text and are used for

; file attach or service requests.  Default is No, i.e. leave empty messages

;

Kill_Transit Yes             ; Kill In-Transit messages

;

; Forces T-Mail to flag In-Transit messages as Kill/Sent, which will result

; in the message being deleted after dispatch.

;

; Transit_Files_From All     ; Allow In-Transit files FROM list

;

; Transit_Files_For All      ; Allow In-Transit files TO list

;

; A list of addresses for which you permit remote systems to send files.

;

; For In-Transit files, the following conditions must be met:

;

; 1) The address of the author of the message must be in the list

; 2) The address of recipient must also be in the list.

;

; Examples:

; ~~~~~~~~

; Transit_Files_From V32B!/100.13

; Transit_Files_From 2:5030/100.1 50/*.* !50 /919.5

; Transit_Files_For @trlist.txt  ; List of allowable recipients

;

; Audit_Send_Msg Yes         ; Audit message

;

; Defines whether confirmation receipts should be handled or ignored.

; If defines as Yes, then messages with the ARQ attributes will cause a

; confirmation receipt message to be generated to the originator.  The

; confirmation receipt is generated from a template file (AUDIT.TPL).

; Default is NO confirmation receipts.

;

; =========================== Process Control ==============================

;

; Description of the mechanisms of activity process control.  External

; programs executed at end of session, receipt of control files, network

; mail, echomail and file echoes.

;

;--------------------------------------------------------------------------

;

; Syntax:

;

; Process <macro> | <filemask> <KeyWord> | <[!] command> | <dvpname>

;

; where <macro>:

; ArcMail              - Processing of inbound netmail and echomail

; Tic                  - Processing of inbound .?IC files

; NetMail              - Mail (.PKT) files

; Online               - specified file(s)

; Files                - Any file

; BBS                  - BBS request

; Session              - At session termination

; Fax                  - Upon receipt of a facsimile

; Empty_Queue          - For Multi-line systems only, launch on Master when

;                        the mail queue becomes empty

; Empty_Fax_Queue      - As for Empty_Queue, but applies to the Fax queue.

;

; EMSI_Timeout         - If EMSI handshake not received or explicit BBS call

;

; <filemask>           - filename, wildcards according to Dos or OS/2

;                        conventions

;

; <KeyWord>            - One of the following key words, which will

;                        determinate the action to be taken

;

;                  Keyword                     Action

;

;                  Exit <errorlevel>   - exit from T-Mail with specified

;                                        Errorlevel, errorlevel - number in

;                                        range 1-255.

;

;                  Set <var> = <value> - Set the new value of the specified

;                                        (T-mail.ctl) variable

;

;                  Poll <addr group>   - Poll the systems specified by <addr

;                                        Group>

;

;                  Control             - Check if the configuration files

;                                        have been modified and if necessary

;                                        re-read them.

;

;                  Repack <addr group> - repack Netmail (rescan outbound).

;                                        If an <addr group> is given, then

;                                        mail repacking occurs for those

;                                        nodes only.

;

;                  Create <filename>   - create file <filename>

;

;                  Kill <filename>     - erase file <filename>

;

;

;                  Nothing             - do nothing.  This may be used to

;                                        remove or create a flag.

;

; <command> -    The name of the program or batch file (.exe, .com, .bat..)

;                which handles the appropriate processing, and the required

;                parameters.  Under Dos, an exclamation mark preceding the

;                command signifies NO MEMORY SWAP (fast execution).  Under

;                OS/2, a preceding exclamation mark signifies that the

;                communications port needs to be closed before the <command>

;                is executed and in this case the macro, *O, has no

;                meaning.

;

; <dvpname> -    The name of the DesqView program, i.e. the name of a

;                .dvp-file, defining the window to be opened.

;

;

; Examples:

; ~~~~~~~~

; Process Online  *.reg  doreg.bat *A *D *B       ; Process key registration

; Process ArcMail c:\squish\squish.exe In Out

; Process Tic anytic.bat -Debug                   ; System directory is

;                                                   searched

; Process Netmail Create flags\anymail.t-m        ; Created flag

; Process *.doc copy files\*.doc \doc             ; File mask used

; Process *.zip Exit 98                           ; Exit with errorlevel

; Process Files c:\tools\fixfiles.com             ; Full path of .exe file

; Process EMSI_Timeout dobbs.bat *P *B            ; BBS if no EMSI

; Process BBS          dobbs.bat *P *B            ; or explicit BBS request

; Process *.exe del ..\inbound\*.exe

; Process test *.* move ..\inbound\test*.* ..\inbound\testfiles

; Process Arcmail Create flags\anyarc.t-m

;

; If a process has not been defined for ArcMail or Tic, the files received

; will receive no special processing.

;

; Macros may be freely used to define variable parameters to <command>.

;

; *A - Full address of the remote system of the current session.

; *B - Connection speed (DCE baudrate);

; *D - Full default filebox name

; *L - Locked port speed (or 0);

; *P - COM-port number;

; *T - Task number;

; *O - (only for OS/2 versions) - handle of open COM port;

; *N - 1 at successful completion of session, otherwise - 0.

;

; The maximum number of Process_Flags - 32.

;

;

; Process Fax fax.bat                         ; If fax input

;

; Defines the process to be executed when a fax file is received, it

; is possible to send the executable file (or batch file), the file name of

; the input fax file, as a parameter.  The fax processor should not use

; substitution macros defined above.

;

; ============================== Modem Control ================================

;

;                               Modem management

;

;-----------------------------------------------------------------------------

;

Include Modem.ctl            ; Modem configuration

;

; The name of the modem configuration file.  This file will contain the

; modem initialization sequence(s), commands, responses etc.

;

; Init_Interval 20           ; Cyclic modem initialization time (min).

;

; The time interval between cyclic modem initialization sequences.  This

; is the maximum time between modem initialization sequences, i.e. if no

; call has answered or no calls made to remote systems within this time-

; period, the modem will be re-initialized to ensure that it is fully

; operational.  If no "OK" response is received from the modem, then the

; process described under "No Modem" will be initiated.  If no "OK" response

; is received as a result of this process, then T-Mail will exit with

; ErrorLevel 5.  The default and recommended value is 20 minutes.

;

Modem_Delay 10               ; Delay before passing a command to the

;                                modem, in units of 0.1 seconds

;

; The delay, in tenths of a second, before sending a command to the

; modem

;

; Init_After_Dial Yes        ; Force modem initialization after dial

;

; Forces re-initialization of modem after each dial attempt.  If set to 'No'

; the modem initialization will take place only if a non-standard

; dial-prefix is used for this dial attempt.  A non-standard dial-prefix is

; any dial-prefix other than the default.

;

Carrier_Lost_Delay 30        ; Carrier loss detection delay (0.1 sec)

;

; T-Mail will ignore a loss of carrier for the time specified here.  This

; allows T-Mail to possibly keep a session "alive" even during short-term

; line drops.  The time is specified in units of 0.1 seconds.

;

; Busy_Hang_Off Yes          ; Go "off-hook" when busy

;

; Defines whether T-Mail should force the modem off-hook when running an

; external program.  This is done such that callers will see a "busy"

; line while T-Mail is not able to answer the phone.  Default is No - leave

; the modem on-hook.

;

; Process No Modem init.bat *P ; Process for modem initialization failure

;

; Defines the executable program, or batch file which will be executed in

; the event of T-Mail being unable to initialize the modem.  Macros may

; be used on this command line as substitution variables defined below:

;

; *B - connection speed (DCE baudrate)  Baud variable in T-MAIL.CTL

; *L - locked port speed (or 0);

; *P - COM-port number;

; *T - task number;

; *O - (only for OS/2 versions) - handle of open COM port;

;

; ============================== Dial Control ==============================

;

;  Modem calling control, dialing frequency, limit, duration etc.

;

;--------------------------------------------------------------------------

;

Time_Dial 50                 ; Dial time-out, in seconds

;

; Sets the length of time, in seconds that the modem will wait for carrier

; before hanging up.  Internal modem register S7, should be set to at least

; the same value as defined here

;

Call_Tries 25                ; Number of time to call to a node

;

; The number of times T-Mail should attempt to call out to each address

;

Max_Rings 8                  ; Maximum number of Rings to remote system

;

; For modems which can return a "RINGING" response when calling to remote

; systems, the maximum number of "RINGING" responses before aborting the

; current dial attempt.

;

;

Circle         45            ; Interval between outgoing calls (sec)

;

; The time interval, in seconds, between calls to remote systems.  The

; delay between calls is necessary to allow remote systems to call in as

; well as to allow event analysis to take place.

;

; Call_Time 0:00-24:00       ; Outgoing call schedule

;

; The time period during which outgoing calls are permitted.  It is

; reasonable to only permit outgoing calls to occur at certain times of

; the day, for example when the call cost is at it's lowest.  Also, not

; allowing T-Mail to make outgoing calls at certain times, will improve

; performance by lowering the overhead required to examine the mail

; queues and statii.

;- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

;

; This section defines the telephone number translation rules.  The format

; of telephone numbers in the nodelist is a universal, international

; standard and the numbers must be modified to accommodate local dialing

; practices.  This is handled by substituting one set of characters

; <substring for replacement> (1 or more) by another set of characters

; <new substring> (0 or more) in the telephone number strings.

; When using the Dial International string, this is forced before the

; telephone number before the call is made.

;

; Syntax:

;

; Dial <substring for replacement> [<new substring>] [|<suffix>]

;         Or

; Dial International <substring for international calls>

;

; The maximum number of replacement dial strings which may be defined

; is 20

;

Dial 7-812                   ; local calls, remove international

;                              and local area code

Dial 7-    8W                ; National calls, remove international

;                              code and add outside line request

Dial International 8W10      ; International calls, add outside line

;                              request and international dial code.

; =============================== Answer Control ===========================

;

;                   Modem answer control, BBS open hours etc.

;

;--------------------------------------------------------------------------

;

Answer 0:00-24:00            ; System open time for remote callers

;

; The time period(s) during which T-Mail will respond to remote calls

;

Answer_Ring 1                ; Number of rings before picking up the phone

;

; The number of RING responses to receive before answering an incoming call.

; Default is 1 ring.

;

BBS_Time 0:00-24:00          ; Allow human Callers

;

; The time period(s) during which human callers are allowed access to the

; BBS software defined in Process BBS

;

Wait_Carrier 30              ; Wait for carrier (sec) on answer

;

; Maximum time to wait for a successful CARRIER DETECT after answering a

; call.

;

; Hardware_Ring Yes          ; Answer by Ring Indicator (RI) signal

;

; Defines how a modem RING will be detected.  This may be by either the

; hardware "Ring Indicator" signal or by the modem sending the string

; "RING" to the computer.  Default is No, indicating that a "RING"

; response is expected as defined in MODEM.CTL.

;

BBS_Banner Banner.txt        ; File displayed if BBS available

;

; The name of the file to be displayed to a human caller, requesting

; access to the BBS, if the BBS IS available.

;

NoBBS_Banner sorry.txt       ; File displayed if BBS unavailable

;

; The name of the file to be displayed to a human caller, requesting

; access to the BBS, if the BBS is NOT available due to time restrictions.

;

Process BBS bbs.bat *P *B *H ; BBS software

;

; Define the program, or batch file, which will be executed to load the

; BBS software into memory and start it.  This process will only be

; executed in the event that BBS access is permitted and the current time

; is within the constraints given in BBS_Time.

;

; Macros may be used on this command line as substitution variables defined

; below:

;

; *B - connection speed (DCE baudrate);

; *L - locked port speed (or 0);

; *P - COM-port number;

; *T - task number;

; *O - (only for OS/2 versions) - handle of open COM port;

; *H - The maximum time (in minutes) that a user may use the BBS.

;      i.e. the time before the period defined in BBS_Time expires.

;

; Force_BBS Press ESC-ESC!   ; BBS announcement text

;

; A short text, given to the remote caller, to identify a human caller

; requiring access to te BBS software.  This is subject to BBS access time

; limitations.

;

; Force_NoBBS Sorry.. Mail only node.; BBS unavailable text

;

; A short text, given to the remote caller, if BBS access is denied.

;

Wait_Mailer 14               ; Time, in seconds, to recognize mailer.

;

; The time, in seconds, during which T-Mail will attempt to identify if the

; remote caller is a mailer of a human caller.  The default is 13 seconds

; and upon expiration will cause an automatic exit to the BBS software if

; time constraints allow.

;

; Min_Baud 1200              ; Minimum speed for incoming calls

;

; Establishes the minimum speed for remote connection.  Calls slower than

; this speed will be rejected.

;

; Process FTS-1 gsz.exe port *P speed *B z pr1 rc *F

;

; Defines an external protocol (X-Modem) for the receipt of mail according

; to FTS-0001.  If not defined, an FTS-0001 session cannot be initiated and

; is thus not supported.  Received files are placed in the directory defined

; by the variable Files.

;

; Macros may be used on this command line as substitution variables defined

; below:

;

; *B - connection speed (DCE baudrate);

; *L - locked port speed (or 0);

; *P - COM-port number;

; *T - task number;

; *O - (only for OS/2 versions) - handle of open COM port;

; *F - name of inbound file (generated by T-Mail automatically).

;

;- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

;

; External mailers are initiated by keywords exchanged during EMSI

; handshaking.  Up to 16 external mailers may be defined:

;

; Syntax:

;

; ExternMail <keyword> <command> [<parameters>]

;

; Example:

; ~~~~~~~

; ExternMail dobb dobbs.bat *P *B *L

; ExternMail uuc uupc.bat *B *P

;

; Macros may be used on this command line as substitution variables defined

; below:

;

; *B - connection speed (DCE baudrate);

; *L - locked port speed (or 0);

; *P - COM-port number;

; *T - task number;

; *O - (only for OS/2 versions) - handle of open COM port;

; *N - number of the description of the Extermail as it appears in T-MAIL.CTL

;

; ================================ EMSI Control ===============================

;

;                         EMSI handshaking parameters

;

;-----------------------------------------------------------------------------

;

; You may define a limited sub-set of systems with which your system will

; communicate.

;

EMSI_Nodes All

;

; Defines the systems with which this system will communicate.  For a

; description of the formation of a list of addresses, see Chapter 2.

;

; Example:

; ~~~~~~~

;

;        EMSI_Nodes 5030/*.* !CM !5030/101

;        EMSI_Nodes @c:\t-mail\in_list.txt

;

Flags MO, MNP                ; EMSI flags

;

EMSI_OH 3:00-8:00            ; Load answer times to EMSI field

;

EMSI_FR 3:00-6:30            ; Load File request time to EMSI field

;

; Flags defining this system's capabilities

;

AKA_Mode Yes                 ; Allow mail and files to AKA addresses.

;

: Set to Yes, this will ensure that any mail and files for ALL listed

; AKA addresses will be exchanged along with mail and files for the

; primary address.  If set to No, only mail and files for the primary

; address will be exchanged.

;

; Ignore_NRQ Yes             ; Ignore remote's NRQ (No_Requests) flag

;

; Set to Yes, will ignore the remote system's "No File Requests" status

; and transmit File Requests anyway.  Set to No, will honor the remote

; system's denial of File Requests.

; ========================== File request control =============================

;

;                     Making and handling File Requests

;

;-----------------------------------------------------------------------------

;

Freq_Time 8:00-3:00,5:00-6:00; Freq times

;

; The definition of the time period(s) when File requests ARE allowed to

; your system.

;

; Freq_Password Fpassw       ; Password for Freqs

;

; The password which is required, when sending a service message to

; perform a File Request.

;

Freq_Dir dir.frq             ; List of directories from which files may be

;                              requested

;

; The name of a file containing a list of directories (full pathnames) from

; which files may be requested.  As many directories as required may be

; specified.

;

Freq_Ndx index.frq           ; Alias list

;

; The name of a file containing a list of alias names and the files to which

; they refer.  As many aliases as required may be specified.

;

Freq_Number 3                ; Max # Of files which may be requested

;

; The maximum number of files which may be requested, by any system,

; at any one time.

;

Freq_Size 512                ; Max. Total size of files (kBytes)

;

; The maximum number of bytes which may be requested, by any system,

; at any one time.   Default is 2048Kb

;

Freq_Day_Lim 2048            ; Daily Freq limit, kB

Freq_Week_Lim 4096           ; Weekly Freq limit, kB

Freq_Month_Lim 8192          ; Montly Freq limit, kB

;

; File Request limits  on a daily, weekly and monthly basis.

; Default is 64Mb

;

Freq_MinBaud 1200            ; Min baudrate for Freqs

;

; Minimum speed at which File Requests will be honored.  Default 300bps.

;

Freq_MaxTime 120             ; Time limit for freqs (min)

;

; The maximum time which a File Request may take.  This time is

; calculated, based upon the fact that the caller's modem does

; not use data compression.  Default value is 120 minutes and if

; the File Request exceeds that estimated time, only those files

; which can be transmitted within the allotted time are sent.

;

Freq_Nodes All               ; Who May request files

;

; Defines the list of systems which are permitted to make File Requests

; to this system.  The list of systems is defined according to the

; rules for creating an <address list> in Chapter 2.

;

Freq_Nodes @c:\t-mail\fr_list.txt

;

; If this variable is not defined, File Requests are not supported

;

; Freq_Transit YES            ; Allow In-Transit File requests

;

; Defines if this system will allow File Requests to be to remote

; systems, VIA this system.  Default is No, disabling the feature.

;

; Freq_Hold YES               ; Allow to pend holded File requests

;

; If this variable is defined as YES, the session is built so:

;        For caller: transmit-receive-transmit

;        For remote: receive-transmit-receive

; If this variable NO (by default), Only the first two phases

; are performed.

;

Freq_Policy policy.frq       ; File to send on Freq failure

;

; Whenever a File Request is refused, T-Mail will send a message to the

; remote system giving the reasons for the failure.  This file will be

; attached to that message.

;

; Freq_Confirm No            ; Disable File Request confirmation

;

; Allows T-Mail to send a confirmation of a File Request upon

; completion.  Set to No = No confirmation sent.

;

; =============================== Fax definitions =============================

;

;        Variables defining the operations of T-Mail with a Fax mode.

.

;                     Available only in commercial version.

;

;-----------------------------------------------------------------------------

;

; Fax_Class 2                ; Fax Class operations

;

; This variable defines the type of fax modem present.  Permitted values

; are 1 or 2, indicating Fax class 1 or Fax class 2.  In the current

; implementation, only class 2 is supported.

;

; Fax_Baud 9600              ; Max fax data rate

;

; Defines the maximum speed for Fax transfers.  Ignored for Fax class 2

;

; Fax_Mode Any               ; Access level to fax-gate

;

; Defines the list of systems who may access the Fax gate.

;

; Possible values:

;

;                Any - Available to anyone

;                Reg - Available only to systems defined by the variable

;                      Fax_Nodes

;                Nod - Available to all systems defined in the nodelist.

;                Psw - Available to all systems supplying the correct

;                      password, which is sent in the Subject field

;                      The password is defined by the variable Fax_password

;

; Fax_Nodes           5030/100.*

;

; Defines the list of addresses which are permitted access to the

; Fax-Gate.  The <address list> format is defined in Chapter 2.

;

; Examples:

; ~~~~~~~~

; Fax_Nodes 5030/*.* !CM !5030/101

; Fax_Nodes @c:\t-mail\f_list.txt

;

; Fax_Password Fax_it!       ; Fax-gate password

;

; Password for access to the Fax-gate.  This password must be supplied in

; the Subject field of the inbound message.

;

; Fax_Call 17:38-8:00        ; Fax open hours

;

; Defines the time at which T-Mail will call out to deliver pending

; Faxes.  Normal mail activity will be interspersed with fax activity

; during fax events.  By default, outbound calls to fax machines are

; not made.

;

; Fax_Tries 25               ; Attempts to send fax message

;

; Defines the number of times to attempt to deliver a fax message.

; Should this number of attempts be made, unsuccessfully, to deliver

; a fax, the System Operator will be informed.

;

; Fax_Answer 18:00-8:00      ; Allow incoming faxes

;

; Defines the times when T-Mail will allow incoming fax calls.  If

; the variable Fax_Adaptive is set to No, T-Mail will only answer

; to fax calls during these hours.

;

; Fax_Inbound d:\faxes       ; Path for received faxes

;

; Defines the directory, where fax-messages will be stored.

;

; Fax_Adaptive Yes           ; Allow adaptive fax/data answer

;

; Defines whether your fax-modem supports adaptive answer, which

; makes a difference for Class 1 fax.  For Class 2 fax, adaptive answer

; is supported but may be inhibited.

;

; Fax_Cover cover.gr3        ; filename of fax cover page

;

; The name of the file which will be sent before each fax message

;

; ============================== Voice definitions ============================

;

;              Parameters for voice Answer-back with ZyXEL modems

;                             Commercial version only.

;

;-----------------------------------------------------------------------------

;

; Voice_Answer 8:00-1:00     ; Allow voice answers

;

; Defines the hours during which voice answer-back will be enabled.

;

; Voice_Password 12345       ; Remote playback password

;

; Defines the password, numeric DTMF tones, which may be used to interrupt

; the pre-recorded message and causes the playback of files from the

; voice_inbound to the caller.  The hash character "#" may be used

; to reset the password if an error occurs.

;

; DTMF tones for the numeric digits 1, 2 and 3 are used for playback control

;

; 1 - next message

; 2 - remove current message, playback next message

; 3 - terminate voice answer-back

;

; Voice_Inbound c:\t-mail\files\voice    ; Voice inbound

;

; The directory in which voice messages are stored

;

; The name of a message file is in the format: ddmmhhnn.zyx

;

; where dd - day of month,

;            mm - month

;            hh - hour

;            nn - minute.

;

; e.g. File 04021355.Zyx was recorded at 13:55 on february 4th.

;

; Voice_Greeting greet.zyx   ; Greeting message

;

; Defines the files, containing a voice message, which will be

; played to a voice caller.

;

; Voice_Max_Time 70          ; Limit (sec) of voice message

;

; Defines the maximum length, in seconds, of a recorded message.

;

; Voice_Silence_Time 4       ; Silence interval (sec)

;

; The maximum length of silence, after which it is assumed that the

; incoming voice message is complete.  If the call did not begin with

; a period of silence, it started with tones, then T-Mail will attempt

; to answer as a modem or fax.  Default interval is 7 seconds.

;

; Voice_Silence_Level 14     ; Silence level, dB

;

; Defines the volume level, below which it is considered silent.

; Default is 16dB.

;

; ============================== Port definitions =============================

;

;                  Parameters defining communication port(s)

;

;-----------------------------------------------------------------------------

;

Port 2                       ; Serial port no.

;

; Defines the number of the COM-port to be used for modem communication.

; A value of 0 may be used when defining a Master system which will

; perform mail packing only.

;

; Xport 2E8                  ; Define the serial port address.  Only needed

;                            ; for non-standard addresses.

;

HardWare_Flow Yes            ; RTS/CTS flow control

;

; Defines whether hardware or software flow control is used.  Hardware

; flow control is handled by RTS/CTS handshaking and the default is Yes.

;

Baud 2400                    ; Baud rate

;

; Maximum speed of the modem

;

; Lock_Baud 9600             ; Locked port speed

;

; Defines the speed at which a communications port is "Locked" by

; a FOSSIL driver.  See the explanation of the FOSSIL driver in

; Chapter 4.

;

; ============================ End of t-mail.ctl ==============================

*  *  *

Modem configuration parameters (modem.ctl)

������������������������������������������

The configuration commands for the modem are defined below and are

contained as a part of T-MAIL.CTL but may also be kept in a separate file

and included with the 'Include' command.

; === T-MAIL 2400 =============== MODEM PROFILE ===============================

;

;

; ------------------------------- Command strings ----------------------------

;

Init1 AT &D2 &C1 X4 S7=255 | ; Modem initialization strings

; Init2 ~~AT E0 M3 |

;

; Modem initialization string(s), sent to the modem when T-Mail is

; started and at the start of each even and queue re-scan

;

; Init_Terminal ATE1 M1 |    ; Terminal mode initialization string

;

; Modem initialization string, sent to the modem, when entering Terminal

; emulation mode

;

Modem_OK AT |                ; Modem "attention" string

;

; String sent to the modem to grab it's "attention".  The modem should

; respond with "OK" if correctly initialized

;

Dial_Prefix ZYX  AT&N35DT    ; ZYX

Dial_Prefix V42  AT\N5DT     ; V42

Dial_Prefix HST  ATL5DT      ; PVT

Dial_Prefix CM   ATL0DT      ; CM

Dial_Prefix FAX  ATK1DT      ; FAX

Dial_Prefix      ATDT        ; Default

;

;  Flag implication is:-

;

;  V32B implies V32

;  V42  implies MNP

;  V42B implies V42

;  ZYX  implies V32B and V42B

;  H14  implies HST

;  H16  implies HST

;  H19  implies HST

;  H21  implies HST

;

; The string, to be sent to the modem, before dialing a telephone number.

; This would normally be ATDP for a pulse dial exchange or ATDT for a tone-

; dial exchange.  The first string matched is used as the dial prefix.

;

; Examples, for a Zyxel modem, for various nodelist flags:

;

;

Dial_Suffix |                ; Dialing suffix

;

; String to be sent following the called number

;

Onhook V~~^~ATH0 |           ; Modem on-hook

;

; String to send to put the modem "on hook"

;

Offhook ~ATH1 |              ; Modem Off-hook

;

; String to send to put take modem "off hook"

;

Modem_Answer  ~~ATA |        ; Force answer string

;

; String to send to force te modem to answer an incoming call

;

; Statistics ATI2 |          ; Statistics command

;

; String to be sent to the modem to request session statistics.  The

; resulting data is written to the log file

; ---------------------------------- Responses -------------------------------

;

Busy     BUSY                ; Busy string

No_Dial  NO DIALTONE         ; No dialtone string

Ring     RING                ; Ring string

Ringing  RINGING             ; Ringing string

;

; Strings, sent by the modem, according to conditions or events.  e.g. when

; an outgoing call is made and the line is "engaged", the response defined

; by the "busy" variable will be expected.

;

; Valid  PROTOCOL: Prot:     ; Valid connection response

; Valid  COMPRESSION: Comp   ; Valid connection response

; Valid  CARRIER             ; Valid connection response

;

; The Valid variable is used to define replacement strings in modem

; responses. The first parameter is the string to be search for and the

; second, the string used to replace it.  If no replacement string is

; given, only the argument is removed.  The response created by this means

; must not exceed 60 bytes and up to 16 response/replacement pairs may be

; defined.

;

AttemptOn NO CARRIER NO ANSWER; Increments call attempt counter

;

; If these responses are received from the modem, a failed attempt is

; recorded and the number of call attempts is incremented.

;

; AnswerOn RING NO DIALTONE

;

; Defines the string(s) which, when received from the modem, will cause

; T-Mail to instruct the modem to answer the incoming call.

;

; External CONNECT FAX c:\fax\recfax.bat

;

; Defines the external program response handler(s).  These are programs

; or batch files which are executed when a specific response is received

; from the modem.  Up to 16 such responses may be defined.

;

Connect_300   CONNECT        ;   300 bps connect string

Connect_1200  CONNECT 1200   ;  1200 bps connect string

Connect_2400  CONNECT 2400   ;  2400 bps connect string

Connect_4800  CONNECT 4800   ;  4800 bps connect string

Connect_7200  CONNECT 7200   ;  7200 bps connect string

Connect_9600  CONNECT 9600   ;  9600 bps connect string

Connect_12000 CONNECT 12000  ; 12000 bps connect string

Connect_14400 CONNECT 14400  ; 14400 bps connect string

Connect_16800 CONNECT 16800  ; 16800 bps connect string

Connect_19200 CONNECT 19200  ; 19200 bps connect string

Connect_21600 CONNECT 21600  ; 21600 bps connect string

Connect_24000 CONNECT 24000  ; 24000 bps connect string

Connect_26400 CONNECT 26400  ; 26400 bps connect string

Connect_28800 CONNECT 28800  ; 28800 bps connect string

Connect_64000 CONNECT 64000  ; 64000 bps connect string

;

; Define the response codes indicating a valid connection has been

; established.  The connection speed indicated here is the modem to

; modem (DCE-DCE) speed and not the computer to modem (DTE-DCE) speed.

;

; ============================== End of modem.ctl ==============================

*  *  *

Events and Mail Routing (events.ctl)

������������������������������������

This section describes the file EVENTS.CTL, which contains information

on the activity of the system, when it is to occur and what the behavior of

the system is to be at any particular time.

The general syntax, of commands in this file, is:

[[<task_ident>]] [Critical] <keyword> <parameters> <time period>

The rules for formulation of the task identifier <task number> has been

described in Chapter 7.

[Critical] - Prefix which describes the status of the defined event

as Compulsory.  Any event flagged as Critical, will

be executed even if the time for that event has expired

or been overrun by another event.

The table below is a list of keywords and parameters used to describe

events which may be used.  The parameters are followed by the time during

which the event should be active.  If the time is specified as a simple

hour, then the event is at most 60 minutes long starting at the specified

hour.  If a non-Critical event is overrun, by BBS activity or some other

reason, the event will be ignored.

Events are run, only until completion, during the time in which they

are active and only once unless EVENTS.CTL has been modified.  An event may

be described as "periodic" in order to run more than once, as described

later in this chapter.

--------------------------------------------------------------------

Key words and parameters              Definition

--------------------------------------------------------------------

Break [<addr list>] <time>     Break any session running at the

defined <time> for any of the defined

systems.

Poll [flag] <addr Group>       Create an empty message for the

defined system(s) and place them

in the mail queue.

Send [flag] <filename> <addr group>

Send the specified file to all

system(s) defined in <addr group>

Post [flag] <filename> <addr group>

Send a message, contents in the file

defined by <filename>, to the

system(s) defined in <addr group>

Freq [flag] <filename> <addr group>

Request a file from the system(s)

defined in <addr group>

UpdReq [flag] <filename> <addr group>

Request a file update, for the file

defined by <filename> from the

system(s) defined by <addr group>.

Note: [flag] may be, Imm, Crash, Hold

or Dir.  POLL, Freq and UpdReq

are always flagged as Direct.

Exit <errorlevel>              Exit with the specified <errorlevel>

* Pack <addr group>              Repack mail for system defined by

(or Repack)                    <addr group>)

* Drop <addr group>              Remove all service messages for the

system(s) and repack mail

Set <parameter=value>          Substitute a <TMAIL.CTL> parameter

value.

Exec <command | dvpname>       Completion of external program or

start of task in DesqView.

--------------------------------------------------------------------

Routing verbs and rules:

* Mail <addr list> <address>     Routing of mail

* Files <addr list> <address>    Routing of Files

Hold [(NN)] <addr list>        Do not call to deliver

Freeze <addr list>             Never call

Priority <address list>        Establishes high priority systems

NoReq <address list>           Prohibit file requests

Boss <address list>            Forces T-Mail POLL a system if

the session was interrupted for

any reason

* Direct <address list>          Override any routing for systems

in <address list> and send

directly

* Deny <address list>            Inhibits the formation of mail

to system(s) in <address list>

MailOnly <address list>        Inhibits the transfer of Files

to system(s) in <address list>

SendOnly <address list>        Inhibits the receipt of files

from systems in <address list>

--------------------------------------------------------------------

* - for multi-line systems, these key words render the action only

on the node nominated as Master.  In BinkMode these key words

are ignored, with the exception of Deny (see chapter 17).

Detailed descriptions of the EVENTS.CTL verbs:-

Break

===== Syntax:

[task ident] Break <addr Group> <time>

examples:

~~~~~~~~

Break 5030/*.* !/124 5:30      ; Break any local connection,

; except to 5030/124 if still

; active at 05:30 (ZMH)

Break 9:00                     ; I need the machine at 09:00

Allows you to define when the system should NOT be running a

session and that, if a session is running, it should be broken.

Note: Care should eb taken using nodelist flags is the definition of

'break' as they require nodelist lookup.

Poll

==== Syntax:

[task ident] Poll <addr Group> <time period | time>

examples:

~~~~~~~~

Poll 5020/35 1:00-6:00         ; Every day at midnight

[1] Poll /2 0.21:00            ; Sunday at 2100 hrs

A Poll is a deliberate call placed to a remote system in order to

transmit and/or receive mail/Files.

This verb forces T-Mail to generate an empty message for the

appropriate addresses with the DIRECT attribute.  If a particular

<time> is given, and T-Mail is busy at this time, the poll will be

activated as soon as T-Mail is started again.  If a <time period>

is given, the poll will take place within that session.

Send

==== Syntax:

[task ident] Send <filename> <address group> <time period>

example:

~~~~~~~

[1] Send c:\files\info.zip /2 /4 /5.6 5070/35 1:00

This verb forces T-Mail to generate a message with the File

Attach attribute for the defined addresses and thus to send the

file, defined by <filename>, to the specified system.

Post

==== Syntax:

[task ident] Post <filename> <address group> <time period>

example:

~~~~~~~

Post c:\files\info.txt /2 /5.6 5070/35 1:00 ; Every day at

midnight

This verb forces T-Mail to generate a message for the

appropriate addresses.  The contents of the message body is

defined by the contents of the file <filename>

Freq and

UpdReq

====== Syntax:

[task ident] Freq <filename> <address group> <time period>

[task ident] UpdReq <filename> <address group> <time period>

examples:

~~~~~~~~

[M] Freq files /2 /4 /5.6 5070/35 3.1:00    ; once on week

UpdReq d:\files\kronvall.zip /2 3:00        ; Daily if the

file was updated

This verb forces T-Mail to generate a File or Update request

to the appropriate addresses.  A file request will be satisfied,

and a file received, if the file exists on the remote system.

An update request will only be satisfied if a NEWER file, than

exist on the local system, exists on the remote system.

Exit

==== Syntax:

[task ident] Exit <errorlevel> <time>

examples:

~~~~~~~~

[1] Exit 99 1:00             ; Daily at midnight

Exit 92 1.2:00           ; Monday at 0200 hrs

Forces T-Mail to exit the current process with an Errorlevel

of <errorlevel>.  If T-Mail is currently busy with, for example

a lengthy session, exit will take place as soon as the session

is complete but not later than 60 minutes after the defined

time.

Pack

RePack

====== Syntax:

[task ident] Pack <time period>

[task ident] Pack <address group> <time period>

examples:

~~~~~~~~

Critical Pack 1:00

Pack /2 /4 5020/4 1:00

Forces T-Mail to delete all mail bundles, or all mail bundles

for <address group> systems, and to pack mail anew.

Drop

==== Syntax:

[task ident] Drop <addr group> <time period>

examples:

~~~~~~~~

Critical Drop /2 1:00

Drop 5020/4 1:00

Forces T-Mail to remove all service messages (generated for

Poll, Freq, Post, Send or UpdReq) for the <addr group> system(s).

Upon removal of the service messages, T-Mail will repack mail

for the <addr group> system(s).

Set

=== Syntax:

[task ident] Set <parameter=value> <time period>

Examples:

~~~~~~~~

[1] Set Time_Dial=100 5:00-9:00

[2] Set Freq_Number=10 5:00-7:00

Set Answer=0:00-12:00 5.20:00-0.22:00

Allows parameters (from T-MAIL.CTL) to be changed on-the-fly.

This facility should be used with care and parameters, such

as port, address etc should not be changed.

The old parameter value is NOT retained for later restoration

and if this is required, then T-Mail must be restarted or the

original values restored by another Set command.

Values may be the same as in T-MAIL.CTL as Set acts

independently.  It is not possible, in all cases, to modify

existing parameters.  In the following cases it is only

possible to add to an existing list:  Highlight, Dial, AKA,

Process, Process_Flag, Direct, MailBox, ExternMail.

SET may be used, with a file name, to set a group of variables.

Example:

SET @morning.ctl 4:00-11:00

(morning.ctl)

sound         1FF

scan          100

scan_circle   300

(t-mail.ctl)

key <Alt-E>    Set  @morning.ctl

Exec

==== Syntax:

[task ident] Exec <[flag]command|desqview program> <time period>

examples:

~~~~~~~~

[M] Exec morning.bat 5:00                 ; Daily

[M] Exec weekly.bat 0.0:10                ; Weekly

Exec mem/c >>t-mail.log 8:30-11:00;

[1] Critical Exec c:\dv\ec-pif.dvp 1.9:00

Forces T-Mail to EXECute the command line given or start

the specified DESQview task.  For executable file (.EXE and .COM)

the command processor is NOT loaded but memory IS swapped out

out to disk.

* note: it is possible to use the following flags-modifiers

on the command line:

'!' - Do NOT swap out memory

'%' - Do NOT preserve screen contents

example:

~~~~~~~

Exec !Weekly.bat 0:10   ; Weekly batch file with NO swap

Mail

==== Syntax:

[task ident] Mail <addr list> <dest_address> [| <alternate dest list>]

[time interval]

examples:

~~~~~~~~

Mail 5020/*.* 5020/28 | 5020/*.*

Mail 1:*/*.* 5030/124 | 1:*/*.* 50/0 5020/140

Mail 5010/*.* 5030/124 | 5010/*.* !~V32

Mail 5020/* !5020/44.2 5040/* 5030/2 ; 0000 - 2359 everyday

Mail 5030/1 5030/6 4:00-9:00         ; 0400 - 0900 everyday

Mail 5030/1 5030/6 3.4:00-6.9:00     ; Wed 0400 - Sat 0900

Files 5030/* 5030/6 6.4:00-6.23:59   ; Sun 0400 - Sun 1259

Mail All 5030/2                      ; 0000 - 2359 everyday

Mail @nodes1.Lst      5030/6         ; addresses from list

Files 50*/*.*         5030/2         ; All region 50 via 5030/2

Mail 5030/* 5020/* Hub               ; Hub routing

Purpose: to establish the rules for routing mail or files.

If mail is not flagged as Direct, T-Mail will examine the routing

rules to determine to which system a mail packet should be sent and

when. T-MAIL wil then add mail to or create mail bundles for those

systems.

Messages with the File Attach attribute are handled according to

the 'Files' keywords, mail is handled according to the 'Mail'

keyword.

Please note that routing rules are examined in the order in which

they appear in EVENTS.CTL until a rules is located which will handle

the current message.  More general rules should therefore be

specified later.

If an alternate address list is specified and a system calls in

presenting one of the alternate addresses, netmail for that node will

be delivered directly in the same session.

Default routing for 4-D addresses, in the event of not locating

a suitable routing rule:-

- If point of current node (i.e. Zone/net and Node of this

system), mail is sent direct.

- Mail addressed to "another's point" (Zone, net and node

are not those of this system) will be dispatched to that

point's Boss node.  i.e. if our address is 5030/15.0,

mail for 5030/6.602 will be sent via 5030/6.0

the <address> field may also use the Hub qualifier:

Mail 5020/* Hub

In this case the mail will be sent to the appropriate Hub node.

If a hub node has not been defined for a particular node, the

mail will be routed to the net Host.  'Host' and 'Hub' are nodelist

definitions used to define default routing methods under FidoNet

technology.  Please see the relevant Fidonet technical specifications

for further details.

Direct

======

syntax: [task ident] Direct <address list> [<time period>]

examples:

~~~~~~~~

Direct 5030/* !5030/2! CM 3:00-5:00

Direct 5030/6.*

Direct @nodes.lst 4:00-6:00

Specifies the routing of messages and files and indicates that

mail will be sent directly (not via any other systems) to the

specified systems.

Mail and files for points are routed, by default, through their

Boss nodes, for example:

Direct 5030/6.*

Mail and files for 5060/6.601 will be bundled with mail for node

5030/6.0  Mail can also be addressed directly to a specific point

system by supplying the full 4-D address of the point system  e.g.

Direct 5030/6.601

This is only effective is T-Mail 'knows' the telephone number of

the point system defined, if not then 'near' direct routing is used

and the mail will be routed to the boss node.

Hold and

Freeze

====== syntax: [task ident] Hold [(NN)] <address list> [<time period>]

[task ident] Freeze <address list> [<time period>]

examples:

~~~~~~~~

Hold 5030/* 3:00-5:00

Freeze 5030/6.*

Hold @nodes.lst 4:00-6:00

Restrictions may be defined using a time period argument

The difference between Hold and Freeze is that mail addressed to

a node defined as "Hold" will only be delivered when that node calls

in to pickup mail or if the Hold status is changed to Immediate,

either by operator keyboard request or a message being packed with

Imm status.  Mail will NEVER be delivered to a node which is under

a Freeze status.

When the Hold mode is given a threshold may be specified for

the system(s), defining the amount of mail (NetMail + FileAttached +

default_MailBox), below which the Hold attribute will be valid.  If

the defined amount of mail is exceeded, the Hold attribute will no

longer apply and mail will be delivered according to normal

schedules.  Default value is 0, meaning no limit.

NoReq

=====  syntax: [task ident] NoReq <address list> [<time period>]

examples:

~~~~~~~~

NoReq 5030/* 3:00-5:00

NoReq 5030/6.*

NoReq @nodes.lst 4:00-6:00

Defines that File requests from the defined system(s) are not

allowed during the specified time period.

Deny

====   syntax: [task ident] Deny <address list> [<time period>]

examples:

~~~~~~~~

Deny 5030/* 3:00-5:00

Deny 5030/6.* 22:00-24:00

Deny @nodes.lst 4:00-6:00

Establishes a time period during which mail bundles will NOT be

created for the listed systems.  This can be useful for easing the

workload of a hub system.  In BinkMode, however, mail and files will

be sent normally.

Priority

========

syntax: [task ident] Priority <address list> [<time period>]

examples:

~~~~~~~~

Priority 5030/1 5030/6 3:00-4:00

Priority @r50.Net 7:00-9:00

Defines that certain systems will be given priority over others

during the selected time period.  When mail queues are scanned for

outbound mail, the priority queue is always scanned first.  When all

priority mail has been dispatched, normal mail will be sent.  This

command may also be given from the Console keyboard.

Note: Care should be taken with this command as priority systems are

ALWAYS serviced before normal systems and if, for some reason,

it is not possible to deliver mail to a system which is marked

as priority, other systems would not be called.  Mail is

delivered normally to systems calling in.

Boss

====   syntax: [task ident] Boss <address list> [<time period>]

examples:

~~~~~~~~

Boss 5030/* 0.3:00-6.5:00

Boss 5030/6.* 22:00-24:00

Boss @nodes.lst 4:00-6:00

Defines how a remote system is to be treated in the event of a

session failure, line drop etc while receiving files.  If the remote

system is defined as a "Boss" system and a session failure occurs, a

poll packet will be generated and the remote system called.  This can

be useful to ensure mail pickup if the mail is on hold at the remote

site.

MailOnly

========

syntax: [task ident] MailOnly <address list> [<time period>]

examples:

~~~~~~~~

MailOnly 5030/*.* 5:00-7:00

MailOnly All! ~CM 5:00-7:00

MailOnly All !/2 !/86 !/100 13:00-15:00

MailOnly @nodes.lst         4:00-6:00

Defines systems to which only mail will be sent during the

specified time period.  Any requests to transmit files, apart from

packed mail, will be ignored.  T-Mail informs the remote system, by

means of the EMSI handshake HXT.

SendOnly

========

syntax: [task ident] SendOnly <address list> [<time period>]

examples:

~~~~~~~~

SendOnly ~Listed 09:00-22:00

SendOnly All !5030/100 13:00-19:00

SendOnly .1

Defines actions to be taken when a system calls in.  If the

calling system is listed, the EMSI flag HAT (Hold All Traffic)

is sent.  If the remote system is capable of processing the EMSI

flag (FSC-0056), then no mail will be sent from the remote system.

Note: For Fidonet systems, it is not allowed to use this verb

during ZMH (Zone Mail Hour).

Cyclic events

�������������

It is sometimes required to establish that an action occurs on a

regular basis, e.g. analysis of system performance, modem initialization,

log-file processing, etc.  To this end it is possible to define a "Periodic"

event, of the form:-

P.01:00

The symbol "P" in the place of the day of the week, signifies a

periodic event.  In this case the given time defines time interval between

cycles.  In the above example, every hour and the periodic events are

effective immediately.

The duration of a periodic event may be from 00:01 to 32000:00, but are

applicable, in the EVENTS.CTL file only for the verbs Poll, Send, Post, Freq,

UpdFreq, Drop, Set, Exec, Exit and Pack.  The Poll and Drop verbs are only

executed by the Master system in a multi-tasking environment.

[2] Exec !Load.bat P.1:00      ; Every hour

The above example will be executed, by task #2, as soon as it is

initiated, then again at 14:15, then at 15:15 and so on every 60 minutes.

Should T-Mail be unable to execute the event because it was busy doing other

things, it will be executed as soon as T-Mail becomes free, however missed

events will not be stacked more than one level deep.

example of EVENTS.CTL:

~~~~~~~~~~~~~~~~~~~~~

; ========== T-Mail EVENTS list for 2:5030/15 ==========

;

;

Exit 99 08:30

Exec daily.bat 06:15

Exec Weekly.bat 0.00:10

;

RePack 06:01

Poll 5030/2 18:10

Poll 5030/6 17:00-20:00

Poll 5020/35 04:00

Boss 5030/* !5030/2 !5030/29 00:00-23:59

;

Hold All 11:00-23:00

MailOnly All !/100 05:00-7:00

Direct 5030/6.*

Mail 5030/7 5030/6

Mail *:*/*.* 5030/2

Files *:*/*.* 5030/2

;

; Examples of "complex" syntax

;

; Mail 5040/* !5040/3.3 5030/* !/*.4 5030/7 1.00:00-5.23:59

; Mail @nodelist\r50.Net 5030/3

; Direct 5030/*.* !/2 !/3 !/5

; Boss * !/2 !/4.1 03:00-04:00

; [1] Hold V32 ZYX 0:00-6:00

; define ZMH 2:20-3:30

; define ex_favor !/100 !/6 !/29

; [2] Hold CM ex_favor ZMH

;

; ======================= End of file events.ctl ====================

*  *  *

Substitution list (SUBST.LST)

�����������������������������

Description of the substitution list verbs and parameters which are used

to signify changes for particular systems or groups of systems.

syntax: [[<task_ident>]] <address> <parameters> or

[[<task_ident>]] <address group> <parameters>

Where:

<task_ident>    -  Task number as detailed in chapter 7.

<address>       -  Address of system as detailed in chapter 2.

<address group> -  Group of addresses as detailed in chapter 2.

<parameters>    -  Zero or more parameters, followed by a parameter

values or tags.  Delimiter is a blank (space)

character.

Parameter

name         Example parameters/tags

----------------------------------------------------------------------

Time         CM

22:00-23:59          ; During a specified time

Speed        1200                 ; The speed defines, at which

2400                 ; speed calls will be made

Phone        123-4567 8W123-22222 ; Substitute one phone number for

; another from the nodelist.  The

; substituted phone number is not

; translated.

PreDial      External load.bat    ; Defines the program, which will

; be executed before dialing the

; appropriate address.

Dial         Prefix AT\N1DP       ; Used as the Dial_Prefix for this

; node

BOX          c:\mybox\            ; Defines the directory to be used

; as a FileBox for this address

Wait         70                   ; Overrides the Time_Dial parameter

; for this system.

Hidden       123-4568             ; Telephone number for alternative

; line of defined system, see HDIAL

HTime        00:00-23:00          ; The Operating time of alternative

; line of defined system.

Hdial        Prefix AT\N1DP       ; Dial prefix to be used on the

; alternative line.

FTime                             ; Additional hold time of file

; requests for this system.

Password                          ; Session password.

Set          freq_size=2048       ; Allows you to override the

; definitions from T-MAIL.CTL

External     copy * F c:\hole     ; Defines an external command to be

; executed.  This function is only

; available in the commercial version

; and must be the last in the node's

; description

AKA          2:5030/500.5         ; Present this address as our primary

; address instead of that defined in

; T-MAIL.CTL

HideAKA      2:5030/500.6         ; "Hide" these addresses

SHORTEMSI                         ; Use small EMSI_DAT fields, IDENT

; will be blank

ZMO -                             ; Inhibit ZedZap and Direct Zap

ZAP -                             ; Inhibit Direct Zap

CM V32 V42                        ; Defines additional flags for

V32B ZYX                          ; the appropriate systems

V42B H16

H14 HST

PVT ISDN

At the beginning of the line will be an <address> or <address group>.

For an <address group>, you may not use BOX, PHONE, HIDDEN, HTIME, HDIAL or

PASSWORD.  If an address is given, even though it may not exist in a

nodelist, it will qualify under the category "Listed" (See chapter 2).

Substitutions for <address group> should always be made BEFORE any

substitutions for <address>.  Lines may be continued by beginning the

continuation line with a hash "#" character.

Use of the term AKA is as follows:-

66:66/*.* Aka 66:66/1.0 Time CM SPEED 2400

When T-Mail calls to any node in zone 66, Net 66 OR is called by any

node presenting zone 66, net 66 as it's primary node number, it will present

66:66/1.0 as it's primary address.

The tag ZAP may be used if it necessary to inhibit the use of the

Direct ZedZap transfer protocol.

The tag ZMO may be used if it necessary to force a Classic Z-Modem-1K

transfer protocol

The command External is described in detail in chapter 27.

The keyword SET may be used to temporarily change the value of T-

MAIL.CFG parameters.  The changes are made at the start of a session and are

restored to their original state when the session ends.

The keyword Hdial defines another dial prefix for an additional

(hidden) number.  If this is not given, the prefix, defined by the keyword

Dial is used and if that is not given, the prefix defined in modem.ctl is

used.

If the dial prefix is terminated by a pipe character (|), it is used,

not as a dial prefix, but as the entire dial string.  Blanks may not be

included in the dial string.

Example (subst.lst):

5030/999 dial at&n14dp123-4567|

The Predial field describes the program, or batch file, which will be

executed before making outgoing calls to the defined system(s).  It may be

used for loading a softmodem profile.  The Predial function must be the last

field on the line in SUBST.LST.  Parameter substitution may be made using one

or more of the following macros:

*A - address of system to which T-Mail will make calls;

*B - connection speed (DCE baudrate);

*L - locked port baudrate (or 0);

*P - COM port;

*T - task number;

*O - (only for OS/2 versions) - handle of opened COM port.

The example: (subst.lst)

~~~~~~~~~~~~~~~~~~~

5030/151 time 2:00-8:00 dial ATN&0DP wait 100

# Set freq_time 0:0-24:00

# PreDial !Load.bat COM*P profile.151

5030/100 time CM PreDial 100.Bat *T -t -g -w

5030/100 time CM

# PreDial 100.Bat -t

# Dial ATDP

The last example is incorrect as the Predial field should be last

The PreDial and External functions can not be used on the same command

line as each of them should be last.

The password file (PASSWORD.LST)

��������������������������������

The list of systems with which we have secure sessions, password

protected, may be listed in the PASSWORD.LST or SUBST.LST files.

Each line of the PASSWORD.LST file takes the form:-

<address> <password>

Where:

<address>  - address of a single system as defined earlier in "address

structure"

<password> - password, which will be used when a mail session is

started with this system.

examples:

~~~~~~~~

2:5030/100 PASS1

5030/99 WORK

86.5 SECOND

����ͻ

� 8. The Principles of schedule organization

���������������������������������������������������������������������������

The mail queue concept and implementation is of vital importance for

the correct functioning of the mailer and a large proportion of the work

undertaken by the mailer, is given over to mail queue processing.

Two basic type of mailer exist today:  The SEAdog/Dutchie/FrontDoor

style (also called ArcMailAttach) which creates a queue of messages, to

which are attached mail bundles, and the BinkleyTerm (Bink) style which

creates routing files in OutBound directories.  T-Mail can operate as either

style of mailer.

In the ArcMailAttach style, the basis of queue creation is netmail

(network mail).  A directory contains files of a specific format, called

messages (*.MSG).  The packer (often built-in) will scan through these

messages and according to the routing rules and service attributes (flags)

will create a dispatch queue.  Each item in the dispatch queue concerns

information to be sent to a defined list of systems and may contain:-

.. messages to be sent directly to this system;

.. messages which may be routed through this system;

.. files, which can be sent together with these messages with FileAttach

attribute;

.. file requests on this system.

analysis of schedules and the formation of target calls

�������������������������������������������������������

The fact that information for a particular system may exist in the mail

queue does not signify that the system will be called.  For this to happen,

other criteria must be met as described below:-

Criteria to be met before a system can be called?

.. Does a system exist with priority status?  If so, does the current

system have priority status?  If so, then a call can be made, if not,

continue scan.

.. Are we within the time period (Call_Time) for making outgoing calls?

.. Analysis of possibility to make outgoing calls to current system:

- Is the telephone number of the system known?  If not, no call can

be made.

- Is the current system in "Freeze" mode?  If so, no call can be made.

- Will the system accept our calls?  This is dependent upon:-

Nodelist - flags OH (FSC0020), Txy (FSC-0062);

On subst.lst - given time/htime.

If it will not accept our call at this time, no call is made unless

the immediate flag is present.

- Is the current system in HOLD mode?  If not then process the call,

if yes then has the hold limit been exceeded, if yes then the call

is processed.

- Has a Hold command from the operator keyboard been given?  If not

then proceed with the call.

.. The next stage of analysis is the type of mail to be sent:

- Does the mail currently exist?

- Do files exist in the system' filebox?

- Are file requests present?

If file requests are present then an analysis is made of the handling

of file requests by the target system:

nodelist   - flags FR

subst.lst  - parameter ftime

events.ctl - parameter NoReq

Calls will only be processed within the specified time-frame

.. Following this, a check is made on the number of unsuccessful calls

already made to the target system, if the value exceeds the value

specified by Call_Tries, no further call attempts are made.

.. On multi-line systems, if a busy flag exists for this node, then no call

is made.

Additional conditions which are also taken into consideration:

- If a message exists, for the target system, with the Crash or

Immediate attribute, any Hold status will be ignored.

- If all messages for the target system are on Hold, no call will be

made.

In operation, the methods shown above may not occur in the same

sequence, but all steps will be taken at some time.

The display and operational control of queues. Status messages

��������������������������������������������������������������

The current queue is shown in the bottom window of the main screen as

two columns.  For each item in the address queue (Node), number of

unsuccessful calls made (Try), quantity of mail (Kb or bytes) to go out

(Mail), quantity of files (Kb or bytes) to go out (File), quantity of mail

and file with Hold status (Hold) and the system status (St).

If the queue is too long to fit in the display window, it may be

scrolled by using the <up>, <down>, <PgUp>, <PgDown>, <Home> and <End> keys.

In the commercial version only, the ALT-F key may be used to switch to the

Fax queue.

The operator may change the operational status of node(s) in the queue

which is indicated by the following letters opposite each address:

H - Hold status

I - Immediate status

P - Priority status

Any system status which is set as a result of operator action will only

be valid during the time period defined by the variable CH_interval, this

has a default of 60 minutes.

Hold status     When a system is placed in the Hold status, no calls will be

made to deliver outbound mail or files to that system.  It

is possible to cancel the Hold status only by the

application of a "stronger" status, such as Immediate.

- Establish with key 'H' for the current system in the queue;

- keys CTRL-H or "Backspace" will open a window requesting

a group of addresses.  When the group is given, all

systems defined by the <address group> will be placed in

Hold mode

- Toggled (alternatively on/off) by repeated depression of

the key 'H' for current system in the queue

- key CTRL-U will open a window requesting a group of

addresses.  When the group is given, all systems defined

by the <address group> will have the Hold status

removed

Immediate       A system holding Immediate status will not be restricted

status          by Hold status or time constraints.  The only restriction

which would stop outgoing calls being made to an "Immediate"

system would be if the system were "Frozen".

- Establish with key 'I' for the current system in the queue;

- keys CTRL-I or "Tab" will open a window requesting a

group of addresses.  When the group is given, all systems

defined by the <address group> will be placed in Immediate

mode

- Toggled (alternatively on/off) by repeated depression of

the key 'I' for current system in the queue

- key CTRL-R will open a window requesting a group of

addresses.  When the group is given, all systems defined

by the <address group> will have the Immediate status

removed

Priority        The purpose of the Priority status is identical to the

status          instruction Priority (EVENTS.CTL).  Priority systems will

always be served first in any queue.

- Establish with key 'P' for current node in queue;

- key CTRL-P will open a window requesting a group of

addresses.  When the group is given, all systems defined

by the <address group> will be placed in Priority mode.

- Toggled (alternatively on/off) by repeated depression of

the key 'P' for current system in the queue

- key CTRL-C will open a window requesting a group of

addresses.  When the group is given, all systems defined

by the <address group> will have the Priority status

removed

����ͻ

� 9. Call processing

���������������������������������������������������������������������������

Response to an incoming call, start of session

����������������������������������������������

When T-mail is neither calling out nor engaged in the execution of an

external program, it will be awaiting an incoming call, provided that the

variable Answer (in T-MAIL.CTL) has permitted answering at this time.

An incoming call is recognized either by the text "RING" sent from the

modem (DCE) to the computer (DTE), or the ring indicator (RI) changing

state.  Which signal will be used is defined by the variable Hardware_Ring

in T-MAIL.CTL.  By default, call answering is turned off, but an incoming

call may be answered manually by using the ALT-A key combination.  T-Mail

will immediately take the phone off-hook and proceed to answer the call.

The number of rings which will be ignored before a call is answered may

be set in the variable Answer_Ring in T-MAIL.CTL, this is normally set to 1

and forces the call to be answered as soon as the RING is detected.  While

the phone is "Ringing", the operator can force immediate answer by pressing

the space bar.  It is also possible to reset the ring counter by pressing

the "ESC" key.  If an incoming call does not complete the required number of

rings, following a delay of 6 seconds, T-MAIL returns to waiting for a call.

example:

~~~~~~~

Answer        0:00-9:00       ; Time during which incoming calls will be

; handled.

Answer_Ring 5                 ; NUmber of rings before answering the phone.

Action following call answer

����������������������������

After answering the call, T-Mail will wait for the modems to establish

a connection according to the variable Wait_Carrier.  This delay maybe

modified by the operator, using the "+" or "-" keys, or aborted using the

"ESC" key.  WHen the modems have establish a connection, a response code is

received by ther DTE indicating the connection type and speed (see

MODEM.CTL), T-Mail passes into the connection processing phase.

In the commercial version, it is possible to specify adaptive answering

which will allow a remote fax machine to call in to deliver a facsimile.

If the modem response matches a definition in one of the Extern

variables in MODEM.CTL, all further processing is handled by the external

command.  For example, CONNECT 2400 could start BinkleyTerm, CONNECT 9600

could start FrontDoor and CONNECT FAX could start the receipt of fax

messages.

If no external process is found, T-Mail will:

1. If an exit to the BBS is permitted (variable BBS_Time in T-MAIL.CTL)

the response described by Force_BBS is given.  If an exit to the

BBS is not permitted, the response described by Force_NoBBS is given.

2. Waiting for information from the line.

The response time for information from the line is defined by the

variable Wait_Mailer (in T-MAIL.CTL).  If nothing sensible is received from

the line during this period, T-Mail will execute the actions described in

Process_BBS.  If information is received, the following actions are

possible:

� If a keyword is received, which matches one of the keywords in the

Extern_Mail table, the commands defined by the matching definition

are executed.

� Allow the removal of a poll message to a node, upon subsequent

successful completion of an external program triggered by an

'External_Mail' command.

� If an FTS-0001 session has been defined, described in Process FTS-1

in T-MAIL.CTL, and FTS-0001 handshaking is completed, receipt of

mail, using FTS-0001, will take place.  T-Mail only supports

incoming mail using FTS-0001 protocol.

� If two successive "ESC" characters or an EMSI page are received:

If an exit to the BBS is allowed at this time (BBS_Time), the file

defined by BBS_Banner is sent to the remote system and control is

transferred to the program defined in Process_BBS.

If an exit to the BBS is not permitted at this time (BBS_Time), the

file defined by NoBBS_Banner will be sent to the remote system.

� If an EMSI page is received, EMSI handshaking will be initiated which

will include password checking, connection speed (>= Min_Baud) for

WaZoo mail session (receive with Zmodem), files and mail and file

requests.

External connection handlers

����������������������������

As well as normal modem messages to inform of a connection status, it

is possible to define up to 16 responses which will trigger the execution of

an external program.  These definitions are detailed in MODEM.CTL:

Extern <modem string> <command>

example:

~~~~~~~

Extern CONNECT 2400 do2400.Bat

Extern CONNECT VOICE do_voice.bat

These definitions are examined before any data is received from the

remote system.  Note the command to be executed cannot contain blanks.

FTS-0001 session

����������������

T-Mail has the ability to accept mail using the FTS-0001 protocol on an

incoming call as defined by the variable FTS-1 in T-MAIL.CTL.  T-Mail will

start the external Xmodem protocol for the receipt of files, which are placed

in the directory specified by the variable Files.  No messages or files will

be transmitted using FTS-0001.

External keyword handlers

�������������������������

T-Mail has the ability to accept a so-called "External Mail" command at

the time of EMSI handshake.  If an External mail command is sent from the

remote computer at this and it matches one of the definitions in T-MAIL.CTL,

the response command will be executed.

ExternMail <login word> <response command> [<parameters>]

example:

~~~~~~~

ExternMail UUpc c:\uupc\uu.bat

ExternMail Guest c:\bbs\dobbs.bat 1 2400

On answering the incoming call, if the text UUpc is given as a login

line, the program uu.bat in the directory c:\uupc, will be executed.  Up to

10 external mail commands may be specified.

Macros may be used on this command line as substitution variables defined

below:

*B - connection speed (DCE baudrate);

*L - locked port baudrate (or 0);

*N - serial number of the remote mailer, for details, see T-MAIL.CTL

*O - (only for OS/2 versions) - handle of open COM port;

*P - COM-port number;

*T - task number;

example:

~~~~~~~

ExternMail UUpc c:\uupc\uu.bat *P *B *N

For COM3 and 2400 bps will run c:\uupc\uu.bat 3 2400 1

At the start of an outgoing session, if an "external" trigger text is

received, the relevant commands will be executed.  Upon successful

completion of the external program (Errorlevel = 0), any outstanding service

messages are removed and the system is removed from the processing queue.

It is possible to "fool" the system into making outgoing calls by using bogus

addresses and overriding with a telephone number as shown in the example

below:

example for UUCP system:

~~~~~~~~~~~~~~~~~~~~~~~~

(t-mail.ctl)

ExternMail login uucico.exe -F

(subst.lst)

.999 time CM phone 123-4567

(events.ctl)

Poll .999 5:00

In the example above, at 0500 hrs in the morning, a poll service

message will be generated for address .999  When this system is called and

the text "login" is received, the program "uucico.exe" will be executed.

Upon successful completion of "uucico.exe", the service message (poll) will

be deleted and the system (.999) will be removed from the processing queue.

BBS operation

�������������

T-Mail can be used as a front-end for Bulletin Board System (BBS)

software.  Currently Remote access 1.x and Maximus 2.x have been certified

for use with T-Mail, but problems should not arise with other software.

Human callers are identified by T-Mail by:-

.. 2 consecutive 'esc' characters being given within "Wait_Mailer"

time.

.. receipt of an IEMSI packet

.. receipt of EMSI_CLI packet

.. EMSI timeout occuring with a 'Process EMSI_Timeout' defined in

T-MAIL.CTL to start the BBS.

In the event of the incoming call not being from a compatible mailer,

keywords previously defined in ExternMail not being given or two consecutive

"esc" characters not being given within "Wait_Mailer" time, T-Mail makes the

assumption that a human caller is present.

The variable BBS_Time defines the interval when it is permitted to fire

up the BBS.  For example, if BBS usage is permitted at night on weekdays and

all day at weekends, the following line would be set in T-MAIL.CTL

BBS_Time 1.22:00-5.7:00,5.0:00-6.23:59

The software to handle the BBS functions is then defined, in T-

MAIL.CTL, by the command Process BBS:

Process BBS <command> [<parameters>]

Macros may be used on this command line as substitution variables defined

below:

*B - connection speed (DCE baudrate);

*H - the time (in minutes) until the end of the BBS event.

*L - locked port baudrate (or 0);

*N - serial number of the remote mailer, for details, see T-MAIL.CTL

*O - (only for OS/2 versions) - handle of open COM port;

*P - COM-port number;

*T - task number;

examples:

~~~~~~~~

Process BBS bbs.bat *P *B *H

Process BBS max.exe -p *P -b *B -t *H -n *T

����ͻ

� 10. Session handling

���������������������������������������������������������������������������

Start of session, EMSI handshake, passwords

�������������������������������������������

Before the exchange of mail between FTN systems, it is possible to

execute an EMSI (Electronic Mail Standard Identification), handshaking

sequence. The sequence is triggered by receipt of an EMSI page command and

consists of an exchange of the main characteristics of the systems, such as

password, system address, akas and capability flags etc.

In the case of an incoming call, a check may be made for password

validity.  If a password has not been set for the calling system, a warning

is issued and the session continues.  If a password has been set for the

calling system, it will be verified and if not correct, the session will be

terminated with an error.  A check is also made for correct passwords to AKA

addresses and if an incorrect password is given for any AKA address, mail

for that AKA will not be delivered.

mailer session

��������������

Following a successful EMSI handshake sequence, T-Mail chooses the

strategy for the mail session, generally (without restrictions) the session

will occur thus:-

To the system being called:-

1) Transmit all mail (*.PKT containing messages) for the primary address

of the remote system.

2) Transmit all files from the remote system's filebox.

3) transmit all file requests;

4) transmit files from file requests (.req);

5) repeat items 1-4 for each AKA address of the remote system;

6) transmit any files in additional fileboxes as defined by the keyword

BOX in the substitution list for the primary address of the remote

system

7) receive files.

8) if a file request is received, process and transmit the reply:-

- file(s) requested

- message with information as to why a request was incomplete or

refused

- message acknowledging the request

To the system calling in:

1) receive files from remote system;

2) process received file requests (.req)

3) transmit all mail (.pkt files, containing messages) for the main

address of the remote system;

4) transmit all files from the default filebox,

appropriate to the main address of the remote system;

5) transmit all files attached to messages;

6) transmit all files from the default hold filebox, appropriate

for main address of the remote system;

7) transmit file requests (.req);

8) repeat items 2 - 6 for each additional address of the remote system;

9) transmit the file, defined by the variable BroadCast in t-mail.ctl as

long as such a variable is defined, the file exists and the address of

the remote system is defined in the list of addresses on the Broadcast

command line.

10) transmit the files from additional fileboxes, given in the substitution

list by the keyword BOX for the main address of the remote system;

11) transmit any answers on file requests, received from the remote systems.

This answer consists of:

- found files;

- message giving the reasons for the failure(refusal) of the request;

- message - acknowledging successful request(s).

12) if file requests are received from the remote system, files are sent.

It is useful, at this point, to consider restrictions which may be

placed upon the system by the System Operator.

It is possible, using the variable Freq_Hold (T-MAIL.CTL) to prohibit 3

phases of the session.  Total inhibition (Freq_Hold No), causes item 8 for

the calling system and item 11 for the called system to be ignored.  Replies

to file requests are sent only in the case of the remote system being

called.

The variable AKA_mode, in T-MAIL.CTL, defines whether it is necessary

to send files for additional addresses (AKA_Mode Yes) or only for the

primary address (AKA_Mode No).  In the latter case, additional addresses are

ignored, causing item 5 for the calling system and item 7 for the called

system to be ignored.

Under defined circumstances, file requests to remote systems will not

be honored.  That is, if the remote system cannot support file requests, or

they are prohibited currently from this system.  Restrictions may be

established on particular systems by:-

- the command NoReq in events.ctl;

- setting the time interval for file requests in the field

Ftime in subst.lst.

If the remote system is not capable of supporting file requests, this

information will be supplied in a special field in the EMSI page.  The

system operator can force this field to be ignored by setting Ignore_NRB to

the value "Yes" in T-MAIL.CTL, although this is not sensible thing to do.

In normal operation, T-Mail will hold file requests on such a node.

For a set of predefined addresses, it is possible to establish a "mail-

only" session, which is handled by giving the command "MailOnly" in the

events definition file, EVENTS.CTL.  In this case, the only exchanges

between such systems will be mail bundles (*.pkt files).  This is the

recommended method of handling the so-called "mail hour(s)", i.e. the time

periods when fast mail delivering is guaranteed.  This is defined by the

EMSI flag HXT.

For a set of predefined addresses, it is possible to establish a

"transmit only" session, which is handled by giving the command "SendOnly"

in the events definition file, EVENTS.CTL.  In this case no files will be

accepted from the remote system.  This type of session is only possible when

the remote system is calling in and understands such a session type.  This

is defined by the EMSI flag HAT.

For a set of predefined addresses, it is possible to inhibit mail

sessions completely.  The variable EMSI_Nodes in T-MAIL.CTL defines the list

of valid addresses with which a session may be granted.  It is also possible

to inhibit sessions according to a minimum modem data rate, defined by

Min_Baud in T-MAIL.CTL.

Exit to the BBS at end of mail session

��������������������������������������

During a mail session, T-Mail can be given the command to start up the

BBS at the end of the session.  This request can be handled in various ways:

Message:  If, when unpacking messages, T-Mail sees a message addressed

to "BBS", it will be considered as a BBS request.  A check will be made to

ascertain if BBS operation is permitted at this time and if not permitted,

the "Subject" field will be checked against the variable T-password in T-

MAIL.CTL.  If a match is found, the BBS request is considered satisfied.

Keyboard:  It is also possible to activate a BBS session by the Alt-T

key on the operator console.  At the end of session, on the T-Mail system

which sends the BBS request, the program defined by the variable Terminal

will be activated.  On the system receiving the request, the command defined

by Process_BBS will be activated.

Conversation (Chat) during the session

��������������������������������������

If a mail session is established between two T-Mail systems or between

T-Mail and Bink/+ (by S. Terekhov), the system operators may take advantage

of the possibility of on-line "chat".  A chat session is begun by either

party hitting the Alt-C key combination and is terminated by Alt-C or a line

drop.

The on-line chat is carried out in two windows, one for the local and

one for the remote system.  The Ctrl-G (Bell) key may be used to signal the

operator on the remote system.  If the variable Chat_log is defined, during

chat sessions, then entire dialogue is recorded to the defined log file.

Keys used during the session

����������������������������

During a session, the following keys may be used:

Alt-H    Terminate the current session

Ctrl-X   On file receipt, skip the current file.  This

may not work correctly with all mailers.

Alt-C    Enter chat mode for two-way discussion.

Alt-T    Following a mail session, enter the BBS.

Alt-F1   Display user defined keys

����ͻ

� 11. Actions at the completion of a mail session.

���������������������������������������������������������������������������

At the completion of a mail session, T-Mail will unpack received

packets (*.PKT files), if mail is received and not unpacked during the

session, it is unpacked and the names analysed for further action.

If the, just completed, session was terminated (broken) before

completion and the instruction Boss (EVENTS.CTL) is in force for the remote

system, an empty message is created to force a poll on the system to resume

the session.

By using the instruction Unpack (T-MAIL.CTL) you can permit or prohibit

the unpacking of mail bundles.  By default, incoming mail will be unpacked.

By using the commands KIll_Empty and/or Kill_Transit (T-MAIL.CTL), you

can permit or prohibit the removal of empty messages and/or the removal of

In-Transit messages which have been transmitted.

The instruction Unpack_Size (T-MAIL.CTL) defines the maximum size, in

bytes,  of a mail packet (.PKT) which will be unpacked immediately upon

receipt (during the session).  The size of this variable should be

determined by the performance of your system.  Too large a size could result

in a session failure due to time-out.  The default size is 8000 bytes.

In a FidoNet Technology Network, special file name extensions are used

when packing broadcast (Echo) mail. i.e. xxxxxxxx.MO1 for the first bundle

generated on Monday.  Upon recognizing such a file extension, T-Mail will

automatically start the ArcMail mail processor if 'Process Arcmail' has

been defined in T-MAIL.CTL.

In a similar manner, files of the form xxxxxxxx.?IC are characterized

as "TICK" files, for which the appropriate processor needs to be started.

For these, and other cases, T-Mail uses the "Process" concept, in which

a file_name type can be defined and linked to a definite series of actions

or defined program.

In T-Mail.ctl:

Process <macro>|<filemask> <KeyWord>|<[!]command>|<dvpname>

Where <macro>: ArcMail -         The processing of ArcMail (echomail)

Tic -             The processing of *.?IC files

NetMail -         The processing of non-empty .PKT-file

Files -           The processing of any file

Session -         At completion of a session

Fax -             At acceptance of a fax

<filemask> -  The file mask (according to the rules of the operating system)

<KeyWord> -   One of the key words below, defining the action:

Exit <errorlevel>     -  Exit from T-Mail with the given

errorlevel, where errorlevel -

in the range 1-255.

Set <var> = <value>   -  Set the value of a variable in

T-Mail.ctl

Poll <addr group>     -  Generate a Poll on a given address or

group of addresses <addr group>.

Control               -  Check if the configuration files

have been updated and if necessary

reread them.

Repack [<addr group>] -  Repack Netmail (or rescan Bink outbound).

If <addr group> is given, only process

for addresses within this group.

Create <filename>     -  Create a file <filename>

Kill <filename>       -  Delete a file <filename>

Nothing               -  Do nothing.

<command>    -  The name of the executable program (.exe, .com), or batch

file (.bat) which will be started to perform the required

actions, followed by any required parameters.  Preceding

the command name with an exclamation mark "!", indicates

that the program or batch file should be executed without

swapping out memory.  Under OS/2, the exclamation mark

indicates, close COM port before executing.

<dvp name>   -  The name of the DesqView programm, .dvp-file, defining

the name of the window to be opened.

examples:

~~~~~~~~

Process ArcMail c:\squish\squish.exe -In -Out

Process Tic anytic.bat -Debug                   ; in system directory

Process Netmail Create flags\anymail.t-m        ; Create a flag

Process * .doc copy files\*.doc \doc            ; Use a mask

Process * .zip Exit 98                          ; Exit with errorlevel

Process Files c:\tools\fixfiles.com             ; full pathname

Process *.exe del ..\inbound\*.exe

Process test*.* move ..\inbound\test*.* ..\inbound\testfiles

Process Arcmail Create flags\anyarc.t-m

If no process is defined for Arcmail or Tic, received Arcmail or Tic

files will be considered as normal files and handled according to the Files

process.

Macros may be freely used to define variable parameters to <command>.

*A - Full address of the remote system of the current session.  Default

address is -1:-1/-1.-1 if no current session is active;

*B - connection speed (DCE baudrate);

*L - locked port baudrate (or 0);

*P - COM-port number;

*T - task number;

*O - (only for OS/2 versions) - handle of open COM port;

*N - 1 at successful completion of session, otherwise - 0.

Process commands are executed in the order, in which they are defined in

T-MAIL.CTL.   The maximum number of Process commands is 32.

Process Fax is an exception to this in that it defines the process to

be performed when a fax has been received.  A parameter may be used, which

will, by default, be replaced wit the full path and name of the received fax

file.  Macros should not be used with the Process fax command.

����ͻ

� 12. File requests

���������������������������������������������������������������������������

If it is required to fetch a particular file or files from a remote

system, the File Request feature may be used.

T-Mail supports WaZOO-style file requests.  Which means, that upon

the receipt of a .req file. the .req file will be assumed to contain a list

of the files to be processed.

T-Mail also supports message style File Request, which is defined by a

message having the name of the required file in the subject field and the

F'req attribute.  This type of File Request can be directly generated by T-

Mail, using the Alt-R key.

New files may also be requested with the use of an Update Request.  In

this case the file request will only be satisfied in the event that the

remote system has a newer file than the calling system.

Restrictions and requirements for processing file requests (Freq)

�����������������������������������������������������������������

T-Mail offers many opportunities for restricting the processing of File

Requests, by time, size, number of files, etc.  In T-MAIL.CTL:

Freq_Time 0:00-5:00,8:00-10:00  ; By time interval

Freq_Number 5                   ; Maximum number of files

Freq_Size 512                   ; Total size, kilobytes

Freq_MaxTime 120                ; Total download time, minutes

Freq_MinBaud 1200               ; By minimum connect rate

Freq_Day_Lim <daily limit>      ; Daily Kilobytes, default 64Mb

Freq_Week_Lim <week limit>      ; Weekly Kilobytes, default 64Mb

Freq_Month_Lim <month limit>    ; Monthly Kilobytes

T-Mail also allows you to define that requests will only be granted for

a particular address or group of addresses.  Requests from other nodes will

not be processed.

The variable Freq_Nodes (T-MAIL.CTL) defines the list of addresses,

from which file requests will be accepted.

Freq_Nodes <address list>

list of directories for file requests

�������������������������������������

The list of directories, files which are accessible for requests, is

contained in a text file, defined by the variable Freq_Dir:

Freq_Dir dir.frq

(if a full path is not given, the file must be located in the system directory

T-Mail).

This file contains lines of text describing the directories from which

File Requests may be made.  Additionally each directory may be further

qualified by a password and/or a list of addresses of system which may make

File Requests on a particular directory.

syntax: <path> [|<password>] [<address list>]

example (file dir.frq):

~~~~~~~~~~~~~~~~~~~~~~

c:\files\info

C:\files\area3 5030/*.* !5030/4 5020/* 5000/*.*

c:\files\area4 | password_of_4 5030/*.* !5030/4 5020/* 5000/*.*

In the first case, the directory only is defined, in the second - directory

and list of addresses, in the third - additional password for requests made

from this directory.

If a password is given for a particular directory, the File Request

must be made somewhat differently from the normal case.  e.g. To request the

files LABUDA.EXT from directory c;\files\area4, the request must be made by

using filename, exclamation mark, password, as in:  labuda.ext !password_of_4

It is also possible to use the pipe symbol "|", instead of exclamation mark.

The maximum number of directories which may be defined is limited by

the amount of available disk storage.

aliases (Magic file names)

��������������������������

It is often more convenient to define a general purpose name for File

Requesting specific information which may changes from time to time.  This

concept is called the "Magic" name or "Alias" naming convention and may be

used to equate one or more files with a particular, externally known, name.

The concept is widely used on BBSs and the following list gives an idea of

some of the more popular magic names in use today:

ABOUT         ..  Text file containing a system description

FILES         ..  List of all files available on the system

POLICY        ..  Operating procedures for the system

STATS         ..  Publicly available system statistics

Aliases are listed in the file, defined by the variable Freq_Ndx in

T-MAIL.CTL.

Freq_Ndx index.frq

In this file, each definition line has the following format:-

syntax: <Alias> <pathname> [<command>]

Where:

Alias -         The Alias (magic name).  Any characters may be used

except blanks, spaces.

Pathname -      The full path and name of the file(s) which will be

transmitted.  Wildcards may be used.

Command -       The name of a program.

If a <command> is defined, it will be executed before the formation of

the list of files, necessary to satisfy the request.  Three parameters will

be send to the <command>, these being: The alias name used, The name of the

remote System Operator and the address of the remote system.  Spaces

appearing in any of the parameters will be replaced by underscores "_".

example:

~~~~~~~~~

Files c:\files\100 all.arj

T-Mail c:\support\T-Mail.lzh do_reg.bat

T-exe c:\support\t-2????.*

In the above example, when a File Request is made for "T-Mail", the

batch file "do_reg.bat" will be executed with, say, the following command

line:-      do_reg.bat T-Mail Anthony_Guetmansky 2:5030/6.0

Any program or batch file execute as <command> should run as quickly as

possible in order not to slow the system down or cause a possible time-out.

If necessary, using the "file" indicator, it is possible to defined the

files to be sent as a list in another file:

example:

~~~~~~~~~

T-Mail @c:\support\t-m.lst

When a File Request is made for "T-Mail", the files listed (one per

line) in the file T-M.LST will be sent to the remote system.  Please note

that data lines within this file cannot be longer than 50 characters.

Another form of file "mask" may be used to define a numeric file name

extension.  The form is .999, which will match with the highest numeric

value for the file being requested.

Net5030 d:\nodelist\net5030.999

If directory d:\nodelist\ contains the files net5030.070, net5030.077

and net5030.177, only the latter will sent.  Priority is given to the

highest numerical value except 000 is considered equal to 1000.

Full pathname requests

����������������������

If a password is defined in the variable T-password, it is possible to

make File Requests based upon a full pathname, for example:-

'c:\support\reg.lst !my_password_no_5'

This would override restrictions on times and file sizes.

Transit and remote file requests

���������������������������������

If a file request is received, accompanied by a message which is NOT

addressed to any of this systems' addresses, the request will be handled in

accordance with the definition of the variable Freq_Transit (T-MAIL.CTL).

If In-Transit File Requests are not permitted, the request and message will

be deleted. If In-Transit File Requests are permitted, a new request will be

generated to the destination system.

If a remote request is generated and, at a later time, satisfied, the

resultant file(s) will remain on this system.  This is currently a

restriction of FTNs in that only direct File Requests are possible.

To overcome this restriction, the concept of "remote Requests" has been

implemented.  The basic method of operation is to Request a remote system

to make a File Request to another system, and then transmit the resulting

files and/or message back to this system.

This is specifically handled by creating a message addressed directly

to T-Mail, as shown below:-

Freq_Password Talon

Example of such a message:

~~~~~~~~~~~~~~~~~~~~~~~~~

===============================================

From: Andy Elkin                      5030/15

To: T-Mail!Talon                      5030/100

Subj: Freq labuda.ext 5030/5

-----------------------------------------------

[body of letter - empty]

===============================================

Upon receiving this message at 5030/100, T-Mail will generate a File

Request from this system to 5030/5, to request the file LABUDA.EXT and a

special file will be created to preserve the information as to where this

Request originated.

If the request is satisfied at a later time, T-Mail will generate a

service message, file attaching the resultant file, back to the originator

of the Request.

Notification of file requests

�����������������������������

Should a File Request fail, a message will be created containing

information about the reason for the failure.  To the body of the text of

this message will be added text defined by the variable Freq_Policy in T-

MAIL.CTL:

Freq_Policy policy.frq

If a File Request is successfully completed, a message will be created

containing a list of the files transmitted as well as other statistics and

file sizes and dates, etc.  The generation of these status messages may be

inhibited, by the use of the variable Freq_Confirm in T-MAIL.CTL.

Freq_Confirm No

����ͻ

� 13. Mailbox (postal boxes)

���������������������������������������������������������������������������

Personal MailBoxes for netmail

������������������������������

FTN mailers are often used, for administration purposes, in conjunction

with mail-robot types of programs.  These are programs which handle the

automatic processing of specific system functions, examples of such robot

programs are AreaFix, FileFix, Raid and so-on.

Traditionally the method of handling traffic for these programs has

been that following the end of a mail session, programs will be started to

scan the incoming mail to search for messages addressed to a particular

robot handler.  This is also possible in T-Mail by defining a process for

inbound NetMail (Process Mail).

It is desirable however to avoid having to scan the entire mail

directory at the end of every session in order to search for service

messages as this is extremely wasteful of system resources.  It is more

logical to entrust the work to mail "packers" and to separate messages for

specific users (SysOp and even robot users) into specific directories.

In order to achieve this objective, the concept of personal mailboxes

has been developed for T-Mail.  This concept does not operate in BinkMode.

When an incoming message is recognized as being for one of the

designated "users" of the system, one which has a personal MailBox, the

message is moved into the directory defined for that user.  If messages

exist in that directory which are bound for other systems, they are, when

packing mail, moved out into the NetMail directory for transmission to the

remote system.

The list of MailBoxes is defined in T-MAIL.CTL:

syntax: MailBox <name> = <path> [<destination address>] [| <command>]

Each entry contains the name of a "specific" recipient, the character

"=", the directory to hold messages, optionally a redirection address and

optionally a command to be executed upon the receipt of a message.

example (T-Mail.ctl):

~~~~~~~~~~~~~~~~~~~~

MailBox FileFix=c:\filefix\ | c:\filefix.bat -Scan -Toss -Hren

MailBox AreaFix=c:\areafix\

MailBox Ivan Pupirin=c:\mail\ 5030/15.99 | echo Vanya! Wake up!

The most interesting example above, is the last, in which the target

directory is the same as the main NetMail directory.  In this case the

message is not moved at all, but will be readdressed to the new system.

This method allows a remote system to receive NetMail addressed to this

system.  It should be noted that the message MUST be re-addressed to a node

other than that used by the current system, otherwise an endless loop could

result.

If you send a message, from your MailBox to an addressee with the same

name, the message will only be sent if the address is different from the

current system.

T-Mail can handle up to 100 MailBoxes and will automatically create any

non-existent directories at start-up.

FileBoxes, MailBoxes for files

������������������������������

While MailBoxes are intended to improve inbound mail flow, FileBoxes

are intended to improve outbound file handling.  Usually, in order to

transmit a file to a remote system, the SysOp must create a message with a

file attach attribute.  This message contains information to the packer

about the file and the system to which it must be sent.  In practice, the

majority of files, such as EchoMail bundles, are not subject to routing and

are sent directly to the target system.

In such cases, an accompanying service message in unnecessary and it is

possible to inform the mailer of the destination address by other means.

The basic concept of FileBoxes is such that for each address it is possible

to associate a FileBox (directory).  When a connection is established with a

system and a FileBox exists, containing files, for that system, they will be

automatically transmitted and deleted.  It is only necessary to copy the

files required to be transmitted into the appropriate directory.

When mail is packed for a defined system, T-Mail checks for the

existence of files in the FileBoxes.  The information on existing files is

used in the formation of the mail queue.  For each address, there are

actually 2 FileBoxes.  One is the "active" FileBox which, if it contains

files, will force T-Mail to call out to deliver those files, and the other

"inactive" which, if it contains files, will only be transmitted if the

remote system calls in.  The latter is similar to using the HoldforPickup

attribute in FileAttach service messages.

The names "active" and "inactive" fileboxes differ by the last letter

of the extension. For example, for the same address the names-fileboxes

might be:

1234567.01      -  "Active" filebox

1234567.01H     -  "Inactive" (hold) filebox.

The actual "name" is calculated from the appropriate address of the

system.  The root directory for all FileBoxes, is defined by the variable

File_Boxes in T-MAIL.CTL:

File_Boxes c:\T-Mail\filebox\

If this variable is not defined, by default, the directory for file

boxes will be <Files>\boxes\, where <Files> - is the path, given to by

variable Files in T-MAIL.CTL.

T-Mail will search for the subdirectories, the names of which are

calculated, from the remote address, using the following algorithm:

ZZNNNFFF.PPX

Where ZZ - radix-32, zone,

NNN - radix-32, net,

FFF - radix-32, node,

PP - radix-32, point.

X - if this filebox is "active", it is blank,

if "inactive" (hold), letter H.

Example, for address 2:5030/100.1 the directory will be named:

c:\T-Mail\filebox\024T6034.01

For more details of the calculations of names, please see the

description in the T-Mail API.  Please note also that radix-32 is the

numeric representation using the symbols 0-9 and A-V.

For T-mail, the existence (or otherwise) of a FileBox directory is

important.  If a FIleBox exists, which contains files, they will be sent and

removed.  Empty FileBox directories are automatically removed to speed-up

scanning.

Apart from the default method of handling FileBoxes, it is possible to

establish permanent FileBoxes for particular systems.  This is handled by

using the Box variable in SUBST.LST and will create a separate and extra

FileBox for a particular system.  The main differences between a FileBox

created in this manner and a default FileBox, are they are not scanned at

mail packing time and are not removed if empty.  Also files from these

FileBoxes are not transmitted if the address of the FileBox is only an AKA

of the remote system.

����ͻ

� 14. Processes and flags (Process_Flag)

���������������������������������������������������������������������������

T-Mail requires the exclusive use of one flag during operation, which

is defined by the variable Flag_Session in T-MAIL.CTL.  This flag is created

by T-Mail when starting a mail session and is erased when the session

terminates.  By default it is created in the directory defined by the

variable Flag_Dir.

All other flags may be designated and used independently.  By using the

variable Process_Flag (described in detail in chapter 7), you can define

the reaction of T-Mail to any particular flag.  This can be particularly

useful for controlling the overall operation of the system and an example

will make this clearer:

Example (t-mail.ctl):

~~~~~~~~~~~~~~~~~~~~

[S] Process Arcmail Create flags\anyarc.t-m           ; create a flag

...

[M] Process_Flag anyarc.t-m c:\squish\squish -In -Out ; start tosser

In the example above, upon successful completion of a session with

received EchoMail, the slave task will create a flag called anyarc.t-m.

During the next scan of the flags, the Master system will execute the

actions defined actions and remove the flag.

����ͻ

� 15. Modem management

���������������������������������������������������������������������������

The "Modem Control" section of T-MAIL.CTL defines the parameters and

configuration of the modem.  The variable Modem defines the name of the file

containing the modem management parameters as well as modem responses under

different situations.

Unless you are familiar with modem technology, it is not recommended to

make changes to this control file, instead simply verify that the commands

will be understood by your modem and remove (comment out) those commands

which are not usable.

It should be noted that each command sent to the modem, except dialing

prefixes, must be terminated by a line-end symbol <cr>.  It is represented

in the MODEM.CTL file by the pipe symbol "|".

The following symbols may be used in modem strings:

v  -  lower DTR

^  -  raise DTR

'  -  pause 0.1 sec.

~  -  pause 0.5 sec.

These symbols cause, only the actions defined and are not, in

themselves, sent to the modem.

example: (modem.ctl):

~~~~~~~~~~~~~~~~~~~~

Init   v~''^ATZ|       ; Lower DTR, delay 0.7 secs, raise DTR,

; send "ATZ" and <cr>.

OnHook v~~^~v~~^ATH0|  ; Lower DTR, delay 1.0 secs, raise DTR,

; delay 0.5 secs, lower DTR, delay 1.0

; secs, raise DTR, send "ATZ" and <cr>.

COM port set-up

�������������������

The section " Port definitions " of T-MAIL.CTL defines the

parameters of the COM port, which T-Mail will use in normal operation.

The number of the COM port (variable Port) must be established:

Port 1                 ; for COM1

The variable Baud, set the maximum speed of connection.  If the modem

allows it's port to be locked, the Lock_Baud variable should also be set to

the speed at which the modem is locked.  For more details, please see your

modem manual and the discussion on FOSSIL drivers.

Baud 9600              ; maximum baud rate

Lock_Baud 19200        ; locked port baud rate

If your modem is capable of supporting flow control by hardware

(RTS/CTS), then it is recommended to set the variable HardWare_Flow:-

HardWare_Flow Yes      ; Allow RTS/CTS

Is in case using Lock_Baud, RTS/CTS flow control is mandatory

for successful operation

.

Modem initialization

��������������������

The variable Init_After_Dial defines whether T-Mail will re-initialize

the modem after each dialed number.  If any dialing prefix was used, except

that defined as the general dial prefix, the modem is re-initialized by

default.

Modem_Delay defines the delay (in 0.1 sec intervals) before sending

any command to the modem.

T-Mail will automatically re-initialize the modem on a regular basis as

defined by the variable Init_Interval (minutes).  If for any reason the

modem does not respond with an acceptable response, T-Mail will retry up to

three more times.  If initialization still fails, then T-Mail will execute

the process defined as No_Modem.  This process could be used to initiate

hardware reset or some other form of "drastic measure".  The modem is tries

once more after this and if it still fails, errorlevel 5 is returned and T-

Mail will exit.

The syntax for Process No modem is:

(t-mail.ctl):

Process No Modem <program> [<macro>]

<program>  -  the name of the program or batch file to execute,

<macro>    -  as defined below, parameters passed.

*B - connection speed (DCE baudrate);

*L - locked port baudrate (or 0);

*P - COM port;

*T - task number;

*O - (only for OS/2 versions) - handle of opened COM port.

example:

~~~~~~~

Process No Modem d:\bat\init.bat *P

Going Off hook when starting an external program

������������������������������������������������

If it is envisioned that T-Mail will be busy, processing, for an

extended period of time, it is wise to put the modem "off-Hook" in order

that calling systems will receive a BUSY signal and do not spend excessive

time on the line waiting for an answer.  The variable Busy_Hang_Off YES will

place the modem on an Off-Hook state whenever an extended exit is expected.

This variable should be used with care as, if the exit is, in fact,

very short, it is possible for the remote computer to actually remain On-

Line during the entire exit.

Delay at loss of carrier (carrier monitor)

������������������������������������������

It sometimes happens, particularly on low-quality lines, that the Carrier

Detect signal can be lost for a short period and then recovered.  It is

necessary to inform the modem of the maximum length of time this can be

allowed to happen before the line is considered to have been dropped.

The variable Carrier_Lost_Delay defines, in units of 0.1 secs, the time

interval during which T-Mail will consider the session still viable.  Set

this value to 0 to disable this feature.

Statistics log on connection termination

����������������������������������������

Some modems maintain internal statistics concerning the performance of

the previous session.  T-Mail allows you to capture this information and to

record it in the log files for later examination, by setting the variable

Statistics in MODEM.CTL to the command necessary to read the required

information.

example (modem.ctl):

~~~~~~~~~~~~~~~~~~~

Statistics ATI6 |           ; Link info from US Robotics

If this variable is not set, in modem.ctl, no statistics will be

collected.

����ͻ

� 16. Terminal emulator

���������������������������������������������������������������������������

T-Mail has built-in terminal emulator support which may be used for

modem tuning and simple remote, on-line, session execution.

Enter terminal mode

�������������������

Terminal mode can be initiated by the following methods:

- key Alt-T is pressed;

- call BBS during session (Alt-T) was requested;

- T-Mail was loaded with the parameter -TT;

The variable Terminal defines the external terminal program to be

executed and if this is not defined, a simple internal emulator will be

started.

The initialization command for terminal mode is defined, in MODEM.CTL,

by the Init_Terminal variable:-

example:

~~~~~~~

Init_Terminal ATE1|

If this variable is not defined, then the modem is not initialized in

Terminal mode.

Terminal mode recognizes and handles ANSI and Avatar code sequences.

Some general information, start protocols, transmission results, etc)

may be logged to a file, defined by the variable Term_log in T-MAIL.CTL

The following keys may be used in terminal mode, using the internal

emulator:

Alt-D  -  Locate a number to be dialed.  If an address is defined

in the nodelist, a telephone number can be extracted.  If

it is necessary to enter the telephone number directly,

it should be preceded by the hash symbol "#".  The

number will be dialed according to the dialing

parameters, Term_inactivity.  Dialing may be aborted by

pressing the ESC key.

PgUp   -  send files

PgDn   -  receive files

Esc    -  Abort the reception of files

Alt-I  -  initialize the modem

Alt-H  -  Hangup the line

Alt-=  -  Toggle (on/off) DoorWay mode

Alt-L  -  Toggle (open/close) capture file

Alt-B  -  change baudrate

Alt-7  -  7-bit characters

Alt-8  -  8-bit characters (default)

Alt-E  -  clear the screen

Alt-O  -  Toggle status line

As well as the above, any keys defined by the variable Key in T-

MAIL.CTL, except Doorway toggle mode, continue to act normally.

External protocols

������������������

If no external protocols are defined, the response to PgUp, PgDn or

Autodownload sequence, will be internal Zmodem.  External protocols for

upload and download may be defined as follows:

Term_Download c:\gsz\gsz.exe port *P speed *B rz -rr

Defines the external download protocol.

Term_Upload   c:\gsz\gsz.exe port *P speed *B sz

Defines the external upload protocol.

The following macros may be used as parameters

*B - connection speed (DCE baudrate);

*L - locked port baudrate (or 0);

*P - COM port;

*T - task number;

*O - (only for OS/2 versions) - handle of opened COM port.

*F - at download - the directory sets, are where received the files

(Term_DownDir) at upload - are set the by name of sent file (files).

Received files are placed in the directory defined by the variable

Term_DownDir (only if the default internal protocol is used).  If files to

be uploaded are not defined by a full path, they are expected to exist in

the directory defined by the variable Term_UpDir:

Term_UpDir      c:\T-Mail\files\

Term_DownDir    c:\T-Mail\files\

Exit terminal mode

������������������

To exit from terminal mode, hit the Alt-Q key combination.  If the

emulator was started from a T-Mail command line with the -TT parameter, T-

Mail will also be terminated.

Operator inactivity interval in emulator mode should be set using the

variable Term_Inactivity in T-MAIL.CTL.

Term_Inactivity 60               ; Terminal inactivity timeout (min)

Defines the maximum time, in minutes, of operator inactivity before the

emulator will exit from Terminal mode.  The default value of 0, specifies an

infinite time-out and it is not in operation during file transfers.

Upon exit from terminal mode, the capture log file, if opened, is

closed.

����ͻ

� 17. BinkleyTerm compatibility

���������������������������������������������������������������������������

As well as the main operational mode (ArcMail Attach), T-Mail is able

to support a BinkleyTerm style of Outbound area.  In this method of

operation, T-Mail does not use the *.MSG files for building queues for

external programs.

To start T-Mail in this mode, T-Mail must be configured with the

variable Bink_mode set to Yes, by default this is set to No.

Bink_Mode Yes                    ; Bink-style mode (no-ArcMail-attach)

A time interval may be defined which will determine the frequency with

which the outbound directory is rescanned:

Scan_Circle 100                  ; Rescan period (seconds)

It is necessary, in Bink_Mode to define a directory which will be used

to hold outbound packets:

Packets d:\outbound\out\         ; Outbound packets directory

Directories for different zones are formed by using a directory

extension of the zone number expressed as a hexadecimal expression:

Address 2:5030/100@fidonet

AKA 228:5001/19@palantir

....

Packets c:\t-mail\outbound\

Outbound for zone 228 will be c:\t-mail\outbound.0e4\

Currently, T-Mail in BinkMode does not support Domain names and this

must be taken into consideration when using other program in conjunction

with T-Mail.

When operating a multi-line system it is still necessary to make use of

the Master/Slave relationship.  The Slave machines are responsible for

forming mail queues and the Master machine is responsible for outbound queue

scanning.

Using T-Mail in BinkMode will limit some of the available features, in

particular, the rules for routing (Mail/Files/Direct) become meaningless.

The rule of Deny remains operational, although with some differences in that

it causes as it creates entries for excluded systems in the mail queue.

When mail requests are formed, however, no effect will be seen

����ͻ

� 18. T-Mail functions

���������������������������������������������������������������������������

Crash and immediate mail

������������������������

This method of operation causes immediate access and mail session with

a particular system or set of systems.  For all practical purposes, T-Mail

does not pause between outgoing calls to wait for incoming calls.

To enter a Crash-Mail event, the system operator should depress the

keys <Shift-Tab> to establish the address, after which T-Mail will

continually call to the specified address until a successful session has

been conducted.  The operator may terminate the Crash-Mail event by pressing

the <Shift-Tab> keys a second time.

It is possible to enter a telephone number directly instead of a node

number by preceding the number with a hash "#" symbol.  Similarly, if you

have a session password established with the remote system, this may be

indicted by separating it from the node or telephone number using the

asterisk "!" or pipe "|" symbol.

examples of Crash-Poll:

~~~~~~~~~~~~~~~~~~~~~~

5030/2                           ;  Node number

5030/1|klimbeer                  ;  Node number and password

5030/2!Kronpass                  ;  Node number and password

#126-6972                        ;  Telephone number

#123-4099!Paswrd2                ;  Telephone number and password

During a Crash-Mail event, T-Mail continues normal operations on the

packing of new mail, completion of events and the analysis of flags.  In the

case of an Exit, the Crash-poll mode status is automatically removed.

Remote control: T-Mail server

�����������������������������

T-Mail server mode is invoked by service messages from remote systems.

These messages are normal messages but are addressed to T-Mail.  The

destination field of such messages should contain:

T-Mail|<T-password>

Where <T-password> - password of the T-Mail server, defined by the

variable in T-MAIL.CTL.  Note, the process REQUEST, will be executed if a

match is made on any of the 2nd passwords defined by the variable T-Password

or Freq_password.

Example:

-----------------------------------------------

From: Andy Elkin                      5030/15.1

To: T-Mail|my_password_no_5         5030/15

Subj: Send C:\T-Mail\files\t-2300.Arj /27 /51

-----------------------------------------------

Note: Spaces are not significant.

The subject field of a service message contains the information

concerning the task to be performed by the T-Mail server.  The task type is

defined by the first keyword and may be:-

Poll [<address>...]         - Forces T-Mail to generate a poll packet and

initiate a mail session with the defined

system(s).

Request <file> <address>    - Forces T-Mail to File Request the defined file

from the defined system.  If a file is received

as a result of the File Request, it will be

dispatched to the originator of the service

request.

Attach <file> <address>    -  Send specified file to specified node(s)

Send <file> <address>      -  Same as Attach

Post <file> <address>      -  Send the text contained in the specified file,

as a message, to the specified system(s).

Exec                       -  Copy the text of the message body to a file

called t-serv.bat and then execute t-serv.bat

(for OS/2 t-serv.cmd is created/executed)

Set <variable> <value>     -  Change the value of the named variable to the

value given.  This command should be used with

great care not to change important variables,

such as Address, Aka, Port etc.

List <file area>           -  Forces T-Mail to generate a list of the

contents of a particular file area.  The

parameter <file area> should contain the same

string information defining the name of the

directory to be listed.  e.g. if the Variable

Freq_Dir, defines the path d:\Files\Windows

the request list should appear thus:

To: T-Mail

Subj: List Windows

If a FILES.BBS file exists in this directory,

then it's contents will be used for the text

body of the reply, otherwise a DIR command

will be executed and the results returned as

the text body of the reply.

To receive a list of File Areas, which are not

protected by a password, it is adequate to

simply fill the destination field with the

text T-Mail.

Users who are aware of the T-Password are able

to request file lists from not only those

defined in Freq_Dir, but from any directory on

the system.  In order to achieve this is is

necessary to specify the full path:

To: T-Mail|password

Subj: List c:\files\private\

The asterisk character "*" may be used to

request a list of all file areas:

To: T-Mail

Subj: List *

examples of requests:

---------------------

To: T-Mail | password

Subj: Poll /2 /3 /6

---------------------

---------------------

To: T-Mail | password

Subj: Request filename.ext /3 /7.1 33:36/*.3

---------------------

---------------------

To: T-Mail | password

Subj: Write c:\txt\my_txt.doc /16 /18

---------------------

---------------------

To: T-Mail | password

Subj: Set Answer 0:00-4:00

---------------------

---------------------

To: T-Mail

Subj: List SUPPORT

---------------------

The use of wildcards in the specification of an address is subject to

the limitation of a single occurrence.  Any address specified, must be

contained in the nodelist in use or it will be ignored.

Service messages are processed immediately upon being unpacked and care

must be used if mail is unpacked "on the fly", especially in the case of the

Exec request as if too much time is taken, the session may be broken.

If the user knows the T-Password, it is possible to set attributes in

messages which will be redirected to other systems.  Generally message

attributes such as Crash, Immediate etc are lost when the message is

unpacked.  This is handled by placing special text on the 1st line of the

body of the message:

Set!<t-password>!Attrib <flags through blank>

Flags can be, for example: LOCAL DIR CRASH HOLD KILL

Set!My_passw!Attrib Dir Hold

When unpacking and creating a new message, the attributes DIR(ect) and

HOLD(forPickup) are added.  The LOCAL attribute is automatically added.  The

first line, shown above, is removed from the outgoing message and this fact

is written in the log file.

service lines in messages and processing

����������������������������������������

Some specific lines, in service messages, are recognized by T-Mail and

cause non-standard actions.  These lines should be placed at the beginning

of the message and are only effective for messages which are generated

locally and thus carry the LOCAL attribute.

Delay the message:            Hold to [hh:mm] [dd[-mm[-yy]]]

or Hold to [hh:mm] [dd[/mm[/yy]]]

This signifies that the message should not be packed and no attempt

should be made to deliver it, before the specified time.

examples:

~~~~~~~~

Hold to 3:00 15/12/92

Hold to 16-12-92

Hold to 16/12

Hold to 4:00 16

Hold to 4:00

The default date is the current day.  If a date, but not a time is

specified, the time is assumed to be midnight.

Direct routing:               Route to <address>

When the message is packed, ignore set routing rules and route to the

address specified.

examples:

~~~~~~~~

Route to 5030/2

Route to /2

Confirmation receipts

���������������������

For an In-Transit message, if the attribute AUDITRECREQUEST (ARQ) is

set, T-Mail will generate a message to the original author informing of the

time and destination of the outgoing message.  The template for the receipt

message is AUDIT.TPL and may contain the following macro strings which will

be replced when the message is created:

~U - name of the author of the message;

~P - name of addressee;

~N - address of addressee;

~A - own address;

~L - address, where the message will be forwarded;

~D - date;

~T - time.

Such a confirmation will only be generated in the event that the T-Mail

variable, Audi_send_Msg is set to Yes.  Default is No.

example (t-mail.ctl):

~~~~~~~~~~~~~~~~~~~~

Audit_Send_Msg Yes

����ͻ

� 19. T-Mail management from the local keyboard

���������������������������������������������������������������������������

The keys listed and defined below are control functions for T-Mail's

mailer mode.  The keys for Terminal mode have been described in a separate

chapter.

�����������������������������������������������������������������Ŀ

�           �                         �                           �

� Key       � T-Mail reacts when      �           Action          �

�           �                         �                           �

�����������������������������������������������������������������Ĵ

�           �                         �                           �

� F1        � waiting for a call      � Displays HELP text        �

� Alt-X     � waiting for a call      � Exit with errorlevel 10   �

� Space     � waiting for a call      � Immediate call out        �

� Alt-O     � waiting for a call      � DOS Shell                 �

� Alt-Y     � waiting for a call      � Repack all mail           �

� Alt-Z     � waiting for a call      � Send the line in modem    �

� Alt-T     � waiting for a call      � Enter Terminal mode       �

� Alt-M     � waiting for a call      � Zoom mail queue to Full   �

�           �                         � screen.                   �

� Shft-Tab  � waiting for a call      � Toggle Crash mode         �

� Alt-P     � waiting for a call      � Poll **                   �

� Alt-S     � waiting for a call      � Send a file to address ** �

� Alt-D     � waiting for a call      � Send a file to address ** �

�           �                         � (direct)                  �

� Alt-R     � waiting for a call      � Request a file from       �

�           �                         � address **                �

� Alt-A     � waiting for a call      � Force the modem to answer �

�           �                         � incoming call.            �

� Alt-K     � always                  � Screen saver.  Do version �

�           �                         � only.                     �

� ?         � waiting for a call      � Information on a system   �

� Alt-=     � waiting for a call      � Change the value of any   �

�           �                         � system parameter          �

� +/-       � waiting for carrier     � Increases/decrease wait   �

�           �                         � time.                     �

� Esc       �                         �                           �

� Space     � waiting for carrier     � Hang up the line          �

� Alt-H     � during transmission of  � Terminates the session    �

�           � file or handshaking     �                           �

� Ctrl-X    � reception of file       � Skip the current file.    �

�           �                         �                           �

� Alt-C     � transmission of files   � Exit to 'Chat' mode       �

�           �                         �                           �

� Alt-T     � transmission of files   � Close down the BBS and    �

�           �                         � end the session.          �

�           �                         �                           �

�������������������������������������������������������������������

���������������������For addresses in queue ���������������������Ŀ

�           �                         �                           �

� Key       � T-Mail reacts when      �           Action          �

�           �                         �                           �

�����������������������������������������������������������������Ĵ

�           �                         �                           �

� Ctrl-H    � waiting for a call      � Hold (*)                  �

�           �                         � call systems (***) during �

�           �                         � next hour                 �

� H         �                         � Toggle status             �

�           �                         � hold (*) for current      �

�           �                         � system in queue           �

�           �                         �                           �

� Ctrl-U    � waiting for a call      � Unhold mail to (***)      �

�           �                         � systems                   �

�           �                         �                           �

� Ctrl-I    � waiting for a call      � Give Immediate status to  �

�           �                         � (*) systems, to (***) for �

�           �                         � current hour              �

�           �                         �                           �

� I         �                         � Toggle Immediate status   �

�           �                         � for current system        �

�           �                         �                           �

� Ctrl-R    � waiting for a call      � Cancel Immediate status   �

�           �                         � (*) for systems (***)     �

�           �                         �                           �

� Ctrl-P    � waiting for a call      � Give Priority status to   �

�           �                         � (*) systems, to (***) for �

�           �                         � current hour              �

�           �                         �                           �

� P         �                         � Toggle priority status    �

�           �                         � for current system        �

�           �                         �                           �

� Ctrl-C    � waiting for a call      � Cancel priority status (*)�

�           �                         � for systems (***)         �

�������������������������������������������������������������������

*   - Statii Immediate, Hold and Priority are

described in the chapter "Queue Management".

**  - the "Address structure", including wildcards,

is permissible as, for example:

5030/6 567:667/4.5 1:3/3 5020/* /1

*** - including format <address list> such as:

1:3/3 5020/* !5020/3 HST !MNP

As wellas the keys described above, it is possible to use the cursor-

control keys arrow, Home, End, PgUp and PgDn to view items in the queue.  In

combination with Ctrl to change the position in the queue.  Pressing the

Ctrl-Enter combination will display information on the current address in

the queue and simply pressing Enter will execute the program T-QUEUE, for

queue management.  This access is also available via the programmatic

interface provided by the T-Mail Application Programmers Interface (API).

����ͻ

� 20. Additional set-up

���������������������������������������������������������������������������

Definition of "hot keys"

������������������������

The System Operator may define up to 20 "hot" keys, which will cause

the execution of external programs or commands.  The keys are defined, in T-

MAIL.CTL with the following entries:-

syntax: Key <hex scancode> [<flag>] <command|dvp> [<parameters>]

or

syntax: Key <mnemonic> [<flag>] <command|dvp> [<parameters>]

Where <hex scancode> - Scancode of the key (see table below)

<mnemonic>      - A mnemonic key definition (see table below)

<flag>          - One or some symbols - modifiers, specifying

start conditions of <command>:

<command|dvp>   - program or full name of DesqView DVP-file;

<parameters>    - optional parameters

'!' - do not swap (for OS/2 - close port);

'%' - do not preserve the screen;

Alongside of the executed command it is possible to define a KeyWord(s),

similar to the method used for Process (see chapter 11):

Exit <errorlevel>   - exit from T-Mail with specified

Errorlevel, errorlevel - number in

range 1-255.

Set <var> = <value> - Set a new value to a variable

defined in T-mail.ctl

Poll <addr group>   - Generate a Poll to the address or

group of addresses, given in <addr

group>.

Control             - Check if the configuration files

have been modified and if necessary

re-read them.

Repack <addr group> - repack Netmail (rescan outbound).

If an <addr group> is given, then

mail repacking occurs for those

nodes only.

Create <filename>   - create file <filename>

Kill <filename>     - erase file <filename>

Nothing             - do nothing.  This may be used to

remove or create a flag.

Parameter substitution may be made using one or more of the following

macros:

*A - address of current system in queue ;

*B - connection speed (DCE baudrate);

*L - locked port baudrate (or 0);

*P - COM port;

*T - task number;

*O - (only for OS/2 versions) - handle of opened COM port.

examples:

~~~~~~~~

Key 5800 Tst.bat *T

Key <Shift-F10> showbox.bat *A *D

Key 5900 c:\dv\bc-pif.dvp

Key 2600 !c:\command.com

Key 3100 !Msgview.exe c:\T-Mail\mail\*.msg

Key 2600 c:\tools\view.exe c:\T-Mail\T-Mail.log

Key 4100 set sound=No

Key 4200 Poll /2 /3 5040/18

Key 4300 _view.exe *A

Key 4100 repack              ; Repack netmail

Key 4200 Exit 99             ; exit with errorlevel

Keyboard scan codes.

�������������������������� Function keys ��������������������������Ŀ

�                                                                   �

� <F1>  3B00    <Shift-F1>  5400    <Ctrl-F1>  5E00  <Alt-F1>  6800 �

� <F2>  3C00    <Shift-F2>  5500    <Ctrl-F2>  5F00  <Alt-F2>  6900 �

� <F3>  3D00    <Shift-F3>  5600    <Ctrl-F3>  6000  <Alt-F3>  6A00 �

� <F4>  3E00    <Shift-F4>  5700    <Ctrl-F4>  6100  <Alt-F4>  6B00 �

� <F5>  3F00    <Shift-F5>  5800    <Ctrl-F5>  6200  <Alt-F5>  6C00 �

� <F6>  4000    <Shift-F6>  5900    <Ctrl-F6>  6300  <Alt-F6>  6D00 �

� <F7>  4100    <Shift-F7>  5A00    <Ctrl-F7>  6400  <Alt-F7>  6E00 �

� <F8>  4200    <Shift-F8>  5B00    <Ctrl-F8>  6500  <Alt-F8>  6F00 �

� <F9>  4300    <Shift-F9>  5C00    <Ctrl-F9>  6600  <Alt-F9>  7000 �

� <F10> 4400    <Shift-F10> 5D00    <Ctrl-F10> 6700  <Alt-F10> 7100 �

� <F11> 8500    <Shift-F11> 8700    <Ctrl-F11> 8900  <Alt-F11> 8B00 �

� <F12> 8600    <Shift-F12> 8800    <Ctrl-F12> 8A00  <Alt-F12> 8C00 �

�                                                                   �

����������������������������� Alt-keys�����������������������������Ĵ

�                                                                   �

�       <Alt-A> 1E00        <Alt-N> 3100        <Alt-1> 7800        �

�       <Alt-B> 3000        <Alt-O> 1800        <Alt-2> 7900        �

�       <Alt-C> 2E00        <Alt-P> 1900        <Alt-3> 7A00        �

�       <Alt-D> 2000        <Alt-Q> 1000        <Alt-4> 7B00        �

�       <Alt-E> 1200        <Alt-R> 1300        <Alt-5> 7C00        �

�       <Alt-F> 2100        <Alt-S> 1F00        <Alt-6> 7D00        �

�       <Alt-G> 2200        <Alt-T> 1400        <Alt-7> 7E00        �

�       <Alt-H> 2300        <Alt-U> 1600        <Alt-8> 7F00        �

�       <Alt-I> 1700        <Alt-V> 2F00        <Alt-9> 8000        �

�       <Alt-J> 2400        <Alt-W> 1100        <Alt-0> 8100        �

�       <Alt-K> 2500        <Alt-X> 2D00        <Alt--> 8200        �

�       <Alt-L> 2600        <Alt-Y> 1500        <Alt-=> 8300        �

�       <Alt-M> 3200        <Alt-Z> 2C00                            �

�                                                                   �

���������������������������� Control keys �������������������������Ĵ

�                                                                   �

�       <Home> 4700      <Ctrl-Home> 7700      <BackSpace> 0E08     �

�        <End> 4F00       <Ctrl-End> 7500         <Insert> 5200     �

�       <PgUp> 4900      <Ctrl-PgUp> 8400            <Del> 5300     �

�       <PgDn> 5100      <Ctrl-PgDn> 7600       <Ctrl-Ins> 9200     �

�       <Left> 4B00      <Ctrl-Left> 7300       <Ctrl-Del> 9300     �

�      <Right> 4D00     <Ctrl-Right> 7400                           �

�         <Up> 4800        <Ctrl-Up> 8D00                           �

�       <Down> 5000      <Ctrl-Down> 9100                           �

�          <5> 4C00                                                 �

������������������������������ Gray keys ��������������������������Ĵ

�                                                                   �

�     <Grey /> E02F         <Grey -> 4A2D     <Grey Enter> E00D     �

�     <Grey *> 372A         <Grey +> 4E2B                           �

���������������������������������������������������������������������

��������������������� Allowable Mnemonic codes ���������������������Ŀ

�        F1 - F12        Ctrl-F1 - Ctrl-F12   Shift-F1 - Shift-F12  �

�    Alt-F1 - Alt-F12          0 - 9             Alt-0 - Alt- 9     �

�         A - Z           Ctrl-A - Ctrl-Z        Alt-A - Alt-Z      �

���������������������������������������������������������������������

Sound effects

�������������

T-Mail is capable is making 9 different sounds which may be used in

different situations.  For each situation, a keyword corresponds to a

particular mask.

�������������������������������������������������������������������Ŀ

�     Keyword   �           situation                    �   Mask   �

�������������������������������������������������������������������Ĵ

�               �                                        �          �

�     Connect   �   CONNECT! with remote system          �     1    �

�               �                                        �          �

�     Begin     �   Start of file transfer               �     2    �

�               �                                        �          �

�     End       �   End of session                       �     4    �

�               �                                        �          �

�     Unpack    �   Unpacking mail (.PKT)                �     8    �

�               �                                        �          �

�     Chat      �   Request for chat from remote         �    10H   �

�               �                                        �          �

�     Human     �   Human Caller                         �    20H   �

�               �                                        �          �

�     Abort     �   File transfer aborted                �    40H   �

�               �                                        �          �

�     Error     �   Unable to initialize modem or FOSSIL �    80H   �

�               �                                        �          �

�     Tick      �   Various cases, i.e. Keyboard buffer  �   100H   �

�               �   overflow.                            �          �

�               �                                        �          �

�     Ring      �   Incoming call                        �   200H   �

�               �                                        �          �

���������������������������������������������������������������������

In T-MAIL.CTL, each keyword described above, corresponds to a melody,

the sound of which is user definable according to individual tastes.  A

utility, TESTSND, is provided for the purposes of annunciating the various

sounds defined.

syntax: SoundOf <key word> <number of recurrences> <sample>

Each sample contains the one or more groups of 4 numbers, interpreted:

The first number of sample - starting tone (Hz) (-1 .. silence)

2nd                    - tone increment

3rd                    - increment duration (0.25secs)

4th                    - number of steps

example:

~~~~~~~

SoundOf Chat         4   3500 100 50 1         3000 0 50 1

�        ����     �   �   � � number of steps

�        �        �   �Ŀ � increment duration (0.25secs)

�        �        �     Tone increment (Hz)

Key   number of    starting tone (Hz)

Word   recurrences

The variable Sound in T-MAIL.CTL defines which sounds are permitted and

which sounds not. The value Yes (1FFH) allows all sounds, the value No

(00H), none.

example:

~~~~~~~

Sound 181   ; the sounds 'Tick', 'Connect' and 'Error' are enabled

Screen colors

�������������

A utility is provided, written by Alex Kopylov, for modifying the

default color scheme.  The utility, caled TCE create a control file caled

T-MAIL.CLR which define the color scheme required.  If T-MAIL.CLR has not

been created according to individual needs, a default color scheme will be

used.

TCE command line format:

TCE InputFile [OutputFile] [parameters]

Where <InputFile> and <OutputFile> - names of color control files.

If OutputFile is not give, the input file is overwritten.

Parameters:

-Shadow: XXX - Decimal value of character for the shadow Dialog box

A value of 0 = transparent.

-Conv        - Disables the editing feature (read and write only)

-Quick       - Disables the copyright notice display.

-Mono        - Use monochrome palette for creating a new file.

-OldClr      - Use T-Mail v2.02) .Clr format of input file.

-Internal    - indicates that the output file should be written in

internal T-Mail format.

The order of parameters on the command line is not significant.

In interactive operation, the following keys are used:

F1         - help

F3         - cancel all changes

F5         - color palette by Andy Elkin

F6         - alternative palette by Andy Elkin

F7         - color palette by Alex Kopylov

F8         - color palette by Alex Kopylov, for 16 background

F10        - Write output file and exit

Esc        - exit without writing output file

Tab        - cursor to next field

ShiftTab   - cursor to previous field

'0'        - map mode

'1'        - queue mode

'2'        - info mode

'3'        - crash mode

'4'        - session mode

'5'        - chat mode

'6'        - terminal mode

'7'        - T-Mail's help;

Enter      - Change to color selection screen

PgUp       - Color selection

PgDn       - Color selection

Ctrl       - Display chosen color

Up, Down   - change foreground color

Left,Right - change background color

Space      - blinking (increased brightness of background): On/Off;

Del        - blinking/16 colors for backgrounds;

Ins        - window with colorful palette;

examples:

~~~~~~~~

Tce T-Mail.Clr           ; edit color in T-Mail.Clr

Tce New.Clr              ; create new .Clr file

����ͻ

� 21. Multi-line and Multi-tasking features

���������������������������������������������������������������������������

T-Mail is a multi-line mailer.  One copy of T-Mail may serve only a

single modem over a single telephone line but several copies of T-Mail may

be used in a multi-tasking (DesqView, Windows, OS/2 etc) or Local Area

Network environment to simultaneously serve a number of modems on separate

telephone lines.  T-Mail is fully Desqview aware and will give back free

time-slices to allow smoothe operation even on 286 computers.

Number of tasks. The Master/Slave(s) concept

��������������������������������������������

In the case of a multi-line system, where several copies of T-Mail are

operating simultaneously, one of the copies of T-Mail is allocated the task

of a mail packer.  All tasks (Slaves) may engage in the unpacking of mail

but only one task, the Master, is used for mail packing.  The passing of

information between Slave tasks and the Master task is handled using special

files and flags.

Each task is given a unique number and is used to identify the division

of duties between tasks.  The task number is specified on the T-Mail command

line with the parameter -N:

T-Mail -N1       Task number 1 - default task

T-Mail -N9       Task number 9

As well as assigning each task a unique number, one of them must be

designated as the Master task (#1), this is handled by the Task_Mode

command.

[1] Task_Mode Master       ; Only task #1 is the Master task

[2] Task_Mode Slave        ; All others are Slave tasks

[3] Task_Mode Slave

...

The copy of T-Mail which was initiated with the command line parameter

-N1 will be designated as the Master task and assigned the tasks of mail

packing.  All other task will be designated as Slave tasks and will handle

the unpacking of mail.

The commands given in T-MAIL.CTL and EVENTS.CTL are generally

applicable to all tasks.  To define a specific command to a specific task,

it can be prefixed with the relevant task number.

[1] Log T-Mail1.Log

[2] Log T-Mail2.Log

...

The use of dedicated packer

���������������������������

The viewing of mail, creation of mail packets and other functions

performed by the Master task can be very time-consuming.  An example of such

would be processing of EchoMail.  It is possible, and even higly desirable,

to have a task dedicated only to this purpose and with no responsibilities

for modem communication.  This is achieved by setting the modem port of the

Master task to 0.

The example (T-Mail.ctl):

[1] Task_Mode Master

[2] Task_Mode Slave

...

[1] Port 0

[2] Port 1

...

In the example configuration fragment, task 0 will be responsible only

for mail packing, execution of external programs etc.

Flags and semaphores for multiline operation

��������������������������������������������

The Master/Slave concept has another possibility to allow distributed

processing by the creation and identification of flags:

example (T-MAIL.CTL):

~~~~~~~~~~~~~~~~~~~~

[1] Task_Mode Master

[2] Task_Mode Slave

...

[1] Process_Flag anyarc.t-m c:\T-Mail\echoes.bat

[2] Process ArcMail Create anyarc.t-m

...

As shown above, task #2 has the possibility to create a flag (anyarc.t-

m) and task #1 will, upon recognizing the existence of the flag, execute the

batch file (echoes.bat).  The same technique may be used to establish the

execution of other processing functions.

����ͻ

� 22. Features specific to the OS/2 version

���������������������������������������������������������������������������

A special version of T-Mail exists for running under the IBM operating

system OS/2, version 2.0 and above.  The vast majority of features of the

Dos and OS/2 versions are identical, but some differences are noted here.

The main difference is the execution of external programs when the "!"

flag is used on the command line.  Under OS/2 this forces T-Mail to close

the Com port before the external program is called, it can be opened again

upon re queueing.  You may start OS/2 tasks as well as Dos tasks.  The main

advantage of using OS/2, is the possibility of using parallel tasks.

example:

~~~~~~~

Process BBS !Do_bbs.cmd *P

(run do_bbs.cmd, but close the COM driver).

For the commands (Process, ExternMailer etc), if the macro *O is used,

it will be replaced by the handle of the open COM port.

Under OS/2, the combination of *O and ! have no meaning as the COM port

is closed at the start of the external process.

The command line parameter -K and the control key Alt-K, also have no

meaning under OS/2.

An additional variable is available in T-MAIL.CTL, Snoop.  It defines

the name of the channel (pipe) for e Snooper program.

example: (t-mail.ctl)

~~~~~~~~~~~~~~~~~~~~

Snoop         \pipe\t-snoop

It is possible also to use the program PMSNOOP by Peter Fitzimmons.

����ͻ

� 23. Command line parameters

���������������������������������������������������������������������������

-L<file> name of the log-file (default T-MAIL.LOG)

-N<Num>  Task number, <Num> should be a number from 0

to 255.  i.e. it is possible to use up to 256

tasks.

-I       forces T-Mail to ignore executed Poll, RePack, Exec and

to execute again.

-W       prohibits the releasing of timeslices under multi-tasking

operating systems (DesqView, Windows, os2

Dosbox, etc).

-D1      terse log.

-TT      Load T-Mail in Terminal emulator mode.

-T[<n>]  the number of internal buffers for stacks of nodelist

Default is 60, minimum 5, maximum 100.

-K       activates the screen blanker.  Not meaningful under OS/2.

����ͻ

� 24. Errorlevels returned by T-Mail

���������������������������������������������������������������������������

Errorlevel 0 - 10 are reserved for T-Mail and defined below:

0  reserved

1  Internal data structure error (self-diagnostics)

2  T-MAIL.CTL not found or not enough memory to handle

the variables found in the configuration file

3  Insufficient free disc space (less then 32 Kb)

4  reserved

5  Cannot initialize the modem

6  reserved

7  Cannot initialize the FOSSIL driver

8  reserved

9  reserved

10  Alt-X key (user break) depressed

If the disc becomes full during the reception of a file, errorlevel 3

will be given.

�����������������������������������������������������������������������������

����Section II����Additional capabilities of the commercial version������

�����������������������������������������������������������������������������

����ͻ

� 25. Operation with FAX messages

���������������������������������������������������������������������������

T-Mail has a built-in capability of operation with Fax modems.  An

incoming call can be from a fax machine or another fax modem and T-Mail will

accept the fax transmission without any predefined limit on the number of

pages.  Messages may be created which are bound for the fax-gateway,

converted to TIFF v5.0 GR3 and together with an accompanying fax message,

are handled in the fax processing queue.

The fax queue is processed in the same manner as a normal mail queue.

The bottom window of the T-Mail display contains a list of the mail and fax

queues, which may be alternated using the ALT-F key.

Additional hardware supported

�����������������������������

The current version of T-Mail supports all fax modems supporting the

Class 2 fax standard.  All keywords in the T-MAIL.CTL file for Fax Class 1

are reserved for further development.

-------------------- Fragment T-Mail.ctl ---------------------

Fax_Class 2                   ; Fax Class operations

; Defines the type of fax. The values 1

; and 2 for Fax Classes 1 and 2 are

; permitted.

Fax_Baud        9600          ; Max fax bit rate

; Defines the maximum speed of a fax

; transfer.  For Fax Class 2, does not

; apply.

------------------------------------------------------------------

Fax-gate

���������

Fax-gate permits users to use the system, upon which a commercial

version of T-Mail is operating, to transfer mail and messages, to a remote

fax machine.

To send a facsimile message to a remote fax machine, a service message

must be received, addressed to FAX, containing control information and the

fax text to transmit.

Before converting the incoming data, T-Mail checks the authority of the

author of the originating message to access the fax gate.  This access is

defined in the variable Fax_Mode in T-MAIL.CTL:

------------------------- Fragment T-Mail.ctl ------------------------------

Fax_Mode      Any       ; Access level to fax-gate

Fax_Mode defines the list of users, permitted to access the fax_gate.

possible values:

Any - fax-gate available to anyone

Reg - only by systems listed in the variable Fax_Nodes

Nod - only by systems,listed in the nodelist

Psw - only by password (given in the subject field),

The password is defined in the variable Fax_password

Fax_Nodes     5030/100.* !.3 !.4

Defines the list of addresses, which are permitted to use the fax_gate.

examples:

~~~~~~~~~

Fax_Nodes 5030/*.* !CM !5030/101

Fax_Nodes @c:\t-mail\f_list.txt

Fax_Password Fax_it!        ; Fax-gate password

-------------------------------------------------------------------------

messages arriving for the fax_gate, must be addressed to this system and

not to the destination (of the fax):

From: John Smith           2:245/76

To: Fax                  2:5030/15

Subj: Fax_It!

��������������������������������������������������������������������

Phone (095)123-4567

Time 0:00-6:00

Hello, dear friend!

[message body]

The first lines of the body of the message define the final destination

(telephone number) of the fax message and the times when that system should

be called.

Service control lines begin with one of the keywords defined below:

These control lines will not be transferred to the outgoing fax.

Phone <number>           -  The telephone number of the remote fax, to

which to message

Time <time period>       -  Defines the time(s) during which the

message may be sent.  Default 00:00 - 24:00

Length <page len>        -  Defines the length of the page (lines).

Default 50.

Draft                    -  Defines normal resolution, received by

default.

Fine                     -  Defines, that the fax should be sent in

fine mode (with double resolution on

vertical).

File <filename>          -  Defines the name of a graphics file, in

TIFF v5.0 G3 format, which will be sent

following the message text. Up to three

lines can be used to define graphics files

The utility TIF2TIF is provided to convert

simple TIFF files to the required format.

The only mandatory command from the above set is the Phone command to

define the number to be called.  The phone number to be called can also be

defined in the "To:" field of the incoming message:

To: FAX (095)123-4567

Examples of task of telephone numbers:

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

123-4567        -      This local number

(095)123-4566   -      long distance number

1(800)345-444   -      international number.

International telephone numbers should be presented in the same form as

in the nodelist and will be converted according to the nodelist processing

rules before being handed to the modem.

Sending fax messages

��������������������

The converted message is entered to an outgoing fax queue for dispatch.

This queue is similar to the mail queue and is displayed in the bottom half

of the screen.  In fact the two queues, mail and fax utilize the same window

and toggling between them is achieved with the Alt-F key combination.  The

arrow keys may be used to "scroll" through the fax queue and chose a

particular fax for processing.  A fax can be deleted by selecting wit the

arrow keys and pressing the "del" key.

T-Mail interrogates the mail and fax queues serially.

As well as the time periods discussed above, a global time interval may

be defined during which it is permitted to send Fax messages.  This is

defined with the variable Fax_Call.  The variable Fax_Tries defines the

maximum number of attemps which will be made to deliver a fax.

-------------------- Fragment T-Mail.ctl ------------------------

Fax_Call 17:38-8:00         ; Allow call to deliver fax

Defines the time, during which, T-Mail will call out to deliver faxes.

Fax_Tries 25                ; Attempts to send fax message

Defines the number of attemps which will be made to deliver a fax

message.  If this number of attempts are unsuccessfully made, an error

message will be reported to the system operator.

---------------------------------------------------------------------

In the case of a successful fax transmission, a message will be sent to

the originator confirming the successful transmission.

The variable Fax_Cover defines the file, which will be sent before each

message.  This file must be in Tiff v5.0 Gr3 format.

example:

~~~~~~~

Fax_Cover cover.gr3

receiving fax messages

����������������������

For fax Class 2 operations, it is possible to define various methods of

operations, i.e. data. data/fax or fax only.  This mode is defined in T-Mail

by the variable Fax_Answer in T-MAIL.CTL which gives the times during which

T-Mail will respond to incoming fax calls.  At all other times T-Mail will

respond only to data or human callers.

Incoming faxes are automatically handled in either Fine or Standard

mode.

The variable Fax_Adaptive defines how the modem will answer an incoming

call and it is recommended that this is set to Yes to allow the processing

of fax as well as data.

-------------------- Fragment T-Mail.ctl ------------------------

Fax_Answer 18:00-8:00       ; Allow incoming fax

Defines the time, during which T-Mail will allow the reception of fax

calls.  If the variable Fax_Adaptive is set to No, during this time, T-Mail

will only accept incoming fax calls.

Fax_Adaptive Yes             ; Allow adaptive fax/data answer

Informs T-Mail whether your modem support adaptive answer.  This makes

a difference for class 1 operations, but makes no difference for class 2

operations as adaptive answer is supported at all time, although it may be

prohibited.

---------------------------------------------------------------------

Format conversion TIFF v.1.1

����������������������������

There are various representations of graphic images collectively known

as the TIF format.  For T-Mail, the following general forms are used:

- uncompressed TIF (further designated as Tif),

- compressed TIF (PackB),

- compressed according to Group 3 standards T4 (Gr3).

For the storage of graphic data, the most economic is the Gr3 format

and it is this which is used internally by T-Mail.

The utility program, Tif2Tif, which is contained in the commercial

version of T-Mail, is intended to perform format conversion for the handling

of Fax images:

- convert uncompressed  TIF or PackB to TIF Gr3

- convert Gr3 to uncompressed TIF or PackB

- Note that only a single page can be converted

Calling sequence:-

TIF2TIF <in_file> <out_file> <switches>

Where:

<in_file>  - input file, any of the three types

<out_file> - target converted file

<switches> - keys, which can be such:

-T - conver to uncompressed TIF

-C - convert to PackB

-F - convert to TIF Gr3

-1 - ignore cover page and preserve the whole

image in one page

-S - Silent mode, do not display messages

examples:

~~~~~~~~

Conversion of a received fax

Tif2tif in.gr3 in.tif -T

.. but in 1 page:

Tif2tif in.gr3 in.tif -T -1

Preparing a fax message for dispatch:

Tif2tif out.tif out.gr3 -F

����ͻ

� 26. Voice answer-back facilities

���������������������������������������������������������������������������

Some modems, for example those supplied by Zyxel, are capable of

operating in voice mode.  This means that the modem has facilities to handle

the incoming voice data, convert it to a digital bit-stream and pass it to

the computer for processing.  This allows facilities to be built such as an

answering machine or voice mail.

T-Mail contains built-in functions to handle the requirements of a

voice answering machine.  The times defined, when T-Mail functions as an

answering machine are given in the variable Voice_Answer.  During this time,

in response to a voice call, T-Mail will play back a pre-recorded message

(defined in the variable Voice_Greeting) and accept an incoming message from

the remote caller.  Should a voice caller be silent for a preset time period

(Voice_Silence_Time), the modem will be switched into adaptive Fax/Data

answer mode.  If a voice call is accepted, the voice message received will

be stored in the directory defined by the variable Voice_Inbound.

Voice recording automatically stops on:

- silence longer then a given time (Voice_Max_Time);

- A BUSY signal

- A Fax CNG tone

As well as the actions above, it is possible to control the actions of

the answer-back system by using the keypad of a touch-dial telephone (DTMF

tones).  A sequence of digits must be defined as an answer_back password

(Voice_Password) and if this sequence is transmitted from the remote caller,

during a call, T-Mail will switch into play_back mode and play any messages

already received.  When listening to pre-recorded messages, the keypad can

be used, for further control, as follows:

DTMF           action

1      Next message message

2      Delete current message, play next

3      End play_back

����ͻ

� 27. Operation in a local area network (external nodes)

���������������������������������������������������������������������������

T-Mail, as well as being able to transact mail sessions between remote

system via a telephone or ISDN line, can also operate through a Local Area

Network.

Remote systems, which will transact mail sessions through an operating

system must have their addresses defined as external nodes and are described

in SUBST.LST as follows:-

<address> <parameters> External [!] <command>

The text following the keyword External, is the command required by the

operating system to be executed in order to "transmit" a file.  The

following macro commands may be used:

*F - The full name of the file to be transmitted

*A - The full destination address

The asterisk "!" may be used to indicate that no memory swap is to take

place before the command is executed.

example (subst.lst):

~~~~~~~~~~~~~~~~~~~

5030/15.77 External !Copy *F c:\points\77

The word External may be preceded by the usual parameters to define

time limitations on the availability of the remote system.

5030/15.77 Time 5:00-9:00 External Mycopy.exe *A *F c:\points\77

�����������������������������������������������������������������������������

����Section III����Additional functionality������������������������

�����������������������������������������������������������������������������

The information given in this section is intended for advanced users

only.  All of the functions described here should be used with EXTREME

caution.

����ͻ

� 28. Zmodem protocol setup

���������������������������������������������������������������������������

The protocol, used by T-Mail to handle file transfer, is Zmodem. T-Mail

supports 3 versions of this protocol:

Plain Zmodem (ZMO)    -  Standard Zmodem

ZedZap (ZAP)          -  Zmodem with a window size determined by the

connection speed.

Direct ZedZap (DirZAP) - Similar to ZedZap with enhanced to remove

protection for some service code (XON, XOFF

etc.)

For the absolute maximum of efficiency in transferring files, the

DirZap variant is the best, but for this to operate correctly it is

necessary to ensure that both the local and remote modems are set up in a

similar manner.  If the connection of the two systems is indirect in some

way, then DirZap is not possible.  The default and recommended protocol is

ZedZap.

The variable Dir_ZedZap in T-MAIL.CTL, configures the use of DirZap by

your system.  By default this is No, making the "best" protocol, ZedZap.

You may limit the ability of remote systems to use DirZap and/or ZAP,

by defining the flags ZMO and ZAP in SUBST.LST

examples:

~~~~~~~~

5030/100.4 ZAP       ; permit ZAP and ZMO

5030/100.5 ZMO       ; permit only ZMO.

Fine-tuning of the Zmodem protocol is also possible using variable in

T-MAIL.CTL.  Each variable can be different for each connection speed and

therefore 11 parameters are allowed on each variable:

ZBlockSize   The maximum size of the transmission window, in bytes

ZTimeOut     Zmodem timeout

ZStartBlk    The initial size of the transmission window, in bytes

ZMinCps      The minimum acceptable transmission speed in bytes/sec.

If the data rate falls below this figure, the session will

be terminated.

Zmodem uses a Cyclic Redundancy Check (CRC) for error detection.  Two

variants exist, CRC-16 and CRC-32, while CRC-32 is significantly more

secure, it can reduce (slightly) the transmission efficiency.  The user may

select the error detection protocol by defining the variable ZCRC32 in T-

MAIL.CTL:

ZCRC32 Always (or Yes)    ; Use CRC-32 if the remote site supports it

Never (or No)      ; Use CRC-16 only

Smart              ; If error detection/correction is used in

the basic connection (MNP, V42), then use

CRC-16, otherwise - attempt to use CRC-32.

By default, Smart is used.

You may define a set of file names which are to be refused from remote

systems.  When an attempt is made to transmit one of these files to your

system, it will be skipped by the Zmodem protocol.  File names are defined,

according to Dos naming conventions, by the variable ZSkipFile.  Up to 16

such names may be defined in T-MAIL.CTL.

examples:

~~~~~~~~

ZSkipFile LPT*.*

ZSkipFile CON

ZSkipFile COMMAND.COM

ZSkipFile *.exe

Before receipt of a file, T-Mail check on the date of creation/update,

size and name.  If a file already exist, in the inbound directory, with the

same name, date and size, it will be skipped.  If a file exists, in the

inbound directory, with the same name and date, but a smaller size, it will

be considered as a "broken" file and file reception will continue from the

point of break.  If the dates differ, the existing file is renamed to allow

the incoming file to be accepted.  Renaming is performed on the LAST

character of the extension of the file name, in that it is incremented { A-

Z, 0-9 } Until a unique file name is found.  If no unique name can be found,

the file will be overwritten.

You may define, up to 16, filenames which will ALWAYS be accepted even

if the file already exists.  File names are defined, according to Dos naming

conventions, by the variable ZOverWrite.

examples:

~~~~~~~~

ZOverWrite *.PNT

ZOverWrite READ.ME

You may also define, using the variable ZFlush, the amount of

information which will be received before a buffer flush is performed.  This

is not necessary or recommended if you are using a disk cache with "lazy

write" configured.

The following fragment of T-MAIL.CTL describes the commands for the

Zmodem setup:

; =========================== Zmodem Definitions===========================

;

;           Definition of parameters for Zmodem protocol

;

;---------------------------------------------------------------------------

;

; Dir_ZedZap       YES              ; Enable Direct ZedZap protocol

;

; Set to YES, will enable the use of the variant Direct ZedZap if

; the remote system supports it, Set to NO will prohibit Direct

; ZedZap even if the remote system supports it.  Default - NO

;

ZCRC32             Smart            ; Options for CRC-32 in Zmodem

;

; Defines the type of CRC checking to use when sending or receiving files.

; Default is Smart.

;

;           Always (or Yes)    - Use CRC-32 if the remote site supports it

;           Never (or No)      - Use CRC-16 only

;           Smart              - If error detection/correction is used in

;                                the basic connection (MNP, V42), then use

;                                CRC-16, otherwise - attempt to use CRC-32.

;

;

; ZBlockSize  1024,1024,2048,2048,4096,4096 ,8192 ,8192 ,8192 ,8192 ,8192

;

; baud rates: 1200,2400,4800,7200,9600,12000,14400,16800,19200,21600,28800

;

; Defines the 11 maximum block sizes for transmission at various connect

; speeds.  The comment "baud rates" is used to indicate at which speed the

; block size is effective.   The values of ZBlockSize are the defaults.

;

;

; ZTimeout    10,  10,  10,  10,  10,  10,   10,   10,   10,   10,   10

;

; baud rates: 1200,2400,4800,7200,9600,12000,14400,16800,19200,21600,28800

;

; Defines the 11 timeouts, in seconds, at various connect speeds.  The

; comment "baud rates" is used to indicate at which speed the ; block size

; is effective.  The values of ZTimeout are the defaults.  If a connection

; is made with MNP, V42 or HST, these values are automatically multiplied by

; three.

;

;

; ZMinCps     20,  80,  100, 200, 200, 300,  400,  500,  600,  600,  600

;

; baud rates: 1200,2400,4800,7200,9600,12000,14400,16800,19200,21600,28800

;

; Defines the minimum acceptable transmission speed in bytes/sec.  If the

; data rate falls below this figure, the session will be terminated.  Great

; care should be used when defining this variable and by default the

; facility is not used.  i.e. all values are set to zero.

;

;

; ZStartBlk   512, 512, 512, 512, 512, 512,  512,  512,  512,  512,  512

;

; baud rates: 1200,2400,4800,7200,9600,12000,14400,16800,19200,21600,28800

;

; Defines the initial sizes of the ZModem window for various connection

; speeds.  The values of ZStartBlk are the defaults.

;

;

; ZFlush_Size 4096

;

; Defines the size of received data before a buffer flush, to disk, occurs.

; By default this is set to 0, indicating no forced buffer flush.  It is not

; recommended to use this variable if a disk cache or FOSSIL driver is

; already in use.

;

;

; ZSkipFile LPT*.*

;

; Defines the name(s) of files which will not be accepted for upload to this

; system.  Dos wildcards may be used and up to 16 such names may be defined.

;

;

; ZOverWrite *.PNT

;

; Defines the name(s) of files which will always be overwritten by incoming

; files of the same name.  Dos wildcards may be used and up to 16 such names

; may be defined.

;

; Zchat Yes

;

; Defines whether 'chat' will be permitted during Zmodem file transfer

; sessions.  Default - Yes.

;

; ZframeType <type>

;

; Defines the type of frames to be sent in the Zmodem transmission.

;

; Types are:-

;

;     ZCRCG (default)  - usual frames without confirmation

:     ZCRCW            - frame, following which confirmation is expected

;     ZCRCQ            - frame, following which confirmation is sent but

;                        not expected.

;

;--------------------------------------------------------------------------

����ͻ

� 29. Connection of key generators

���������������������������������������������������������������������������

T-Mail permits the use of so-called "key generators."  Basically this

means the automatic processing of arriving files, with a pre-specified file

name.

The function is handled by the 'Process' variable with the first

parameter defined as 'online' i.e.

Process online ......

This is followed by the definition of the arriving file to be detected:

Process online LAS*.REG ..........

This is followed by the program or batch file which should be executed

in response to this file:

Process online LAS*.REG Makereg.bat *A *D *B

In this manner ANY arriving file may be handled on-line and the results

returned to the caller.

It should be borne in mind however, that the online processing should

not be a lengthy process as the session could timeout and be abnormally

terminated.  T-Mail is capable of holding the line for about 90 seconds, but

other mailers may be less "tolorant".  Any on-line

����ͻ

� 30. Additional functionality

���������������������������������������������������������������������������

This section describes additional commands which may be included in T-

MAIL.CTL

Show_Dial_Prefix Yes

Defines whether the information, sent to the modem before

the telephone number will be displayed on the screen and

written to the log file.  By default this facility is

switched off (No).

(Yes)

... Calling 2:5030/15.0, ATDP\N4%C0580-2383,,,...

(No)

... Calling 2:5030/15.0, 123-4567...

Purge_Kbd Yes

Defined as Yes will cause the keyboard buffer to be

cleared before initializing the modem.  Default - Yes.

CH_Interval <time> Defines the time, in minutes, during which a queue status,

activated by keyboard command, should be active.  It also

defines the time for traffic delay according to HAT.

Default value is 60 minutes.

EMSI_Log <fname>   Defines the name of the log file name for EMSI messages.

Default is no log of EMSI message is maintained.

Dial Insert <char> Defines the symbol, to be inserted between each character

in the dial string.

Example:

~~~~~~~

Dial Insert ,

The line, sent to modem when dialing 235-7719 will be:

"2,3,5,-,7,7,1,9".

Force_Fossil No    Defines if the FOSSIL driver should be initialized before

each initialization of the modem.

FnReq <filename> <listname> [<command | dvpname>]

If <filename> is received during a session, a flag file

is created, called CALLER.INF, the <command> is executed

or the task <dvpname> is started under Desqview.  In this

manner the files listed in <listname> can be processed.

The file, CALLER.INF, contains information on the calling

system as follows:

line 1: Full address of remote system

line 2: name of remote system

line 3: name of system operator

line 4: directory, where the received file is stored

line 5: 'No Key' string for compatibility with D'Bridge.

����ͻ

� 31. Additional command-line parameters for T-Mail

���������������������������������������������������������������������������

When starting T-Mail, as well as those command-line parameters

described in chapter 23, the following may be used:

-C<file>   Use the defined configuration file instead of the default

T-MAIL.CTL

-X         Copy the nodelist(s) and substitution lists from the Master

system to the Slave system in a multi-line environment.

-F         In ArcMail Attach mode, inhibit the nodelist display within

queues.  In BinkleyTerm mode, inhibit the display of *.?LO

-S         Inhibit the display of modem lights, recommended if you are

running a slow computer ro using non-standard addresses.

-2F        Forces the use of DPMI for giving up timeslices.

�����������������������������������������������������������������������������

�����Section IV�������Help information������������������������������

�����������������������������������������������������������������������������

����ͻ

� I. Examples and additional help information

���������������������������������������������������������������������������

Given below is an example of an elementary command file, demonstrating

the ability to work with Errorlevel completion codes, returned by T-Mail.

@echooff

:begin

T-MAIL.EXE

rem

rem           --- Check errorlevels, Requeued T-Mail ---

rem

if errorlevel 200 goto weekly

if errorlevel 100 goto daily

if errorlevel 10 goto altx

if errorlevel 7 goto fossil

if errorlevel 5 goto modem

if errorlevel 3 goto done

if errorlevel 2 goto done

if errorlevel 1 goto diag

if errorlevel 0 goto done

:weekly

rem

rem           --- Run weekly.bat, once on week ---

rem

call weekly.bat goto begin

:daily

rem

rem           --- Run daily.bat, daily ---

rem

call daily.bat goto begin

:altx

rem

rem           --- Log information on completion reason ---

rem

echo Alt-X pressed, exit>> t-mail.log

goto done

;fossil

rem

rem           --- FOSSIL ERROR ---

rem

Echo FOSSIL initialization error>> t-mail.log

call fossil.bat

goto begin

:modem

rem

rem           --- Completion of external modem initialization ---

rem

call init.bat

goto begin

:diag

rem

rem           --- reload on internal error ---;

rem

goto begin

:done

rem

rem           --- Completion of work ---

rem

����ͻ

� II. Tools and libraries used to create T-Mail

���������������������������������������������������������������������������

T-Mail was created using the Borland C++ compiler and debugger.  All

source code has been created using the integrated environment, BC++.  All

initial code, except for the swap procedures (Mikhail Likhachev), X-files

(Sergey Terehov) and color setup, TCE (Alex Kopylov), are written and

organized by the author.  No additional modules, other than those supplied

with Borland C++, have been utilized.

����ͻ

� III.  Standards and documents

���������������������������������������������������������������������������

1. FTS-0001 A Basic FidoNet (r) Technical Standard

Revision 15

Randy Bush, Pacific Systems Group

August 30, 1990

2. FTS-0005 The Distribution Nodelist

Original by Ben Baker, 7:44/76@AlterNet

Amended by Rick Moore, 1:115/333@FidoNet

February 5, 1989

3. FSC-0008 Fundamentals of FOSSIL implementation and use

Draft Version 4 August 10, 1987

Vincent E. Perriello, VEP Software

4. FSC-0009 Nodelist Flag Draft Document

Primary Author: Ray Gwinn

Secondary Author: David Dodell

Version 1 (11-15-87)

5. FSC-0020 Alternate Nodelist Flag Proposal

By Marshall Presnell, (109/639.106)

November 13, 1987

6. FSC-0025 AVATAR

Advanced Video Attribute Terminal Assembler and

Recreator

George A. Stanislav, 129/39

7. FSC-0041 MSGID/REPLY

A proposal for unique message identifiers

And reply chain linkage

Jim nutt, 1:114/30@fidonet

8. FSC-0046 A Product Identifier For FidoNet Message Handlers

Draft II

Joaquim Homrighausen, 3:632/308@fidonet

9. FSC-0056 EMSI/IEMSI Protocol Definitions

Joaquim H. Homrighausen, May 3, 1991

10. FSC-0062 A Proposed Nodelist flag indicating Online

Times of a Node

David Thomas, 2:253/600@fidonet, May 10, 1992.

����ͻ

� IV. Thanks

���������������������������������������������������������������������������

The enormous task of putting this product together, testing,

documenting, improving etc, has only been possible with the widespread help

of SysOps from Russia, The Ukraine, Belorus, Estonia, Chekhia and Germany.

The amazing quantity of ideas, which have appeared in T-mail during the last

year, has been the result of a never-ending stream of feedback from the

relentless participation of many Beta-testers around the world.  To them and

to the many others who have helped to make T-Mail the product which it is

today, I express my sincere and unreserved thanks.

Special thanks go to:

Peter Suchkov     - For preparation of the manual

T-mail.doc.

Antony Guetmansky - for the design of screens etc.

Alex Korchmar , Andry Fridman , Gleb Yalchik

- for editing documentation.

Dmitry Osipov     - For the design of the Fax fonts and

documentation editing

Alexey Saminsky   - for help in the nodelist compiler.

Serge Terekhov    - For the library for handling files

(X-files).

Michael Likhachev - For solving problems with swapping.

Alex Kopylov      - For the nodelist compiler TCE.

����ͻ

� V. Trademarks

���������������������������������������������������������������������������

T-Mail        Is a registered trademark of Andy Elkin

Fido, Fidonet are registered trademarks of Tom Jennings and Fido Software

MS-DOS        Is a registered trademark of Microsoft Corporation

PC-DOS        Is a registered trademark of International Business

Machines Corporation

OS/2          Is a registered trademark of International Business

Machines Corporation

Borland C ++  is a registered trademark of Borland International, Inc.

DesqView      is a registered trademark of Quarterdeck, Inc.

Remote Access is a registered trademark of Continental Software

Maximus       is a registered trademark of Scott J. Dudley

FrontDoor     is a registered trademark of Joachim Homrighausen

BinkleyTerm   is a registered trademark of Bit Bucket Software Co.

Hayes         Is a registered trademark of Hayes Microcomputer Products

ZyXEL         Is a registered trademark of ZyXEL Communication Corp.

����ͻ

� VI. Conclusion

���������������������������������������������������������������������������

Almost without doubt it is impossible to produce a software program, of

this complexity, which is 100% bug-free.  It is to be hoped however that any

remaining bugs in T-Mail are minor in nature and will not detract from your

use and enjoyment of the product.  For support of this and associated

products, an international support EchoMail conference has been established,

called T-Mail.support.

*      *      *

#