VGATEST: Utility for testing LOD's VGA support on your hardware

(c) Scott M. Baker, 1995

Description

-----------

LOD versions 4.20 and above include support for SVGA graphics, in the

modes 800x600x256 and 640x480x256. This program is designed specifically

to test the operation of the 800x600x256 mode.

VGATEST should be operated from within your LOD directory because it makes

use of several of LOD's files including FONT8x8.FNT, FONT8x12.FNT,

LODVGA.PAL, BMPSTD.RES, and BMPBIG.RES.

Command Line Parameters

-----------------------

/NOACCEL  ... disable S3 accelerator checking & tests

Benchmarks/tests

----------------

The tests in VGATEST's benchmark section are listed below in order of

occurrence:

[compatibility tests. These are automatic tests used by VGATEST to ensure

that certain portions are working correctly. They are only executed once

and probably occur too fast to be seen]

- S3 test. If your video card has an S3 coprocessor, then VGATEST will

tell the S3 to draw a horizonatal line at coordinates (0,0) and check

to make sure the S3 did what it was supposed to. This test happens

very quickly and you probably won't notice it.

- Flipper test. The flipper test will draw approximately 10 rectangles

along the left hand side of the screen. VGATEST will use this to

determine if the page flipper is working properly. This test executes

very fast and has only one iteration, so you probably won't even

see it.

[multiple iteration tests. These tests are executed repeatedly for a total

of 5 seconds per test. The number of iterations that occurred during the

5 second interval is recorded and used as a benchmark]

- Cleardevice test. The entire screen is repeatedly cleared to a random

color.

- Fillrect test. Random filled rectangles are drawn.

- Horizontal line test. Horizontal lines are drawn in random colors.

- Vertical line test. Vertical lines are drawn in random colors.

- Terrain bitmap test. Small (51x31) terrain bitmaps are drawn randomly

in the top of the screen.

- Bigpic bitmap test. Large (181x121) bigpic bitmaps are drawn randomly

in the top of the screen.

- 8x8 font test. An 80x25 text window is defined in the middle of the

screen and text is written and scrolled through it. The message

should read:

"Screen scrolling simulation. ****** TESTING ****** TESTING ******"

- 8x12 font test. Same as the 8x8 test, but with a larger font.

- 8x14 font test.

Warning Messages

----------------

- "WARNING: S3 coprocessor test FAILED. Perhaps no S3 is present?"

VGATEST detected an S3 coprocessor, however when VGATEST tried to use

it, the coprocessor didn't work. Either VGATEST was wrong and you never

had one to start with, or some incompatibility is present.

You should probably use the /NOACCEL switch when using LOD and/or GTERM.

- "WARNING: VESA flipper had to be forced to int mode!"

VGATEST had to switch the VESA flipper to interrupt mode to get pages

to flip. VBE supports two methods of switching pages: By use of a far

call and by an interrupt instruction. The far call is more efficient,

but not all VBE adapters work properly with it. Thus, using the interrupt

mode flipper may be necessary for compatibility reasons.

You will need to use the /FORCEINT switch when using LOD and/or GTERM.

- "WARNING: VESA flipper had to be forced to dual mode!"

VGATEST had to switch the VESA flipper to dual mode to get pages to

flip. The VBE standard supports up to two "windows" into video memory.

Some VBE adapters designate one window as read and one as write. In this

case, both windows may need to be moved to perform the desired graphics

operations.

You will need to use the /FORCEDUAL switch when using LOD and/or GTERM.

VGATEST revision history

------------------------

5/16/1995

- S3 coprocessor code added. Tested on Diamond Stealth 64 & Stealth ISA

- OAK support added. Untested.

- Support for dual window VBE adapters added. Dual window is selected if

(WinAAttr and 7)!=7. (i.e. WinA is not supported, not readable, or

not writable)

- VGATest will detect page flipper failure.

- If Page Flipper Failure occurs on a VBE system, then VGATest will

attempt Dual and Int mode VBE flippers.

- Removed a confusing and possibly erroneous function call.

- Modified TSENG flippers to flip both windows.

- Modified benchmarks to run for a specified amount of time (5 seconds)

rather than a specified number of iterations.

- Fixed support for VBE adapters which use 1024 bytes/line in 800x600

mode (incl. Diamond Stealth ISA)

5/22/1995

- Added error checking to exit gracefully if fonts or palette cannot be

loaded

- Tests 8x12 font and 8x14 font in addition to 8x8 font

- Correct palette (LODVGA.PAL) file is now loaded

- Documentation update