CACHECHK

v2 11/2/95

Copyright (c) 1995 by Ray Van Tassle.

This is NOT freeware.  This is postcard-ware.  Send me a nice

picture post card as the registration fee.

Ray Van Tassle

1020 Fox Run Lane

Alqonuin, Il. 60102 USA

(708)658-4941

CACHECHK performs memory access timing tests, to see if you have a

cache, how many caches, and to check the access speed.

There are two kinds of caches,  the on-chip cache (level one, or L1)

which is in the CPU processor chip, and off-chip cache (level two, or

L2) which is on the motherboard.

Some 386 motherboards have 64KB of off-chip cache.

Some CYRIX 386/486 chips have a very small L1 cache.

486's have an on-chip cache, and most new motherboards also have a L2

cache.

The AMD 486 has 8KB of L1 cache.

The Intel 486 has 16KB of L1 cache.

Intel Pentium has 16KB of L1 cache, 8KB for data and 8KB for code.

The typical 486 MB has 256KB of L2 cache, although many will let you

install 512KB or even 1MB.

Q: HOW MUCH CACHE DO I NEED?

For a 486, get a 256K cache.

All the writeups that I have seen say:

64KB cache gets a LOT of improvement.

128KB gets a bit more.

256KB gets a teeny bit more.

512KB gets only a teensy weensy bit more.

(Note: these are for DOS and WINDOWS3.  A real OS, like LINUX, OS/2,

and perhaps WIN-95 and WIN-NT may be different.)

In general, this seems to be the way all caches work---the first bit

gets a lot of bang, and each additional bit gives smaller and smaller

improvements.

It is claimed that, as long as you have a decent L2 cache, that an AMD

(with an 8k L1) is virtually identical in performance to an Intel 486

(with a 16K L1).

256K uses 8-(32K x 8) SRAM chips (8 * 32K = 256KB), in two banks,

and the MB can interleave accesses to the two banks.

512K uses 4-(128K x 8) SRAM chips (4 * 128K = 512KB, but this is only

one bank, so the access time is slower, because the MB can't do bank

interleaving.

So, 512K costs a LOT more than 256K and gives only a marginal improvement

in performance, so stick with 256K.

Q: Why does my main memory show faster with the cache enabled, even

out beyond the cache size?

A: The L1 cache (on a 486) is filled in granularity of 16 bytes (this

is the "cache line size").  When you read a byte, all 16 bytes of that

line are read into the cache, in 4 quadbyte (32 bit) units.  The quadbyte

which is addressed is read from memory first, put into the cache, and

transferred to the CPU.  Then the other 3 quadbytes are read into the cache.

So, if you are accessing the memory in sequential order (like CACHECHK does),

the next bytes have been (or are being) automatically sucked into the cache,

a "read-ahead", if you will.

If the cache is disabled, this read-ahead does not take place.

CACHECHK will run the access tests using all the memory in your

machine, so that you can check to make sure that all the memory is

cached.

Usage: CACHECHK -tn -hfvwqz? [Optional comments]

Cache memory detector & timer.  Runs only on 80386 (or better) CPU.

-h? = Print this help text.

-f  = Perform tests with cache disabled.

-n  = Don't calibrate timer.

-q  = Quick. Faster but not as accurate.

-qq = Each 'q' is 2 times quicker. But less accurate.

-tn = Top of memory to test. n = nth MB

-v  = Verbose

-w  = Do memory write (otherwise memory read).

-z  = Slower.  Like q, but the other direction.

Probably won't be needed on anything less than a 586DX4/200.

The "optional comments" just get logged, so you can identify test information

along with the results.  Mostly useful if you redirect the output to

a file. For example:

cachechk -q Test with bios set to 1 wait state >before.dat

The basic timing loop is 1/2 second per size.  Each 'q' cuts the time

in half.  Each 'z' doubles the time.  If the timing figures aren't

steady, you probably have too many q's.

You can run the tests with the cache disabled, with the 'f' option (on

a 486 or Pentium).  Naturally, it is re-enabled again when it's done.

This generally turns off BOTH the L1 and L2 cache.  Your bios setup

may (or may not) let you individually enable/disable the caches.

Times cache & memory access, and figures out cache size(s).

Results are in:

microseconds per KB, MB per seconds, and nanoseconds per byte.

Memory is access in quadbytes, in flat 32-bit protected mode. For base

memory (first MB, MB#0) only 640KB is accessed. Memory accesses are in

various block sizes, from 1KB to 2MB. Each megabyte is tested starting

at the beginning of that megabyte.  CACHECHK will work under a memory

manager (HIMEM, EMM386, QEMM, Windows, etc.), but the results may be

inaccurate, the machine might crash, and it won't be able to test all

of the memory. It will run under WINDOWS, but results are wildly

inaccurate. For best results, boot clean--on DOS 6 & above, hit F5

while it boots.

It will NOT touch extended memory that is already allocated or in use.

If you have a memory manager installed, it usually occupies the first

portion of the 2nd megabyte, so CACHECHK will not be able to check that.

SOME TIMINGS I HAVE TAKEN

CPU	L1	L1speed	L2	L2Speed	Mem	Speeds

type	siz	ns/byte	size	ns/byt	ns/byt	�sec/KB

------	----	-------	----	------	------	----------

386/25	0	n/a	 64k	59.2	90	62......94

486/33	8k	30.7	128k	43.6	70	31..45..73	(Intel)

486/66	8k	16.1	0	n/a	50	15......52	(Intel)

486/100	8k	11.1	0	n/a	46	10......48	(AMD)

486/100	16k	10.0	256k	18.8	26	10..19..27	(Intel)

P-75	8k	10.2	256k	16.4	24	10..17..24

Timer

-----

CACHECHK re-programs the timer chip to get a high-precision timer (about

1,200,000 ticks per second).  In some motherboards (notibly reported

to be "UMC with fake cache chips"), there is a flaw with this timer.

I worked around this in version 2, but there may be some boards where my

work-around still doesn't work.

Normally, CACHECHK calibrates this timer against the Real-Time-Clock, which

is in all systems 286 and above.  If your RTC isn't standard, this may hang

or give bogus values.

I count the number of timer ticks in 1/16th second. When four in a row

give the same value (within .01%), I take the average of those four counts,

and use this as the calibration value.

If there is some problem with this calibration, use the "-n" switch, and

it will use the pre-defined value of 1,193,728 (0.84 microseconds per tick).