Connecting to Compu-Data CSLIP with Windows 95

This procedure requires the CD-ROM version of Windows 95 or the DSCRIPT.EXE file. The Plus Pack is not required nor has it been tested for compatibility with this procedure.

Install Dial Up Networking

Open the Windows help topic Connecting to the Internet using Dial Up Networking.  Follow the step by step instructions.  When asked enter the following information:

- Username - the first and last name you use when logging on to Compu-Data separated by a single space.

- Password - Your Compu-Data Password

- Compu-Data's phone number - 232-7747 (add any prefixes unique to your location).

- Host Name - none

- Domain Name - compudata.com

- Domain Name Server (DNS) Address -  204.183.163.2

Install CSLIP & Script Utility

If you do not have the CD-ROM version run DSCRIPT.EXE before continuing

- Open Control Panel

- Double Click on Add/Remove Programs

- Click on the Windows Setup Tab

- Click on Have Disk

- Fill in the Copy manufacturers files from field with X:\Admin\Apptools\Dscript  (Substitute the letter of your CD-ROM drive for the X)  If you are using DSCRIPT.EXE enter the directory path that contains the expanded files.

- Under Components  click on the empty box to the left of SLIP and Scripting for Dial-Up Networking

- Click on Install

- When installed click on OK to close the Window

- Open the Dial-Up Networking folder

- Right click on the connection icon for Compu-Data

- Click on Properties

- Click on Server Type button

- In the Type of Dial-Up Server window select CSLIP:Unix Connection with IP header Compression

- Check both Log on to Network and TCP/IP

- Click on OK

- Click on the Configure button

- Click on the Options tab

- Uncheck all boxes except Display Modem Status

- Click on OK

- Click on OK

Setting up the automatic logon script

- Copy the SLIPlog.scp file to the C:\ProgramFiles\Accessories directory

- Open the Dial-Up Scripting Tool in the Start Menu under Programs/Accessories

- You should have a listing for the Compu-Data Dial-Up connection you created when you installed the Dial-Up Networking

- In the file name window fill in the location of the SLIPlog.scp file

- Uncheck the Step through Script box

- Check the Start Terminal Screen Minimized box

- Click on Apply, then Close

Connecting to Compu-Data

- Double click on the Connection Icon you created for Compu-Data in Dial-Up Networking.  (My Computer/Dial-Up Networking).  If everything is properly configured  the Connect To Window will open.  Fill in your User name and Password, check the save password box if you do not wish to enter your password every time you log on.

- Click on Connect

- The Modem will Dial

- The Connect To window will indicate verifying User Name and Password,  after a few moments it will indicate logging on to Network, then it will indicate connected and show the connect speed and elapsed time on line

- Start Netscape or other Internet client.

It didn't work! - Checking your settings

You didn't really think it would work on the first try, did you? ;)

Checking the Dial-Up Networking settings

- Open the Dial-Up Networking folder

- Click on Connections/Settings

- Redial should be checked

- The numbers in the boxes should be set to your personal preferences

- Prompt to use Dial-Up Networking  should be checked

- Click on OK

Checking the Compu-Data Connection

- Open the Dial-Up Networking folder if it is not already open

- Right click on the Icon for you Compu-Data Connection

- Click on Properties

- Check the telephone number and correct if required

- Under Connect Using make sure the correct modem is shown in the window and change if required

- Click on the Configure button

- Choose the General tab

- Is the COM Port correct?

- Is the Maximum Speed correct - try 38400 or 57600 for a 14.4 modem, double that for a 28.8 modem. Lower the speed if you are getting errors on file transfers or if the system seems sluggish

- Check Only connect at this speed

- Click on the Connection tab

- Set Data Bits to 8, Parity to None, Stop Bits to 1

- Under Call preferences check all boxes

- Choose the Options tab

- You should have a check next to Display Modem Settings, all other boxes should be unchecked.

- Correct as needed and click on OK.

- You have returned to the prior window, under Connect using, click on the Server type button

- Under Type of Dial-Up Server select CSLIP:Unix Connection with IP header Compression

- The Log on to network and TCP/IP boxes should be checked

- Click on the TCP/IP Settings button

- Server assigned IP address, Server assigned name server, Use IP header compression, and Use default gateway should all be checked

- Click on OK, then click on OK , then click on OK  again to close all the open windows

Checking the Network settings

- Double click on the Network icon in the Control Panel

- Choose the Configuration tab

- In The following network components are installed window you should see at least three items listed.  They are Client for Microsoft Networks, Dial-Up Adapter, and  TCP/IP.

- Highlight Client for Microsoft Networks and click on the Properties button

- Quick Logon  should be checked, other items should all be unchecked

- Click on OK

- Highlight Dial-Up Adapter and click on the Properties button

- Choose the Driver Type tab

- Enhanced mode (32-bit and 16-bit) NDIS  driver should be checked

- Choose the Bindings  tab

- TCP/IP should be checked - any other listed items should not be checked

- In the Primary Network Logon  window, Client for Microsoft Networks  should be listed

- Click on OK

- Highlight TCP/IP and click on the  Properties button

- Choose the Gateway tab - All items should be blank

- Choose the WINS Configuration tab - all items should be blank

- Choose the IP Address tab

- Obtain an IP address automatically  should be checked

- Choose the Bindings tab - Client for Microsoft Network should be checked

- Choose the Advanced tab - it should be empty

- Choose the DNS Configuration tab

- Enable DNS should be checked, Host  is None, in the DNS search order window the address should be 204.183.163.2  -  If it is not, delete any addresses listed and Add it

- Click on OK , then click on OK  again to close.

- REBOOT YOUR SYSTEM  - some changes may not take effect unless the system is rebooted.

I am sure that somewhere along the line I have missed a few things.  If you find any errors or omissions please drop me a note on the Main Board (Joe Fitzpatrick) or via e-mail (joe.fitzpatrick@compudata.com).

9/2/95