There is an "MID" (Message IDentifier) associated with each message.

If an MID is not given explicitly with the "Send" command, one is

created automatically from the message number and callsign of the MailBox

into which the message was initially entered. It has the form nnn_call.

There are 3 types of messages:

1) Personal.	If sent with SP, or with S and to a callsign.

2) NTS traffic. If sent with ST.

3) Bulletins.	If sent with SB, or with S and NOT to a callsign.

For backward compatibility, any message sent with Sx, where x is NOT

B, P, or T is also considered a bulletin. These types have been

used in the past for bulletin type sorting.

Each type of message gets somewhat different handling:

For NTS traffic, the LT, KT, and ET commands are active.

For Bulletins, a BID (the MID) is sent when forwarding to MBL systems.

For Personal, the message can only be rad by the sender, addressee, and sysop.

There are several "flags" associated with each message. These are shown in

the "message status" position in the "list message" display. Note that each

flag has an associated "L" command, and some have associated "K" commands.

F - The "Forwarded" flag:

This indicates the message has been forwarded to all

its destinations, but has not yet been killed.

H - The "Hold" flag:

This indicates the message is held.

It will not forward, and can only be seen by the sysop.

I - The "In process" flag:

This indicates the message is in the process of being forwarded.

K - The "Killed" flag:

This indicates the message is killed, but has not yet been purged

from the system. Killed messages are purged with the GM command.

O - The "Old" flag:

This indicates the message has been hanging around

un-forwarded and un-read for too long.

Y - The "Read" flag:

This indicates the message has been read by its addressee,

but has not yet been killed.

How do BID's work?

The various commands (S, M, CM) work exactly the same.

The basic command is S[type] TO [@ AT] [< FROM] [$[MID]]

Data inside [] may be omitted.

Messages differ in the following ways:

TO gets translated.

TO is a callsign.

TO is an interest group.

AT gets translated.

AT is a callsign.

AT is a distribution list.

$ field is present.

$ field is present, with BID.

Type is B

Type is P

Type is T

Type was not specified.

Message is held.

A type B or P message gets a BID if the command that creates

the message has the "$" field. A message of type B or P gets

a default BID if none was specified and it has a distribution

list. A message of type T never gets a BID. In the discussion

below, the same rules apply whether the message was created

using the S, M, or CM commands.

Here is how the system behaves:

1) If the user sends the message with "$ID" given in the command,

the message is assigned identifier "ID". If this identifier

has been seen before, the message is rejected and the text

"NO - Already have it." is displayed.

2) If the user sends the message with "$" given in the command,

the message is assigned a unique MailBox generated BID. This BID

is generated from the message number and callsign of the MailBox.

The message is accepted, since this BID cannot have been seen before.

3) If there is a distribution list, and a BID is not given

with the command, a unique MailBox generated BID is assigned.

This BID is generated from the message number and callsign

of the MailBox of origin. If this BID has been seen before,

the message is put on hold.

5) If the user (sysop) attempts to change the BID of a message,

and the new BID is one that has been seen before, the change is

rejected, the text "NO - Have it already" is displayed,

and the message retains its current BID.

6) If a message is received from another MailBox, and has a BID

sent along with it, and has a distribution list that includes

the MailBox from which the message was received, the message is

marked as already forwarded to that MailBox.

Some results of applying these rules:

1) A message entered into the system without using "$" in the command

and without a distribution list may loop within the system.

There is no rejection mechanism for this type of message.

2) A message which was entered with a "$" given in the command

will be rejected when it is forwarded back to any system it

previously passed through.

3) Messages of type B or P may have a distribution list,

messages of type T may not.

4) There will be no attempt to pass a message which has a BID

back to the station that sent it to you.