CMD Technology

INSTALL.DOC

August 15, 1994

The purpose of INSTALL.INI file is to be able to control the configuration

of the installation program.  The INSTALL.INI file has several sections,

each headed by a word enclosed in brackets.  The following is the list of

sections in this file:

[DOS UTIL FILES]

[DOS DRIVER]

[NETWARE 3.1X DRIVER]

[WINDOWS DRIVER]

[INSTALL OPTIONS]

[MISC]

The order in which the sections appear is not critical.  Each header is

followed by a number of lines.  The structure of the lines under each

header is defined under the header type.

[DOS UTIL FILES] Section

------------------------

This header lists all the utility files to copy from the distribution

disk to the destination directory.  Each line consists of the name of the

file to copy.

example:

[DOS Util Files]

DISKPREP.EXE

VIEWIDE.EXE

.

.

[DOS DRIVER] Section

--------------------

This section contains the name of the DOS driver.  See DOSDRVR.DOC

for more information on DOS driver selection.

example:

[DOS Driver]

CMD640X.SYS

[NETWARE 3.1X DRIVER] Section

-----------------------------

This section contains the name of the Netware driver.

example:

[Netware 3.1x Driver]

C640X31X.DSK

[WINDOWS DRIVER] Section

------------------------

This section contains the name of the Windows drivers.

example:

[Windows Driver]

CMDINT13.386

CMD640X.386

[MISC] Section

------------------------

Each line in this section begins with a keyword followed by a equal sign.

Each keyword controls some aspect of the way the installation program

installs on your system.  On the right side of each equal sign are values

assigned to the keyword.  The order of these line is not critical.  The

following is the only keyword that is used in this section:

SYSTEM.INI REMOVE LINE

If the above keyword or a whole section of a keyword is missing then

a default value is used in the installation.  If you type a semicolon ";"

in front of a line then that line is ignored. The following is the

explanation of all the keywords used in this section:

SYSTEM.INI REMOVE LINE

----------------------

This option can be used to comment any line from the SYSTEM.INI file.  You

can specify any key word(s) on the right hand side of the equal sign.

Installation program will search the SYSTEM.INI file for the specified

key word and comments that line out.  It is not case sensitive.  If you

want to comment more lines in SYSTEM.INI with differnt key word(s), then

you can add another "SYSTEM.INI REMOVE LINE" keyword under "MISC" section.

example:

SYSTEM.INI REMOVE LINE =vmd

SYSTEM.INI REMOVE LINE =device=*vfd

In this example, all the lines containing the word "vmd" or "VMD"

and "device=*vfd" or "DEVICE=*VFD" in SYSTEM.INI will be commented out.

[INSTALL OPTIONS] Section

-------------------------

Each line in this section begins with a keyword followed by a equal sign.

Each keyword controls some aspect of the way the installation program

installs on your system.  On the right side of each equal sign are values

assigned to the keyword.  The order of these line is not critical.  The

following is the list of all the keywords:

Destination Directory

Windows/WFWG 3.1x Directory

Netware 3.1x Directory

Advanced Timing

Local Bus Speed

Secondary Drive

Secondary Irq

LBA Mode

Install DOS Driver

Install Windows Driver

Install Netware Driver

Other Options

If any of the above keywords or a whole section of a keyword is missing then

a default value is used in the installation.  If you type a semicolon ";"

in front of a line then that line is ignored.  If the value on the right

side is a question mark "?" or blank, the user is always prompted

for that option during the installation.  The following is the explanation

of all the keywords used in this section:

Destination Directory

---------------------

The installation program copies all the driver(s) and utility files to

a directory on the user's hard disk.  The user can type the name of the

directory on the right hand side of the equal sign or type a question mark.

If the name of the directory is typed in this option, then the user is not

prompted during installation.  If a question mark typed, then the user is

prompted for the directory.  If this keyword is not present at all, then

the default directory "C:\QUICKCMD" is used in the installation program.

example:

Destination Directory=c:\IDE

or

Destination Directory=?

Windows/WFWG 3.1x Directory

--------------------------

The installation program installs the FastDisk driver only if the user

has Windows/WFWG 3.1x installed on their system. The value on the right

hand side of the equal sign must be the user's Windows/WFWG 3.1x directory

or it can be a question mark.  If the right hand side is a question mark,

then the user is prompted for the directory.  If this keyword is not

present at all, then it is assumed that the user does not have

Windows/WFWG 3.1x installed on their system.

example:

Windows/WFWG 3.1x Directory=c:\windows

or

Windows/WFWG 3.1x Directory=?

or

;Windows/WFWG 3.1x Directory=

Netware 3.1x Directory

----------------------

The installation program install Netware drivers, if the system will be

used as Novell Netware 3.1x server.  The value on the right hand side of

the equal sign must be the directory where the Netware Drivers will be

installed.  If it is a question mark, then the user is prompted for this

options during installation.  If this keyword is not present at all, then

it is assumed that the user will not use the system for Novell Netware 3.1x

server.

example:

Netware 3.1x Directory=c:\netware

or

;Netware 3.1x Directory=c:\netware

Advanced Timing

---------------

This option allows user to use the Fast IDE drive timings.  The value on the

right hand side of the equal sign can be a YES or NO or a question mark.

If the value is YES then the Local Bus speed is obtained from the option

"Local Bus Speed" defined in the next options.  This installation programs

skips this options if the value is a NO.  If it is a question mark, then the

user is prompted for this options during installation.  If this keyword is

not present at all, then it is assumed that the user does not want to  use

Fast IDE drive timings.

example:

Advanced Timing=YES

or

Advanced Timing=NO

or

Advanced Timing=?

Local Bus Speed

---------------

If the user selects to use Fast IDE drive timings, then the user is prompted

to select their Local Bus Speed.  The user can type a value on the right

hand side of the equal sign (Mhz) or a question mark if they do not know

the speed of their local bus.  This option is valid only if user selects YES

for Advanced Timing option.

example:

Local Bus Speed = 33Mhz

or

Local Bus Speed = 50Mhz

or

Local Bus Speed = ?

Secondary Drive

--------------

If the user has a drive connected to their secondary IDE port then the

installation program scans the secondary IDE port for the drive(s).  The user

can type YES or NO for this option.  If the value is YES selected then the

installation program starts scanning the secondary IDE port without prompting

the user before scanning.  The installation program skips this option if a

NO is entered.  If it is a question mark, then the user is prompted for

this options during installation.

example:

Secondary Drive=Yes

or

Secondary Drive=NO

or

Secondary Drive=?

or

;Secondary Drive=

Secondary Irq

------------

This option is valid only if Secondary Drive option have a value YES.  The

right hand side of the equal sign is the IRQ for the secondary drive.

example:

Secondary Irq=14

or

Secondary Irq=?

LBA Mode

--------

This options stands for Logical Block Addressing mode.  It can have a value

of ON or OFF.  If ON is selected then the installation program uses the

Logical Block Addressing Mode.  If OFF is selected then CHS mode is selected.

example:

LBA Mode=ON

or

LBA Mode=OFF

or

LBA Mode=?

Install DOS Driver

------------------

This options allows the options to install DOS driver or skip DOS driver

installation.  It can have a value YES or NO.  If this keyword is not

present under "install options" section, then DOS driver is installed.

examples:

Install DOS Driver=YES

or

Install DOS Driver=NO

or

;Install DOS Driver=NO

Install Windows Driver

------------------

This options allows the options to install Windows driver or skip Windows

driver installation.  It can have a value YES or NO.  If this keyword is not

present under "install options" section, then Windows driver is installed.

examples:

Install Windows Driver=YES

or

Install Windows Driver=NO

or

;Install Windows Driver=NO

Install Netware Driver

------------------

This options allows the options to install Netware driver or skip

Netware driver installation.  It can have a value YES or NO.  If this

keyword is not present under "install options" section, then Netware

driver is installed.

examples:

Install Netware Driver=YES

or

Install Netware Driver=NO

or

;Install Netware Driver=NO