Driver for Lantastic/AI Installation

To install Lantastic/AI v4.x or Lantastic/AI v5.x software on a workstation,

follow the instructions.

****** Installation Porcedure ******

1. Run your existing DOS system as you usually do.

2. Place the Lantastic/AI Network Operating System Disk #1 in

floppy drive A: or B:

3. Change to the drive containing the diskette and type

"INSATLL <Enter>" at the system prompt.

4. Choose NE2000 as your Ethernet Hardware Adapter.

5. Follow all the operations as you are prompted on the screen.

6. After installing the Netware Operating System, reboot your system.

--- Sample AUTOEXEC.BAT file ---

:

call C:\LANTASTI\STARTNET.BAT

:

:

--- Sample STARTNET.BAT file for Lantastic/AI v4.x ---

:

@ECHO OFF

PATH C:\LANTASTI;%PATH%

SHARE /L:200

NEX000 IRQ=3 IOBASE=300 16-BIT VERBOSE

AILANBIO

REDIR ADMIN LOGINS=3

SERVER C:\LANTASTI.NET

NET LOGIN/WAIT \\ADMIN ADMIN

NET LPT TIMEOUT 10

:

:

--- Sample STARTNET.BAT file for Lantastic/AI v5.x ---

:

@ECHO OFF

SET LAN_DIR=C:\LANTASTI.NET

SET LAN_CFG=C:\LANTASTI

PATH C:\LANTASTI;%PATH%

SHARE /L:200

NEX000 IRQ=3 IOBASE=300 16-BIT VERBOSE

AILANBIO

REDIR ADMIN LOGINS=3

SERVER C:\LANTASTI.NET

NET LOGIN/WAIT \\ADMIN ADMIN

NET LPT TIMEOUT 10

NET POSTBOX

:

: