Driver for PowerLan Installation

��������������������������������

To install PowerLan software on a workstation, follow the following

instructions.

****** Installation Porcedure ******

1. Run your existing DOS system as you usually do.

2. Place the PowerLan install Disk #1 in floppy drive A: or B:

3. Change to the drive containing the diskette and type

"INSATLL <Enter>" at the system prompt.

4. Choose ET200 as your Ethernet Hardware Adapter. (you may choose

WD8013 as your Ethernet Hardware Adapter, if your are installing

jumperless adapter in your PC and you had set the adapter to

Share Memory Mode.)

5. Follow all the operations as you are prompted on the screen.

6. After installing the Netware Operating System, reboot your system.

--- Sample AUTOEXEC.BAT file ---

:

PATH=C:\PL\PROGRAMS;%PATH%

call C:\PL\ADMIN\SYS\PLSTART.BAT

:

:

--- Sample PLSTART.BAT file ---

:

@echo off

if %PLTREE%. == . set PLTREE=C:\PL

if %PLSO%. == . set PLSO=CON

echo Starting POWERLan... >%PLSO%

if %1. == . goto skip_xfer

goto %1

:skip_xfer

REM

REM		    [-------------------------------------]

REM		    [  POWERLan Configuration Batch File  ]

REM		    [-------------------------------------]

REM

REM

REM - - - - - - - - - - - Initialization  - - - - - - - - - - - - - - - - - -

REM

share	/f:2048 /l:20 >>%PLSO%

if errorlevel 1 pause

REM

REM - - - - - - - - - - - - - - - - - - Start NETBIOS - - - - - - - - - - - -

REM

REM

%PLTREE%\PROGRAMS\nxne2000  /ln:0 /ses:16 /nam:17 /lp:300 /li:3

if errorlevel 1 pause

REM

REM - - - - - - - - - - - - - - - - -  Start Name Server - - - - - - - - - - -

REM

%PLTREE%\PROGRAMS\nameserv  /names:50 >>%PLSO%

if errorlevel 1 pause

REM

REM - - - - - - - - - - - - - - - - -  Start Unspooler - - - - - - - - - - -

REM

%PLTREE%\PROGRAMS\prtunspl ADMIN /p:LPT1 /netbuf:256 /n:LPT1 >>%PLSO%

if errorlevel 1 pause

REM

REM - - - - - - - - - - - - - - - - -  Start Remote Viewing - - - - - - - - -

REM

%PLTREE%\PROGRAMS\netshow ADMIN  /adapter:0 /l:1024 >>%PLSO%

if errorlevel 1 pause

REM CFG=%PLTREE%\PROGRAMS\alert  >>%PLSO%

REM

REM - - - - - - - - - - -(NOT ACTIVE)  Start Cache - - - - - - - - - - - - -

REM

REM CFG=%PLTREE%\PROGRAMS\cache 0 /e >>%PLSO%

REM CFG=%PLTREE%\PROGRAMS\cachectl  >nul

REM

REM - - - - - - - - - - - - - - - - -  Start Redirector - - - - - - - - - - -

REM

%PLTREE%\PROGRAMS\redir /n:ADMIN /d:26 /s:4 /k:1 /b:2 /p1:1 /p2:1 /p3:1 /q >>%

if errorlevel 1 pause

REM CFG=%PLTREE%\PROGRAMS\net lpt  /LPT1:9 /LPT2:9 /LPT3:9 >>%PLSO%

REM

REM - - - - - - - - - - - - - - - - -  Start Beacon - - - - - - - - - - - - -

REM

%PLTREE%\PROGRAMS\beacon ADMIN /type:7 >>%PLSO%

if errorlevel 1 pause

REM

REM - - - - - - - - - - - - - - - - -  Start Non-dedicated Server - - - - - -

REM

:server

%PLTREE%\PROGRAMS\server  /p:C:\SPOOL /r:12 /u:8 /k:4 /b:2 /q >>%PLSO%

if errorlevel 1 pause

%PLTREE%\PROGRAMS\net sharesys /spool:C:\SPOOL /start >>%PLSO%

if errorlevel 1 pause

%PLTREE%\PROGRAMS\net sharesys C:\PL\ADMIN\SYS\PLSHARES.SYS >>%PLSO%

if errorlevel 1 pause

%PLTREE%\PROGRAMS\net sharesys /logon:C:\PL\ADMIN\USERS /start >>%PLSO%

if errorlevel 1 pause

%PLTREE%\PROGRAMS\net sharesys /admin:C:\PL\ADMIN /start >>%PLSO%

if errorlevel 1 pause

REM

REM - - - - - - - - - - -(NOT ACTIVE)  Start Dedicated Server - - - - - - - -

REM

REM

REM - - - - - - - - - - -(NOT ACTIVE)  Start Popup Redirector  - - - - - - -

REM

REM CFG=%PLTREE%\PROGRAMS\plr CTRL-ALT-R >>%PLSO%

REM

REM - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

REM

if exist %PLTREE%\PROGRAMS\PLADMIN.EXE set PLFOUND=YES

REM

REM - - - - - - - - - - - - - - - - -  Log User On - - - - - - - - - - - - -

REM

:logon

%PLTREE%\PROGRAMS\net logon ADMIN

REM

REM - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

REM

if %PLFOUND%. == YES. set PLTREE=

set PLFOUND=

if %PLSO%. == CON. set PLSO=

REM

REM		 [--------------------------------------------]

REM		 [  End of POWERLan Configuration Batch File  ]

REM		 [--------------------------------------------]

:

: