NDIS Driver for Banyan VINES Installation

�����������������������������������������

To install Banyan VINES 5.xx software on a DOS base workstation, follow the

instructions.	DOS must be installed on the target workstation C: drive, or

you must have a bootable DOS floppy diskette.

1. Installation Porcedure:

a.  Use the DOS MD command to create a subdirectory called \VINES.

b.  Copy the contents of the VINES LAN S/W (DOS) Install diskette

into the \VINES subdirectory.

c.  Copy the contents of the NDIS subdirectory on the VINES LAN S/W

(DOS) 2 of 2 diskette into the \VINES subdirectory.

d.  Copy all files from \MSLANMAN.DOS\DRIVERS\ETHERNET\AE-200

subdirectory on the driver diskette to the \VINES subdirectory.

e.  Edit the PROTOCOL.INI file to read as follows:

[PROTOCOL MANAGER]

DRIVERNAME = PROTMAN$

[VINES_XIF]

DRIVERNAME = NDISBAN$

BINDINGS = AE-200

[AE-200]

IOBASE = 0x300

INTERRUPT = 3

DRIVERNAME = AE-2000$

f.  Run the PCCONFIG.EXE program located in the \VINES subdirectory.

g.  Select option 1, Network Card Settings.

h.  Select the "NDIS Ethernet" option.

i.  The NDIS workstation screen appears.  Enter the IRQ that you

selected for the AE-200 adapter.

j.  Press 'F10' when done, press <ESC> to exit this screen.

k.  Select "2 - Login Environment Settings. ", Press <Enter>.

l.  Select "1 - Select Default Communications Driver. ", Press <Enter>.

m.  Choose "NDIS Ethernet", Press <ESC> to exit this menu.

n.  Edit the CONFIG.SYS file and add the following lines, substituting

A: for C: for a floppy diskette installation.

DEVICE = C:\VINES\PROTMAN.DOS /I:C:\VINES

DEVICE = C:\VINES\AE-200.DOS

o.  Add the BAN command to the AUTOEXEC.BAT file.  BAN loads the NDISBAN

drivers and reads the PCCONFIG.DB file to invoke changes made during

the PCCONFIG stage.