Note: Before following installation steps to install two different

NetWare client drivers, please refer to NET.CFG file in this

directory to set a correct line speed and operating mode

(also refer to "AUTONEGOTIATION with the attached device"

section of the \README.1ST file for the line speed and mode)

to the adapter to communicate with the attached device (hub or

repeater).

=====================================================

=     CN100TX(B) PCI-BUS FAST ETHERNET ADAPTER	    =

=     Driver Installation Procedure		    =

= >>> NetWare DOS ODI Workstation Client Driver     =

=====================================================

The NetWare DOS ODI client driver files are in the \DOS_ODI subdirectory of

this diskette.

1. Power on the PC. Put this diskette in a floppy-disk drive (the example

uses Drive A:). At the DOS prompt type --

A:\DOS_ODI\LSL <enter>

A:\DOS_ODI\DC21X4 <enter>

A:\DOS_ODI\IPXODI <enter>

A:\DOS_ODI\NETX <enter>

2. Refer to the Novell Installation documentation for more details.

###

=====================================================

=     CN100TX(B) PCI-BUS FAST ETHERNET ADAPTER	    =

=     Driver Installation Procedure		    =

= >>> Personal NetWare 1.x			    =

=====================================================

The driver for Personal NetWare 1.x is stored in the \DOS_ODI subdirectory.

1. Install Personal NetWare on the PC's hard disk. Refer to the Personal

NetWare manual for instructions.

2. Copy the proper driver files from this diskette  to the Personal NetWare

subdirectory. With this diskette in Drive A:, at the DOS prompt type:

COPY A:\DOS_ODI\DC21X4.INS C:\NWCLIENT <enter>

COPY A:\DOS_ODI\DC21X4.COM C:\NWCLIENT <enter>

3. Update the version of LSL.COM in the Personal NetWare subdirectory. Type:

COPY A:\DOS_ODI\LSL.COM C:\NWCLIENT <enter>

4. Configure Personal NetWare to use the CN100TX adapter:

a).Run Personal NetWare's SETUP.EXE program. The main menu will appear.

b).Choose "Primary Network Interface Card" and locate "CNet CN100TX(B)

Fast Ethernet Adapter". Press <enter> to select this card.

c).Return to the main menu and select "Configure Primary Interface Card"

to configure the "Media Frame Type(s)".

d).After setting the frame type(s), return to the main menu. Press "Save

Changes and Exit".

5. Run Personal NetWare's STARTNET.BAT program.

###