This section provides two installation procedures. The first is for

NetWare 3.11, followed by the procedure for NetWare 3.12.

====================================================

=     CN100TX(B) PCI-BUS FAST ETHERNET ADAPTER     =

=     Driver Installation Procedure                =

= >>> NetWare v3.11 Server Driver                  =

====================================================

The NetWare 3.11 server driver files are in the \NW_31X subdirectory of this

diskette.

1. Power on the file server and execute the Novell SERVER.EXE program.

2. Insert this diskette in the server's floppy disk drive (the example

uses Drive A:) and load the file LSLENH.NLM first. This will

automatically load PATCHMAN.NLM.

At the server's prompt, type:

LOAD A:\NW_31X\LSLENH.NLM <enter>

** NOTE: The examples here detail how to key-in the LOAD command

and some of its parameters. Most users prefer to put the LOAD

command into the AUTOEXEC.NCF file which is automatically executed

each time the server is booted. Refer to the NetWare documentation

for details on setting up auto-execution files.

3. Load the LAN driver file DC21X4.LAN and bind it. This will automatically

load MSM31X and ETHERTSM.NLM. The format for LOAD is --

LOAD <driver name> <PARAMETER NAME>... <PARAMETER NAME[=VALUE]>

To include the Ethernet frame type in the command, type --

LOAD A:\NW_31X\DC21X4.LAN SLOT=16 FRAME=Ethernet_802.2 NAME=DC21X4_802.2

BIND IPX TO DC21X4_802.2 NET=2

and/or --

LOAD A:\NW_31X\DC21X4.LAN SLOT=16 FRAME=Ethernet_802.3 NAME=DC21X4_802.3

BIND IPX TO DC21X4_802.3 NET=3

The LOAD command can also contain other parameters which will force the

CN100TX or the driver to function in certain ways. See the NetWare

documentation for standard NetWare parameters. CN100TX driver-specific

parameters are listed below.

PARAMETER NAME  DESCRIPTION

---------------- --------------------------------------------------

SLOT=xx         On a PCI machine, starts from slot number 16

for the first Ethernet device.

TP_NO_LINK_TEST selects twisted pair connection, but disables

the LINK test for specific Hubs that do not

support LINK.

TP_FULL_DUPLEX  forces twisted pair connection to work in

dedicated Full-Duplex mode. Include both this and

the LINE_SPEED=xxx parameters if the attached

device (e.g., switching hub) is also set to

dedicated Full-Duplex.

LINE_SPEED=xxx  sets CN100TX line speed to 10Mbps or 100Mbps

Format: LINE_SPEED=<value>

Allowed values are: 10,100

Include this if TP_FULL_DUPLEX parameter is used.

** NOTE: The TP_FULL_DUPLEX and LINE_SPEED=xxx parameters are for use

when connecting the CN100TX to a Full-Duplex network device.

Do not include these two parameters in the driver command if

the link partner is a Half-Duplex or Auto-Negotiation device.

If attached to a Half-Duplex or Auto-Negotiation device , the CN100TX

driver can automatically set the correct line speed and operating

mode.

###

====================================================

=     CN100TX(B) PCI-BUS FAST ETHERNET ADAPTER     =

=     Driver Installation Procedure                =

= >>> NetWare v3.12 Server Driver                  =

====================================================

NetWare v3.12 server driver files are in the \NW_31X subdirectory of this

diskette.

1. Power on the file server and execute the Novell SERVER.EXE program.

2. Put this diskette in the server's floppy disk drive (the example

uses Drive A:),

3. Load the LAN driver file DC21X4.LAN and bind it. This will automatically

load MSM31X and ETHERTSM.NLM. The format for LOAD is --

LOAD <driver name> <PARAMETER NAME>... <PARAMETER NAME[=VALUE]>

To include the Ethernet frame type in the command, type --

LOAD A:\NW_31X\DC21X4.LAN SLOT=16 FRAME=Ethernet_802.2 NAME=DC21X4_802.2

BIND IPX TO DC21X4_802.2 NET=2

and/or --

LOAD A:\NW_31X\DC21X4.LAN SLOT=16 FRAME=Ethernet_802.3 NAME=DC21X4_802.3

BIND IPX TO DC21X4_802.3 NET=3

The LOAD command can also contain other parameters which will force the

CN100TX or the driver to function in certain ways. See the NetWare

documentation for standard NetWare parameters. CN100TX driver-specific

parameters are listed below.

PARAMETER NAME  DESCRIPTION

---------------- --------------------------------------------------

SLOT=xx         On a PCI machine, starts from slot number 16

for the first Ethernet device.

TP_NO_LINK_TEST selects twisted pair connection, but disables

the LINK test for specific Hubs that do not

support LINK.

TP_FULL_DUPLEX  forces twisted pair connection to work in

dedicated Full-Duplex mode. Include both this and

the LINE_SPEED=xxx parameters if the attached

device (e.g., switching hub) is also set to

dedicated Full-Duplex.

LINE_SPEED=xxx  sets CN100TX line speed to 10Mbps or 100Mbps

Format: LINE_SPEED=<value>

Allowed values are: 10,100

Include this if TP_FULL_DUPLEX parameter is used.

** NOTE: The TP_FULL_DUPLEX and LINE_SPEED=xxx parameters are for use

when connecting the CN100TX to a Full-Duplex network device.

Do not include these two parameters in the driver command if

the link partner is a Half-Duplex or Auto-Negotiation device.

If attached to a Half-Duplex or Auto-Negotiation device , the CN100TX

driver can automatically set the correct line speed and operating

mode.

###