====================================================

=     CN100TX(B) PCI-BUS FAST ETHERNET ADAPTER	   =

=     Driver Installation Procedure		   =

= >>> Packet Driver				   =

====================================================

The packet driver file is in the \PCTCP subdirectory of this diskette.

1.  Load the packet driver using the software interrupt and any

optional switches if required.

Usage: E100PK [options] <software_int_no>

Example: E100PK -d 0x7e

The only required parameter is the software interrupt. A packet

driver needs to have a software interrupt assigned to it for other

programs to access it.  The recommended interrupt 0x7e.  If you need

any of the options listed below, place them before the software

interrupt on the command line.

-#	  Specify line speed. -0 is 10 MBps , -1 is 100MBps

-f	  Enable UTP full-duplex mode

-i	  Force driver to report itself as IEEE 802.3 instead of Ethernet II.

-d	  Delayed initialization.  Used for diskless booting

-n	  NetWare conversion.  Converts 802.3 packets into 8137 packets

-w	  Windows hack, obsoleted by winpkt

-u	  Uninstall

2.  Type command, E100PK 0x60, the screen will appear following

message:

Packet driver software interrupt is 0x60 (96)

Interrupt number 0xA (10)

I/O port 0xE800 (59392)

My Ethernet address is 00:80:AD:17:E4:0A

Adapter: 100TX(B), 10M