**********************************************************

*     Installation for NetWare 386 v3.11 server driver	 *

**********************************************************

The server driver file is stored in the \NW_31X subdirectory.

1. Power on the file server and execute the Novell SERVER.EXE program.

2. Load LSLENH.NLM first. This will automatically load the PATCHMAN.NLM.

At the server's prompt, type:

LOAD A:\NW_31X\LSLENH.NLM

3. Load LAN driver E935.LAN and bind it. This will automatically load

MSM31X.NLM and ETHERTSM.NLM. The general way of using these parameters

is:

LOAD <driver name> <PARAMETER NAME>

Example:

LOAD A:\NW_31X\E935.LAN SLOT=16 FRAME=Ethernet_802.2 NAME=E935_802.2

BIND IPX TO E935_802.2 NET=2

LOAD A:\NW_31X\E935.LAN SLOT=16 FRAME=Ethernet_802.3 NAME=E935_802.3

BIND IPX TO E935_802.3 NET=3

NOTE: This assumes that this diskette is in drive "A:".

Driver specific parameters list  (as for NetWare standard parameters,

please see the NetWare user's manual):

SLOT=xx	   On a PCI machine use slot number from 16,

for first ethernet device, and upwards.

TP		   selects TP port

BNC		   selects BNC port

AUTODETECT	   enables autodetection of media type when driver

is loading. Note that this ONLY! works at

initialization time, i.e. when the driver first

loads for a specific network card. (default)

TP_NO_LINK_TEST selects twisted pair connection, but disables

the LINK test for specific HUBs that do not

support LINK.

TP_FULL_DUPLEX  selects twisted pair connection to work in

full duplex mode.

BIOS16	   instructs the driver to access the PCI BIOS

using 16-bit real mode calls, instead of the

PCI32 mechanism.

This is not recommended, and should be used

ONLY! on motherboards that have a known problem

in the PCI BIOS implementation.

4. The server program will load and execute the E935.LAN driver module.

Please refer to the NetWare 386 v3.11 Installation Manual for more

information to make an AUTOEXEC.NCF batch file to load the E935.LAN

module.

**********************************************************

*     Installation for NetWare 386 v3.12 server driver	 *

**********************************************************

The server driver file is stored in the \NW_31X subdirectory.

1. Power on the file server and execute the Novell SERVER.EXE program.

2. Load LAN driver E935.LAN and bind it. This will automatically load

MSM31X.NLM and ETHERTSM.NLM. The general way of using these parameters

is:

LOAD <driver name> <PARAMETER NAME>

Example:

LOAD A:\NW_31X\E935.LAN SLOT=16 FRAME=Ethernet_802.2 NAME=E935_802.2

BIND IPX TO E935_802.2 NET=2

LOAD A:\NW_31X\E935.LAN SLOT=16 FRAME=Ethernet_802.3 NAME=E935_802.3

BIND IPX TO E935_802.3 NET=3

NOTE: This assumes that this diskette is in drive "A:".

Driver specific parameters list  (as for NetWare standard parameters,

please see the NetWare user's manual):

SLOT=xx	   On a PCI machine use slot number from 16,

for first ethernet device, and upwards.

TP		   selects TP port

BNC		   selects BNC port

AUTODETECT	   enables autodetection of media type when driver

is loading. Note that this ONLY! works at

initialization time, i.e. when the driver first

loads for a specific network card. (default)

TP_NO_LINK_TEST selects twisted pair connection, but disables

the LINK test for specific HUBs that do not

support LINK.

TP_FULL_DUPLEX  selects twisted pair connection to work in

full duplex mode.

BIOS16	   instructs the driver to access the PCI BIOS

using 16-bit real mode calls, instead of the

PCI32 mechanism.

This is not recommended, and should be used

ONLY! on motherboards that have a known problem

in the PCI BIOS implementation.

3. The server program will load and execute the E935.LAN driver module.

Please refer to the NetWare 386 v3.12 Installation Manual for more

information to make an AUTOEXEC.NCF batch file to load the E935.LAN

module.