***********************************************

*  CNet ISA-Bus PowerNIC Driver Installation  *

*  > NetWare 4.x ODI Server                   *

***********************************************

The NetWare 4.x ODI server driver file is stored in the \NW_4

subdirectory of the PowerNIC diskette.

1. Power on the file server, and execute Novell's SERVER.EXE program.

2. Load the LAN driver E200P.LAN. (Two different methods are shown here.)

METHOD 1 --

At the server's prompt, load the driver E200P.LAN and bind it

(this also automatically loads MSM.NLM and ETHERTSM.NLM). The general

way of using these parameters is:

LOAD <driver name> <PARAMETER NAME>

Example:

LOAD A:\NW_4\E200P.LAN PORT=300 INT=3 FRAME=Ethernet_802.2 NAME=E200P_802.2

BIND IPX TO E200P_802.2 NET=2

LOAD A:\NW_4\E200P.LAN PORT=300 INT=3 FRAME=Ethernet_802.3 NAME=E200P_802.3

BIND IPX TO E200P_802.3 NET=3

NOTE: This assumes that this diskette is in drive A.

METHOD 2 --

a) At the server's prompt type: LOAD INSTALL

b) When the install menu appears, choose the menu item.

c) Install the LAN driver E200P.LAN according to the NetWare manual,

or use the on-line instructions.

3. The server program will load and execute the E200P.LAN driver module.

Please refer to the NetWare 4.x Installation Manual for more information

on making an auto-execution batch file to load the E200P.LAN module.