***********************************************

*  CNet ISA-Bus PowerNIC Driver Installation  *

*  > Packet Driver                            *

***********************************************

The packet driver for PC/TCP is located in the \PCTCP subdirectory

of the PowerNIC diskette.

1. Run the 200EPD.COM program by entering the following

at the DOS command line --

A:>\PCTCP\200EPD  <Packet_INT#> [IRQ#] [IO_Addr#]

(NOTE: This assumes the driver diskette is in drive A.)

where --

<Packet_INT#>:   interrupt vector number, which can be

decimal, hexadecimal or octal format.

if:

decimal format -- from 96 to 128

hexadecimal ----- from 0x60 to 0x80

octal format ---- from 0140 to 0200

[IRQ#]:          interrupt setting.

The following settings can be selected 3, 4, 5, 9,

10, 11, 12, 15.

[IO_ADDR#]:      I/O Base Address setting.

The following settings can be selected:

0x300 0x320 0x340 0x360 0x380 0x3A0 0x3C0 0x3E0

0x200 0x220 0x240 0x260 0x280 0x2A0 0x2C0 0x2E0

EXAMPLE:    200EPD  0x60 3 0x300

(manual selection)

--> Packet_INT# = 0x60, IRQ# = 3, IO_Addr# = 0x300

200EPD  0x60

(automatic selection)

--> Packet_INT# = 0x60, IRQ# & IO_Addr# = automatic selection

2. Execute the ETHDRV.EXE device program, which is included with the

FTP PC/TCP software package.

The Ethernet adapter should now work with FTP PC/TCP.