COMPEX ENET32-VL Ethernet Adapter (940128)             Checked by Alwyn Song

Jan 28, 1994                                           January 29 1994

���������������������������������������������������Ŀ

�                                                   �

�               DIRECTORY STRUCTURE                 �

�                                                   �

�����������������������������������������������������

1) INTRODUCTION

This floppy diskette contains network drivers and a configuration

utility. The files are arranged in the following directories.

a) <Root>\WFW31

Contains:

- Files for installing a DOS NDIS driver in Windows

For Workgroups.

b) <Root>\DOS

Contains:

- NetWare ODI workstation driver.

- NET.CFG, a sample configuration file for your quick reference.

- ENET32.INS, a driver installation information file used by the

DOS Requester INSTALL program.

c) <Root>\OS2ODI

Contains NetWare OS2 Requester.

d) <Root>\NETWARE.4X

Contains NetWare ODI server driver and its installation

file.

f) <Root>\MSLANMAN.DOS

Contains files for installing DOS NDIS driver in a Microsoft LAN

Manager network. Refer to the section "LAN MANAGER" in this

document for more details.

e) <Root>\MSLANMAN.OS2

Contains the driver for installation of OS/2 workstation or OS/2

server. Refer to the section "LAN MANAGER" in this document for

more details.

f) <Root>\UTILITY

Contains a configuration utility for you to change the settings

of the board.

g) <Root>\PKT

Contains a Packet Driver for the network adapter.

���������������������������������������������������Ŀ

�                                                   �

�                 NOVELL NETWARE                    �

�                                                   �

�����������������������������������������������������

1) INTRODUCTION

The floppy diskette contains drivers and utilities which enable the

ENET32-VL to operate in the following NetWare Operating system:

a) NetWare Lite v1.x

b) NetWare      v3.1x

c) NetWare      v4.x

The main features and functions of the drivers are listed below.

a) NetWare DOS Workstation Driver (EN32VL.COM)

This ODI driver is for DOS workstations when using Novell ODI

driver stack.

b) NetWare OS2 Requester (EN32VL.SYS)

This ODI driver is for OS2 workstations when using Novell ODI

driver stack.

c) NetWare Server Driver (EN32VL.LAN)

This Ethernet driver is for NetWare v3.x or v4.x file Servers and

complies with Novell ODI (Open Data-link Interface) Specifications.

The functions of the utilities are listed below.

a) NET.CFG

- sample configuration file for DOS ODI driver.

b) EN32VL.INS

- driver installation information file used by the

DOS Requester INSTALL program.

c) EN32VL.LDI

- driver installation information file used by the

NetWare 4.x server INSTALL program.

2) Installing the NetWare DOS ODI Client Driver

The DOS ODI driver may be installed using the Novell client INSTALL

program (available in NetWare v3.12 and above). Please refer to the

Novell user manuals for specific instructions to load additional

drivers from an external floppy diskette.

The driver may also be loaded manually through the DOS command line

or automatically through a batch file such as AUTOEXEC.BAT.  The

driver is located in the DOS subdirectory on the driver diskette.

To install the ENET32-VL driver, follow the instructions given below:

i)   Insert the Driver diskette into the system's floppy drive.

ii)  Create a subdirectory in your client's main boot drive with

the following files:

LSL.COM    - Link Support Layer

EN32VL.COM - The ENET32-VL DOS ODI Driver

IPXODI.COM - The IPX protocol stack file

NETX.EXE   - NetWare Shell (Used with conventional memory

workstations)

NET.CFG    - Optional; for configuring options different

from the default settings

VLM.EXE    - Use this instead of NETX.EXE to utilize NetWare

Directory Service (NDS) for Netware 4.x and

above

NETX.EXE, LSL.COM, VLM.EXE and IPXODI.COM can be obtained from

the Workstation Driver Diskette in the Novell Client Software

Package. Copy the client files EN32VL.COM from the DOS

directory of the Driver Diskette to the subdirectory.

Please note, when loading the VLM, the CONFIG.SYS must

have the following line to operate correctly.

LASTDRIVE=Z

iii) To load these files manually, execute the following commnads

at the DOS prompt:

LSL

EN32VL

IPXODI

NETX  (Use VLM instead for NetWare 4.x)

iv)  To automatically load these files, use a DOS text editor to

create and/or modify  the AUTOEXEC.BAT file, then save it to

the disk. The AUTOEXEC.BAT file should look similar to the

following:

\<subdirectory>\LSL

\<subdirectory>\EN32VL

\<subdirectory>\IPXODI

\<subdirectory>\NETX  (Use VLM instead for NetWare 4.x)

Reboot the workstation so the above commands are executed.

v)   After NETX (or VLM) is executed, and if a server is present,

it will attach to the server and display the server name on

the monitor. You can now login to the network.

Keyword Examples:

To change any default settings when the driver is loaded, the NET.CFG

file must exist in the workstation. A sample NET.CFG section of

EN32VL.COM driver is in the DOS directory of the driver diskette.

You can append the contents of this sample to your existing NET.CFG

file and edit it according to your needs.

3) Installing the Netware OS/2 Client Driver

To install the OS/2 ODI driver, the Novell OS/2 Requester should be

used. Load Netware OS/2 ODI driver from the OS/2 CONFIG.SYS file in

the sequence below.  For more information on installing the OS/2 ODI

driver, refer to the NetWare Requester for OS/2 installation

documentation.

i)   Insert the driver diskette into the system's floppy drive.

ii)  Using the Netware OS/2 disks provided by Novell, install the

Netware OS/2 requester files on the target hard disk and the

file server.  Follow the instructions on screen to configure

the Netware OS/2 requester for a Netware-supported LAN adapter

board. Select any LAN adapter (e.g., NE2000).

iii) Copy the OS/2 ODI driver, EN32VL.SYS, from the Driver Diskette

to the C:\NETWARE directory of the boot drive through path:

A:\OS2ODI

iv)  Edit the CONFIG.SYS file and replace the Netware-supported LAN

adapter driver with the name of the OS/2 ODI driver, EN32VL.SYS.

For example:

DEVICE=C:\NETWARE\EN32VL.SYS

The following is an example of NetWare 3.x of the CONFIG.SYS

load sequence for OS/2 ODI driver support for the IPX protocol

stack.

REM ---Netware Requester Statements Begin ---

DEVICE=C:\NETWARE\LSL.SYS

RUN=C:\NETWARE\DDAEMON.EXE

DEVICE=C:\NETWARE\EN32VL.SYS

(Located in OS2ODI directory on Driver Diskette)

DEVICE=C:\NETWARE\IPX.SYS

DEVICE=C:\NETWARE\NWREQ.SYS

IFS=DEVICE=C:\NETWARE\NWIFS.IFS

RUN=DEVICE=C:\NETWARE\NWDAEMON.EXE

REM ---Netware Requester Statements End ---

The default settings may be modified when the driver loads by

using the file NET.CFG. Also, the OS/2 ODI driver may also be

installed to support multiple protocol stacks and Ethernet frame

types. For more information, refer to the Netware Requester for OS/2

installation documentation.

Keyword Examples:

To change any default settings when the driver is loaded, the NET.CFG

file must exist in the workstation. A sample NET.CFG section of

EN32VL.SYS driver is in the DOS directory of the driver diskette.

You can append the contents of this sample to your existing NET.CFG

file and edit it according to your needs.

4) Installing NetWare Server 4.x Driver

The NetWare server driver EN32VL.LAN is located in the NETWARE.4X

subdirectory of the driver diskette. The INSTALL utility of

NetWare 4.x can be used for automatic installation of the driver.

Follow the guidelines in the Novell NetWare documentation for more

details to install the ENET32-VL driver from the driver diskette.

To manually load the drivers through the Server command console, use

the LOAD and BIND commands as described below. For more information

on these commands, refer to the NetWare LAN driver installation

documentation. To load the server driver, either use the

server command line prompted by ":" or edit AUTOEXEC.NCF. If you

use the server command line, you will need to load the driver each

time the system is booted. If you edit AUTOEXEC.NCF, it will

automatically load the driver each time your system is booted.

To install the NetWare Server, follow the instructions given below:

i)   Copy the ODI server driver, EN32VL.LAN, from the Driver

Diskette to the hard disk directory where the NetWare system

files are located. The path on the Diskette should be:

A:\NETWARE.4X

ii)  Boot NetWare by executing SERVER.EXE. Refer to NetWare

documentation on loading disk drivers and mounting volumes.

iii) To load the driver manually, simply follow the instructions

given below:

type the following command at the file server console:

LOAD EN32VL <enter>

(You will need to specify DOS path if the driver is

located in a DOS partition).

No keywords need to be specified for interrupt channel

and I/O address. Make sure that MSM and ETHERTSM files are

present in the same directory as EN32VL.  The EN32VL driver

will autoload these files.  These files are provided with

Novell NetWare.

Then, bind the driver to the IPX protocol stack by typing:

BIND IPX TO EN32VL NET=xxxx  <enter>

(Bind to ipx or any other protocol stack).

iv)  To load the driver using AUTOEXEC.NCF, you must edit

AUTOEXEC.NCF and add the above LOAD and BIND command lines to

the file. You should use the Novell server utility INSTALL to

edit the AUTOEXEC.NCF file.

Keyword Examples:

The FRAME keyword should be used with the LOAD command to designate

the desired frame type.  In addition, the BOARD keyword must be used

in conjunction with the FRAME keyword if no IO Address is specified

to support multiple FRAME types.  The available frame values are:

Ethernet_802.2

Ethernet_802.3

Ethernet_SNAP

Ethernet_II

Please refer to the Novell NetWare documentation for details regarding

each frame type.  The BOARD keyword value is designated by the user.

The range available for BOARD is 0 - 99999999. For example, to load

multiple frame types for one physical ENET32-VL board, the following

should be entered:

LOAD EN32VL FRAME=Ethernet_802.2 BOARD=1 <enter>

LOAD EN32VL FRAME=Ethernet_802.3 BOARD=1 <enter>

LOAD EN32VL FRAME=Ethernet_II BOARD=1    <enter>

LOAD EN32VL FRAME=Ethernet_SNAP BOARD=1  <enter>

Finally, the driver must be binded to the IPX protocol for each frame

type according to the procedure provided in the Novell documentation.

Please note that if memory is too low to support multiple frames, the

buffer size must be increased in the STARTUP.NCF in NetWare. For

example, the "ReservedBuffersbelow16Meg" should be set to below 32

from the default setting of below 16. The correct syntax is as follow:

Set ReservedBuffersbelow16Meg=32

Single FRAME type support

The FRAME keyword should be used with the LOAD command to designate

the desired frame type. For single FRAME support, no BOARD keyword

is needed to designate one frame type.  The available values and

description to load FRAME types is explained above in the Multiple

FRAME type support section.

Loading Multiple Boards

The driver must be loaded again for each new adapter board installed

in the system.  For example, to load two boards in the system for

the NetWare environment, the following must be entered.

For first adapter board: LOAD EN32VL

For second adapter board: LOAD EN32VL

Loading Multiple Boards With Multiple Frames

The following example can be used:

For first adapter board:

LOAD EN32VL FRAME=Ethernet_802.2 BOARD=1 <enter>

LOAD EN32VL FRAME=Ethernet_802.3 BOARD=1 <enter>

For second adapter board:

LOAD EN32VL FRAME=Ethernet_802.2 BOARD=2 <enter>

LOAD EN32VL FRAME=Ethernet_802.3 BOARD=2 <enter>

5) Installing NetWare Server 3.1x Driver

For NetWare 3.1x server, the ENET32-VL server driver is same as the

one used on NetWare 4.x server. The driver (EN32VL.LAN) is located

under the NETWARE.4X subdirectory in the Driver Disk.

NetWare 3.1x INSTALL utility cannot be used to install the server

driver onto the server. You have to use the LOAD and BIND commands to

load the NetWare server driver as described below.  For more

information on these commands, refer to the NetWare 3.1x LAN driver

installation documentation.

To install the NetWare Server 3.1x, follow the instructions given

below:

i)   Copy the server driver EN32VL.LAN from NETWARE.4X directory of

the driver diskette to the hard disk directory where

NetWare 3.1x system files are located.

ii)  Boot NetWare by executing SERVER.EXE. Refer to the NetWare

documentation on loading disk drivers and mounting volumes.

iii) To load the driver manually, follow the instructions given

below.

For NetWare 3.11, type the following command at the file

server console:

LOAD LSLENH.NLM    <enter>

LOAD MSM31X.NLM    <enter>

LOAD ETHERTSM.NLM  <enter>

LOAD EN32VL	    <enter>

Note: The LSLENH.NLM, MSM31X.NLM and ETHERTSM.NLM are updates

for NetWare 3.11 and are not found on the original

NetWare 3.11 diskettes. These updates are available from

Novell. The original MONITOR.NLM also cannot be used

since it has a bug and will crash the system. A new

version of MONITOR.NLM is needed.

For NetWare 3.12, type the following command at the file

server console:

LOAD EN32VL	<enter>

(You will need to specify DOS path if the driver is located

in a DOS partition).

Then, bind the driver to the IPX protocol stack by

typing:

BIND IPX TO EN32VL NET=xxxx  <enter>

(Bind to ipx or any other protocol stack).

iv)  To load the driver using AUTOEXEC.NCF, edit the

AUTOEXEC.NCF. Use the "INSTALL" utility to add the above

command lines.

Keyword Examples:

See Keyword Examples under Installing NetWare 4.x Server

driver.

���������������������������������������������������Ŀ

�                                                   �

� MICROSOFT LAN MANAGER AND WINDOWS FOR WORKGROUP   �

�                                                   �

�����������������������������������������������������

1) INTRODUCTION

The ENET32-VL has two NDIS drivers which comply with Microsoft NDI

Specifications v2.0.1.  The ENET32-VL NDIS 2.0.1 drivers (EN32VL.DOS

and EN32VL.OS2) are located in the MSLANMAN.DOS and MSLANMAN.OS2

directories. The directory structure provided on the driver diskette

has been setup for automatic OEM installation. Follow the guidelines

in the Microsoft LAN Manager Installation Guide for more details to

install the ENET32-VL driver from the driver diskette.

The three files necessary for proper driver installation are:

a) EN32VL.NIF

A Network Information file. It contains information needed by the

LAN MANAGER SETUP program to install the NDIS driver.

b) PROTOCOL.INI

A sample initialization file. It holds the hardware settings of

the network adapter and the necessary information for the driver

to configure itself to work in different environments.

c) EN32VL.DOS (or EN32VL.OS2)

NDIS driver.

2) LAN MANAGER INSTALLATION

Use the installation procedure described here as general guidelines.

Your exact installation requirements may vary slightly.

LAN Manager has a SETUP program that functions to import and configure

drivers.  The SETUP utility for LAN Manager will install the ENET32-VL

NDIS driver that is provided in the Driver Diskette.

To install the NDIS driver, follow the instructions given below:

i)   Insert the driver diskette into the floppy drive.

ii)  From the LAN Manager subdirectory on your hard disk, run

SETUP to begin installation.

iii) Use the ALT key on your keyboard and the highlighted

letter to pull down the Configuration menu (ALT + C).

iv)  From the Network Adapter Drivers dialog box, select the

Compex ENET32-VL driver from the menu, if shown. Otherwise,

select Other Driver to import the ENET32-VL driver from the

driver disk.

v)   Enter the disk drive letter for the driver diskette.

No path is required because the driver diskette is structured

for the Microsoft LAN Manager OEM setup.

vi)  Select the ENET32-VL Ethernet adapter and choose OK.

vii) Select the desired protocol from the Network Protocol dialog

box and choose OK.

viii)In the Workstation Configuration dialog box, select the

ENET32-VL Ethernet adapter and choose OK.

ix)  Follow the remaining instructions from LAN Manager to complete

installation.

3) Windows for Workgroups 3.1 Driver Installation

The installation for the NDIS 2.0.1 driver is menu driven through the

Windows interface.  The driver and other relevant files are located in

the root directory. The directory structure provided on the driver

diskette has been setup for automatic OEM installation. Follow the

guidelines in the Microsoft Windows for Workgroups  User's Guide for

more details to install the driver from the driver diskette.

To install the NDIS 2.0.1 driver, follow the instructions below:

i)   Insert the driver diskette in the system's floppy drive.

ii)  From the Main group in Windows, choose the Control Panel icon.

iii) In the Network Settings dialog box, choose the Adapters button

and the Network Adapters dialog box should appear.

iv)  Choose the Add button to install the driver from the diskette.

v)   In the list of network adapters, select the "Unlisted or

Updated Network Adapter" option.

vi)  Enter the drive letter and the WFW31 path to load the NDIS driver.

For example, A:\WFW31.

vii) Select the ENET32-VL Driver and choose OK.

viii)If the network adapter driver requires additional settings, a

setup dialog box appears displaying the default settings.

Modify the settings if necessary.

ix)  In the Network Adapters dialog box, choose the Close

button.  Then choose the OK button in the Network Settings

dialog box.

x)   Follow the remaining Window instructions, to restart the

computer and have the driver take effect.

4) NDIS driver keyword

The following are the supported keywords for the NDIS driver.

IOADDRESS - IO address of the board. Supported values are 0x220,

0x240, 0x280, 0x2A0, 0x300, 0x320, 0x340 and 0x380.

Use this keyword if you have more than one board on the

system and you want to choose a particular board to run

the driver.

INTERRUPT - Interrupt no of the board. Supported values are 10, 5, 3

and 15.

TXBUFFERS - Transmit buffer size. A valid value is a number which is

a power of 2. Default value is 4.

RXBUFFERS - Receive buffer size. A valid value is a number which is

a power of 2. Default value is 4.

5) Special Notes on ENET32-VL Rev.C board.

Due to a minor bug in the EEPROM contents, the NDIS driver will

detect the interrupt channel of the board as 0 which is incorrect.

You should use the the VLSET.EXE program to reprogram the EEPROM.

To correct the bug, run the VLSET.EXE program, change the default

settings to what you need or just load the default settings again,

and then reprogram the board.

6) Other installations

To install the NDIS driver for use with non-LAN MANAGER based

products, follow the directions supplied with that product.

�����������������������������������������������Ŀ

�                                               �

�               PACKET DRIVER			�

�                                               �

�������������������������������������������������

1) INTRODUCTION

The Packet driver EN32VLPK.COM for DOS conforms to the Packet Driver

Specification v1.09" developed by FTP Software Inc.

2) INSTALLATION

To install the driver, copy the Packet Driver EN32VLPK.COM from the

PKT directory in the Driver Diskette to your desired directory. The

packet driver can be loaded from the DOS prompt or from the

AUTOEXEC.BAT file.  The command line format for Packet Driver is:

EN32VLPK [-n] [-d] [-w] <INT=packet_int_no> [IOADDR=io_addr]

The <INT=packet_int_no> keyword is the software interrupt used to

communicate with the packet driver and must be included in the command

line. The valid range is from 0x60 to 0x80. The most commonly used

number if 0x60. For example:

EN32VLPK INT=0x60

Other optional options are explained below.

[-n]	-	The [-n] option converts Ethernet type 8137 and

Novell PSO-like packets. This is required if you

use a standard boot ROM and use the Packet Driver

and IPX included in the Packet Driver package, as

the drivers loaded from the boot image file.

[-d]	-	The -d option delays initialization. The -d option

will keep the initialization code after loading. This

is required for making the PROM boot work.

[-w]	-	The -w switch is used for Windows. Install the Packet

Driver before running MS-Windows. This switch does not

prevent Windows from swapping the network application

out of memory, it simply detects when that has

happened, and drops the packet.

IOADDR	-	Used to specify the IO address of the board. This

option is usually not necessary as the driver will

search for the installed board if this option is not

specified.

���������������������������������������������������Ŀ

�                                                   �

�                     UTILITY                       �

�                                                   �

�����������������������������������������������������

The configuration utility, VLSET.EXE, allows you to view the current

setttings of the ENET32-VL board and to change the settings. Settings

that can change include the IO address, Interrupt channel, Cable

type, Boot ROM address and Burst Mode.

���������������������������������������������������Ŀ

�                                                   �

�       HISTORY OF DRIVERS AND UTILITIES            �

�                                                   �

�����������������������������������������������������

1) INTRODUCTION

This section records the modifications made on the

device drivers and utilities.

1.1) ODI Workstation Driver

A) EN32VL.COM v1.00

a) First release.

1.2) OS2 Requester

A) EN32VL.SYS v1.00

a) First release.

1.3) ODI Server Driver for NetWare 4.x and 3.x

A) EN32VL.LAN v1.00

a) First release.

1.4) DOS NDIS Driver

A) EN32VL.DOS v1.00

a) First release.

1.5) OS/2 NDIS Driver

A) EN32VL.OS2 v1.00

a) First release.

1.6) Packet Driver

A) EN32VLPK.COM v1.00

a) First release.

1.7) Configuration Utility

A) VLSET.EXE v1.00

a) First release.

Provided by:  COMPEX Inc

4055, La Palma

Unit C, Anaheim

CA 92807, USA