Jun. 1995

DE-220 SERIES ETHERNET CARD

AT&T UNIX SVR4.2.2 DRIVER PROGRAM

===============================================

1. login UNIX as "root"

login: root

Password: XXXXXX

2.  Create directory /usr/lib/tcprt/dlk

#mkdir /usr/lib/tcprt

#mkdir /usr/lib/tcprt/dlk

#cd /usr/lib/tcprt/dlk

3.  Copy the object and install files to /usr/lib/tcprt/dlk

Insert the floppy diskette to drive A

#doscp a:de220.o /usr/lib/tcprt/dlk/de220.o

#doscp a:d22e /usr/lib/tcprt/dlk/d22e

#cd /usr/lib/tcprt/dlk

#chmod +x d22e

4.  Install the Driver

#./d22e

Do you wish to install or delete the network driver(i/d/q)?     i

/* first adapter install */

Do you have another adapter to install?                         y

Interrupt vector number[5]:                                     5

IOBase Address( in hexadecimal ) [300]:                         300

There are the device(s) available on your system:

1   d22e_0

Type the number of the device(s) to be configure with inet?     1

Please enter the IP host name for the device d22e_0(default:xx ) XXXX

Please Initialize the IP Address for host XXXX:                 n.n.n.n

Configure host XXXX with default Ethernet(TM) ifconfig option?  yes

/* second adapter install */

Do you have another adapter to install?                         y(n)

Interrupt vector number[5]:                                     10

IOBase Address( in hexadecimal ) [300]:                         340

There are the device(s) available on your system:

1   d22e_0

2   d22e_1

Type the number of the device(s) to be configure with inet?     2

Please enter the IP host name for the device d22e_1(default:xx ) YYYY

Please Initialize the IP Address for host YYYY:                 n.n.n.n

Configure host YYYY with default Ethernet(TM) ifconfig option?  yes

/* the third adapter */

Repeat the above steps ....