Jun. 1995

DE-220 SERIES ETHERNET CARD

Setup/Diagnostic PROGRAMS

===============================================

********************************************************************

*                                                                  *

*  DE-220 series Ethernet cards do not have any on-board jumpers   *

*  or switches. All the hardware settings can be specified         *

*  through our SETUP220.EXE program. Please run SETUP220.EXE       *

*  program to set up and test the hardware configuration.          *

*                                                                  *

********************************************************************

1. The files for setup and diagnostics:

README.DOC              : Description of how to use the setup programs

\INSTALL\SETUP220.EXE   : SETUP main program

\INSTALL\IPXODI.COM     : IPX protocal driver for ODI

\INSTALL\LSL.COM        : Link support layer for ODI

\INSTALL\DE22X.COM      : DE-220 ODI driver

To run the program, insert the DE-220 driver program diskette in drive

'A' and do the following:

> SETUP220           --run the SETUP220.EXE program

or

> SETUP220 /F        --run the SETUP220.EXE with parameter /F

if the data in your EEPROM has been corrupted

or you cannot configure your card through

normal procedure.

DE-220 Series Adapter parameter file table

+----------------+----------------------------------+

| KEYWORDS       |  AVAILABLE SELECTIONS            |

+----------------+----------------------------------+

| Connector Type |  UTP, BNC, AUI                   |

+----------------+----------------------------------+

| IRQ            |  3, 4, 5, 9, 10, 11, 15          |

+----------------+----------------------------------+

| I/O Base       |  300H, 320H, 340H, 360H          |

| Address        |                                  |

+----------------+----------------------------------+

| ROM Size       |  8K/16K/32K                      |

+----------------+----------------------------------+

| ROM Address    |  C000 C800 D000 D800             |

|                |  C200 CA00 D200 DA00(8K,16K)     |

|                |  C400 CC00 D400 DC00(8K,16K)     |

|                |  C600 CE00 D600                  |

|                |                                  |

|                |  DC00 (Ethernet II packet type)  |

|                |                                  |

+----------------+----------------------------------+

2.  SETUP220 lets you configure the hardware settings of DE-220.

You have the option to view/change :

Main Menu Options :

===================

1> Auto Setting Configuration

2> Set up Configuration

3> Adapter Basic Diagnostic

4> NetWork Diagnostic

Viewed items: Your Current Configuration

<1> Node ID

<2> Bus Mode

<3> Card Type

Viewed/Changed items : Set up Configuration

<1> Connector Type

<2> I/O Base Address

<3> Interrupt Number

<4> Remote Boot ROM Size

Base Memory Address

+-----------------------------------------------+

| Detailed Description of DE-220 Setup Programs |

+-----------------------------------------------+

The following is the setup program's main menu:

������      Main Menu    �����ͻ

�  Auto Setting Configuration  �

�  Setup Configuration         �

�  Adapter Basic Diagnostics   �

�  Network Diagnostics         �

������������������������������ͼ

1. Program auto-configure your DE-220 series card.

Such as connector type, I/O base address, interrupt number, and

remote boot function.

2. Use the Setup Configuration option configure your DE-220 series

card. Here you specify settings for connector type, I/O base

address, interrupt number, and whether or not to enable the remote

boot function.

3. The Adapter Basic Diagnostics option checks the card for proper

operation.

4. The Network Diagnostic option performs a traffic test for

checking the card's network performance. This test requires

that LSL.COM, DE220X.COM and IPXODI.COM be located in the

same directory as the setup program.

SETUP CONFIGURATION MENU

------------------------

���������� Setup Configuration ����������Ŀ

�                                         �

� Connector Type ------> UTP (RJ-45)      �

� I/O Base Address ----> 300H             �

� Interrupt Number ----> 3                �

� Remote Boot ---------> Disable          �

�������������������������������������������

1. The Connector Type setting lets you specify the connector to be

used with your DE-220 series card.

2. The I/O Base Address setting lets you specify the address

through which your microprocessor communicates with your card.

3. The Interrupt Number or IRQ setting is used by your card to

make a request for service from the microprocessor.

4. The Remote Boot setting allows you to use the remote boot

function of your card.

* To use the remote boot function, a boot ROM must be

installed on the card.

5. The Boot ROM Address setting indicates the starting memory

address for the boot ROM. (When Boot ROM enable)

6. The Boot ROM Size setting indicates the memory size for the

boot ROM. (When Boot ROM enable)

ADAPTER BASIC DIAGNOSTICS

-------------------------

The Adapter Basic Diagnostic option lets you run a test for

checking your DE-220 series card.

The following items are tested:

1. Current I/O Base Address

2. DC-DC Convert, Oscillator, Crystal T7213

3. EEPROM

4. LAN Setup Registers

5. Memory

6. LAN Controller Functions

7. Cable Connection

8. Loopback Mode

������������������� Adapter Diagnostic �����������������ͻ

�  Node Address     : 00 80 C8 00 00 01                  �

�  I/O Base Address : 300H                               �

�  Connector Type   : BNC (Thin Coax)                    �

�  Interrupt Number : 15(16-bit)                         �

����������������������� Cycle      ���������������������Ķ

�  Current I/O Base Address ...................  Pass    �

�  DC-DC Convert, Oscillator, Crystal T7213 ...  Pass    �

�  EEPROM .....................................  Pass    �

�  LAN Setup Registers ........................  Fail    �

�  Memory .....................................  Unknown �

�  LAN Controller Functions ...................  Unknown �

�  Cable Connection ...........................  Unknown �

�  Loopback Mode Testing ......................  Unknown �

�                                                        �

��������������������������������������������������������ͼ

NETWORK DIAGNOSTICS

-------------------

��� Advanced Diagnostics ��ͻ

� Setup as a master station �

� Setup as a slave station  �

���������������������������ͼ

The Network Diagnostic must have one master station. If you want to

use this workstation as a master, choose 'Setup as a master station'.

The other station must act as a slave station. To use this workstation

as a slave, select 'Setup as a slave station'.

Before running this network diagnostic, copy LSL.COM, IPXODI.COM,

DE220ODI.COM, and NET.CFG to the same directory as the setup program.

Slave and Master workstations must have the same frame type as

specified in net.cfg.

MASTER STATION

������� Master Station �����ͻ

� Waiting Sending Receiving  �

�    \       \       \       �

����������������������������ͼ

SLAVE STATION

���������������������������� Slave  Station  ��������������������������ͻ

� Node Address     : 00 80 C8 00 00 02                                  �

� I/O Base Address : 300H                    Test started at : 20:06:10 �

� Interrupt Number : 15(16-bit)              Current time    : 20:10:50 �

� Connector Type   : BNC (Thin Coax)                                    �

�����������������������������������������������������������������������Ķ

�                                                                       �

� Transmit Packet Length ..................................   1234      �

� Transmit Packet Counter .................................   1111      �

� Transmit Time Out Counter ...............................      0      �

�                                                                       �

� Received Packet Length ..................................   1234      �

� Received Packet Counter .................................   1111      �

� Received Error Counter ..................................      0      �

� No Response Counter .....................................      0      �

� Data Mismatch Counter ...................................      0      �

�                                                                       �

�����������������������������������������������������������������������ͼ