Installation Guide for Sun PC-NFS

=======================================

This file contains the information about how to install NIC100.SYS Driver

into Sun PC-NFS. Driver into PC-NFS as following procedures :

A. First-time installation on a dual-floppy PC without a hard disk :

-------------------------------------------------------------------

a. Create a bootable DOS diskette (containing the necessary DOS

programs).

1. Format a blank diskette using DOS FORMAT command with /s option.

2. Use DOS COPY command to copy ANSI.SYS file to the formatted

diskette.

3. Make a directory "DRIVERS" under the root directory.

4. Copy the NIC200.SYS driver file from \PCNFS directory on the

NIC100 Ethernet Adapter Driver Diskette to the \DRIVERS

directory on the formatted diskette.

b. Install PC-NFS on a Bootable DOS diskette as a PC-NFS Bootable

diskette for your PC-NFS workstation.

1. Insert the PC-NFS Installation Diskette (#1) into drive A:

and type A: and press <Enter>.

2. Insert the formatted bootable DOS diskette into drive B:.

3. Type install and press <Enter>.

4. Select a proper display mode for your PC.

5. Enter the letter of the drive where you want to install PC-NFS.

Since the formatted diskette is inserted in drive B:, you should

type B: and press <Enter>.

6. Select the type of network connection. "Ethernet" is highlighted.

Press <Enter> to select "Ethernet".

7. Select the network adapter you will use for PC-NFS.

Since NIC100 Ethernet Adapter does not appear in the list of the

supported Ethernet adapters, highlight "Other" and press <Enter>.

8. When you select "Other", the next screen asks you to enter a line

specifying the device driver you want to use as shown below. The

installation program adds the line you type to the CONFIG.SYS file

in the formatted DOS diskette in drive B:.

Config.sys line for your driver

:_

At the prompt, enter the following line and press <Enter>.

DEVICE=A:\DRIVERS\NIC100.SYS /pxx /ixx

p for io_addr : The i/o balse address setting on the Adapter

The value can be one of 200, 220, 240, 260

300, 320, 340, and 360 for the NIC100

adapter. The default value is 300.

i for int_level : Hardware interrupt setting on the Adapter

The value can be one of 2,3,4,5,10,11 or

12 for the NIC100 adpater.

The default value is 3.

9. Follow the on-screen instructions to copy the files from the

PC-NFS distribution diskettes to the bootable DOS diskette,

until the installation program displays the final screen,

instructing you to remove the distribution diskette and give

the Server Diskette to your system administrator.

10. The bootable DOS diskette now becomes an PC-NFS bootable

diskette for you computer. PC-NFS installation is complete.

11. Press <Enter> to start the NFSCONF configuration program.

12. Please follow the instructions described in Chapter 3 of the

"PC-NFS Installation Guide" for configuring PC-NFS for your PC

and network.

c. Power off your computer and install the NIC100 Ethernet Adapter

onto  your computer as the instructions described in the "NIC100

Ethernet Adapter Installation Guide".

d. Reboot your computer with the bootable PC-NFS diskette.

e. Now, your computer is ready for your PC-NFS connection.

B. If you have a single-floppy PC without a hard disk :

------------------------------------------------------

You cannot install PC-NFS on a single-floppy PC without a hard disk;

however, you can create a PC-NFS boot diskette that contains the

necessary files to run PC-NFS on your single-floppy PC. Follow the

following procedure.

a. Create a bootable DOS diskette (containing the necessary DOS

programs).

1. Format a blank diskette using DOS FORMAT command with /s option.

2. Use DOS COPY command to copy ANSI.SYS file to the formatted

diskette.

3. Create an \DRIVERS directory on the boot diskette using the DOS

MKDIR command.

4. Create an \NFS directory on the boot diskette using the DOS MKDIR

command.

b. Copy the necessary PC-NFS files from the PC-NFS distribution

diskettes to the \NFS directory on the DOS boot diskette.

Please refer to the page 31 and page 32 for the file names required

to be copied from the distribution diskettes to the DOS boot

diskette.

c. Copy the NIC100.SYS driver file from \PCNFS directory on the NIC100

Ethernet Adapter Driver Diskette to the \DRIVERS directory on the

DOS boot diskette.

d. Create AUTOEXEC.BAT and CONFIG.SYS files, using any PC text editor,

such as Edlin. The sample AUTOEXEC.BAT and CONFIG.SYS are shown

below:

AUTOEXEC.BAT

--------------------------------------------

PATH = A:\NFS

SET NFSDRIVE=A

PRT *

NFSRUN

--------------------------------------------

CONFIG.SYS

--------------------------------------------

DEVICE = ANSI.SYS

BUFFERS = 20

FILES = 20

DEVICE = \NFS\PCNFS.SYS

DEVICE = \NFS\SOCKDRV.SYS

DEVICE = \DRIVERS\NIC100.SYS /pxx /ixx

LASDRIVE = V

--------------------------------------------

Note : the options for DEVICE = \DRIVERS\NIC200.SYS /pxx /ixx

are described as below:

p for io_addr : The i/o balse address setting on the Adapter

The value can be one of 200, 220, 240, 260

300, 320, 340, and 360 for the NIC100

adapter. The default value is 300.

i for int_level : Hardware interrupt setting on the Adapter

The value can be one of 2,3,4,5,10,11 or

12 for the NIC100 adpater.

The default value is 3.

e. Remove your boot diskette and insert the PC-NFS Configuration

Diskette (#3) in drive A.

1. Type SET NFSDRIVE=A and press <Enter>.

2. Type b:nfsconf and press <Enter>

3. When prompted, insert your boot diskette into drive A.

4. Follow instructions in Chapter 3 of "PC-NFS Installation Guide"

to configure PC-NFS.

f. Power off your computer and install the NIC100 Ethernet Adapter

onto your computer as the instructions described in "NIC100

Ethernet Adapter Installation Guide".

g. Reboot your computer with the bootable PC-NFS diskette.

h. Now, your computer is ready for your PC-NFS connection.

C. First-time installation on a bootable Hard Disk or Partition :

----------------------------------------------------------------

a. Install PC-NFS on a bootable hard disk

1. Create a \DRIVERS directory in your hard disk drive C, for

example.

2. Copy the NIC100.SYS driver file from \PCNFS directory on the

NIC100 Ethernet Adapter Driver Diskette to the \DRIVERS

directory on your hard disk drive C.

3. Insert the PC-NFS Installation Diskette (#1) into drive A:

and type A: and press <Enter>.

4. Type install and press <Enter>.

5. Select a proper display mode for your PC.

6. Enter the letter of the drive where you want to install PC-NFS.

Press <Enter> to accept the default drive C, or type another

drive letter and press <Enetr>. In the following instructions,

we use drive C as an example.

7. Select the type of network connection. "Ethernet" is highlighted.

Press <Enter> to select "Ethernet".

8. Select the network adapter you will use for PC-NFS.

Since NIC100 Ethernet Adapter does not appear in the list of the

supported Ethernet adapters, highlight "Other" and press <Enter>.

9. When you select "Other", the next screen asks you to enter a line

specifying the device driver you want to use as shown below. The

installation program adds the line you type to the CONFIG.SYS file

in the formatted DOS diskette in drive B:.

Config.sys line for your driver

:_

At the prompt, enter the following line and press <Enter>.

DEVICE=C:\DRIVERS\NIC100.SYS /pxx /ixx

p for io_addr : The i/o base address setting on the Adapter

The value can be one of 200, 220, 240, 260

300, 320, 340, and 360 for the NIC100

adapter. The default value is 300.

i for int_level : Hardware interrupt setting on the Adapter

The value can be one of 2,3,4,5,10,11 or

12 for the NIC100 adpater.

The default value is 3.

10. Follow the on-screen instructions to copy the files from the

PC-NFS distribution diskettes to the bootable DOS diskette,

until the installation program displays the final screen,

instructing you to remove the distribution diskette and give

the Server Diskette to your system administrator.

11. Press <Enter> to start the NFSCONF configuration program.

12. Please follow the instructions described in Chapter 3 of the

"PC-NFS Installation Guide" for configuring PC-NFS for your PC

and network.

b. Power off your computer and install the NIC100 Ethernet Adapter

onto your computer as the instructions described in the "NIC100

Ethernet Adapter Installation Guide".

c. Now, your computer is ready for your PC-NFS connection.

D. If your computer already has PC-NFS software installed :

----------------------------------------------------------

a. Locate the directory path where you place your previous network

driver file.

We choose C:\DRIVERS as an example.

- Copy the NIC100.SYS driver file from \PCNFS directory on the NIC100

Ethernet Adapter Driver Diskette to the \DRIVERS directory on your

hard disk drive C.

b. Change the current directory to the root directory of your installed

PC-NFS files by using DOS CD command.

c. Start nfsconf program.

1. Type nfsconf and press <Enter>.

2. Select "Network" from the Main Configuration Screen.

3. Select "Direct connect"

4. Back to main screen.

5. Select "Adapter" configuration.

6. Select <Ethernet E>.

7. Select " <Other> Use a manufacturer's driver: "

8. When you select "Other", the next screen asks you to enter a line

specifying the device driver you want to use as shown below. The

installation program adds the line you type to the CONFIG.SYS file

in you hard disk drive C.

Config.sys line for your driver

:_

At the prompt, enter the following line and press <Enter>.

DEVICE=C:\DRIVERS\NIC100.SYS /pxx /ixx

p for io_addr : The i/o base address setting on the Adapter

The value can be one of 200, 220, 240, 260

300, 320, 340, and 360 for the NIC100

adapter. The default value is 300.

i for int_level : Hardware interrupt setting on the Adapter

The value can be one of 2,3,4,5,10,11 or

12 for the NIC100 adpater.

The default value is 3.

9. Review or change other Configurations as instructions described

in your "PC-NFS Installation Guide".

d. Power off your computer and install the NIC100 Ethernet Adapter onto

your computer as the instructions described in the "NIC100 Ethernet

Adapter Installation Guide".

e. Now, your computer is ready for your PC-NFS connection with NIC100

Ethernet Adapter.