Packet Driver for FTP PC/TCP Installation

===============================================

The NIC100.COM program is complies with the FTP PC/TCP Packet driver

Specifications. This packet driver allow you to use NIC100 Jumperless

Ethernet Adapter with FTP PC/TCP software.

Installation Procedures :

a. Execute the NIC100.COM program by follows command line :

> NIC100  [<SOFTWARE_INT> <IRQ> <IO_ADDR>]

<SOFTWARE_INT> :  interrupt vector number, which can be

decimal, hexadecimal or octal format.

if:

decimal format -- from 96 to 128

hexadecimal ----- from 0x60 to 0x80

octal format ---- from 0140 to 0200

<IRQ> :		interrupt setting. The setting can be selected

2,3,4,5,10,11 or 12 for NIC100 Ethernet Adapter.

The default setting is 3.

<IO_ADDR>  :	I/O Base Address setting on the NIC100 Ethernet

Adapter. The setting can be selected 0x300, 0x320,

0x340, and 0x360 .

The default setting is 0x300.

Note: user can use default settings by only type "NIC100". The

program will automatic search NIC100 Ethernet adapter,

and uses its internal setting.

b. Execute the \ETHDRV.EXE device program, which is provided by FTP

with PC/TCP software package together.

Now, your NIC100 Ethernet Adapter is work with FTP PC/TCP.

NOTE: Except the FTP software's PC/TCP package, most of the DOS

version's TCP/IP software also can be run with packet driver.

Please refer to the software package installation for packet

driver install procedure.