The Guide of SETUP.EXE program

====================================

This file is used to tell the user : how to use the file "SETUP.EXE".

The "SETUP.EXE" is built for auto-configuring the NIC100 Jumperless

Ethernet Adapter and auto-installing the driver.

When user wants to execute this program, just types the "SETUP" in DOS

prompt. The program will instruct the user to auto-install the driver step

by step.

a. First, the program will auto-configure IOBASE and IRQ of the NIC100

Jumperless Adapter to avoid the conflict with other add-on cards in

user's computer.

b. After a moment, the screen will show the Hardware Auto-Configuration

of user's NIC100 adapter and the Software Auto-Install menu.

If hardware configuration is not wanted, user just presses "F10"

function key to manually configure the hardware IOBASE and IRQ.

c. In Software Auto-Install menu, user can chooses one of driver to

install. When Installation is completed, the user also can auto-login

to NetWare server if installed NetWare driver and the link was OK.