NETWORK DRIVER FOR IBM LAN SERVER 3.0 DOS CLIENT

================================================

Install NDIS driver for IBM LAN Server DOS Client

-------------------------------------------------

1. Use DXMAID to install the protocol and adapter driver

of DOS Requester for the client.

2. Select WD driver as a temporary driver.

3. DXMAID program created a sub-directory \lsp.

4. Copy L8029.DOS NDIS driver from the network adapter driver

diskette. L8029.DOS is located in

\NDIS\L8029.DOS

5. Edit the CONFIG.SYS, and PROTOCL.INI as the following samples:

*** CONFIG.SYS ***

device=\lsp\protman.dos /I:\lsp

NDIS driver -> device=\lsp\l8029.dos

device=\lsp\dxma0mod.sys 001

device=\lsp\dxmj0mod.sys

lastdrive=Z

*** PROTOCOL.INI ***

[PROTMAN_MOD]

DriverName = PROTMAN$

[DXMJ0MOD_MOD]

DriverName = NETBEUI$

Bind to le8029r ->  Bindings = LE-8029R

NCBS = 15

ADAPTRATE = 1000

WINDOWERROR = 0

PIGGYBACKACKS = 1

DATAGRAMPACKETS = 2

PACKETS = 50

PIPELINE = 20

MAXTRANSMITS = 6

MINTRANSMITS = 2

LANABASE = 0

CHAINX5C = 0

[LE-8029R]

DriverName = L8029$