LAN Manager Installation

for DOS and OS/2

========================

Introduction

------------

This directory contains the following files for installing the following

Ethernet adapters on a LAN Manager DOS workstation, LAN Manager OS/2

workstation or server with the associated NDIS compliant network device driver.

The file layout in the current directory is shown as below:

INSTALL.DOC    This file

MSLANMAN.DOS

|

|___ DRIVERS

|

|___ ETHERNET

|       |

|       |___ LE8029R

|               |___ L8029.DOS

|               |___ PROTOCOL.INI

|

|___ NIF

|

|_____ LE8029R.NIF

MSLANMAN.OS2

|

|___ DRIVERS

|

|___ ETHERNET

|       |

|       |___ LE8029R

|               |___ L8029.OS2

|               |___ PROTOCOL.INI

|

|___ NIF

|

|_____ LE8029R.NIF

Install the adapter on an MS-DOS LAN Manager workstation

--------------------------------------------------------

1. LAN Manager new installation

----------------------------

a. Start the computer and access the MS-DOS command line.

b. Insert LAN Manager DOS Setup disk into disk drive A:.

c. Start the Setup program by typing

setup

d. Follow the on-screen instructions until "Select Available

Network Drives:" message appears.

- select <Other Driver>.

- insert "LE-8029R Driver Diskette"

in drive A:.

- type the drive name A:\MSLANMAN\   ; type the drive where

the driver working

diskette is inserted

- when a list of LE-8029R Ethernet Adapter

select the adapter for the installation.

- follow the on-screen instructions for the rest

installation procedure.

If your computer already has LAN Manager software installed

-----------------------------------------------------------

a. Make the LAN Manager directory the current directory.

For example, if the LAN Manager software is installed

in C:\LANMAN, you would type

c:

cd \lanman

b. Execute LAN Manager Setup program. You would type

setup

- select Configuration               ; Press <Alt>+C

- select Network Drivers             ; Select network driver

configuration

- select <Remove Config>             ; Remove current network

driver configuration

- select <Add New Config>            ; Standard driver list

appears.

- select <Other Driver>.

- insert "LE-8029R Driver Diskette"

in drive A:.

- type the drive name A:\MSLANMAN\   ; type the drive where

the driver working

diskette is inserted

- when a list of LE-8029R  Ethernet Adapter

select the adapter for the installation.

- Review or change the other configuration for your

workstation

2. The PROTOCOL.INI together with the driver is a text file containing

hardware configuration data for the network adapter. The following

table summarizes the possible entries and values for each section

name in PROTOCOL.INI.

--------------------------------------------------------------

Entry        Units     Range                        Default

--------------------------------------------------------------

DRIVERNAME    -          -                          L8029$

--------------------------------------------------------------

If the adapter is not found or not initialized successfully, the driver

shows error message.

Install the adapters in one LAN Manager OS/2 server or workstation

------------------------------------------------------------------

The installation procedure for LAN Manager OS/2 is very similar to

that for MS-DOS LAN Manager installation.

Start LAN Manager Setup program, then follow the same procedure as

that for MS-DOS LAN Manager.

Install Multiple Adapters in one LAN Manager OS/2 server or workstation

-----------------------------------------------------------------------

You may install multiple adapters (four at maximum) in one

system for your OS/2 LAN Manager. You must select the  driver

the times equal to the number of adapters installed, during

running LAN Manager Setup program.

After installation for multiple adapters, the master PROTOCOL.INI

in LAN Manager directory will be configured as the following

example:

PROTOCOL.INI Example ( for example, three LE8029R adapters installed )

.

.

[LE8029R_NIF]

DRIVERNAME = L8029$

Adapter ID = xxxx

[LE8029R_NIF2]

DRIVERNAME = L8029$

Adapter ID = xxxx

[LE8029R_NIF3]

DRIVERNAME = L8029$

Adapter ID = xxxx

.

.

In regard to CONFIG.SYS, you will find the file contains only one DEVICE

line for L8029.OS2 instead of three as shown in the following example.

CONFIG.SYS Example

.

.

DEVICE=C:\LANMAN\DRIVERS\ETHERNET\LE8029R\L8029.OS2

.

.

Since L8029.OS2 driver can support four adapters at the same time,

LAN Manager Setup program configures CONFIG.SYS to load L8029.OS2

image just once during booting. This advantage can save your system memory.