How to install the driver with the adapter

for NCSA's TCP/IP softwares on an MS-DOS computer

=================================================

A. How to install Packet Driver

----------------------------

1. Execute the LE8029.COM program located on the directory

\PKTDRV in this driver diskette.

Command line :

>LE8029 <packet_int_no>

packet_int_no : software interrupt vector number, which

can be decimal, hexadecimal, or octal

format.

decimal format -- from 96 to 127

hexadecimal ----- from 0x60 to 0x7F

octal format ---- from 0140 to 0199

Now, you are ready to use NCSA's TCP/IP softwares

with the Ethernet Adapter.

B. A sample file for "CONFIG.TEL" for NCSA's TCP/IP softwares using

Packet Driver

-----------------------------------------------------------------

Sample file "CONFIG.TEL"

#myip=RARP

myip=132.147.160.5   # ip types are

#  bootp        - sends out a BOOTP request for your IP

#  rarp         - Uses reverse ARp to get your IP

#  xx.yy.zz.qq  - your PC IP address

netmask=255.255.0.0  #  needed if not using BOOTP.

#  otherwise not needed because Bootp gets your netmask

hardware=packet

#  ^       # harware choices

#  |       #  3com         - 3com 3c501

#  |       #  3c523        - 3com 3c523

#  |       #  wd800        - Western Digitial 800E

#  |       #  nicpc        -

#  |       #  nicps        -

#  |       #  ni5210       - Micom Interlan NI5210 card

#   -----> #  packet       - FTP packet Driver spec

#          #  (currently only Ethernet class devices are supported)

#interrupt=3         # hardware IRQ interrupt, If usign packet drive,

# don't care about the former IRQ setting

address=0            # base memory address or packet driver class

# if using packet driver (0 == default == ethernet)

# or class=6 for slip

ioaddr=61            # I/O address or packet int vector if using packet driver

#  ^           # If = 0 and packet driver, telbin looks for first

#  |           # packet driver found between 60H and 7FH

#  |

#   -------->  # Packet driver interrupt vector <packet_int_no> number

# The number between 61H and 7FH

# The figure of the Default settinh is hexadecimal format.

video=auto           # video choices

#  auto         - automatically determines video type

#  cga

#  ega

#  hercules

#  pga

#  no9

bios=no              # Bios=Yes -> uses bios, bios=no -> goes direct to screen

# If you have a snowy CGA, use bios=yes

erase=delete         # use delete code or backspace code for <- key?

# legal values are "delete" and "backspace"

clearsave=yes        # save visible lines in scrollback when

# clearing screen

scrollback=100       # number of lines of scrollback per session

# following are screen colors in a session

# you can also place them after a host declaration

# below, to set the color on a host by host basis

# a foreground color that is capitalized

# sets the bold bit, if a background color

# is capitalized it sets the blink bit.

ftp=yes              # yes to allow incoming FTP sessions, no otherwise

rcp=no               # Remote Copy, yes if you want to allow it

# otherwise No.

## *********************************************************************

arptime=10           # arp timeout in seconds

# affects machines on your local network

retrans=1            # starting retransmit time out in ticks

# 1/18ths of sec MAX 100, min 1

mtu=1024             # maximum transmit unit in bytes

# outgoing packet size, MAX=1500

maxseg=1024          # largest segment we can receive

# whatever the hardware can take, MAX=4096

rwin=2048            # most bytes we can receive without ACK

# =TCP window size, MAX=4096

contime=20           # timeout in seconds to try connection

# before returning error to user

#name=mygateway

host=scosysv         # your host name

hostip=132.147.160.1 # xx.yy.zz.qq - your host IP address

gateway=1            # This machine is a gateway for me