LE8029R DRIVER INSTALLATION

FOR NETWARE 3.12

===========================

Installing the Adapter in a NetWare 386 v3.12 File Server

==============================================================

a. **** Load LAN driver ****

After the following tasks have been done, use LOAD LAN driver to link

the appropriate drivers with the network boards installed in the file

server.

* Run SERVER

* Load loadable modules for the disk drivers

* Load INSTALL

Load LAN drivers Command format

:LOAD [path LAN_driver [driver_parameter..]

Command parameters

Path

Replace path with a full path; begin with a DOS drive

letter or a NetWare volune name. If you do not specify

a path and the SYS volume is mounted, the operating

system assumes that the loadable LAN driver module

is located in the SYS:SYSTEM directory.

LAN_driver

All NetWare 386 loadable LAN drivers end with a .LAN

extension.

The loadable LAN driver module included in the

driver diskette is located in the subdirectory

\NW312.

For easy installation, you may copy the module to the

drive containing SERVER.EXE, either the hard disk or

diskette.

Driver_parameters

Most drivers use one or more of the following parameters.

DMA=number              FRAME=name

INT=number              MEM=nember

NAME=board name         NODE=number

PORT=number             RETRIES=number

SLOT=number

Please refer to NetWare 386 System Administration manual

for the detail definition of each parameters.

* The LAN driver prompts you for required parameters (not optional) if

you do not specify them on the command line.

* The LAN driver does not prompt you for optional parameters; the default

values are coded into the driver. To modify the default value, you

must specify the new value at the command line when you load the driver.

b. **** Bind the protocol to the LAN driver ****

NetWare 386 allows protocols other than IPX to function on the network.

The steps below use IPX for an example protocol to bind each LAN driver

in the file server.

Command format

BIND protocol [TO] LAN_driver | baord_name

[driver_parameter..][protocol_parameter..]

1) Bind IPX to the LAN driver.

Replace the protocol with IPX.

Replace LAN_driver with either the name of the driver you

loarded for the netowrk board or the board name you assigned

when you loaded the driver.

Include a driver parameter when you have more than

one netowrk board of the same type in the server.

You do not need driver parameters if you bind a protocol

to a board name rather than a LAN driver.

Use protocol_parameter for the parameters unique to

the communication protocol. IPX has one, NET.

2) Assign the network number.

NET=number

Identifies a unique network number for the cabling system

attached to the board.

Each LAN driver must be assigned the network number that

identifies the cabling system to which it communicates

with similar LAN drivers on the network.