Network Driver for PathWay Runtime

==================================

Introduction

------------

PathWay Runtime (release 1.2) provides the low level services that permit

TCP/IP applications (such as PathWay Access) to function on your computer.

PathWay Runtime must be installed and configured before PathWay applications

or PathWay API can be installed and used.

The following sections describe how to configure network adapter driver for

PathWay Runtime.

PathWay Runtime can support the following driver specifications in different

environments that co-exist with PathWay applications:

Driver  Environment     Typical Network Software

------  -----------     ------------------------

NDIS    Shared          MS LAN Manager, VINES, LAN Requester

DEC Pathwork

------------------------------------------------

ODI     Shared          Novell Netware

------------------------------------------------

PDS     Shared          LANs with Packet driver

------------------------------------------------

PDS: Packet Driver Specifications

Where are the Drivers ?

-----------------------

The ODI driver LE8029.COM is located on the subdirectory \DOSODI of

the driver diskette.

The DOS NDIS driver L8029.DOS is located on the subdirecotry

\NDIS\ of this driver diskette.

Together with L8029.DOS, you can find a sample PROTOCOL.INI file.

For Packet driver, you can find LE8029.COM on the subdirectory \PKTDRV

of the driver diskette.

The above drivers can work with the follwoing models with each associated

available settings:

LE-8029R

LE-8029RB

LE-8029RT

Information Common to all Driver Files

--------------------------------------

No matter what the driver specification, the AUTOEXEC.BAT file always

contains the following entries for PathWay Runtime:

pwconfig -n:65

drivername

pwtcp

Please refer to the PathWay Runtime Installation manual for details.

PathWay Runtime in a DOS dedicated environment with NDIS driver

---------------------------------------------------------------

1. Prepare a PROTOCOL.INI as the example below and place it on a driver

sub-directory, for example C:\LE8029R:

PROTOCOL.INI   Examples lines for LE8029R

[LE8029R_NIF]

DRIVERNAME = L8029$

2. Copy PROTMAN.EXE of PathWay Runtime to C:\LE8029R. Or, if you have

PROTMAN.DOS available from other vendor, copy it into C:\LE8029R.

3. Copy NDIS driver "L8029.DOS" to C:\LE8029R.

4. Edit CONFIG.SYS to add the following two lines:

device=c:\LE8029R\PROTMAN.EXE /i:\LE8029R

device=c:\LE8029R\L8029.dos

* where /i:\LE8029R specifies the directory path of PROTOCOL.INI

5. Install your network adapter into your workstation and boot up

the workstation to load PROTMAN.EXE and L8029.DOS into memory.

This step must be done before you execute PWSETUP program.

6. Run PathWay PWSETUP program to install PathWay Runtime.

7. Select NDIS driver and complete the installation procedure.

8. Verify the configuration of AUTOEXEC.BAT

AUTOEXEC.BAT   The following lines were added by setup program when

PathWay Runtime was installed.

set path=c:\pathway;%path%

pwconfig -n:65

ndis -d:1 -i:3

pwtcp

* ndis -d:1 -i:3 where -i:3 is the IRQ setting of you network adapter.

Due to the bug of PWSETUP program, one line of "pwbind" was missing

between the lines of "ndis -d:1 -i:5" and "pwtcp". You have to add

the line into AUTOEXEC.BAT as below by yourself:

set path=c:\pathway;%path%

pwconfig -n:65

ndis -d:1 -i:5

--------->  pwbind

pwtcp

PathWay Runtime in a NetWare environment with ODI driver

--------------------------------------------------------

1. In this section, Netware ODI for DOS is assumed to have been installed

in you workstation. Otherwise, install Netware software with ODI driver.

2. Bootup and load Netware LSL and ODI driver, before you install PathWay

Runtime. This procedure is necessary to be done before you can install

PathWay Runtime.

3. Run PathWay PWSETUP program to install PathWay Runtime.

4. Select ODI driver and complete the installation procedure.

5. After installation, verify the configuration of the AUTOEXEC.BAT and

NET.CFG files.

AUTOEXEC.BAT    The following lines should be included in the file:

lsl

le8029

ipxodi

netx

set path=c:\pathway;%path%

pwconfig -n:65

odi -i:3

pwtcp

* odi -i:3 where -i:3 is the IRQ being used by the network adapter.

NET.CFG         Example

Protocol IPX

Bind LE8029

Protocol IP

Bind LE8029

Protocol ARP

Bind LE8029

Link Driver LE8029

frame ETHERNET_802.3

Protocol IPX 000000000000 ETHERNET_802.3

frame ETHERNET_II

Protocol IP 000000000800 ETHERNET_II

Protocol ARP 000000000806 ETHERNET_II

PathWay Runtime in a dedicated environment with PDS driver

----------------------------------------------------------

1. Copy the PDS driver LE8029.COM to a sub-directory, for example, C:\PKT.

2. Please review the install.doc file, which is together with LE8029.COM.

3. Add the following command lines of loading Packet driver LE8029.COM in

AUTOEXEC.BAT:

set path=c:\pkt;%path%

LE8029 <packet_int_no>

4. Run PathWay PWSETUP program to install PathWay Runtime.

5. Select PDS driver and specify the directory C:\PKT (same as that in point 1)

and complete the installation procedure.

6. After installation, AUTOEXEC.BAT should have the following lines:

set path=c:\pkt;%path%

LE8029 <packet_int_no>

set path=c:\pathway;%path%

pwconfig -n:65

pktdrv -i:3

pwtcp

* pktdrv -i:3 where -i:3 must match the value of the adapter's IRQ setting