STANDARD MICROSYSTEMS CORPORATION

Novell NetWare 386 3.1 Server driver

For EtherCard PLUS Family LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

The purpose of this document is to briefly explain the

installation of the NetWare 386 LAN drivers that are used

with the Standard Microsystems EtherCard PLUS Family LAN adapters.

The files in the release diskette are:

INSTALL.DOC     This file.

RELEASE.DOC     Version number and specific release

information.

SMCPLUSS.LAN    NetWare 386 3.1 Server driver.

1.  Refer to the NetWare 386 Installation manual and follow

the instructions to partition the hard disk and to copy

the necessary system files to drive C:.

2.  Copy SMCPLUSS.LAN from this diskette to drive C:.

3.  Follow the "Run SERVER" and the "Load loadable modules"

sections of Novell's Installation Manual to Step (3) -

Load LAN driver module.

load c:smcpluss <Enter>

4.  Answer the prompts for the hardware attributes of each

adapter. Depending on the type of adapter you have, there

are up to four adapter specific configuration parameters

the driver may ask you to supply:

1) I/O Base Address.

2) Interrupt Request (IRQ) Level.

3) RAM Buffer Base Address.

4) Size of RAM Buffer - Enter 200 for  8K bytes

800 for 32K bytes.

5) MCA Bus Slot number (Micro Channel Only).

For the SMC8003EP, WD8003EB, SMC8003W, SMC8013EP,

SMC8013W, SMC8013EW, SMC8003WC, SMC8013WC, SMC8013EPC,

and SMC8013EWC, the driver will only prompt you for the

I/O Base Address.

For the WD8003E, WD8003WT, WD8003EBT, WD8003S, and WD8003SH,

the first four parameters listed above are required.

For the WD8013EBT adapter, the first three parameters

listed above are required.

For all Micro Channel adapters, only the bus slot number

is required.

If you do not want the prompts to appear, you can include

the required configuration parameters in a command line.

Refer to the NetWare 386 3.1 System Administration manual

under LOAD LAN drivers.

For the SMC8003EP, 8003EB, 8003W, 8013EP, 8013W, 8013EW,

8003WC, 8013WC, 8013EPC, and 8013EWC:

load c:smcpluss port=280 <Enter>

For the 8003E, 8003WT, 8003EBT, 8003S, and 8003SH:

load c:smcpluss port=280 int=3 mem=d0000:200 <Enter>

For 8013EBT adapters:

load c:smcpluss port=280 int=3 mem=d0000 <Enter>

For all Micro Channel adapters.

load c:smcpluss slot=1 <Enter>

5.  Proceed with Step (4) - Load other loadable modules - to

continue the installation of NetWare 386 3.1.

NOTE: The driver has to be bound to start working (refer to the

section "Bind the protocol to the LAN driver").

Novell NetWare 386 3.1 Server driver Custom Variables

1.      CollisionsCount

This counter contains the total number of collisions

that have been detected by the hardware when this driver has

attempted to transmit a packet accross the network.

2.      ExcessCollisionsCount

This counter contains the total number of times that the

hardware has not been able to transmit a packet because of

collisions. The hardware will automatically re-attempt to transmit

a packet up to 16 times if it detects that a collision has occured.

On the 16th consective collision for a given packet, this variable

is incremented.

3.      TxTimeOutCount

The number of times a Transmit complete interrupt did not occur

within 2 seconds of starting a packet transmission. When this occurs,

the NIC is reset and an atttempt is made to re-transmit the packet.

4.      BadNetworkErrorCount

This count contains the number of times that a packet was not

transmited. If a Transmit Timeout occurs 10 times in a row for

a given packet, the packet is discarded, this variable is incremented

and an attempt to send the next packet is begun.

5.      UnderrunErrorCount

This variable counts the total number of times that the transmision

FIFO has been underrun by the DMA controller. The NIC contains a

16-byte FIFO. During transmission the Transmit Serializer reads

data from the FIFO and the DMA writs data into the FIFO from memory.

If the FIFO becomes empty because the DMA does not write to it and

the transmision byte count register on the NIC indicates that the

packet has not been completely transmitted, a FIFO Underrun occurs.

Transmission of the packet is aborted when this event happens.

6.      RxCRCErrorCount

This count contains the total number packets received with CRC

errors. During transmission, CRC logic in the hardware generates

a 32 bit CRC for the transmitted bit sequence and appends these

four bytes as the last four bytes of a transmited packet. During

reception, CRC logic grenerates a CRC field from the incoming packet

and compares it to the last to four bytes of the packet. If the

values do not match, a CRC Error is generated.

This count is also incremented when Frame Alignment errors

occur. A Frame Aligment error indicates that the incoming packet

did not end on a byte boundary and consequently the CRC did not

match at the last byte boundary.

7.      RxFrameCRCErrorCount

This count contains the total number packets received with a

Frame Aligment error. This error indicates that the incoming packet

did not end on a byte boundary and the CRC did not match at the

last byte boundary.

8.      RxFIFOOverrunErrorCount

This countains the total number of times that the FIFO has

been overrun when receiving a packet. The NIC contains a

16-byte FIFO. During reception the Receive Deserializer writes

data into the FIFO and the DMA reads data from the FIFO. If the

DMA cannot begin a bus access to write a byte of the received

packet within the time it takes to fill the FIFO, a FIFO Overrun

occurs. Reception of the packet is aborted when this event happens.

9.      RxMissedPacketCount

This count contains the total number to times that packets have

not been received because the packet receive buffer queue is full.

10.     GotNothingCount

The number of times a receive packet interrupt occured without

the NIC having placed any data in a recieve buffer.

11.     NICResetCount

Whenever the NIC is reset, this variable is incremented. This

counter is incremented when the board is initialized for the

first time, the board was in a SHUTDOWN state and then restarted,

or a Transmit Timeout occured.

12.     UnsupportedMediaCount

The number of packets that have been received of a frame

type that the driver has not been configured for (ETHERNET_802.2,

ETHERNET_802.3, ETHERNET_SNAP, ETHERNET_SNAP) or does not support.

13.     EnqueueSendCount

The number of times the driver had a packet ready to transmit while

the NIC was still transmitting the previous packet.

14.     DequeueSendCount

The number of times packets have been removed from the transmission

queue where they have had there transmision delayed. EnqueueSendCount

minus DequeueSendCount will be the current number of packets queued

for transmission. On a properly operating network this number should

be zero.

15.     E8023PacketsAcceptedCount

Number of ethernet 802.3 packets that have been received by this

network node.

16.     EiiPacketsAcceptedCount

Number of Ethernet II packets that have been received by this

network node.

17.     E8022PacketsAcceptedCount

Number of Ethernet 802.2 packets that have been received by this

network node.

18.     ESNAPPacketsAcceptedCount

Number of Ethernet SNAP packets that have been received by this

network node.

19.     BackToBackTxCount

Indicates the number of times a Transmit Complete, Transmit Abort

or Transmit Timeout interrupt occured, and, there was another

transmit packet already buffered to the LAN Adapter's RAM ready

for immediate tranmission.

20.     NICBusyCount

Indicates the number of times that a transmit packet was moved

to the LAN Adapter's RAM and transmission was not started because

the NIC was currently transmitting another packet.

21.     NICNeedsServiceCount

The number of times that a transmit packet was moved

to the LAN Adapter's RAM, and, the NIC was not busy transmitting

a packet, and, the NIC had a Transmit Complete or Transmit Abort

interrupt that needed servicing before the next packet could

be transmitted.

Novell NetWare 386 3.1 Server driver Error Messages

1.      Error: Verifying LAN Adapter at Specified I/O Port.

The driver was unable to find a lan card signature id

at the spcified base I/O port.

2.      Error: Verifying LAN Adapter in Specified Slot.

The driver was unable to find a matching lan card Adapter

Id byte in a Micro Channel system.

3.      Error: Maximum LAN boards loaded for this driver.

This error message indicates that the maximum number of lan cards

that a single driver can support has been exceded. The current

limit is eight lan adapter cards.

4.      Error: Could not find matching virtual adapter.

This message is displayed when attempting to load a new frame type

for an adapter that has already been loaded, and the parameters

describing the adapter do not match any of the currently running

adapters.

5.      Error: Unable to get resource tag.

This is an internal OS error that should occur only when the

system is out of memory.

6.      Error: Unable to get memory.

This is an internal OS error that occurs when the driver is unable

to allocate memory needed for its local data.

7.      Error: Unable to set hardware interrupt.

This is an internal OS error that should occur only when the

system is out of memory or, an attempt was made to share an

un-sharable interrupt, or an invalid interrupt was specified.

8.      Error: Hardware configuration conflict.

This is an internal OS error that is displayed when the hardware

configuration of a lan adapter has conflicts and does not match

completely a previously configured adapter, or an invalid hardware

parameter was specified.

9.      Hardware Error: Node entered matches a multicast address.

A node address was given on the command line or the board is

configured for a node address that is reserved for multicast

address. This can be fixed by zeroing the Least Significant

Bit of the Most Significant Byte of the node address.

10.     Hardware Error: Unable to initialize ethernet card.

The driver is unable to read the base port register of the NIC.

11.     Hardware Error: Ethernet buffer memory failure.

A write/read of the the last word of memory on the Lan Adapter

failed. This usually indicates a problem with the size of memory

that the adapter is configured for, or a conflict with other

memory in the system.

12.     Error: Unable to register MLID with the LSL.

This is an Internal OS error message that is displayed when

registering a new board with the OS. This message usually

indicates that the OS is out of memory.

13.     Possible cable disconnect detected on LAN adapter.

This message is displayed whenever the driver is unable to transmit

a packet after 10 Transmit Timeouts have occured.

Novell NetWare 386 3.1 Server driver Warning Messages

1.      Unneeded Memory Address ignored

The user specified a memory address parameter when

loading the driver that was un-needed. A sofware specified

memory address will be used instead.

2.      Unneeded Memory Length ignored

The user specified a memory length parameter when loading

the driver that was un-needed. A sofware specified memory

length will be used instead.

3.      Unneeded Interrupt number ignored

The user specified a interrupt number parameter when

loading the driver that was un-needed. A sofware specified

interrupt number will be used instead.

EtherCard PLUS, TokenCard, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks of Standard Microsystems

Corporation. Other trademarks mentioned herein belong to their respective

companies.