STANDARD MICROSYSTEMS CORPORATION

FTP Packet Driver

For EtherCard PLUS Family LAN Adapters

RELEASE INFORMATION

Version 2.03            14 August 1991

VERSION HISTORY

-- Version 2.03 (Current Release) --

Fixed problem where board would fail memory test during initialization

on 16-bit LAN Adapter boards, when installed in a system with a 16-bit

VGA board that was configured in forced 16-bit mode (ie. not autosense).

Added support for 8K, 32K, and 64K ram sizes on

1. SMC8013EP/A

2. SMC8013WP/A

LAN adapter cards (16K already supported).

-- Version 2.02 --

This version includes support for the following SMC Ethernet cards.

1. SMC8013EP/A

2. SMC8013WP/A

This version fixes the following bugs found in version 2.01.

1. Interrupts 10, 11 and 15 now work with the WD8013EB and WD8013EBT.

2. Loading, unloading, and then reloading the Packet Driver now works.

-- Version 2.01 --

This version contains a modification which will allow Novell's

NetBIOS to be loaded after IPX.COM and before the packet driver

(8003PKDR). Previously 8003PKDR would function only if loaded

prior to Novell's NetBIOS.

This version contains modifications for the WD83C690 Network

Interface Controller. This version or later versions must be

used with adapters which contain the WD83C690.

NOTE

Use of adapters which contain the WD83C690 with previous

versions of this driver may lead to data integrity problems.

-- Version 2.00 --

This release of the Packet Driver conforms to the FTP

Software's Packet Driver Specification v1.09. It can be

loaded as a "Stand-Alone" driver as well as be loaded on

top of the NetWare Shell (IPX.COM) driver sharing the LAN

adapter with the NetWare Shell. This driver has been tested

with FTP's PC/TCP, Wollongong's WIN/TCP, and NCSA's TELNET

software packages. The INSTALL.DOC contains information

on installing with each these software packages.

-- Version 1.00 --

This is the first official release of this PC/TCP Packet

Driver v1.00. It conforms to the FTP Software Packet

Driver Specification v1.08, and implements the "basic"

functions.  The driver runs on top of the Netware Shell

driver (IPX.COM), and the two drivers share one adapter to

transfer both TCP/IP packets and Netware packets.  In

order to use this driver, you must have the Netware IPX.COM

generated with driver (.OBJ & .LAN) release v3.07 files or

later. This driver can not be used with the StarLAN driver

using the Novell/AT&T StarLAN packet format.

EtherCard PLUS, TokenCard, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks of Standard Microsystems

Corporation. Other trademarks mentioned herein belong to their respective

companies.