WESTERN DIGITAL

"PLUS"-FAMILY LAN ADAPTER BOARDS

JUMPER SETTINGS

5 April 1989

Each Western Digital LAN adapter board provides several

parameters that are configured by setting appropriate jumpers

on the board.  The following tables summarize the jumper

configurations for certain adapter boards.

For more detailed information on any board, or for information

on jumpers for an adapter board that is not listed here, refer

to the Western Digital "User Installation Guide" that comes

with the adapter board.

NOTES REGARDING XT/AT BUS BOARDS:

o Each "PLUS"-family LAN adapter board contains a RAM buffer

of either 8 or 32 Kbytes that occupies PC memory space.

The base address for this RAM memory is set by the software

you use, not by jumpers on the board.

o Current network software distributed by Western Digital

uses a default RAM base address of D0000 (hex); some earlier

software used a C4000 default base address.

o EGA and VGA adapters also have memory that occupies this

area and could conflict.  D0000 is often a good RAM base

address for users of EGA adapters; D4000 is often good for

users of VGA adapters.

o Other add-on boards may also introduce conflicts with this

memory space.  Other conflicts can occur with the I/O base

address and interrupt request level, which are controlled

by jumpers as described below.

For a Micro Channel bus board, the RAM base address, I/O base

address, boot ROM base address, and interrupt level are set by

the system configuration procedure when the board is installed.

Every software driver obtains this information automatically

and adjusts its operation accordingly.

"*" denotes the settings of jumpers installed at the factory.

== EtherCard PLUS (WD8003E) ==

And

== EtherCard PLUS With Boot ROM Socket (WD8003EBT) ==

JUMPER  PIN              FUNCTION SELECTED

BLOCK   NUMBERS   NAME   WHEN JUMPER IS INSTALLED

------  --------  ----   ----------------------------

W1       1-2      Zero   Select zero wait states for optimized

Wait   operation in a 6MHz AT.  (Factory

setting is jumper removed, which

ensures correct operation in faster

AT bus systems.)  This jumper is

ignored in XT bus systems.

I/O Base Address (W1/3-4, 5-6, 7-8, 9-10):

W1       3-4*, 5-6*,     280 -- the factory setting

9-10*

5-6, 9-10       2A0

3-4, 9-10       2C0

9-10            2E0

3-4, 5-6, 7-8   300

5-6, 7-8        320

3-4, 7-8        340

W2       1-2      IRQ7   Interrupt request level.

3-4      IRQ6   Only one jumper may be installed at a

5-6      IRQ5   time.

7-8      IRQ4

9-10*    IRQ3   (Factory setting is IRQ3)

11-12    IRQ2

W3       1-2*            Ethernet type: pins all jumpered to use

3-4*            J1 BNC connector for thin Ethernet cable

5-6*            (*-the factory setting).  Jumpers all

7-8*            removed to use J2 AUI connector (thick

9-10*           Ethernet cable).

11-12*

W4       1-2     Version Ethernet version: jumper removed for thin

Ethernet and IEEE 802.3/Version 2 thick

Ethernet (the factory setting); jumper

installed for thick Ethernet Version 1.

W5       1-2*    Segment Thin Ethernet segment length:  pins

Length  jumpered for IEEE 802.3 standard 185m

length (*-the factory setting); jumper

removed for 300m extended length.

== Additional Jumpers On ==

== EtherCard PLUS With ROM Socket Only (WD8003EBT) ==

JUMPER  PIN         FUNCTION SELECTED WHEN

BLOCK   NUMBERS     INDICATED JUMPERS ARE INSTALLED

------  --------    --------------------------------------------

ROM Base Address (W6/1-2, 3-4, 5-6):

W6      Open          C0000 (16K, 32K, 64K ROMs)

1-2           C4000 (16K ROMs)

3-4           C8000 (16K, 32K ROMs)

1-2, 3-4      CC000 (16K ROMs)

5-6           D0000 (16K, 32K, 64K ROMs)

1-2, 5-6      D4000 (16K ROMs)

3-4*, 5-6*    D8000 (16K, 32K ROMs) -- factory setting

1-2, 3-4, 5-6 DC000 (16K ROMs)

(NOTE:  Do not use addresses C0000

or C4000 for Boot ROMs)

ROM Size (W6/7-8, 9-10) (W9/1-2, 3-4) (W10/1-2, 3-4):

W6      7-8*, 9-10* ROM Disabled -- the factory setting

W9      3-4*          "

W10     3-4*          "

W6      Open        16 Kbytes ROM

W9      3-4           "

W10     3-4           "

W6      7-8         32 Kbytes ROM

W9      3-4           "

W10     1-2           "

W6      9-10        64 Kbytes ROM

W9      1-2           "

W10     1-2           "

RAM Buffer Size (W7/1-2, 3-4) (W8/1-2, 3-4) (W11/1-2-3):

W7      3-4*        8 Kbytes RAM -- the factory setting

W8      3-4*         "

W11     2-3*         "

(NOTE:  valid 8 Kbyte RAM base addresses

are C0000, C2000, C8000, CA000, D0000,

D2000, D8000, and DA000.)

W7      1-2         32 Kbytes RAM

W8      1-2           "

W11     1-2           "

(NOTE:  valid 32 Kbyte RAM base addresses

are C0000, C8000, D0000 and D8000.)

== WD8003EB Ethernet Adapter ==

JUMPER

BLOCK   NAME                  FUNCTION

------ ------ -----------------------------------------------

W1     INIT   If jumper is installed when power is turned on,

280    I/O Base Address is temporarily set to 280.

Setup Program must then be run to store new I/O

Base Address, and jumper must be removed (*).

W3     AUI-   Ethernet type:  use AUI position for thick

BNC    Ethernet (external transceiver); use BNC (*)

position for thin Ethernet (BNC connector).

W9     ROM    Size of optional ROM installed (2 jumpers).

SIZE                 LEFT JUMPER     RIGHT JUMPER

POSITION         POSITION

16 Kbytes(*)      Top              Top

32 Kbytes       Bottom             Top

64 Kbytes       Bottom           Bottom

Jumpers are ignored if no ROM is installed.

== EtherCard PLUS For Unshielded Twisted Pair Wiring (WD8003WT) ==

JUMPER  PIN       FUNCTION SELECTED

BLOCK   NUMBERS   WHEN JUMPERS ARE INSTALLED

------  --------  --------------------------------------------

W1       1-2      Zero   Select zero wait states for optimized

Wait   operation in a 6MHz AT.  (Factory

setting is jumper removed, which

ensures correct operation in faster

AT bus systems.)  This jumper is

ignored in XT bus systems.

I/O Base Address (W1/3-4, 5-6, 7-8, 9-10):

W1       3-4*, 5-6*,     280 -- the factory setting

9-10*

5-6, 9-10       2A0

3-4, 9-10       2C0

9-10            2E0

3-4, 5-6, 7-8   300

5-6, 7-8        320

3-4, 7-8        340

W2       1-2      IRQ7   Interrupt request level.

3-4      IRQ6   Only one jumper may be installed at a

5-6      IRQ5   time.

7-8      IRQ4

9-10*    IRQ3   (Factory setting is IRQ3)

11-12    IRQ2

W3       1-2*            Ethernet type: pins all jumpered to use

3-4*            RJ-45 twisted pair wiring connector

5-6*            (*-the factory setting).  Jumpers all

7-8*            removed to use AUI connector.

9-10*

11-12*

ROM Base Address (W6/1-2, 3-4, 5-6):

W6      Open          C0000 (16K, 32K, 64K ROMs)

1-2           C4000 (16K ROMs)

3-4           C8000 (16K, 32K ROMs)

1-2, 3-4      CC000 (16K ROMs)

5-6           D0000 (16K, 32K, 64K ROMs)

1-2, 5-6      D4000 (16K ROMs)

3-4*, 5-6*    D8000 (16K, 32K ROMs) -- factory setting

1-2, 3-4, 5-6 DC000 (16K ROMs)

(NOTE:  Do not use addresses C0000

or C4000 for Boot ROMs)

ROM Size (W6/7-8, 9-10) (W9/1-2-3) (W10/1-2-3):

W6      7-8*, 9-10* ROM Disabled -- the factory setting

W9      2-3*          "

W10     2-3*          "

W6      Open        16 Kbytes ROM

W9      2-3           "

W10     2-3           "

W6      7-8         32 Kbytes ROM

W9      2-3           "

W10     1-2           "

W6      9-10        64 Kbytes ROM

W9      1-2           "

W10     1-2           "

RAM Buffer Size (W7/1-2-3) (W8/1-2-3) (W11/1-2-3):

W7      2-3*        8 Kbytes RAM -- the factory setting

W8      2-3*         "

W11     2-3*         "

(NOTE:  valid 8 Kbyte RAM base addresses

are C0000, C2000, C8000, CA000, D0000,

D2000, D8000, and DA000.)

W7      1-2         32 Kbytes RAM

W8      1-2           "

W11     1-2           "

(NOTE:  valid 32 Kbyte RAM base addresses

are C0000, C8000, D0000 and D8000.)

== EtherCard PLUS/A For Micro Channel (WD8003ET/A) ==

JUMPER  PIN         FUNCTION SELECTED WHEN

BLOCK   NUMBERS     INDICATED JUMPERS ARE INSTALLED

------  --------    --------------------------------------------

W3      1-2*        Ethernet type: pins all jumpered to use

3-4*        J1 BNC connector for thin Ethernet cable

5-6*        (*-the factory setting).  Jumpers all

7-8*        removed to use J2 AUI connector (thick

9-10*       Ethernet cable).

11-12*

Boot ROM Size (W9-1/2/3) (W10-1/2):

W9      2-3*        16 Kbytes ROM -- the factory setting

W10     Open*         "

W9      2-3         32 Kbytes ROM

W10     1-2           "

W9      1-2         64 Kbytes ROM

W10     1-2           "

== StarCard PLUS/A For Micro Channel (WD8003ST/A) ==

Boot ROM Size (W9-1/2/3) (W10-1/2):

W9      2-3*        16 Kbytes ROM -- the factory setting

W10     Open*         "

W9      2-3         32 Kbytes ROM

W10     1-2           "

W9      1-2         64 Kbytes ROM

W10     1-2           "

== StarCard PLUS (WD8003S) ==

And

== StarLink PLUS (WD8003SH) ==

JUMPER  PIN              FUNCTION SELECTED

BLOCK   NUMBERS   NAME   WHEN JUMPER IS INSTALLED

------  --------  ----   ----------------------------

W2       1-2      Zero   Select zero wait states for optimized

Wait   operation in a 6MHz AT.  (Factory

setting is jumper removed, which

ensures correct operation in faster

AT bus systems.)  This jumper is

ignored in XT bus systems.

I/O Base Address (W2/3-4, 5-6, 7-8, 9-10):

W2       3-4*, 5-6*,     280 -- the factory setting

9-10*

5-6, 9-10       2A0

3-4, 9-10       2C0

9-10            2E0

3-4, 5-6, 7-8   300

5-6, 7-8        320

3-4, 7-8        340

W1       1-2      IRQ7   Interrupt request level.

3-4      IRQ6   Only one jumper may be installed at a

5-6      IRQ5   time.

7-8      IRQ4

9-10*    IRQ3   (*-Factory setting is IRQ3)

11-12    IRQ2

W3       1-2*            StarCard PLUS (WD8003S):

Jumper must be installed

1-2             StarLink PLUS (WD8003SH):

Jumper must be removed

(for compatibility with 802.3 1BASE5

StarLAN standard.)

== Additional Jumpers On ==

== StarLink PLUS Only (WD8003SH) ==

JUMPER  PIN         FUNCTION SELECTED WHEN

BLOCK   NUMBERS     INDICATED JUMPERS ARE INSTALLED

------  --------    --------------------------------------------

W4      11-12       Boot ROM:  enabled when jumper installed;

disabled (the factory setting) when jumper

removed

Boot ROM Base Address (W4/3-4, 5-6, 7-8, 9-10):

W4      3-4*, 7-8*  C8000 (16K, 32K ROMs; factory setting)

9-10*

7-8, 9-10   CC000 (16K ROMs)

3-4, 5-6,   D0000 (16K, 32K, 64K ROMs)

9-10

5-6, 9-10   D4000 (16K ROMs)

3-4, 9-10   D8000 (16K, 32K ROMs)

Boot ROM Size (W5/1-3-5, 2-4-6) (W6/1-3-5, 2-4-6):

W5      1-3*, 2-4*  16 Kbytes ROM -- the factory setting

W6      1-3*, 2-4*    "

W5      1-3, 4-6    32 Kbytes ROM

W6      1-3, 4-6      "

W5      3-5, 4-6    64 Kbytes ROM

W6      3-5, 4-6      "

##############################################################################################################################