WESTERN DIGITAL

"PLUS"-FAMILY LAN ADAPTER BOARD DRIVERS

FOR USE WITH MICROSOFT OS/2 LAN MANAGER

INSTALLATION GUIDE

May 4, 1989

These drivers are designed to be used with 3Com 3+Open, the

Microsoft OS/2 LAN Manager OEM distribution release 1.0 or later,

and other compatible OEM versions of LAN Manager.  The drivers

named MACWD conform to the Network Driver Interface Specification

(NDIS) defined by Microsoft/3Com.

This directory provides drivers for the "PLUS" (WD8003) family of

Western Digital LAN adapter boards:

DRIVER             WESTERN DIGITAL ADAPTER BOARD TYPES

MACWD.XXX          EtherCard PLUS  (WD8003E, WD8003EBT,

WD8003EB, WD8003WT)

EtherCard PLUS for Micro Channel

(WD8003ET/A, WD8003E/A)

StarCard PLUS (WD8003S)

StarLink PLUS (WD8003SH)

StarCard PLUS for Micro Channel

(WD8003ST/A)

Included in this directory are the following files:

INSTALL.DOC              This File.

RELEASE.DOC              Version history and release info

MACWD.OS2                The OS/2 NDIS MAC driver

MACWD.DOS                The DOS NDIS MAC driver

PROTOCOL.INI             A sample PROTOCOL.INI file

BEGINNING INSTALLATION

1) Install the adapter card.  Proper installation can be verified

by running the diagnostic program under DOS.  Note that the

diagnostic will not run properly from the OS/2 compatibility box.

2) Decide what protocol stack will be used.  The supplied drivers

support the DLC/NETBEUI protocol stack supplied by Microsoft,

3Com and most other LAN Manager vendors.  They also support the

XNS stack supplied by 3Com.  Support for other protocol stacks is

in development. Contact WD Communication Technical Support for

the latest information.

3) Install but do not start LAN Manager using the vendor's

instructions, selecting one of the adapter/drivers supplied with

the vendor's package.  The adapter selected does not matter

because the references to it will be replaced once the

installation is complete.  The examples below assume the 3Com

EtherLink II has been selected with 3+Open.

INSTALLING ON OS/2

1) Copy the OS/2 driver from this diskette to the driver

directory used by LAN Manager.  For example, when installing a

3+Open server using the normal directory structure type:

COPY A:\LANMAN\MACWD.OS2 C:\3OPEN\SERVER\LANMAN\DRIVERS

When installing a 3+Open OS/2 workstation type:

COPY A:\LANMAN\MACWD.OS2 C:\3OPEN\OS2WKSTA\LANMAN\DRIVERS

2) Modify the config.sys or config.os2 file to reference this

adapter rather than the adapter selected during LAN Manager

installation.  For example, if the 3Com EtherLink II was selected

during a server installation, use a text editor to change the

line:

DEVICE=\3OPEN\SERVER\LANMAN\DRIVERS\ELNNKII.OS2

to the line:

DEVICE=\3OPEN\SERVER\LANMAN\DRIVERS\MACWD.OS2

If the 3Com EtherLink II was selected during a workstation

installation, use a text editor to change the line:

DEVICE=\3OPEN\OS2WKSTA\LANMAN\DRIVERS\ELNNKII.OS2

to the line:

DEVICE=\3OPEN\OS2WKSTA\LANMAN\DRIVERS\MACWD.OS2

Note that other varieties of LAN Manager will keep their drivers

in different directories.  The point is to get the CONFIG.SYS to

reference the Western Digital driver in place of the adapter

driver selected during installation.  Note also that the LAN

Manager installation will add several drivers to the CONFIG.SYS

file.  Only the MAC driver referring to a particular board should

be replaced. The other drivers such as PROTMAN.OS2 should be left

unmodifed.

3) Modify the PROTOCOL.INI file to include an entry for this

driver.  The PROTOCOL.INI file contains sections defining

parameters for all the protcol drivers used by LAN Manager.  To

add a section for this driver, use a text editor to add the

following lines to the file:

[WD8003XMAC]

drivername = MACWD$

irq = 3

ramaddress = 0xD000

iobase = 0x280

The line containing "[WD8003XMAC]" defines the name for the

driver and will be used in the bindings command described below.

The drivername "MACWD$" is used by the protocol manager.  The

other lines:

irq =

ramaddress =

iobase =

define parameters for the driver and should be modified

appropriately depending on how the adapter and PC are configured.

The values shown correspond to the current factory defaults for

the adapters supported.  A more complete explanation for each of

these parameters and its appropriate values is given at the end

of this document.

Note that on Micro Channel adapters, these values are read from

the POS registers on the adapter, defined during installation.

The values given on these lines are ignored and do not have to

be added to the PROTOCOL.INI file.

4) Modify the PROTOCOL.INI file to change the bindings of the

protocol stack to reference this driver in place of the driver

selected during installation.  Use a text editor to modify the

"bindings =" line to reference the "WD8003XMAC" name defined in

the section added in step 3 rather than the name set during

installation.

For example: under 3+Open with the XNS protocol stack and 3Com

EtherLink II selected during installation, in the PROTOCOL.INI

file section labeled "[XNS_NETBIOS]", the line:

BINDINGS = ETHERLINKII

must be changed to:

BINDINGS = WD8003XMAC

If the DLC/NETBEUI protocol stack was selected to be used with

3+Open, the same lines must be changed in the PROTOCOL.INI file

section labeled "[DLC]".

5)  The installation is complete.  Reboot and bring up LAN

Manager using the instructions supplied by the LAN Manager

vendor.

INSTALLING ON DOS WITH THE DLC/NETBEUI PROTOCOL STACK

The procedure for installing on DOS with the DLC/NETBEUI protocol

stack is very similar to the procedure used on OS/2.  Only the

directory names and file extensions will be changed.

1) Copy the DOS macwd.dos driver from this diskette to the driver

directory used by LAN Manager.  For example, when installing a

3+Open DOS workstation using the normal directory structure type:

COPY A:\LANMAN\MACWD.DOS C:\3OPEN\DOSWKSTA\LANMAN\DRIVERS

2) Modify the config.sys or CONFIG.DOS file to reference this

adapter rather than the adapter selected during LAN Manager

installation.  For example, if the 3Com EtherLink II was selected

during a workstation installation, use a text editor to change

the line:

DEVICE=\3OPEN\DOSWKSTA\LANMAN\DRIVERS\ELNNKII.DOS

to the line:

DEVICE=\3OPEN\DOSWKSTA\LANMAN\DRIVERS\MACWD.DOS

Note that other varieties of LAN Manager will keep their drivers

in different directories.  The point is to get the CONFIG.SYS to

reference this driver in place of the adapter driver selected

during installation.  Note also that the LAN Manager installation

will add several drivers to the CONFIG.SYS file.  Only the MAC

driver referring to a particular board should be replaced. The

other drivers such as PROTMAN.DOS should be left unmodifed.

3) Modify the PROTOCOL.INI file to include an entry for this

driver.  The PROTOCOL.INI file contains sections defining

parameters for all the protcol drivers used by LAN Manager.  To

add a section for this driver, use a text editor to add the

following lines to the file:

[WD8003XMAC]

drivername = MACWD$

irq = 3

ramaddress = 0xD000

iobase = 0x280

The line containing "[WD8003XMAC]" defines the name for the

driver and will be used in the bindings command described below.

The drivername "MACWD$" is used by the protocol manager.  The

other lines:

irq =

ramaddress =

iobase =

define parameters for the driver and should be modified

appropriately depending on how the adapter and PC are configured.

The values shown correspond to the current factory defaults for

the supported adaptpers.  A more complete explanation for each of

these parameters and its appropriate values is given at the end

of this document.

Note that on Micro Channel adapters, these values are read from

the POS registers on the adapter, defined during installation.

The values given on these lines are ignored and do not have to

be added to the PROTOCOL.INI file.

4) Modify the PROTOCOL.INI file to change the bindings of the

protocol stack to reference this driver in place of the driver

selected during installation.  Use a text editor to modify the

"bindings =" line to reference the "WD8003XMAC" name defined in

the section added in step 3 rather than the name set during

installation.

For example: under 3+Open with the DLC/NETBEUI protocol stack and

3Com EtherLink II selected during installation, in the

PROTOCOL.INI file section labeled "[DLC]", the line:

BINDINGS = ETHERLINKII

must be changed to:

BINDINGS = WD8003XMAC

5)  The installation is complete.  Reboot and bring up LAN

Manager using the instructions supplied by the LAN Manager

vendor.

INSTALLING ON DOS WITH THE XNS PROTOCOL STACK

This procedure is somewhat different from the ones above because

the protocol manager and PROTOCOL.INI file are not used.  The

adapter driver used is the 3Com 3+ driver from the Western

Digital SuperDisk.  Additional information about this driver can

be found in the documenation supplied with the driver on the

SuperDisk diskette, which is supplied with retail versions of the

adapters.

Begin the installation the same way by installing LAN Manager on

the workstation and selecting one of the adapters supplied by

3Com such as EtherLink II.  Insert the SuperDisk diskette into

the A drive, then follow the instructions below:

1) Copy the wd.sys driver from the SuperDisk to the driver

directory used by LAN Manager.  For example, when installing a

3+Open DOS workstation using the normal directory structure type:

COPY A:\3PLUS\WD.SYS C:\3OPEN\DOSWKSTA\LANMAN\DRIVERS

2) Modify the config.sys or CONFIG.DOS file to reference this

adapter rather than the adapter selected during LAN Manager

installation.  For example, if the 3Com EtherLink II was selected

during a workstation installation, use a text editor to change

the line:

DEVICE=\3OPEN\DOSWKSTA\LANMAN\DRIVERS\ETH503.SYS 3 300 1

to the line:

DEVICE=\3OPEN\DOSWKSTA\LANMAN\DRIVERS\WD.SYS 3 280 D000

The final parameters on this line "3 280 D000" must be modified

if the adapter is not installed in its factory configuration of:

IRQ = 3

I/O Base Address = 0x280

RAM Address = 0xD0000

For example, if the adapter was installed with IRQ = 2, I/O

Address = 0x300 and RAM Address = 0xC8000, then the line in

CONFIG.SYS must be modified:

DEVICE=\3OPEN\DOSWKSTA\LANMAN\DRIVERS\WD.SYS 2 300 C800

3) The installation is complete.  Reboot and bring up LAN Manager

using the instructions supplied by the LAN Manager vendor.

PROTOCOL.INI PARAMETERS

Several parameters are typically specified in the MACWD section

of the PROTOCOL.INI file.  The only parameter that must be

specified is the "drivername".  All the others have default

values supplied by the driver and can be left out of the

PROTOCOL.INI file if the installed adapter matches the default

configuration.  Micro Channel adapters will ignore the values of

IRQ, IOBASE and RAMADDRESS and instead use the values in the POS

registers set up when the card was installed.

irq         Specifies the interrupt used by the adapter.  The

factory default is 3.  The interrupt is selected

with a jumper on the adapter or on some boards via

the SETUP program supplied with the adapter.  Consult

the installation guide for information on how to

determine or modify the interrupt. This parameter

will always be a decimal number in the range of

2 - 15.  Not all values are supported by all

adapters.

iobase      Specifies the base I/O address used by the adapter.

The factory default is 0x280.  This address is

selected with a jumper on the adapter or on some

boards via the SETUP program supplied with the

adapter.  Consult the installation guide for

information on how to determine or modify the address.

This parameter will always be a hex number in the

range of 0x200 - 0x3E0.  Note that hex numbers must be

preceded by "0x" to distinguish from decimal numbers.

Not all values are supported by all adapters.

ramaddress  Specifies the Shared RAM segment address used by the

adapter.  This value is established under software

control by the driver.  If this line is not present

in the PROTOCOL.INI file, the driver will use the

value 0xD000 which corresponds to address 0xD0000.

This address is recommended unless some other device

in the PC conflicts.  Consult the adapter

installation guide or print the file SYSMAP.DOC on

the SuperDisk for more information on how to select a

shared RAM address.  The driver will accept hex

values in the range of 0xA000 - 0xEC00.

Trademarks:

Microsoft OS/2 LAN Manager is a trademark of Microsoft

Corporation.  3Com, 3+, 3+Open and EtherLink II are registered

trademarks of 3Com Corporation.  EtherCard PLUS, StarCard PLUS,

StarLink PLUS and SuperDisk are trademarks of Western Digital.