WESTERN DIGITAL

NETBIOS/OSI INTERFACE PROGRAM

FOR

"PLUS"-FAMILY StarLAN/ETHERNET PC ADAPTER BOARDS

INSTALLATION GUIDE

ADAPTER BOARDS

This program is for use with Western Digital "PLUS" (WD8003)

family LAN adapter boards as described in the RELEASE.DOC

file.

LOADING THE PROGRAM

After booting DOS, type the following when the DOS prompt

is displayed:

WPLUSNBE   ... parameter list ...

"... parameter list ..." is an optional list of parameters

in the form "/parameter_name:value".  These are described

in detail in the following section.  The parameters which

may be selected include:

o    length of the user frame to be transmitted by the

transport layer

o    LAN address for this node

o    base I/O address of the LAN adapter board (*)

o    base address for the on-board shared RAM memory (*)

o    PC IRQ level to receive LAN adapter board interrupts (*)

o    Storage allocated for various internal data structures

o    Select use as a second adapter configuration

* These parameters are not used with Micro Channel

bus adapter boards because their values are

established during system configuration when

you install the adapter board.

Any one or more of these parameters may be selected in

any order.  Defaults are assigned to each parameter so

that none of them need be specified if the standard

configuration is desired.

To remove the NETBIOS/OSI Interface Program or change any

of its parameters after it has been loaded, DOS must be

rebooted.

PARAMETER SPECIFICATION

The format for specifying parameters for WPLUSNBE.EXE is:

WPLUSNBE /parameter_name:value /parameter_name:value

where "/" is the DOS switch character, "parameter_name" is the

parameter name described below, and "value" is an acceptable

value for the parameter as described below.  The command line

may specify none or a combination of parameters.  Each

parameter must be preceded by the switch character "/".  If

the parameter can be assigned a value (all except for "v" and

"?"), the colon (:) delimiter must follow the parameter_name.

The following parameters may be selected as command line

options:

/base:n   Specifies the base I/O address (n) of the LAN

adapter board.  (Not used for Micro Channel

bus boards.)

"n" must be a three digit hexadecimal value in the

range 200 through 3E0.  Acceptable values are

200, 220, 240, 260, 280, 2A0, 2C0, 2E0, 300, 320,

340, 360, 380, 3A0, 3C0 and 3E0.

Default: /base:280

/frmsiz:n Specifies the maximum Transport Protocol Data Unit

size (n).  This is approximately the amount of

application data that is sent out per message frame.

"n" is a decimal value.  Acceptable values are

128, 256 and 512.

Default: /frmsize:512

/irq:n    Specifies the interrupt level (IRQ) that the

LAN adapter board will use to notify the PC.

(Not used for Micro Channel bus boards.)

Possible values are 2, 3, 4, 5, 6 and 7.

Default:  /irq:3

/page:ppp Page number to be used for the base address of

the on-board memory that is shared with the PC.

The range of possible base addresses is 80000

to FE000 in steps of 2000.  This corresponds

to 512K to 1M decimal in steps of 8K.

(Not used for Micro Channel bus boards.)

The page number "ppp" must be specified as the

first three hexadecimal digits of the base

address.  Acceptable values for "ppp" are

therefore 800 to FE0 in steps of 20 (hex).

Certain base addresses are frequently used for

other functions in the PC and are therefore

not suitable:

C0000, C8000, CA000 and E0000-FE000 are

typically used for BIOS ROMs

A0000-BE000 are used for graphics RAM.

Default:  /page:C40

/node:a   Specifies the IEEE 802 standard LAN address to

be used for this network node if there is a

need to override the hardware address of the

LAN adapter board.

"a" must be a six byte (12 digit) hexadecimal

value entered in ascending byte order.  That is,

the least significant byte of the address must

be the first byte of the parameter value.

Default:  the node address contained in the LAN

Address ROM on the LAN adapter board.

/cmd:n    Specifies the number of NCB commands supported.

Each NCB command consumes 18 bytes.

Default: /cmd:32   Max: /cmd:32

/ses:n    Specifies the number of sessions supported.

Each session consumes 249 bytes.

Default: /ses:32   Max: /ses:32

/nlb:n    Specifies the number of large buffers supported.

Each large buffer consumes 607 bytes.

Default: /nlb:10   Max: /nlb:40

/nsb:n    Specifies the number of small buffers supported.

Each small buffer consumes 147 bytes.

Default: /nsb:30   Max: /nsb:60

/wz:n     Specifies the size of the transport window.  This

is the number of packets the protocol will transmit

before waiting for an acknowledgement.  Making this

number larger improves efficiency but uses more

buffers.

Default: /wz:4   Max: /wz:4

/2nd      Tells the driver to operate as the 2nd LAN adapter.

This permits it to be used in the same machine as

an IBM Token Ring adapter.

/v        Displays program version and banner information

without permanently loading and running the

program.

The version and banner information are

automatically displayed whenever the program is

loaded normally.  The "v" parameter does not

take a value.

/?        Displays HELP information (the list of valid

parameters) without permanently loading and

running the program.

The "?" parameter does not take a value.

The "v" and "?" parameters can also be invoked when the

NETBIOS/OSI Interface Program is already loaded and running.

####################################################################################