WESTERN DIGITAL

NETBIOS/OSI INTERFACE PROGRAM

FOR

"PLUS"-FAMILY LAN ADAPTER BOARDS

RELEASE INFORMATION

Version 3.1.7                21 November 1988

The purpose of this document (file RELEASE.DOC) is to briefly

explain the contents of this diskette.  The file WPLUSNBE.EXE

is the NETBIOS/OSI Interface Program, intended for use with the

following Western Digital LAN adapter boards:

WD8003E                 (EtherCard PLUS)

WD8003EBT, WD8003EB     (EtherCard PLUS with Boot

ROM Option)

WD8003ET/A, WD8003E/A   (EtherCard PLUS/A For Micro

Channel Bus)

WD8003WT                (EtherCard PLUS/TP For Twisted

Pair Cable)

WD8003S                 (StarCard PLUS)

WD8003ST/A              (StarCard PLUS/A For Micro

Channel Bus)

WD8003SH                (StarLink PLUS)

Refer to the WD8003-NB NETBIOS/OSI Interface Program Installation

Guide (or file INSTALL.DOC) for an explanation of the command line

parameters that can be used with this program.

This version of the WD8003-NB NETBIOS/OSI Interface Program is

intended for use with:

o IBM PC Local Area Network Program

o Microsoft MS-NET

or other NETBIOS compatible software.

OPERATIONAL CONSIDERATIONS

Please consider the following when using the NETBIOS/OSI

Interface Program:

o The ViaNet 3.06 Network Operating System will not function

with the NETBIOS/OSI Interface Program loaded.

o The specified frame size includes OSI protocol headers and thus

the bytes available for actual data transfer in a datagram are

somewhat less than the frame size.

o This version includes parameterized storage allocation.  When

used in a network operating system server configuration,

especially with the IBM PC LAN Program, the default number of

buffers should be increased for optimal performance.

(Use /nlb:20, /nsb:60)

MODIFICATION HISTORY

<< Version 3.1.7 -- Current Version: November 1988 >>

There is no functional difference in this version.  The

organization of the code was changed for compatibility with

a ROM version.

<< Version 3.1.6 -- September 1988 >>

This version provides command line parameters for better

control of the storage allocated by the program.  It also

saves memory by deallocating initialization code.  Other

features include the ability to use the full 32K RAM available

on WD8003EBT and WD8003WT boards, and support for use as a

second adapter.

<< Version 3.1 -- 26 May 1988 >>

This version handles Western Digital adapter boards for

Micro Channel bus systems as well as the original XT/AT bus

adapters.  The Program determines on its own which type of

board is installed and adjusts its operation accordingly.

Certain command line parameters are no longer used with

Micro Channel bus adapters (see INSTALL.DOC instructions).

<< Version 3.0 -- 1 March 1988>>

This and future versions of the NETBIOS/OSI Interface

Program use an improved communication protocol that makes

them incompatible with previous versions (2.1 and earlier)

of the Program.  Version 3.0 or subsequent versions must

now be used with every adapter board in the network.

Only 8K of the 32K RAM buffer on WD8003EBT adapter boards

is supported by this version.

#############################################################################################################################