WESTERN DIGITAL

"PLUS"-FAMILY LAN ADAPTER BOARD DRIVERS

FOR USE WITH NOVELL ADVANCED NETWARE v2.0a

INSTALLATION GUIDE

This directory provides a single driver for the entire "PLUS"

(WD8003) family of Western Digital LAN adapter boards for XT/AT

and Micro Channel bus systems:

StarLAN:   StarCard PLUS (WD8003S)

StarLink PLUS (WD8003SH)

StarCard PLUS/A (WD8003ST/A)

Ethernet:  EtherCard PLUS (WD8003E, WD8003EBT, WD8003EB)

EtherCard PLUS/A (WD8003ET/A, WD8003E/A)

Ethernet Twisted Pair:

EtherCard PLUS/TP (WD8003WT)

Included on this directory are the following files:

INSTALL.DOC           ; This document

RELEASE.DOC           ; Notes about this release

INSTALL.BAT           ; Installation batch file

ASK.COM               ; Program used by INSTALL.BAT

ACOMDRV.OBJ           ; \

BCOMDRV.OBJ           ;  \

CCOMDRV.OBJ           ;   > WD8003 Server drivers

DCOMDRV.OBJ           ;  /

SHELLDRV.OBJ          ; WD8003 Shell driver

The INSTALL batch file automates the installation of the

Western Digital LAN drivers on the Novell GENOS-1 and GENSH-1

diskettes.  Once the drivers are installed, the GENOS and

GENSH procedures can be run exactly as described in the

Advanced NetWare v2.0a Installation Manuals.

The actions performed by the INSTALL batch file are described

in detail in Appendix B of the NetWare 286 Installation-Part 1

Manual and Chapter 2 of the NetWare 86 Installation Manual.

The GENOS-1 and GENSH diskettes each contain a number of

subdirectories.  Each subdirectory contains the driver for a

specific LAN adapter board.  The INSTALL batch file will

add a subdirectory and the appropriate drivers to both Novell

diskettes.  This procedure is described in Step 2 below.

The GENOS-1 diskette supplied by Novell does not have sufficient

free space to install any more drivers.  Prior to running the

INSTALL batch file, you must therefore delete from GENOS-1

the drivers and subdirectory for one or more LAN adapter types

that are not needed, taking care not to erase the NULL

subdirectory.  This procedure is described in Step 1 below.

Be sure that you make working copies of the NetWare diskettes

as instructed by Chapter 1 of the NetWare Installation

Manuals.  This will preserve your original NetWare diskettes.

STEP 1 -- MAKING ROOM ON THE GENOS-1 DISKETTE

Performing a DIR of the GENOS-1 diskette will reveal that it

contains a number of subdirectories, each containing the

drivers for a different LAN adapter.  The number of these

subdirectories will vary with different versions of NetWare.

There is no room on the diskette to add any more drivers.

To add the Western Digital "PLUS"-family LAN drivers,

select one subdirectory on the GENOS-1 diskette corresponding

to a LAN adapter that you never plan to use.  Delete the driver

files in the selected subdirectory first, and then remove the

subdirectory itself.  The subdirectory names correspond to

LAN adapter boards as follows:

ASYNC      IBM ASYNC Remote Dial-In Line

CLUSTER    IBM PC Cluster

CPRONET    Proteon ProNET With Checksum

ETHERNET   3COM EtherLink

GENBIOS    Generic NETBIOS (Interrupt 5Ch)

GNET       Gateway G/NET

ISTAR      Novell Star Intelligent NIC

NESTAR     Nestar PLAN 2000

NETHER     3COM 3C505 EtherLink Plus

OMNINET    Corvus Omninet

PCN        IBM PC Network

PCNET      Orchid/Santa Clara/AST PCnet

SMC        SMC/Pure Data ARCNET

STARLAN    AT&T StarLAN

TRN        IBM Token Ring

VISTA      Allen-Bradley VistaLAN/PC

For example, to delete the GNET (Gateway G/NET) driver files

and subdirectory, use the following DOS commands:

cd \gnet

erase *.*

cd \

rmdir gnet

IMPORTANT

Do NOT erase the NULL subdirectory from the GENOS-1 diskette.

This subdirectory is used by the GENOS procedure.

STEP 2 -- RUNNING INSTALL

Place this driver diskette in drive A: and type

cd NETWAR20  <Enter>

INSTALL  <Enter>

The batch file will lead you through the steps necessary to

add the "PLUS" (WD8003) family drivers to both the GENOS-1

and GENSH-1 diskettes in a new subdirectory named WD8003.

STEP 3 -- RUNNING GENOS AND GENSH

Now that the Western Digital LAN drivers have been added to

the GENOS-1 and GENSH-1 diskettes, you are ready to run the

GENOS and GENSH utilities as described in the NetWare

Installation Manuals.  At the point where you are directed

to select the LAN driver, the WD8003 drivers will appear in

the list of choices as follows:

DRIVER             MENU CHOICE

WD8003        WD Star/EtherCard PLUS

SERVER DRIVER CONFIGURATION (GENOS)

For XT and AT bus boards, there are various configurations of

the drivers that you can select.  Some are designated for

adapter boards with 8 Kbyte RAM buffers, and some for boards

with 32 Kbyte RAM buffers.

The drivers configured for 32 Kbyte RAM buffers can be used

with any size RAM buffers.  This is becuase they will

automatically revert to using 8 Kbyte RAM if they find that

the adapter board only contains 8 Kbyte RAM or is configured

to only use 8 Kbyte RAM.

For XT/AT bus servers, choose one of the first 16

configuration selections that has an appropriate interrupt,

I/O Base, and RAM address.

For Micro Channel bus servers, a Micro Channel configuration

must be selected; these are all described as "Micro Channel

Adapter Auto Configuration".  The usual configuration

parameters such as interrupt, I/O base address and RAM base

address have already been established by the system hardware

configuration procedure when you installed the adapter board

in your system.

NOTE

If your NetWare server contains more than one Micro Channel

LAN adapter board, be sure that you manually selected a

different Interrupt Request (IRQ) channel for each adapter

before running the Novell GENOS procedure.  To do this,

you must use the manual configuration option of the Set

Configuration program on your PC system Reference Diskette

when you install the boards.

SHELL DRIVER CONFIGURATION (GENSH)

This Western Digital shell driver automatically adapts to

either Micro Channel or XT/AT bus boards based on the type of

adapter board that is actually installed in each workstation.

For Micro Channel bus workstations, the shell driver

automatically uses the interrupt, I/O Base and RAM address

established by the system hardware configuration procedure

when you installed the adapter board in the computer.

For XT/AT bus workstations, the shell driver uses interrupt

IRQ3, I/O base 280 (hex), and RAM buffer address D000.

All RAM buffer sizes up to 32 Kbytes are supported.

If an XT/AT bus workstation requires some other interrupt,

I/O Base or RAM address, you can use the NWPATCH program to

create a different configuration.  This patch utility is

available from Western Digital.

#########################