WESTERN DIGITAL

** SuperDisk **

LOCAL AREA NETWORK SOFTWARE DRIVERS

FOR "PLUS"-FAMILY StarLAN, ETHERNET AND

ETHERNET TWISTED PAIR ADAPTER BOARDS

Version 1.6

June 1989

This Disk Copyright Western Digital Corporation 1989.

All Rights Reserved.

== CURRENT VERSIONS ==

If you are not certain you have the most recent version of the

SuperDisk or the drivers contained on this SuperDisk call

714-474-2033 x34700.  This number reaches the Technical Support

voice mail system.  Select the LAN driver update option for the

most recent versions of LAN drivers.  This message recording

will provide you with the most recent version number for all

LAN drivers as well as information on how to obtain them.

== CONTENTS ==

The following files are located in the main (root) directory of

SuperDisk:

File                       Contents

README.DOC      This file.

JUMPERS.DOC     Jumper settings for Western Digital LAN

adapter boards.

SYSMAP.DOC      A map showing how computer system resources

are usually used.  This will help you

configure a LAN adapter board.

INSTALL.BAT     Batch file to copy this diskette to another

disk.  To avoid possible conflicts, it puts

the contents in a subdirectory named

"WDPLUS" that it creates.  (See Note 1).

@6FC0.ADF       Configuration file for Ethernet Micro

Channel (*) adapters (on 3-1/2 inch

diskettes only).

@6FC1.ADF       Configuration file for StarLAN Micro Channel

adapters (on 3-1/2 inch diskettes only).

xxx.LAN,        NetWare v2.1 driver files that are required

xxx.OBJ           to be in the root directory.

Drivers for each LAN operating system are located in separate

subdirectories as follows:

Subdirectory                 Contents

NETWAR20 <DIR>       Novell Advanced NetWare (*) v2.0a

NETWAR21 <DIR>       Novell Advanced NetWare, SFT NetWare,

and ELS NetWare Level II v2.1 and

later versions

NWPATCH <DIR>        Patch Utility for NetWare Drivers

NETBIOS <DIR>        Western Digital NETBIOS/OSI Interface

Program for use with NETBIOS-based

software such as the IBM (*) PC LAN

Program and Microsoft MS-NET (*)

3PLUS <DIR>          3Com 3+ (*)

DECNET <DIR>         DEC DECnet-DOS (*) and DEC PCSA (*)

LANMAN <DIR>         3Com 3+ Open, Microsoft LAN Manager,

and NDIS driver (DOS and OS/2)

On all 3-1/2 inch diskettes and 5-1/4 inch diskettes labeled

Diagnostics:

UNIX53 <DIR>         UNIX (*) V.3 streams

In addition to the driver itself, each subdirectory contains:

o an INSTALL.DOC or USER.DOC file that explains how to

install and use the software; and

o a RELEASE.DOC file that states the current version,

release date, and any improvements that were made

in this version of the software.

Note 1:  The INSTALL program only copies files to the target

drive.  If you are installing NetWare then you MUST use the

Driver disk itself.  Novell requires a certain volume label on

the driver disk, and this label is not copied to the target disk

(whether it is a floppy or hard disk).  DISKCOPY should be used

if you need additional Driver disks for Novell installations.

The 5.25 inch SuperDisk set includes a Diagnostic Disk.

Included on the Diagnostic disk is a network utility called NADD

which is used to identify LAN boards in your PC.  Please refer

to the README.DOC file on the 5.25 inch SuperDisk Diagnostics

disk for more information.  These programs are on the single 3.5

inch Micro Channel SuperDisk.

All of the drivers on the SuperDisk function with both XT/AT bus

systems and Micro Channel bus systems:

StarLAN:     StarCard PLUS (WD8003S)

StarLink PLUS (WD8003SH)

StarCard PLUS/A For Micro Channel (WD8003ST/A)

Ethernet:    EtherCard PLUS (WD8003E)

EtherCard PLUS With 32K RAM Buffer

Boot ROM Socket (WD8003EBT,8003EB)

EtherCard PLUS/A For Micro Channel

(WD8003ET/A, WD8003E/A)

Ethernet Twisted Pair:

EtherCard PLUS TP (WD8003WT)

== DRIVERS AVAILABLE FROM BULLETIN BOARD ==

Western Digital operates a Technical Support Electronic Bulletin

Board for its resellers and customers.  This Bulletin Board

contains documentation, diagnostic software, and drivers for use

with Western Digital products.  You can call it at (714) 756-8176

using a 1200 or 2400 bps modem.

In addition to the latest versions of drivers and other software

on SuperDisk, the Bulletin Board contains:

o A Micro Channel board driver for Western Digital's

ViaNet 3.06 LAN Operating System

o SCO (*) XENIX (*) drivers

o Sun PC-NFS (*) driver

== OTHER AVAILABLE PRODUCTS ==

Most other LAN operating systems provide drivers to use with

Western Digital "PLUS"-family LAN adapters.  Some of these

products that are available from Western Digital are:

Software                   Description

PC/TCP     A complete set of TCP/IP protocols that allows PCs

to communicate with minicomputers, mainframes, and

engineering workstations using the DoD standard

TCP/IP protocols.  PC/TCP (*) features include file

transfer, remote host login and 3270/VT100 terminal

emulation, remote program execution, electronic

mail, and a NETBIOS interface.

ViaNet     Western Digital's fully distributed LAN operating

system that lets all PCs be servers as well as work-

stations.  It makes all network resources available

to every user, and operates with the DOS 3.1

operating system and file structure on every PC.

ViaNet also allows UNIX and DOS systems to

interoperate on the same network.

== TECHNICAL SUPPORT ==

For assiatance call your dealer or distributor's

technical support department.

== VERSION HISTORY ==

-- Version 1.6 (Current Version, June 1989) --

DIAG8003 has been removed and replaced with a completly new

menu-driven Diagnostic Program which is on the Diagnostic disk

of the SuperDisk set.  NADD, the network address utility, has

also been moved to the Diagnostic Disk.  The Micro Channel

SuperDisk remains on one disk and includes the Diagnostic

Program and NADD.

Both the DECnet DOS and 3Com 3+ drivers have been revised to

operate with Micro Channel adapters.  LAN Manager drivers for DOS

and OS/2 have been added.  The UNIX V.3 driver has been placed

on the 5.25 inch disks again.  It is located on the Diagnostic

Disk.

Please review the appropriate RELEASE.DOC and INSTALL.DOC files for

complete information on all of these changes.

-- Version 1.5 (February 1989) --

Some NetWare v2.1 driver files have been moved to the root

directory and the SuperDisk volume label has been changed to

LAN_DRV_200 to comply with new Novell requirements.

-- Version 1.4/1.3 (January, 1989) --

The NetWare drivers and NETBIOS/OSI Program have been upgraded.

The NWPATCH program, NADD program, and SYSMAP.DOC file have

been added.  PC-NFS and UNIX drivers would not fit on 5-1/4 inch

diskettes and have been removed, but are available from the

Technical Support Bulletin Board.

-- Version 1.2 (July, 1988) --

NetWare v2.0a drivers have been upgraded from Version 2.06 to

Version 2.07.  NetWare v2.1/2.11 drivers have been upgraded from

Version 2.10.0 to 2.11.1.  Improvements have been made to the

JUMPERS.DOC file and to the documentation in the DECNET and DIAG

subdirectories.

-- Version 1.0 (May, 1988) --

Original version.

ViaNet is a registered trademark and StarCard PLUS, StarLink PLUS,

EtherCard PLUS, and SuperDisk are trademarks of Western Digital

Corporation.

(*) NetWare is a registered trademark of Novell, Inc.

3Com and 3+ are registered trademarks of 3Com Corporation.

PC-NFS is a trademark of Sun Microsystems, Inc.

LANWatch and PC/TCP are trademarks of FTP Software, Inc.

UNIX is a registered trademark of AT&T.

DEC, DECnet and PCSA are trademarks of Digital Equipment

Corporation.

IBM is a registered trademark and Micro Channel and OS/2 are

trademarks of International Business Machines Corporation.

SCO is a trademark of The Santa Cruz Operation, Inc.

Microsoft and MS-NET are trademarks and XENIX is a

registered trademark of Microsoft Corporation.