Fido's Complete Operating Manual         Page FIDONET-1

FIDONET Table of Contents

Introduction to FidoNet ________________________ Page 2

FidoNet Description ____________________________ Page 3

Costs __________________________________________ Page 5

FidoNet Operation ______________________________ Page 6

Basic FidoNet Packeting (Figure A) _____________ Page 8

FidoNet Routing and Scheduling (Figure B) ______ Page 9

FidoNet Packet Layout (Figure C) _______________ Page 10

FidoNet Glossary _______________________________ Page 11

Fidonet topologies _____________________________ Page 13

Addressing FidoNet Nodes _______________________ Page 16

Routing: Message Forwarding ____________________ Page 17

Route Files ____________________________________ Page 18

Route File Contents and Syntax _________________ Page 19

Route Statement ROUTE-TO _______________________ Page 22

Route Statement SEND-TO ________________________ Page 23

Route Statement ACCEPT-FROM ____________________ Page 24

Route Statement NET ____________________________ Page 25

Route Statement SEND-ONLY and RECV-ONLY ________ Page 26

Route Statement EXTERNAL-MAIL __________________ Page 27

Route Statement PICKUP _________________________ Page 28

Route Statement POLL ___________________________ Page 29

Route Statement NO-ROUTE _______________________ Page 30

Route Statement HOLD ___________________________ Page 31

Routing Examples _______________________________ Page 32

Using the Scheduler ____________________________ Page 33

The Node List __________________________________ Page 35

Installation of FidoNet ________________________ Page 39

FidoNet Section Index __________________________ Page 41

Fido's Complete Operating Manual         Page FIDONET-2

Introduction to FidoNet

This manual explains (hopefully) what FidoNet does,

how it does it, and how to install and run it. FidoNet is

built into, but seperate from, Fido the bulletin board

system.

Please read the FidoNet Operation chapter for

details on how Fidonet works; briefly, FidoNet is a method

of connecting many Fido Bulletin Boards in different

locations for the purpose of transferring messages and files

automatically. The net operates for a short period of time

each day; for instance, Fido the Bulletin Board runs for 23

hours each day, and Fidonet for 1 hour. During this hour,

any messages marked as mail are combined into packets, and

transferred to the each of the Fidonet programs. They are

then unpacked, and normal BBS operation resumes.

In most local areas (St. Louis, Los Angeles, London,

Karlstaad, etc) local networks have been set up for local

traffic, and use centralized hosts to send mail nationally.

If there is a local net in your area, you must join it or

the organization in the geographical area. The host or

region manager can supply you with lists of other systems,

newsletters, and other timely information on FidoNet.

FidoNet inherently "knows" about hosts and regions,

which operate similar to the telephone companies' area code

system. FidoNet routes messages accordinly; this is covered

further in a later section.

There are numerous things to control the setup, such

as available remote systems, cost control (or prevention)

and so on. Once setup, FidoNet is totally automatic,

requiring no sysop or user intervention.

Fido's Complete Operating Manual         Page FIDONET-3

FidoNet Description

Fidonet's purpose is to link Fido based Bulletin

Board systems together for the purpose of automatic message

and file transfers.

FidoNet is a true dialup packet switch network

system, that supports many different topologies. It supports

routing, message forwarding, scheduling, and uses a tuned

collision detection algorithm over normal phone lines, for

the lowest possible cost and highest effeciency.

The simplest scheme, and the one to setup initially,

supports point to point messages. Most major geographical

areas have a host that will accept mail for itself and it's

local nodes.

After you have contacted any other Fido sysops in

your area, you can tie into their local network, and take

advantage of the lower cost. Each local area runs things

differently, and cannot be covered here. If you can't find

your local region or host, contact Fido #1 at 415-864-1418,

where you can find the latest node list and other files to

help steer you in the right direction.

The original design was built around the current

Bulletin Board architecture (if there is such a thing) which

is basically: an unknown number of completely independent,

standalone systems, with extremely low overhead in both

maintenance and cost. FidoNet was designed to be compatible

with this, in that it should be:

(1)  No extra work for the sysop,

(2)  No effect on normal BBS operations,

(3)  No unexpected extra costs,

(4)  No effect upon system reliability.

FidoNet is totally automatic, and requires no extra

work, once set up. Other than the effect of allowing

network-wide message traffic, the only other affect to the

current BBS is that it is "down" to normal (human type)

traffic during the mail periods.

Costs, if any, are controlled by the sysop. Unless

specifically enabled, mail will not be sent out from a node.

(Sending mail costs; receiving mail is free) Fido provides

accounting and cost limitation functions (all automatic) to

prevent unauthorized mail from being sent. There can also be

"free" traffic to non-toll call nodes. The usual privelege

levels can be applied to each of the Mail commands, to

control their use.

FidoNet message traffic success/failure does not in

any way affect BBS operation. (Unless of course of hardware

or other unavoidable failure.) Failure to make a connection

and transmit a packet, or errors during incoming packets,

Fido's Complete Operating Manual         Page FIDONET-4

FidoNet Description

affect only the mail sent or received. In the case of

transmission, the messages will not be sent, nor will

charges (if any) be applied to the senders credit account.

For a paying system, the sysop must occasionally set

the users credit, using the SYSOP command. If reasonably

large sums are used as a minimum ($10.00 or more) this will

not need to be done more than once every few months.

Fido's Complete Operating Manual         Page FIDONET-5

Costs

Each node maintains a node list, that contains the

accessible remote systems. Systems not in the list cannot be

mailed to. The sysop controls this file, updating or

changing it with any text editor. It can only be read from

within Fido; users cannot otherwise access it, and cannot

see the actual phone numbers used to dial.

If the bulletin board system is a public "free" one,

then probably no toll call systems will be allowed for

general use. All users credits default to zero, so that mail

will not be allowed to non-free systems. Toll call systems

can always be accessed by the sysop.

Occasional local networks have a host that will

accept toll call mail for free; contact the Fidos in your

area to find out about this.

There are also a number of ways to control access to

the mail area; these are described in more detail in the

other sections of this manulal.

If a system in the node list is marked as free, then

anyone may send messages to it. Fido controls all costs, and

balances the accounts after messages are sent sucessfully.

Each user record contains a credit and a debit.

Messages entered are charged to debit, and cannot exceed the

credit. Deleteing a message (that has not been sent yet)

entered lowers the debit by the cost of the message; the

actual cost of the message is stored within the message.

Fido's Complete Operating Manual         Page FIDONET-6

FidoNet Operation

Fido and FidoNet are both contained in one program,

run as specified by the scheduler. Fidonet is a time driven

system, and the national time slot is at 1:00 AM Pacific

Time. (4:00 AM Eastern Time) During normal Fido operation,

users can enter FidoNet messages, and during the FidoNet

time, these messages are made into packets and sent to the

right destination. The messages may be destined to any one

or more of the available remote nodes in the nodelist.

At the predetermined time, Fidonet takes control. It

then creates mail packets, one per node, containing the

messages for each node. If there is no mail to a node, no

packet is created, and no call is made to that system.

After the outgoing packets are made, FidoNet

alternately waits for calls and attempts to place calls.

Mail packet transfers are done on a collision detection

basis. After the first few collisions, the network

synchronizes. If there are a number of nodes to send mail

to, each one is called in turn, until all are sent, or mail

time is over. If it fails with one node, it goes to the

next, and repeats the failed one only after trying all of

the others first. All of these steps have the appropriate

error checks and limits. The list of nodes to call is the

same order in which they appear in the node list.

In between outgoing calls (if any) FidoNet delays a

random interval, during which it waits for incoming calls.

This interval, along with the redial algorithm, synchronizes

the net after the initial collisions.

If an incoming call is detected, it attempts

connection with it. The baud rate is determined (same as a

normal caller would in Fido) and a message to human callers

is displayed (warning them that it is accepting only other

FidoNodes) and a synchronization process is started. This

process must complete within 60 seconds, or the call is

terminated. Once synchronized, the packet transfer is made.

The receiver just stores that packet for later use, and then

disconnects. Note that at this time, Fidonet has no idea who

originated the packet it just received.

After an incoming call is received, FidoNet calls

out immediately afterwards (assuming there are calls to be

made) since there is a high probability that the line is now

clear. This helps synchronize the network.

To place an outgoing call, the sender dials the

number, performs the sync process mentioned above, and

transfers it's outgoing packet. (Messages to a given node

are again checked against the node list at mail time; if

they do not match, the packet is not sent, and an error is

logged.) If the transfer was successful, the destination

node number is deleted from the senders list of nodes to

call.

Fido's Complete Operating Manual         Page FIDONET-7

FidoNet Operation

The collision detect algorithm is optimized such

that during the first few minutes of mail time, there are

many collisions, after which the net synchronizes, and none

or few collisions occur. Of course assumptions have been

made on average traffic per node; for heavy (or low) traffic

nodes, the collision detect algorithm can be modified by

command line parameters, the scheduler, and the various

routing control files.

When mail time is over, FidoNet deletes all it's

outgoing packets that were assembled, and for each one that

was sent sucessfully, marks those messages (in the mail

area) as SENT, so the originator can tell if they went out

or not. Then, the incoming packets are unassembled, and the

messages placed sequentially in the mail area. These packets

are then deleted.  A complete log of all call attempts,

incoming calls, and messages sent and received are kept in

the log file.

If any mail at all was sent, the user credits are

balanced. This is somewhat unsatisfactory, as it balances

the accounts even if the mail was not sent. This is to

prevent extremely long processing time necessary to account

for each message and user. (Users lists run upwards of 600

entries typically; on a floppy based system this would

become unworkable.)

FidoNet then terminates, and if there were no fatal

errors invokes Fido for another day. Messages received are

then accessible like any other message.

Fido's Complete Operating Manual         Page FIDONET-8

Basic FidoNet Packeting (Figure A)

Fido's Complete Operating Manual         Page FIDONET-9

FidoNet Routing and Scheduling (Figure B)

Fido's Complete Operating Manual         Page FIDONET-10

FidoNet Packet Layout (Figure C)

Fido's Complete Operating Manual         Page FIDONET-11

FidoNet Glossary

First, a small glossary of terms used in this manual

to describe FidoNet and all it's permutations.

Network:

A group of two or more Fidos, with the same net

number. A "network" witihn FidoNet implies that there is

routing between the systems, and that one of the systems is

a host.

Region:

A geographical area of a continent, arbitrarily

chosen during the First FidoNet Conference in St. Louis.

Regions are where Fido systems that do not belong to a

network are put; it is used only for keeping track of

systems. A region has a manager like a network does.

From Fidos point of view, a region is just like a

network except it does nto do automatic routing to the regin

host.

Node:

One Fido, running FidoNet. It may be a host as well.

Host:

A node that handles mail for itself as well as other

nodes. It may accept mail, and send it on to another node.

The sysop of the host system is usually the manager for that

area.

Manager:

The person who manages the node list for a region or

network. Usually, the sysop of the zeroth node in the region

or net.

Zeroth Node

The node that acts as host for a region or network

has two node numbers; the normal node number (1 - 32767) and

zero; zero indicates that it is the "hub" or central node

for that region or network. For a newtwork, the zeroth node

is where all mail for that network is routed to automaticall

by Fido.

Local Net:

A group of nodes, presumably within a small

geographical area, (ie. non-toll call or cheap calls) that

consists one or more nodes and a host. Basically, a small,

self contained Network.

Local Node:

Really the same as just "Node", but implies that it

is a node that has a host serving it.

Topology:

Basically, if you were to draw a map showing all the

nodes and their interconections. Some of the "shapes" made

Fido's Complete Operating Manual         Page FIDONET-12

FidoNet Glossary

by the map are called "topologies" if they show some

recognizable pattern, such as star, ring, etc.

Traffic:

The actual messages being sent. If you picture

messages as things that travel along the phone connection,

it makes more sense.

Bombing Run

The technique of sending the same message to all or

many FidoNet nodes. Frequently used for advertising purposes

in the past (highly frowned upon; a sure fire way to make

enemies these days) and very useful for disseminating

information rapidly throughout the net.

Fido's Complete Operating Manual         Page FIDONET-13

Fidonet topologies

Fido works by having "time slots", or windows, in

which it sends mail. The national window is one hour, at the

same time across the country; 1:00AM Pacific Time, 4:00 AM

Eastern Time. During this time, all Fidos send their mail to

wherever it is supposed to go. When the time is over, Fido

is run again, and callers can read the messages sent and

received.

Simple topology

Fido's simplest layout is "amorphous", ie. a nice

name for a randomly interconnected blob. All nodes have

access to all other nodes; to send a message, a call is

placed directly to the desired other Fido. To illustrate

this topology, draw 50 dots on a piece of paper,

representing each Fido, and from each, draw a line to every

other Fido. (You will end up with a lot of lines)

The disadvantages of this are mainly the fact that

the window is very, very crowded; it is sometimes difficult

to "get into" a busy system, as many others may be calling

at the same time.

The advantages are mainly simplicity. New Fidos can

come into the net just by letting others know of it's

existence. A "down" system does not affect any other system.

It is very easy to set up a FidoNet, as all you have to do

is get a node number and run FidoNet.

This is how the original FidoNet was organized

(until Sept 84 or so). It collapsed into utter confusion at

about the 50 node point.

Two Layer topology

If you draw in all the Fidos on a map of the world,

you will find that they tend to occur in geographical areas.

St. Louis, Los Angeles, San Francisco, London, Boston, etc.

There are occasional "stragglers" by themselves.

To alleviate the crowding problem, the new

arrangement is to break the current FidoNet system with all

it's many nodes, into local geographic areas and make those

into self-contained FidoNets, of the "amorphous" type. Mail

traffic within the local area works as it always did; if

there is mail to another local system, it is sent directly.

Each area is given a net number, to distinguish it from the

other nets and regions.

There are two types of local organizations; nets and

regions. Nets are where a number of systems are all within a

non-toll call area, ie. within basic service. Regions are

just geographical areas of the country, used to logically

organize a group of systems. Except for the fact that Fido

does automatic routing to nets, the two are the same.

Fido's Complete Operating Manual         Page FIDONET-14

Fidonet topologies

To interconnect all of the local nets together, each

local area designates one node as a "host". This node then

has two node numbers; the regular one, plus 0. For local

traffic, this has no effect; mail is just sent as necessary.

The host is only used to access nodes outside the local

area. This is completely automatic and transparent to users.

In other words, net and node numbers work the same

as ATT's area codes. Systems in a limited area all share the

sam enet number (area code), and have unique node numbers.

Systems in another network may have the same node number,

but will have a different nuet number, just as the seven

digits in your home phone number may be the same as someone

elses on the other side of the continent.

When a person on one net enters a message for a

person on another net, at mail time Fido routes that message

for effecient use of time. All messages for nodes within a

given network are packaged into one packet, and sent to the

zeroth node in that net, not directly to the desired net.

Later, the zeroth node (the host) unpacks the messages, and

passes them on to the desired destination.

The advantages of this are mostly effeciency and low

cost. Since all outgoing messages are bundled together,

costs go down because more messages are sent in a single

call (by way of the phone companies one minute minimum and

cheaper rates for longer connections) and because messages

from two nodes to another single node are sent at the same

time. The traffic in each of the time slots is lessened,

because of the smaller number of nodes using a particular

window.

The disadvantages are that if a host goes down, all

local nodes do not send nor receive mail that cycle.

However, the mail will be again attempted when the next time

slot comes around.

The "stragglers" mentioned above just connect to the

net through the national window, and act like hosts and

nodes by themselves, because Fido does not do this automatic

routing for regions; only nets.

This is the defacto standard organization designed

at the First FidoNet Conference, and implemented on June

11th, 1985. Who knows what it will be by the time you read

this.

This method has both advantages and disadvantages,

as you may have already guessed. Most nodes (even local

ones) still run the national window; this means that if you

want, you can send mail directly. This is still necessary,

because File Attach (logically attaching MSDOS disk files to

messages) subverts the automatic routing, and send them

Fido's Complete Operating Manual         Page FIDONET-15

Fidonet topologies

directly.

Regardless of the configuration, any node may access

any other node during the national time window.

Fido's Complete Operating Manual         Page FIDONET-16

Addressing FidoNet Nodes

As mentioned before, Fidonet uses net numbers and

node numbers. How this is done is described in detail here.

To send a message to another system in FidoNet, you

need to know the other systems net/node number. There are a

number of ways to determine this.

For a telephone system type analogy, net numbers can

be thought of as the Area Code, and the node number as the

Phone Number. To place a call (or send a message) within

your own area code (net) you do not need to specify the net

number; the node number will do. For an out of state (to

another net) message, you pick the net first.

Unlike the phone company, you can either enter the

net and node numbers directly, or let Fido give you a list

from which you can pick one or more. Fido will let you enter

more that one at a time as well, so that the message you

enter can be sent to a number of places at the same time!

When entering a FidoNet message, you see the prompt:

Currently NET #125

?=Help, #=List NODES /=List NETS

This says that you are in Net #125. Entering / gives

you a list of networks and regions; this is like getting a

list of phonebooks. If you pick another Net, then the prompt

will tell you the one you have selected.

Entering # lists the nets within the net; this is

like reading the phone book, only a lot smaller. You then

enter the node number.

There is also a shorthand for all these steps, and

the syntax will be a familiar one by the end of the manual:

NET/NODE

For instance, you could go through the lists and

enter 125 for the net, and 1 for the node, you just enter

"125/1" at the first prompt. This is obviously handy when

you send mail to the same node frequently.

At any time Fido expects a node number, you can

enter this shorthand format. This includes the routing

control files.

Fido's Complete Operating Manual         Page FIDONET-17

Routing: Message Forwarding

Routing is accomplished by the router and the

scheduler. The scheduler controls when to run FidoNet (ie.

when to send and receive mail) and the router controls where

to send the mail. Fido does most routing automatically, but

you can also completely control routing with route control

files.

As an example, here is a very simple system,

consisting of four nodes, two of which are acting as hosts.

(Net #1)                   (Net #2)

Node 1 ------------------- Node 3

|                          |

|                          |

Node 2                     Node 4

The interconnecting lines are phone calls. (While it

is still true that any node can call any other, that does

not use the routing.) Assume that Node 1 to Node 2 is a

local call (ie. no charge) and 3 and 4 also. Node 1 to Node

3 is a toll call.

Someone at node 2 wants to send a message to someone

on Node 4. The message is entered normally; Node 4 is chosen

for the message. (Entering mail is covered elsewhere) This

is identical to previous FidoNet mail.

When Fidonet is run (because the scheduler says so)

FidoNet determines what mail to send, and to where. It

creates packets of mail, places phone calls, and sends them

out.

Without the router, Node 2 would call Node 4

directly, and send the mail. The router however, tells it to

send it to Node 1 instead. Once it is sent, that is the last

that is heard of it as far as Node 2 is concerned.

When Node 1 runs FidoNet, it also makes the packets

as do all other Fidos. However, it's router tells it to send

all mail destined for Node 4 to Node 3 instead. (Since in

this case Node 3 is a different net, and Fido need not be

told to do it; it's done automatically but you could tell it

to manually.)

Similarly Node 3 sends mail to Node 4. Since the

message has reached it's destination, it is forwarded no

longer, and stops. Node 4 has received mail from Node 2.

Even though this discussion is about mail in only

one direction, it can actually be going in both (or any, or

all) directions simultaneously, and in practice does.

Mail that is "in transit" disappears when the

message has finally be forwarded.

Fido's Complete Operating Manual         Page FIDONET-18

Route Files

Previous Fidos had only one route file, called

ROUTE.BBS, and all it did was control routing. Route files

now control many different parameters for packet and mail

operation.

Instead of just one file, there can be one per

schedule. The Schedule Tag also reflects the name of the

route file used:

Schedule A      ROUTE.A

Schedule B      ROUTE.B

...             ...

Schedule W      ROUTE.W

If the specified route file cannot be found, then it

looks for ROUTE.BBS; if that isn't found, one is not used.

IMPORTANT NOTE:

Schedule A is a very special schedule. Fido normally

will not send any mail unless you very specifically tell it

to do so; for this reason, Schedule A was made to simplify

life for sysops not part of a local net.

Schedule A should be used only if your only schedule

is the single, national FidoNet event. If you run any kind

of routing or network, please do not use A.

Schedule A basically basically executes the

following routing statement without the need for a route

file:

SEND-TO ALL

If you need to do fancy routing from Fido to Fido,

you will need to set up schedules for each leg of the

journey, as covered above. It is recommended that you make

each schedule a seperate tag, so that you can have a route

file to control each.

The best way (well, the only way, since this is how

I'm going to cover it all) to get an idea of the

capabilities of the route language is to read about each

statement.

Fido's Complete Operating Manual         Page FIDONET-19

Route File Contents and Syntax

The route file contains an ASCII text "language",

that is more or less format free. There are a number of

keywords, that take a variable number of arguments. The

commands are:

ROUTE-TO

SEND-TO

ACCEPT-FROM

NET

SEND-ONLY

RECV-ONLY

EXTERNAL-MAIL

PICKUP

POLL

NO-ROUTE

HOLD

The command modifiers and metanumbers are:

ALL

OURNET

HOSTS

NOT

Arguments to commands are always numbers or

metanumbers; the numbers specify node and net numbers.

"Metanumbers" represent useful groups of node and net

numbers.

Route file contents are completely free form;

keywords can appear anywhere, in any order. Leading and

trailing spaces, commas or tabs are ignored, though you need

at least one between each keyword and number.

You can use the same keyword more than once. The

following two blocks of code accomplish the same thing:

SEND-TO 99

SEND-TO 1,77

SEND-TO 99,1,77

Fido will let you put in contradictory commands; the

last one encountered will have effect. The two lines below

are treated as if the first one does not exist.

ROUTE-TO 99,1

ROUTE-TO 1000,1

Comments

Comments are considered to be anything starting with

Fido's Complete Operating Manual         Page FIDONET-20

Route File Contents and Syntax

a semicolon and continuing to the end of the line. Comments

can be put anywhere in the route file.

Node Numbers

Most route file commands require numerical

arguments, usually a list of nodes. There are a few ways you

can specify this; use which ever looks nicer or is shorter.

99              a plain number

A number by itself means a node number within your

current network or region. For example, if your net is #125,

99 means node 99 in net 125.

125/99          net/node

This very explicitly means net 125 node 99; if your

net were not 125, then you need to specify the net and this

is one way to do it.

Another way is to change the "default" net; see the

NET command description.

For all route file commands, you can enter any

number of node or net/node numbers, either on the same line

or continuing onto the next. For example, the following are

all OK:

SEND-TO 99

SEND-TO 129/99

SEND-TO 1,33,76,55

5,33,77/456,          5/23,5/24

SEND-TO

1

99

Metanumbers

While you can, and frequently need to, specify long

lists of numbers, you usually do this in ordered sets, such

as member of your own net, all nodes, or all nodes except

your own net, and so on. Fido supports some special

"metanumbers" to accomodate this.

ALL

Means "every node in the node list"

OURNET

Means "every node in out network or region".

HOSTS

Means "all network and region hosts". (All node

number 0's, the zeroth node for each net and region.)

Fido's Complete Operating Manual         Page FIDONET-21

Route File Contents and Syntax

To extend the above example, you could use:

SEND-TO ALL

SEND-TO OURNET

SEND-TO HOSTS

NOT

"Undo" the command for the list of nodes to follow.

The following example should explain it's main side effect:

SEND-TO ALL NOT OURNET

If your net is 125, then this means SEND-TO all

nodes except those that are in net 125. It is not exactly

the same as the word "except"; Fido does not use the word a

way a person does. This is a very subtle difference, and

exactly how people can get burned by computers.

Extremely literally, the above statement means:

"Mark ALL nodes as SEND-TO, then remove the marks

from nodes in OURNET".

Using NOT with the SEND-TO command "undoes" the

effect of the SEND-TO command; it is like saying:

SEND-TO ALL

(UN-SEND-TO OURNET)

There is no UN-SEND-TO command of course. Fido scans

the line from left to right, first marking all nodes as

SEND-TO, then it goes back and removes the marks in OURNET

nodes.

The only time this will matter is if you had done

something like:

SEND-TO 99

SEND-TO ALL NOT OURNET

The first line says to send to 99, in our net. The

second line does what you think it does, EXCEPT that NOT

OURNET means "undo" all of OURNET, including the 99 from the

previous command.

I may be overemphasizing the importance of this, but

better warned than not.

Fido's Complete Operating Manual         Page FIDONET-22

ROUTE-TO

ROUTE-TO is the most basic and powerful routing

command. It modifies the destination of the basic message

unit. It has the syntax:

ROUTE-TO <dest-node> <node, node, node, ...>

Which means in English "Route to dest-node, messages

for node, node, node ...".

Please keep in mind that internally, Fido will route

all mail for nodes inanother net, to the host (node 0) of

that net. This could be done via a ROUTE-TO statement (and

in previous versions was) but is so universally necessary

that it was made implicit. You can use ROUTE-TO to override

Fido's default host routing.

The list of nodes can be replaced with either ALL or

OURNET; for an outgoing host, you could specify ROUTE-TO

<host> ALL, which means send to the host node all outgoing

mail.

;

;Send all of our outgoing mail to fido 99. Fido 99

;makes all long distance phone calls.

;

ROUTE-TO 99 ALL

;

;Route mail for nodes 100/51 and 100/22 to 125/99 instead.

;

ROUTE-TO 125/99 100/51,100/22

HINT:

There are certain things that can override any

routing that may be in effect. The following things cause a

message to go directly to it's final destination rather than

to the host or ROUTE-TO node.

File attach

NO-ROUTE <nodes>

POLL <nodes>

Fido's Complete Operating Manual         Page FIDONET-23

SEND-TO

SEND-TO tells FidoNet that during this schedule,

whatever it is, that it is OK to send mail to the list of

nodes that follow. If none are specified, Fido will not send

any outgoing mail.

The exception to this is Schedule A, which

implicitly allows mail to all nodes; this is done for

convenience, and historic backwards compatibility.

In a multiple schedule system, you may set aside a

schedule to send mail to and from your local nodes, say 1/2

hour before or after than national time. Since you only want

to deal with your local nodes, the statement:

SEND-TO OUTNET

Tells Fido to send mail to only the local systems,

as defined in the node list. You could of course specify any

legal node number, including ALL.

If you do not specify a SEND-TO command, then Fido

will not send any mail!  Most Fidos run the national mail

schedule using:

;

;Normal national schedule route file.

;

SEND-TO ALL

;

;We use ROBOT.EXE to generate messages to all the

;hosts in FidoNet (net/0 and region/0) and now we want

;to send to them.

;

SEND-TO HOSTS

Fido's Complete Operating Manual         Page FIDONET-24

ACCEPT-FROM

ACCEPT-FROM is another basic command, and is used

for cost control and security purposes. It controls whose

mail your Fido will forward onward.

Normally, your Fido system will send only mail that

originated on your system; this is to prevent another system

from accidentally forwarding it's long distance mail to you

for further mailing.

There are many cases where you do want to forward

others' mail. If you are a host for a number of local

systems, you need to accept mail from anyone, for forwarding

to your locals only. The statements:

;

;We are an incoming host, and receive all mail for the

;other nodes in our net. We need to pass this on to them,

;and only them!

;

SEND-TO OURNET          ;our net only

ACCEPT-FROM ALL         ;but from anyone

During a local schedule tells Fido to send mail to

only the local systems, and to accept it from anyone. If you

were an outgoing host for your locals (you take their

outgong mail and send it around the world) you could do that

by:

;

;Accept mail from only our local nodes, for mailing anywhere

;in the world.

;

ACCEPT-FROM OURNET      ;from our own net only

SEND-TO ALL             ;but send to anywhere

NOTE:

ACCEPT-FROM does not mean that Fido will reject

messages from any other Fido; it means that your Fido will

not accept them for FURTHER MAILING.

Fido's Complete Operating Manual         Page FIDONET-25

NET

For all routing commands, Fido assumes, unless told

otherwise, that the network is your Fido's network. That is,

if you specify a node number without a net number, as in:

ROUTE-TO 99,1,2,3,5,77,88,101 ...

Fido assumes all of those nodes are in your net. The

NET command changes the default network for further routing

commands. For example, if you were part of net 125, and ou

wanted to specify a number of nodes in another net, the

following two are equivalent:

ROUTE-TO 99,101/1,101/2,101/3,101/4 ...

NET 101

ROUTE-TO 125/99,1,2,3,4 ...

Remember that Fido will remember the last set NET;

use NET 125 to change it back if necessary.

Fido's Complete Operating Manual         Page FIDONET-26

SEND-ONLY and RECV-ONLY

You may be able to guess at the purpose of these

two. They do not control how Fido routes, but how it dials.

SEND-ONLY tells Fido not to pause waiting for

incoming calls, but to dial as quickly as possible. This is

a very special purpose command, only for outgoing-only

systems used as outgoing hosts.

For example, in the SF Bay Net, there is an unlisted

Fido that does outgoing mail only; since it is unlisted,

there are no other Fidos trying to call it.

RECV-ONLY does just the opposite. Fido makes all of

it's outgoing packets normally, but never makes any outgoing

calls. this command is useful for incoming hosts that want

to stay maximally available, or for systems the rely on

other systems using PICKUP or POLL to retrieve their mail.

;

;We never send mail during this schedule, only receive

;it. Fido 99 sends it. So sit and receive only.

;

RECV-ONLY

;

;Fido 1 passes all it's outgoing mail to us, Fido 99.

;Since its an unlisted number, no ne ever calls us; do

;nothing but make outgoing phone calls.

;

SEND-ONLY

Fido's Complete Operating Manual         Page FIDONET-27

EXTERNAL-MAIL

This is another very special purpose command. It is

meant for use as a gateway from Fido to different networking

system.

All it does it cause Fido to not delete it's packet

files and file list files when a schedule ends. Normally,

after Fido finishes a schedule, it deletes any unsent

packets and lists.

Using EXTERNAL-MAIL, you can coerce Fido into

creating it's packets and file lists in a coherent fashion,

and leave them for you to handle. The way to do this is to

make a route file that contains any routing you need, and

the EXTERNAL-MAIL command, and make the schedule only ONE

MINUTE LONG. (If it is longer, Fido will still attempt to

make outgoing calls!)

When EXTERNAL-MAIL is specified, Fido will treat all

the packetized messages AS IF THEY WERE SENT; messages are

marked (SENT) or deleted, as appropriate.

It is assumed that you will deal with the packet

files yourself.

There is a fixed naming format to these files. For

each node that there is mail for, there are two files:

nnnnmmmm.OUT            Message Packet

nnnnmmmm.FLO            File List

The .OUT file is a FidoNet Packet, as defined in

FIDOMAIL.DOC, and contains all the outgoing messages.

The .FLO file is a list containing all of the

pathnames of any files that were attached by messages in the

a variable length null terminated string.

The net/node number is the filename portion. Both

net and node numbers are expressed as zero filed hexadecimal

numbers:

nnnnmmmm = net/node

007D0001 = 125/1

Fido's Complete Operating Manual         Page FIDONET-28

PICKUP

PICKUP is a command that lets one system "pick up"

it's own mail when it calls another system. For example, if

two seperate systems have mail for each other, normally Fido

requires that each system make an outgoing call to send it's

packet to the other. (This is done in the interest of

accountability; it totall yeliminates arguments ...)

If PICKUP is specified, in the above example who

ever called first would transfer it's packet as it normally

would, then additionally ask the system it just called for

if there is any mail to "pick up" while it's connected. If

there is, the calling system receives the packet that

normally would have been delivered by the other system

making an outgoing call.

Instead of relying on chance determining who calls

out first, the RECV-ONLY command can be used on one system.

The syntax for PICKUP is:

PICKUP n1, n2, n3, n4, n5, ...

The calling system requests to pick up mail only if

commanded to do so; the receiving system will allways let a

calling system pickup it's own mail.

Note that routing will affect this command in a very

important way. If you desire to PICKUP mail for say, 100/99,

your FIdo has to actually call that system, not the host

(100/) that it will normally. It is your responsibility to

ensure that routing is setup properly.

Please refer also to the POLL command, which has a

similar function.

If you want to send mail normally, but leave one

nodes mailfor it to pickup, refer to the HOLD command.

;

;I have a WATS line, so why not pick up mail when I

;make outgoing phone calls. (I wish)

;

PICKUP ALL

Fido's Complete Operating Manual         Page FIDONET-29

POLL

POLL is used to force a node to make outgoing calls

to the specified nodes even if there i sno mail to send. The

purpose of this is to support polling systems that may have

mail for pickup.

POLL is similar to PICKUP, in that it tells FidoNet

to request pickup mail from the specified system; POLL

additionally causes FidoNet to place a call and transfer an

empty packet even if there are no messages to send.

POLL overrides any routing in effect for the listed

nodes.

The syntax for POLL is the same as PICKUP:

POLL n1, n2, n3, n4, ...

Call each of the specified nodes even if there is no

mail for them, and request any pickup mail.

One use of this might be where you have a centrally

located system acting as host, and a numebr of locals. To

make billing easier, you can arrange it so that the locals

never make phone calls; the host calls them in turn and

picks up it's own mail.

;

;Host: call all of our nodes and pick up the mail.

;

POLL OURNET

;

;Local: make packets, but do not send them; let the host

;call and get it.

;

ROUTE-TO 99 ALL

RECV-ONLY

Refer to the HOLD command for more ways to hold mail

for another system to pick up.

Fido's Complete Operating Manual         Page FIDONET-30

NO-ROUTE

This command causes Fido to ignore any routing

specified for the list of nodes specified. This command is

necessary in many cases to make PICKUP work as desired.

The syntax for NO-ROUTE is:

NO-ROUTE n1, n2, n3 ...

If you have mail for a number of nodes waiting to do

PICKUP, make sure you specify them all as NO-ROUTE. The

reason is that if the nodes are in another net, then Fido

will address the packet to the host, not the nodes

themselves; when a node calls in, it will find no mail,

because it is addressed to their host!

NO-ROUTE tells Fido to not do routing for the

specified nodes.

;

;Fido's 100/22 and 100/51 call in for their mail; make

;sure that Fido doesnt address it to 100/0, their host.

;

NO-ROUTE 100/22, 100/51

Fido's Complete Operating Manual         Page FIDONET-31

HOLD

The HOLD command causes Fido to not make outgoing

phone calls for the nodes listed. It's main use is so that

another system may call in using PICKUP or POLL and retrieve

their own mail, by preventing Fido from calling that system

to deliver mail.

An example might be where you want to deliver most

mail normally (let Fido packet it and make phone calls)

except for one system; for that system, you want Fido to

wait for the other node to come and pick it up.

The syntax for the HOLD command is:

HOLD n, n2, n3, n4 ...

For instance, you could have your system send it's

mail out as would any other Fido, except make one system

call it to get it's mail.

;

;Normal outgoinging mail, but make 100/111 call in

;and pick up its mail.

;

SEND-TO ALL             ;OK to send to all,

NO-ROUTE 100/111        ;no routing for this one,

HOLD 100/111            ;hold it

Fido's Complete Operating Manual         Page FIDONET-32

Routing Examples

I feel the best way to describe routing commands is

by example. The example here is for a network, it's incoming

host and local nodes. The host receives mail from the rest

of the network and passes it on to the locals. There are two

schedules for the host:

Schedule B      1:00 - 1:50

Schedule C      1:50 - 2:00

Note that this schedule accomplishes all local

system routing within the national time slot. It does not

need any further schedules, but does assume that the host to

local traffic is fairly small.

The locals just run one schedule, during the normal

national mail time. The schedule tag is arbitrary; it can be

anything A to W. A is handy in this case because it does not

need a route list.

;

;Schedule B route file for the host; just do

;normal FidoNet mail, as most of the systems in the

;world. Since there is no ACCEPT-FROM, it will only

;send mail actually entered on this system.

;

SEND-TO ALL             ;we can send to anyone

;

;Schedule C route file for the host; the locals are still

;running schedule A, and are waiting for mail, and maybe

;sending some of their own. This passes any mail received

;during schedule B to the locals.

;

SEND-TO OURNET

ACCEPT-FROM ALL

Fido processes the route files sequentially; you can

take advantage of this to make it easier to create custom

routing. For example, if you wish tou route all mail to a

certain system except for one or two, it is not necessary to

list them individually merely to exclude the one or two. You

can do it by:

ROUTE-TO 99 ALL

ROUTE-TO 500 500

With the esception of POLL and NO-ROUTE, commands

can overwrite the effect of any previous command. The above

effect could be had by:

ROUTE-TO 99 ALL

NO-ROUTE 500

Fido's Complete Operating Manual         Page FIDONET-33

Using the Scheduler

The scheduler is the way that Fido controls when, as

opposed to how, FidoNet (and other things) are run. The

details of operating the scheduler commands is described in

the Sysop Only Commands section. This section tells how to

use the events.

The scheduler contains a list of "events", each of

which consists of a day and time and other information on

what to do at that time. While Fido is waiting for a caller,

it is also watching the scheduler.

When Fido accepts a caller, it watches the time until

the next scheduled event; if it is less than that callers

time limit, it sets their limit to be the time until the

event, and warns the user that they have less time than

normal. Fido will enforce this limit to ensure that the

caller logs off so that the event can be run.

The scheduler keeps information on what is to be done

to execute the event. The schedule Tag is what determines

what kind of event it might be.

Tags A to W are reserved as FidoNet schedules; when an

event tag A to W is found, FidoNet is run. This tag controls

which nodes in the list mail is sent to; this allows sending

mail to certain other systems during certain times, by

setting a schedule with each desired tag.

This example covers making Fido run FidoNet in the

national time slot. The time of day depends on your

location, that is your time zone. The example below assumes

Pacific Time; adjust accordingly.

Pacific Time        Central Time        Eastern Time

01:00 AM            03:00 AM            04:00 AM

60 minutes          60 minutes          60 minutes

Do two things: Set Fido to run FidoNet every day at

1:00AM for one hour (normal FidoNet operation) and allow the

Yell command to be used only between the hours of 9:00AM and

5:00PM.

(Enter command '6')

Event # [0 - 34]: 0             (over write the default one)

Day of week [All, 0=Sun ...]: A (every day)

Start Time [hh:mm]: 9           (9AM)

Window Width: 480               (8 hrs)

Schedule Tag: [A - W, X, Y]: Y  (Yell command)

(Yell command now enabled from 9:00 AM to 5:00 PM)

(Enter command '6')

Event # [0 - 34]: 1             (New event)

Fido's Complete Operating Manual         Page FIDONET-34

Using the Scheduler

Day of Week [All, ...] A        (every day)

Start time [hh:mm]: 1:00        (at 1 AM)

Window Width: 60                (1 hour)

Schedule Tag [A - W, X, Y]: A   (Normal FidoNet)

That is it. It was not necessary to choose events 0 and

1; any two could have been used. Using 0, however, replaced

the default yell command enable (10:00 AM til 10:00 PM) with

the desired one.

To add other events, just choose another event number

and enter them as above. There is room for up to 35 events.

External Events

The scheduler can be used to cause Fido to terminate

to DOS (to the RUNBBS.BAT batch file, actually) with an

ERRORLEVEL value that can be used to run programs at certain

times of the day.

This isnt a general purpose way to run programs like

data based programs, etc. It is meant for use with programs

that can run unattended, such as Fido's RENUM.EXE utility

program.

You set a schedule normally, except select X for the

tag. When you do this, it asks an additional question:

ERRORLEVEL [4 - 255]:

This is the ERRORLEVEL passed back to DOS. This

value can be tested for in the batch file by:

IF ERRORLEVEL (xx) GOTO LABEL

Where (xx) is the errorlevel you specify, and LABEL

is a label in the batch file.

You need to understand how MSDOS batch files work;

it is too complex a subject to cover here. One reminder

though: the IF ERRORLEVEL (xx) GOTO LABEL does not mean "if

errorlevel EQUALS (xx) goto label", it means "if errorlevel

IS EQUAL TO OR GREATER THAN (xx) goto label". This means

that you must put the tests in decreasing order:

FIDO_XXX switches ...

if errorlevel 10 goto x1

if errorlevel 4 goto x2

if errorlevel 1 goto end

Fido's Complete Operating Manual         Page FIDONET-35

The Node List

The format of the node list has been expanded in

version 11 of Fido, over previous versions.

The node list is the list of remote systems, the phone

numbers, etc. When you apply for a permanent node number,

you will receive the latest node list. Node lists will be

broadcast to all FidoNodes as it is updated.

Most of this information is academic; the node list

is too large, and too complicated, to manage manually. The

LISTGEN program, by John Warren (see seperate documentation)

was written specifically to handle the processing needed to

customize the node for your specific area code, modem, dial

requirements, etc.

As of August 1985, the nodelist is published weekly

by Fido 100/51. Please refer to the current lists, or

contact a local Fido system to obtain the latest node list.

The nodelist distribution process may change without notice.

The node list, called NODELIST.BBS, describes each

remote system, one per line. You do not need to create one;

new ones are distributed on a regular basis, and they are

easy to find if you lose one.

Note that each component of the nodelist cannot

contain spaces or commans; spaces and commas are used to

seperate fields. Therefore, use the underscore character to

seperate words in a given field. For example, to put in a

name of "Fido's Board" do it as "Fido's_Board". See the node

below about special text processing done on the node list

lines.

The basic format for a node list entry is as

follows:

(KEY) 1 0 1200 Board_Name (415)-864-1418 SF_CA 24_hrs_etc

^   ^ ^ ^    ^          ^              ^     ^

|   | | |    |          |              |     |

|   | | |    |          |              |     +------ Cmt

|   | | |    |          |              +------------ City

|   | | |    |          +--------------------------- Phone

|   | | |    +-------------------------------------- Name

|   | | +------------------------------------------- Baud

|   | +--------------------------------------------- Cost

|   +----------------------------------------------- Node

+--------------------------------------------------- *text

Each of these items are seperated by one or more

spaces or tabs. Do NOT have spaces, commas or tabs in the

middle of any of these.

Fido's Complete Operating Manual         Page FIDONET-36

The Node List

Key

This word, if present, tells Fido what kind of line

this is. Currently, there are three possibilities:

(blank)

If no word is present, then this line defines a

normal Fido node. The number field is the systems node

number.

NODELIST

This doesn't define a node, but is used to identify

the nodelist itself. Fido will display the LIST entry when

listing the nodelist when entering messages.

In this case, the node number field is the same as

the extention of the filename itself; the day of the year in

which the file was created. (000 for 1 Jan, etc) This number

cannot be zero. All other fields in the node list entry can

have any contents.

HOST

This line defines a Network Host. The number is this

networks Net Number. All nodes that follow this line are

considered part of this Net until another HOST line is

encountered.

REGION

Same as HOST, but has different meanings to Fido;

refer to earlier chapters.

COUNTRY

This word is reserved; COUNTRY entries are otherwise

treated as HOST for all FidoNet purposes.

Number

This is that systems net or node number, depending on

the key field. It cannot be zero.

Cost

This is the cost, in cents per message, that you want

to charge a user for sending mail to this node. If the cost

is 0, it is assumed to be a local call. If 0, then anyone

may send messages to this node. Otherwise (if not zero) the

user must have enough credit in their user record, or Fido

will not allow them to enter mail. (Set by the SYSOP.EXE

Utility program.)

Baud Rate

This is the speed to dial this system with when it is

time to send them mail. It is any number that is a multiple

of 300; i.e. 300, 600, 1200 ... 9600. Fido will attemt to

Fido's Complete Operating Manual         Page FIDONET-37

The Node List

dial at this baud rate even if your modem cannot handle it.

Name

This is just the descriptive name (mnemonic) for this

system. Luckily, most systems have some cutesy name, like

Fido's Board, Demon, etc. It must be one word. (Use

underlines.) It must also be less than 25 characters long

total.

Please see the note below about special text

processing for this entry.

Phone

This is the phone number for the system. It is

basically a standard D.C. Hayes SmartModem phone number,

with one exception. If you use Sprint, etc, you must put it

in each number. Do not leave spaces in the number. The legal

characters in a phone number are:

1,2,3,4,5,6,7,8,9,0 # *

-              ignored

( )            ignored

.              1 second delay

?              Wait for tone

@              Wait for quiet

_              See note below

If a function is not supported by your modem, the

command is ignored or converted into the closest command,

usually a delay. Note that comma is not the delay character;

it is used to seperate items in lists. A dot is translated

to a comma when dialing.

T              Touch Tone

P              Pulse

May be inserted at any point in the number as needed.

Fidonet dials in Touch Tone by default, if P is not

specified. The maximum length of the phone number is 39

characters.

City

This is another single word field, usually the city and

state. Line the name field, underscores are converted by

Fido into spaces. See the note below on how text is

proccessed for this entry.

Comment

The rest of the nodelist entry is considered to be

part of the comment field. For future compatibility, all

comments should be done as other fields.

Fido's Complete Operating Manual         Page FIDONET-38

The Node List

NOTE:

Underscore characters found in nodelist lines are

converted to spaces after processing. This allows a neater

looking nodelist. Underscores can be used in the Name, City,

Phone or Comment fields.

Fido's Complete Operating Manual         Page FIDONET-39

Installation of FidoNet

Assuming you already have Fido running, setting up

FidoNet is simple. If you dont have Fido running, then skip

this part until you do.

The basic steps to do are:

(1)  Set the basic mail parameters shared by Fido and

FidoNet.

(2)  Set up the MAIL message and file area.

(3)  Make up a working node list (list of other systems.)

(4)  Set the scheduler to run FidoNet.

(5)  If you have a host system, make a route list.

(6)  Obtain a node number.

That is all there is to it. Each step is described

below. Also, I can't emphasize how important it is to ...

HAVE THE TIME AND DATE SET ACCURATELY!!!!!!!!

Fidonet depends heavily on the correct time and date.

Within 5 minutes or less is fine, its not that critical, but

since this is a time driven system, it is very important.

Assuming you already have Fido running completely,

pick or create a message area for FidoNet messages. This can

be an existing message area, or a totally new one. Install

it as you would a regular Fido message area, then:

1 <number>     Select your mail area,

1 1\A          Set MAIL attribute

1 S            Save the changes

This informs Fido that entering messages in this

area should prompt for net/node numbers, change prompts, and

other small internal differences.

The "4" command is used to set the net and node

numbers, the message path and the file path used for

attached files. The message path is the duplicate of what

you entered for the FidoNet Message Area; the file path is

where Fido will place any files coming in over the FidoNet.

Please do not leave it blank! If this path is blank, Fido

will put files in the default Fido area, and could overwrite

files such as the user list, bulletins, etc.

It's a good idea to make the file path the same as

your upload path, or possibly an area of it's own. It does

not have to be a Fido File Area at all; just an MSDOS

directory or disk.

Fido's Complete Operating Manual         Page FIDONET-40

Installation of FidoNet

4 ?             help!

4               lists current settings

4 msgpath\P     Set Message Path

4 filepath\F    Set File Path

If you have a net and node number, set it now:

4 nn\T          Set Net Number

4 nn\N          Set Node Number

If you are a host, or belong to another network, set

your alternate net and node to that, otherwise make it the

same as your main net and node numbers:

4 nn\H          Set Alternate Node Number

4 nn\A          Set Alternate Net Number

The last item is to set the time to run FidoNet with

the Scheduler. The national FidoNet Schedule is based upon

1:00 AM Pacific Time, either Standard or Daylight Savings.

You will have to adjust this of course for your local time

zone.

Follow the Using the Scheduler section, and set the

time.

Fido's Complete Operating Manual         Page FIDONET-41

FidoNet Section Index

ACCEPT-FROM............................. 19

ALL..................................... 19,20

Architecture............................ 3

Batch files............................. 34

Baud Rate............................... 36

Bombing Run............................. 12

Collision detect algorithm.............. 7

Collision detection..................... 3,6

Collisions.............................. 6

Cost.................................... 5,36

COUNTRY................................. 36

Credit.................................. 5

Debit................................... 5

ERRORLEVEL.............................. 34

Errors.................................. 7

Events.................................. 33

External Events......................... 34

EXTERNAL-MAIL........................... 19

FidoNet................................. 39

FidoNodes............................... 35

File attach............................. 22

Geographic area......................... 13

Geographical areas...................... 3

HINT:................................... 22

HOLD.................................... 31,29,28,19

HOST.................................... 36,11,19,23,20

Local call.............................. 36

Local Net............................... 11,3

Local Node.............................. 11

Mail parameters......................... 39

Mail time............................... 7

Maintenance............................. 3

Manager:................................ 11

Message traffic......................... 3

National time slot...................... 33

Net number.............................. 13,16

NET/NODE................................ 16

Nets.................................... 13,20

Network:................................ 11,19

NO-ROUTE................................ 19,22

Node list............................... 35,6,5

Node Numbers............................ 20,16

Node:................................... 11

NODELIST.BBS............................ 35,36

NOT..................................... 21,19

Fido's Complete Operating Manual         Page FIDONET-42

FidoNet Section Index

OURNET.................................. 19,20

Overhead................................ 3

Packets................................. 6,7

Phone number............................ 37,37

PICKUP.................................. 26,19,28,31,30

Point to point messages................. 3

POLL.................................... 19,31,26,22,28

RECV-ONLY............................... 28,19

Redial algorithm........................ 6

REGION.................................. 36,11,13

Remote systems.......................... 5

RENUM.EXE............................... 34

ROUTE-TO................................ 19

ROUTE.BBS............................... 18

Router.................................. 17

RUNBBS.BAT.............................. 34

Scheduler............................... 33,17

SEND-ONLY............................... 19

SEND-TO................................. 19

Synchronization......................... 6

SYSOP.EXE............................... 36

Tags A to W............................. 33

Terminate to DOS........................ 34

Time and date........................... 39

Time of day............................. 33

Time zone............................... 33

Toll call............................... 5

Topology:............................... 11

Traffic:................................ 12

Two Layer topology...................... 13

User credits............................ 7

Users credit............................ 4

Yell command............................ 34

Zeroth Node............................. 11

Zeroth Node............................. 11