Fido's Complete Operating Manual         Page INSTALL-1

Table of Contents

Introduction to Fido ___________________________ Page 2

Getting it all together ________________________ Page 4

Step 1: Setting Up Message and File Areas ______ Page 5

Step 2: The Environment ________________________ Page 7

Step 3: Starting Fido __________________________ Page 10

Step 4: Setting Up Areas _______________________ Page 11

Completing Installation ________________________ Page 14

System Messages and Bulletins __________________ Page 15

Making a Public System _________________________ Page 16

Semi-Private (Restricted) Public System ________ Page 18

Making Fido a Private System ___________________ Page 19

Changing Command Privilege Levels ______________ Page 21

Hints and Tips _________________________________ Page 22

Files Section Organization _____________________ Page 24

Command Line Switches and Setting Time Limits __ Page 27

HELP Files _____________________________________ Page 30

Installing the Modem ___________________________ Page 31

Commanding the modem: FIDOMDM.BBS ______________ Page 34

Specific Computers and Modems __________________ Page 35

Specific Modems ________________________________ Page 36

Modem Problem Determination ____________________ Page 39

Install Index __________________________________ Page 40

Fido's Complete Operating Manual         Page INSTALL-2

Introduction to Fido

If you are really lucky, this manual will help you

create and customize a Fido Bulletin Board system to your

liking. If you are not lucky, you will become hopelessly

lost and confused.

This manual covers installing and operating the

software itself. If you are attempting to become part of the

world wide FidoNet, please contact a FidoNet sysop who is

currently in the net. The problem is that since it's

inception in May 1984, the basic net organization has

changed radically, every few months, and all attempts at

putting down information in a such slowly changing text such

as this is hopelessly outdated by the time it reaches your

hands.

To get information on the current FidoNet, contact

Fido 125/1 (formerly known as Fido #1) and look for the

latest Fido History document, or other related files.

As an example of the problems involved, when the

Third Edition of this manual was distributed in November

1984, there were 50 or so FidoNet nodes in the US, in a

simple amorphous network; today (Aug 85) there are over 350

systems in the US, Canada, England, Sweden, Holland and

France, in a multitiered tree shaped network. So much for

putting dynamic information is a static place as this.

Fido is a software system, not just a single

program. Besides the program itself, Fido requires a number

of disk files, subdirectories or other disks to hold

messages and downloadable files, and obviously a modem and

cable.

While Fido requires many disk files to run properly,

almost all of them are created and maintained by Fido

itself. The exceptions are the batch file to actually run

Fido, the welcome messages and other text messages that Fido

can display, and various other lists and messages. These are

covered in detail later, and are simple to create manually,

if need be, with an ordinary text editor.

You can think of Fido as having an inside and an

outside. The inside is what you can change and see by

running the Fido program. These are things such as setting

up message and file areas, privilege levels for the various

commands, scheduled events such as FidoNet, and the like.

These are all accessed by Fido commands.

The outside consists of the disks used, the modem

and cable, the MSDOS batch file, subdirectories for the

message and file areas, signon and welcome messages that the

system operator creates, and so on. While these are of

course used by Fido (messages are displayed, modem is used

to answer and dial, etc) they cannot be created or modified

Fido's Complete Operating Manual         Page INSTALL-3

Introduction to Fido

from within Fido.

Simply put, there are four stages to follow to get

Fido properly installed. These are:

(1)     The Environment:

Making subdirectories, formatting blank diskettes if

necessary, determining how many and where to put the message

and file areas.

(2)     Inside:

Running Fido from the local keyboard (instead of the

modem) in order to set up Fido to match the environment you

created. Fido must be told about each message and file area

subdirectory or diskette set up earlier.

(3)     Outside:

Customizing the batch file to run Fido from; this

contains the command line parameters for users time limits,

and other operating details.

(4)     Hardware:

Making the modem go. Up to this point, the modem is

not needed at all, and can be ignored. If you have a modem

already connected and running with a communications program,

then this step is easy.

Fido's Complete Operating Manual         Page INSTALL-4

Getting it all together

This part wasn't mentioned above, and really should

be done before you even start. Ask yourself some very basic

questions: do you have a machine that is capable of running

Fido? (MSDOS version 2 or later, a supported machine or IBM

clone, enough memory, etc)? Do you have all the files? A

supported modem, or reason to believe yours will work?

Most people will not have trouble with this, but be

warned that things are not always what the computer store

salesman said they were. "Hayes Compatibility" means more

than just executes AT commands. It implies things about the

signals on the rear panel connecter (Mainly Data Terminal

Ready and Carrier Detect) and other not so obvious

capabilities. A good test here is: does it run SMARTCOM from

Hayes? Standard PCTALK?

If you are running "vanilla" hardware, such as an

IBM XT, DOS 2.1, a Hayes 1200 and a ribbon cable, you won't

have any problem. Those of you with less popular hardware

should be very careful.

Points to Check

MSDOS 2 or higher

A supported machine

A serial port

192K or more memory

A usable modem

A usable cable

Lots of diskettes

All the files you need

Fido's Complete Operating Manual         Page INSTALL-5

Step 1: Setting Up Message and File Areas

The first step is to figure out what you want your

system to look like. This depends on your needs, your

hardware (mainly, how much disk space do you have) and the

needs of your users.

It is not possible to cover all the combinations

that Fido can be configured in, and the one you end up with

will probably change as your needs change. Therefore, this

manual will cover a single simple configuration; others are

set up in the same way, and once it is understood, is fairly

straightforward.

Fido's Complete Operating Manual         Page INSTALL-6

Step 1: Setting Up Message and File Areas

If you follow this installation guide exactly, you

will end up with a Fido Bulletin Board system that has the

following characteristics:

- A public access system

- Two independent Message Areas

- Two independent File Areas

- An additional, special Message Area where users

can find special messages from the system

operator.

- Ready to be used with a DC Hayes SmartModem 1200

Any and all of this can be modified, either now (if

you are confident) or later, after you get this example one

running. While this manual is arranged in the order covered

in the introduction, you may actually perform any steps in

any order; however, this order has been found, by trial and

error, to be the simplest and easiest to follow.YOu can of

course do anything you want!

Fido's Complete Operating Manual         Page INSTALL-7

Step 2: The Environment

You will note that Fido uses MSDOS subdirectories in

huge quantities. If it's a subject you aren't comfortable

with, please learn to use them before tackling Fido.

Subdirectories aren't as hard as Microsoft makes you think

with their terrible manuals.

HINT:

All of the Fido documentation assumes you have a

working knowledge of MSDOS paths. If you don't understand

what they do and how they work, you probably won't be able

to get through this manual, nor be able to maintain your

system if you do. Paths are just not that hard, and are very

handy even if you aren't intersted in Fido.

There isn't really any difference, as far as Fido is

concerned, between hard disks and floppy disks. However,

Fido loves to consume disk space, and floppies aren't that

generous. Before we start, some general notes on hard disks

and floppies.

HINT:

If you are new to subdirectories and all that, you

can make your life easier if oyu set your system prompt to

tell you where you are:

PROMPT ($P) $S

Execute this manually, or put it in AUTOEXEC.BAT.

Installation on a Hard Disk System

The easiest thing to do is start at the root, and

create a new subdirectory just for Fido:

CD \                            log into the root

MD FIDO                         make a FIDO directory

CD FIDO                         enter the FIDO directory

Do the above, and you will be in the newly created

subdirectory. You can then ignore ALL drive letters shown in

the examples below. (B:, etc)

Installation on Floppy Systems

You are going to need all the space you can get. You

should format two new diskettes, and make one bootable

(format it with /S, to install the system on it). This will

become your A: diskette. On it, put the minimum tools you

will need: some sort of editor or word processor (for making

Fido's Complete Operating Manual         Page INSTALL-8

Step 2: The Environment

welcome messages, etc) and the fido .EXE files. Put the

other formatted diskette in B:. YOu may have to do a lot of

disk swapping to get everything created right.

If your drives aren't called A: and B:, then just

substitute the letters that you want.

For all systems, you should set the number of

BUFFERS in CONFIG.SYS on the booted diskette. The

performance increase will be enormous. If you do NOT do

this, then your Fido system will run very slowly, and annoy

all of your users, and generally drive you crazy. If you

know how to use CONFIG.SYS, then just do it. If not, then on

your boot disk do the following:

A:

CD \                            log into the root

COPY CON CONFIG.SYS             copy keyboard to CONFIG.SYS

BUFFERS = 50                    type this exactly,

FILES = 20                      this too

^Z                              Enter Control-Z, then return

Copied 1 file(s)                Message from MSDOS

Next time you boot, MSDOS will read CONFIG.SYS and

set the number of buffers to 50. (It will not do it until

then.)

Anyways ...

Before even thinking about running any programs, the

subdirectories and such need to be created. As mentioned

above, we will end up with two message areas and two file

areas.

Message areas:

GENERAL         Msg area #1

TECH            Msg area #2

File Areas:

IBMPC           File area #1

CPM80           File Area #2

Special Areas:

ANSWERS         Special Msg area

Please remember that the names of the subdirectories

are totally arbitrary; you can make them anything you want.

If the names reflect what's in them, though, it's much

easier a few months from now.

Fido's Complete Operating Manual         Page INSTALL-9

Step 2: The Environment

First, create these subdirectories using the MD or

MKDIR command:

B:              on drive B: ...

MD GENERAL

MD TECH

MD IBMPC

MD CPM80

MD ANSWERS

Now the subdirectories exist on your disk. These are

where Fido will put messages and files that can be accessed

by users. (Of course, there areen't any messages or files

yet, so don't worry about it.)

Now for a tedious part. Now you must log into each

area, and make a file called DIR.BBS. This is a very small

file, and contains a one line description of what the

subdirectory is for. (No, Fido cannot read; this small

description is merely displayed to a user when they log into

an area.)

HINT:

Make these good descriptions, as they are what the

user sees, and is all they have to tell what is in there.

Normally, Fido will display the actual subdirectory name; an

option lets you hide it, and display the DIR.BBS contents

only. This is the /Y command line switch.

makinbg DIR.BBS can be done with your word

processor, or more simply with the COPY command.

CD GENERAL                      enter the GENERAL subdir

COPY CON DIR.BBS                make DIR.BBS

General Interest Messages       one line descrption

^Z                              Control-Z, then ENTER

1 File(s) Copied                MSDOS message to you

CD \FIDO                        log into previous subdir

Repeat this for each area, entering the description

for each, instead of the "General Interest Messages" in the

example above.

At this point, you have made all the basic

subdirectories for Fido. Changes or additions can be made

later, following the same basic steps.

Fido's Complete Operating Manual         Page INSTALL-10

Step 3: Starting Fido

Now that the basic subdirectories have been made,

you must run Fido and tell it where to find them. There is

one last step before you do this; it is to make sure that

Fido has the minimum files it needs to run.

While they are not absolutely necessary to run Fido,

the HELP files are handy, since you probably havent

memorized all the Fido commands yet. All that needs to be

done is to copy them into the main Fido directory. While

logged into the main Fido directory, or the root for floppy

systems, put the Fido distribution diskette into B:, and:

COPY B:*.HLP

You also need as a minimum the initial user list,

USER.BBS, and the initial system file, SYSTEM.BBS. You will

also need the Fido program, of course. Copy these from the

distribution diskette, to the Fido subdirectory.

HINT:

If you can't figure out why you are having problems

with the user list, USER.BBS, or can't remember the

password, etc, refer to the SYSOP.EXE program in the Table

of Contents.

COPY B:USER.BBS

COPY B:SYSTEM.BBS

COPY B:FIDO_IBM.EXE     (or _DEC for DEC, etc)

Now, Fido has all it needs to be run. Fido has many

command line options, which are covered in a later section.

Here, you need only one:

/T              Test Mode

FIDO_IBM /T             (or _DEC for DEC, etc)

After a few seconds, you will see the signon

message, and a prompt for your name. You will NOT see any

nice welcome messages and so on; these are created later.

At the prompt for your name, fill in as follows:

Your FIRST Name: SYSOP <enter>

Your  LAST Name: <enter>

Wait ...

Password: WHAT <enter>

When you enter the password, it will be displayed as

dots, so sthat someone looking over your shoulder can't see

your password. After some messages from Fido, you will be at

the MAIN Command prompt.

Fido's Complete Operating Manual         Page INSTALL-11

Step 4: Setting Up Areas

There are a number of commands that do NOT show up

in the prompt. These are the sysop commands, and are the

digits 1 to 9. Only a user with SYSOP privileges may use

them. (The initial user list contains one user, named SYSOP,

whose privilege level is also SYSOP. That's what you entered

at the example above.)

The sysop-only commands are covered in detail in

elsewhere; however the minimum needed information will be

provided here.

HINT:

One point to remember about the numeric sysop only

commands; you can get help with most of them by entering:

n ?

Where n is the command number.

Fido keeps track of message and file subdirectories

by storing them in small lists kept in diskfiles named

SYSTEMx.BBS. Each list contains one Message Area

subdirectory, one File Area subdirectory, and one HELP

subdirectory (not used here). There can be up to 99 lists.

When a user chooses a message or file area, they are really

choosing one of these lists.

You must now store the names of the subdirectories

in these lists, so that Fido will know where to find them,

and so that users can select them.

The "1" command is used to maintain these lists. The

lists are numbered 0 to 99; you select one by:

1 <n>           Where <n> is a number 0 - 99

First we set area #1, using the subdirectory names

chosen before. (Only what you enter is shown here; Fido will

display the Main Command prompt and other messages also.)

1 ?             Get HELP!

1 1             Select list 1

1 B:\GENERAL\M  Set Message Area name

1 B:\IBMPC\F    Set File Download Area name

1 B:\IBMPC\U    Set File Upload Area

1 S             Save the changes

The first set of areas is now set. You can view your

changes by entering "1" by itself.

Now repeat this process for the second set of areas.

1 2             Select list 2

1 B:\TECH\M     Set Message Area

Fido's Complete Operating Manual         Page INSTALL-12

Step 4: Setting Up Areas

1 B:\CPM80\F    Set File Download Area

1 B:\CPM80\U    Set File Upload Area

1 S             Save changes

The third list will contain only the ANSWERS message

area, there is no corresponding File Area. This is set in

the same way as the other areas.

1 3             Select list 3

1 B:\ANSWERS\M  Set Message Area

1 F             Set BLANK File Area

1 U             Set BLANK Upload area

1 S             Save the changes

The last list to set is the "special" list, number

0. Only the Message Area is used from this list. This area

is where comments to the sysop go, using the G (GoodBye)

command. Usually (and in this example) this area will be the

same as one of the regular areas; ANSWERS in this instance.

This is so that the sysop can reply to messages left at

GoodBye, and users can read them by choosing the ANSWER

area.

1 0             Select list 0

1 B:\ANSWERS\M  Set message area

1 F             Blank areas, as above

1 U

1 S

HINT:

If you leave the message path for area #0 BLANK,

then Fido will not ask "Leave a message to the sysop? (y,n)"

at the G)oodbye command, but merely disconnect.

At this point Fido is more or less usable in Test

Mode (/T). You can now go into the various areas, enter

messages and operate most of the commands. There will not be

any files available yet, nor any messages. You should go

through each area and make sure it is set up as you think it

is.

Message Areas

From the MAIN Command prompt, use M to enter the

Message Section. The A)rea command displays the message

areas available, and lets you choose one. You should see

three areas, and the small descriptions you entered

previously. If not, something is wrong; go back over the

previous steps and check. Probably just an incorrectly

entered name. If all is well, pick one of the areas, for

example "1" for area #1, GENERAL.

HINT:

Use the E command to enter a sample message. After

saving it, it should say "Saving your message". If it does

Fido's Complete Operating Manual         Page INSTALL-13

Step 4: Setting Up Areas

not, but says "DISK FULL: Cannot save your message" it is

either that you disk is really full (not likely at this

point!) or the subdirectory name you entered is incorrect.

You must manually create, with the MSDOS MKDIR command, the

subdirectories where Fido expects to find them.

Return to the MAIN Command prompt, then enter the

Files Section, and check the areas there, using the A

command as before. You should see the area descriptions.

Note however, that there will not be any files for

downloading.

Fido's Complete Operating Manual         Page INSTALL-14

Completing Installation

The software portion of Fido/FidoNet is more or less

ready to be used. At this point, there are a number of

options. The sections you need are listed below; be sure to

browse through the others for ideas and tips.

ALL SYSTEMS:

Fido Message Files

How to customize the welcome messages, bulletins,

etc., in common with all Fido configurations.

Hints and Tips

Tips on speed, problems to watch out for, and ways

to improve overall performance. Some things you can do and

shouldnt do.

PICK ONE:

Public System

How to make a public access bulletin board, so that

new users can log in for themselves, and how to handle

special cases.

Semi-Public System

How to make a semi-private system, where new users

can log themselves in, but have restricted privileges.

Private System

Where new users are not allowed; the sysop has to

manually enter new users. How to keep security absolute.

ALL SYSTEMS:

Modem Installation

Installing and configuring the modem. Many modems

are listed as installed in actual installations. Tips on how

to fix problems.

AND THEN ...

FidoNet Installation

Setting up your Fido as a node in the public domain

electronic mail network, and what this means.

Fido's Complete Operating Manual         Page INSTALL-15

System Messages and Bulletins

There are a number of text files that Fido will

display at various points; welcome messages, the initial

signon, the bulletins and so forth. You will need to

customize them to your taste, or delete them if they areen't

needed. Fido does not require the presence of any of these,

though you should probably use most of them.

These are the ones in common with all Fido setups.

There is a section for each configuration: public access,

private, and semiprivate.

WELCOME1.BBS

This is the first text that a user sees, after

determining baud rate by hitting the CR or ENTER key. Most

of the public Fidos use the "doggie", already provided on

the distribution diskette.

WELCOME2.BBS

Presented after a user enters their name and

password. Just a chance to give logged in users news or

other information. Many systems run without WELCOME2.BBS.

BULLETIN.BBS

Usually where frequently changing information is

posted, for all user to see. This bulletin can also be read

with the B (Bulletin) command from the MAIN Command prompt.

EDTORIAL.BBS

Similar to BULLETIN.BBS, except Fido does not

display it automatically. It can be accessed by the E

command from the Main Section. (Note that the name is

missing the "i"; EDTORIAL not EDiTORIAL.

YELL.BBS

This file is displayed to the user if they attempt a

Y)ell command and yelling is not scheduled. You can use this

to inform them what the Y)ell schedules are. IF you do not

have this file, then Fido displays "Yelling at the sysop

isn't allowed right now.".

Fido's Complete Operating Manual         Page INSTALL-16

Making a Public System

When set up as in the previous sections, Fido is by

default a public access system. However, there are special

files that you can use to make life easier for you and your

users.

NEWUSER1.BBS

This is displayed once for new users, just before

Fido requests their city and state, and to choose a

password. It should probably explain the need for a

password, etc.

NEWUSER2.BBS

This is displayed right after a new user enters

their city, state, and password. Usually, a simple tutorial

of the system, what to expect, where to find things.

If a previously logged in user forgets their

password, after three attempts at entering a password, and

no match is found, then there are a few options.

The presence or absence of QNOPWD.BBS, a special

questionaire, determines how Fido treats new users.

OPTION 1: LEAVE A MESSAGE TO THE SYSOP

Make sure that you do not have a file named

QNOPWD.BBS; if it exists, then Option 2 will be used.

NOPWD.BBS is displayed, then they get to leave a

message to the sysop, identical to when a logged in user

uses the "G" command to leave a comment. After leaving a

comment, they are logged off. They have 10 minutes (or the

specified /S value) to complete the message.

OPTION 2: FILL OUT A QUESTIONAIRE

OPTION 3: MESSAGE THEN LOG OFF

For these two options, QNOPWD.BBS is used. This is a

special file, a questionnaire. Briefly, a questionaire

causes Fido to ask the user questions contained in it, and

records the answers (in this case) in ANOPWD.BBS. If used,

you should look at this file daily to check for new users.

You can delete it whenever necessary, Fido will create a new

one.

Alternatively, QNOPWD.BBS can contain only text to

be displayed, and no questions at all. Usually, this is the

easiest thing to do; just tell them to log on again, but use

a new name and password.

Fido's Complete Operating Manual         Page INSTALL-17

Making a Public System

If QNOPWD.BBS exists, the user gets to fill it out

(or not) and is then logged off; they do not get to leave a

message to the sysop as in Option 1.

Fido's Complete Operating Manual         Page INSTALL-18

Semi-Private (Restricted) Public System

If you want to run a public board, but restrict new

users, this can be easily done.

COMMAND LINE SWITCH:

/P      New Users get DISGRACE privilege

When new users log on, they are assigned DISGRACE

privilege. Then, by using the "3" command to set the

privilege level for the Fido commands, you can restrict the

commands available to new users. For instance, they can read

messages, but not enter any, not be able to change their

name or password, access the file area or the message area,

or any other Fido command.

If you use this method, then new users have to be

upgraded manually, using the SYSOP utility program.

Other than the new users privilege level, the is

identical to a public system.

You can accomplish the same thing by leaving new

users' privilege as NORMAL, and useing the 3 command to

change the privilege levels of the commands themselves to

limit thier use.

Fido's Complete Operating Manual         Page INSTALL-19

Making Fido a Private System

After following the previous steps to get Fido up as

a public system, follow these to make it a private system.

A private system is one where only already existing

callers can log on; new callers are prevented from entering

the system. You can let new callers leave a message to the

sysop, fill out a questionaire, or merely log them off, with

or without a messaeg as to why.

When running a private system, callers must be

entered manually, using the SYSOP utility program. Operation

of SYSOP is covered elsewhere.

The presence or absence of QNOPWD.BBS, a special

questionaire, determines how Fido treats new users.

COMMAND LINE SWITCH:

/M              Private System

Add this switch to the command line in RUNBBS.BAT.

It prevents new users from being added to the user list.

OPTION 1: NEW USERS CAN LEAVE A MESSAGE TO THE SYSOP

Make sure that you do not have a file named

QNOPWD.BBS; if it exists, then Option 2 will be used.

This is one way in which you can make new users

request entry to your system; NOPWD.BBS is displayed, then

they get to leave a message to the sysop, identical to when

a logged in user uses the "G" command to leave a comment.

After leaving a comment, they are logged off. They have 10

minutes (or the specified /S value) to complete the message.

NOPWD.BBS is just a text file, that is displayed to

new users, or users that forgot their password. In it, you

can explain that this is a private system, and request that

they enter whatever information you require.

OPTION 2: NEW USERS FILL OUT A QUESTIONAIRE

OPTION 3: NEW USERS GET LOGGED OFF IMMEDIATELY

For these two options, QNOPWD.BBS is used. This is a

special file, a questionnaire. Briefly, a questionaire

causes Fido to ask the user questions contained in it, and

records the answers (in this case) in ANOPWD.BBS. You should

look at this file daily to check for new users. You can

delete it whenever necessary, Fido will create a new one.

Alternatively, QNOPWD.BBS can contain only text to

be displayed, and no questions at all. This is frequently

Fido's Complete Operating Manual         Page INSTALL-20

Making Fido a Private System

used to tell users that the system is totally private, and

access to it cannot be had.

If QNOPWD.BBS exists, the user gets to fill it out

(or not) and is then logged off; they do not get to leave a

message to the sysop as in Option 1.

Fido's Complete Operating Manual         Page INSTALL-21

Changing Command Privilege Levels

To further customize Fido, you can change the

privilege level needed to operate each of the commands. This

is done with the 3 command (covered in detail in Fido's

Operating Manual).

Entering 3 by itself lists the current settings.

Each command area has it's own privilege tables:

Main Section,           MAINPRIV.BBS

Message Sections,       MSGPRIV.BBS

Mail Sections,          MAILPRIV.BBS

File Sections,          FILEPRIV.BBS

Read Command Prompt     READPRIV.BBS

CHANGE Command Prompt   CHGPRIV.BBS

For example, you can change the Raw-Display command

in the file areas to be NORMAL, and let everyone use it, or

you can make the EnterMessage command in the Message areas

privil, so that new callers (first time callers) cannot

enter messages until you up their privilege level with the

SYSOP.EXE program. (They can always leave Messages to the

Sysop at G)oodbye.)

Briefly, to change a command privilege, enter:

3 <x> <priv.>

Where:

<x>     Is the command letter, as entered normally

to Fido. (E for E)nter message, K for K)ill, etc.)

<priv.> Is one of the Fido privilege levels below.

You MUST spell it out fully.

TWIT            Lowest

DISGRACE

NORMAL

PRIVEL

EXTRA

SYSOP           Highest

Fido's Complete Operating Manual         Page INSTALL-22

Hints and Tips

There are a number of things you can do that make

Fido faster. If you have enough memory for a memory disk,

put the messages in it. This will speed things up by a

factor of ten or so. Dont worry about the .BBS files or .HLP

files; they are small and infrequently accessed. The

downloadable files are generally too large, and again,

areent accessed in a way that they speed up from a memory

disk.

Also, you should use a large number of FILES and

BUFFERS in CONFIG.SYS. This is probably the limiting speed

factor in most systems without memory disks. Use the maximum

FILES = 20, (each "file" takes only 36 bytes, so dont worry

about that) and lots of buffers. The following is an

example:

FILES = 20

BUFFERS = 50

More than 70 buffers or so starts to actually slow

the system down, as DOS spends all of its time fiddling

around with the buffers instead of the data inside them.

If you run out of memory, then lower the number of

buffers. Buffers are 512 (1/2 K) bytes each on IBM PC type

machines.

HINT:

MSDOS is not too smart about checking for enough

memory when you use BUFFERS = . The actual buffers arent

allocated until they are needed; if at that time there isnt

enough memory, then you get the message:

"Insufficient Memory"

And the system halts, making you reboot. Make sure

you have enough memory. 40 to 50 buffers is plenty, and will

work fine with 256K of memory.

Use GOOD diskettes. If you get a disk error, the

user will just see the system hung, and the "... error"

"Retry Ignore Abort" message on the local console. Not good.

Initially, use the RUNBBS.BAT batch file from the

distribution set of files. Modify it to include any switches

needed. Later on, you can include other things for system

maintenance, using the scheduler. See the scheduler section

for that.

When Fido terminates to DOS, it returns an ERRORLEVEL,

that can be detected from within RUNBBS.BAT. Some of the

ERRORLEVEL values are built into Fido; others can be set by

Fido's Complete Operating Manual         Page INSTALL-23

Hints and Tips

the sysop. The fixed ones are:

0         No errors or special conditions

1         Control-C from console or stack overflow

2         Fido Disk File error

3         Other fatal errors

If there is some fatal error, you'd probably like to

see it, and in any case there's not any sense in running

again. Also, you have to type Control-C on the local console

to terminate Fido (for maintainance, etc) and this causes it

to terminate the batch file.

The Fido software is not prone to crashing. Most

troubles will be hardware ones, or power failures and

brownouts. The system cannot be crashed by callers hanging

up at random points, and there is no way in which debuggers

can be run, etc.

WARNING:

Beware of other programs such as NEWKEY, pop up

alarm clocks, SIDE KICK, automatic screen blank programs,

and anything like them. None of them are compatible with any

program that makes extensive use of interrupts, such as

Fido.

Most of the programs do illegal things with the

hardware, and will bomb Fido, and other systems. You may

never have problems; probably you will, but it may take

hours or days to show up. One symptom is that the system

just "stops". If in doubt, start the system up again; if it

happens again, reboot again, this time leaving out any such

program, and see if that cures it. It may take several days

to discover.

It is recommended that you do not use such programs

while running Fido.

Fido is very friendly to multitasking programs such

as Multilink, and has a command line option, /I, to

acocmodate them. Please refer to the Command Line Switch

section for details.

Fido's Complete Operating Manual         Page INSTALL-24

Files Section Organization

There are a number of ways you can run your Files

Section, if you have one at all. You can of course combine

the features any way you want, but here are some common

ones:

OPTION 1: FREE ACCESS UPLOAD AND DOWNLOAD

Users can upload and download files at will. New

files are uploaded into the same area and added to the files

list, so that they are available immediately.

OPTION 2: FREE ACCESS DOWNLOAD, UPLOAD BY SUBMISSION

Users can download any file, but newly uploaded

files cannot be downloaded until the sysop allows it. There

are two ways to do this.

These are accomplished with three things:

File Download Path      In the list of areas

File Upload Path        In the list of areas

FILES.BBS               The files list

The upload and download paths are accessed by the

"1" command, as mentioned before. FILES.BBS is a special

file in each file area, that contains the list of available

files.

Before describing the Options above, here is some

information:

FILES.BBS is the list in each Files area of the

files available for downloading. You must create the first

one, but it can be automatically updated in a number of ways

later.

FILES.BBS is basically just a text file, that

contains the names and a description. Fido puts in the file

size, and aligns the columns neatly. For instance:

(Sample Files display)

AFILE.EXE      4545 Just a sample file.

PROG.BIN  323432434 A very large file

FOO.FIZ     MISSING Another sample

(The actual FILES.BBS contents)

AFILE.EXE Just a sample file.

PROG.BIN A very large file

FOO.FIZ Another sample.

You must enter the names as shown; the DOS DIR

format (FILE     EXT) will not work, it must be (FILE.EXT).

Fido's Complete Operating Manual         Page INSTALL-25

Files Section Organization

Note that if a file is really missing (not in the directory)

the file size will say MISSING instead.

There are special lines within FILES.BBS, that

control how it is listed by the various commands. The

special lines are determined by the first character. They

are:

- This is a comment only line

So is this (starts with a space)

@ This is the "end" of the files list.

The first two are merely comments; you can use them

for whatever you wish. The third is a special marker, and

tells Fido to not list anything after this line, including

this line.

When a file is uploaded, Fido asks for a one line

description for that file. The filename and description is

added to the END of the files list, above.

If there is a line beginning with @, then the new

files are added after it. Since nothing is listed beyond

this point, these files are not listed, and are esentially

unavailable until the sysop edits FILES.BBS and deletes the

line containing the @, or moves it past the last file in the

list. This lets the sysop check out uploaded files before

making them available.

Uploading and Downloading

As mentioned before, Fido has an upload and a

download path. The F)iles command lists the files in the

download path, using the FILES.BBS list there.

Uploaded files go into the upload path, and their

names and descriptions are added to the FILES.BBS list in

the upload path.

If the upload and download paths are the SAME, then

Fido will not let an uploaded file replace (or destroy ...)

an already existing downloadable file. This prevents good

files from accidentally being destroyed or changed.

If the upload and download paths are DIFFERENT, then

Fido will let existing files be overwritten. Usually this

means there is a seperate upload area, not accessible to

users.

You can use the above characteristics to your

advantage. Instead of trying to describe them all, here are

three examples.

OPTION 1: FREE ACCESS UPLOAD AND DOWNLOAD

Fido's Complete Operating Manual         Page INSTALL-26

Files Section Organization

Users can upload and download files at will. New

files are uploaded into the same area and added to the files

list, so that they are available immediately.

Make the upload and download paths the same. New

files go into the download area. Make sure there is no @

line in FILES.BBS; when a new file is uploaded, it's name

and description is added to the end of the files list, where

it is immediately available for downloading.

OPTION 2: FREE ACCESS DOWNLOAD, UPLOAD BY SUBMISSION

Users can download any file, but newly uploaded

files cannot be downloaded until the sysop allows it. There

are two ways to do this.

VARIATION A:

As in Option 1, make the paths the same, but put a

line beginning with @ in FILES.BBS. When a file is uploaded,

it is added to the download directory, but it's name and

description will not be displayed with the "F" command.

This is the easiest to maintain way to do things; to

make new files available, all you have to do is delete the

line beginning with @, or move it to the end of the list.

The disadvantage is that the file really is

accessible; it just does not show. If a user knows it's

there, it can be downloaded.

VARIATION B:

Make the upload and download paths different, where

the upload path is a disk or directory not available as a

File Area. It doesnt matter if there is an @ line in

FILES.BBS.

Since new files are uploaded into a totally seperate

area, they cannot be accessed by users. (You can put the

path in a separate file area, with SYSOP privilege level, so

that you can get in there to look.)

The disadvantage is that in order to make these

files available, you must use the DOS COPY command, and

update the FILES.BBS in the download area manually.

This is the most secure method, is is reccomended

only you require absolute security on uploaded files.

Fido's Complete Operating Manual         Page INSTALL-27

Command Line Switches and Setting Time Limits

There are many command line options that control

Fido operation. You will have to edit these into the

batchfile that runs Fido. The switches are:

<n>/J   Set Modem Type

Selects the type of modem you are using. <n> is a number

that specifies the modem type:

1/J     Hayes

2/J     DEC DF03

3/J     Racal Vadic VA212

4/J     Novation/ IBM PC Junior

5/J     US Robotics

7/J     Prentice POPCOM

8/J     IBM PC Junior

9/J     Ventel MD212 with WECO option

10/J    GDC modem

11/J    Hayes 2400

12/J    US Robotics Courier

See the section on modems for details.

/Y      Don't display pathnames

Causes Fido to NOT display the subdirectory name

when listing file ans message areas with the A)rea command.

Normally, Fido displays the pathname as set with the "1"

command, followed by the contents of the DIR.BBS file.

Adding /Y to the command line causes Fido to display the

DIR.BBS contents only.

<task>/I        Set multi-task environment

This does only two simple things: it changes the

names used for the log files, and affects the way messages

are created.

The names for the logs will incorporate the task ID

number; ie. if you specify 9/I the log files will be:

SYSOP9.LOG

MAILER9.LOG

You cannot use 0 as a number. The other effect of

setting a task ID is to force Fido to recount the messages

before saving a new message. This hopefully prevents

duplicate message numbers making a mess of things, or at

least minimizes it. It has the unfortunate side effect of

greatly slowing down saving messages.

Fido's Complete Operating Manual         Page INSTALL-28

Command Line Switches and Setting Time Limits

If you use /I in test mode, you will not be able to

see anything. Do not use /I with /T.

/T      Test Mode

Test mode. Described early in this manual, not used

for a remote Fido system. Not a good thing to put into the

batch file.

/M      Private System

Private message system. Please read the chapter on

private systems.

/P      Low Default User Privilege

Set the default privilege level for new users to

DISGRACE. See the chapter on semiprivate systems for

details.

(number)/S      Log On Time Limit

Set the time limit for logging on. This limit starts

as soon as carrier is detected, and stops after a valid

password is entered. At that point, the normal user or first

time user time limit starts. The minimum is 5 minutes; the

default is 10.

(number)/L      Normal User Time Limit

Set the time limit per call to (number), where

(number) is the number of minutes. 0 means unlimited use,

i.e. no time limit. Must be 0 - 65535. Read a later section

on time limits. If not specified, then the limit defaults to

60 minutes.

(number)/F      First Time Users Time Limit

Set the time limit for new users. The default here

is 60 minutes also. You can give first time callers a low

limit.

(number)/D      Daily Time Limit

Set the total time limit for a 24 hour period. This

is usually 4 or 5 times the /L limit. This is the maximum

number of minutes a caller is allowed in a 24 hour period,

and prevents long term hogging, and keeps them from hanging

up and calling right back when the per-connect limit is

Fido's Complete Operating Manual         Page INSTALL-29

Command Line Switches and Setting Time Limits

reached. The default is 300 minutes.

(number)/K      K byte Download Limit

Limit the number of K bytes a caller can download in

a 24 hr. period. Along with /F, /L, and /D, allows full

control over call-in-once-download-everything

and-never-call-back type users.

<number>/V      Carrier Detect Mask Bit

Define the bit used to determine the state of the

Carrier Detect signal from the modem. The default is CTS, or

16. For most machines, use 128/V. See the modem section in

this manual for details.

/U      No File Transfer Display

Disables the file transfer information displayed

when a file upload or download is happening. Normally, Fido

will display status messages about each block being

transferred, and all errors that happen. This is very useful

when a caller is having trouble. Enter /U to disable this

display.

<n>/G     Set FidoNet Attempts with Connect

Sets the number of attempts with connect for

FidoNet. The default is one. This is the number of times

that Fido/FidoNet will call a particular number, if the

modems connect but no mail is transferred.

/1      Set COM1

/2      Set COM2

Select which physical IO device to use. The default

is /1, COM1 on the IBM PC. /2 Selects COM2. Not used on

machines other than the IBM PC, unless otherwise noted.

These switches are entered thusly:

Fido 90/l 30/f 200/k 180/d

90 minute time limit per call (normal users)

30 minute time limit (first time callers)

200K bytes/48 hrs download limit

180 minutes time limit per user per 48 hrs

Fido's Complete Operating Manual         Page INSTALL-30

HELP Files

Fido has a number of HELP files, that are displayed

by entering "?" at almost any point within Fido. There is a

help file for each place ? can be entered. The HELP path is

used to find the file needed. The standard Fido setup is a

single subdirectory called HELP, which contains all the help

files.

All the help files have the extention .HLP without

exception. These are:

1.HLP           Sysop 1 command

2.HLP           Sysop 2 command

3.HLP           Sysop 3 command

5.HLP           Sysop 5 command

6.HLP           Sysop 6 command

8.HLP           Sysop 8 command

MAIN.HLP        Main Section command prompt

C.HLP           Within the C command, Main Section

MSG.HLP         Message Section command prompt

MQUERY.HLP      Within the R command, Message, Mail Section

ENTERCMD.HLP    Within the E command, Message Section

MAIL.HLP        FidoNet Mail Area command prompt

FILES.HLP       File Section command prompt

XFERTYPE.HLP    Within the D and U commands, File Section

NODES.HLP       At the "Fido to send to" prompt,

entering a FidoNet message.

You can customize these if necessary, but at least

initially don't worry about them.

Fido's Complete Operating Manual         Page INSTALL-31

Installing the Modem

The modem should be installed ONLY after the

software is ready for it. There is no need for a modem while

configuring the software.

In order to test Fido and the modem, you will need a

second computer and modem. This may be difficult, but is the

only way by which to test the modem. Any modem and computer

or terminal may be used; all you require is some sort of

communications program to call your Fido with.

For IBM PC versions, you can use a command line

switch to select either COM1 or COM2. The default is COM1.

If you need to use COM2, add to your command line in

RUNBBS.BAT:

/2      COM2

If you remember to set the modem type (/J) right,

Fido will initialize the modem for you; you need not set any

switches, except as mentioned below. If you have a modem

that is not listed, then you are on your own; if it is a

Hayes compatible, by all means try that.

Please check that you modem is not on the list of

problem modems, further below.

It is entirely possible you will have to modify your

modem cable or replace it; even if you have a modem/comm/

program combination that works, it may not have all the

lines needed to operate a bulletin board. The extra lines

are Carrier Detect (CD) and Data Terminal Ready (DTR). These

two are not normally needed by most communications programs;

they are however needed by Bulletin Boards, since they need

to be able to detect incoming calls and to force them to

disconnect.

USING A STANDARD CABLE

You can change the RS232 pin that Fido uses to

determine the state of the Carrier Detect signal. This is

done with the /V switch; see the section on Command Line

Switches, and the section on Specific Computers, next. No /V

switch at all is the same as entering:

16/V      default

Which is NOT what is needed by most machines. Refer

to the Specific Computers section for /V values. Legal

values for /V are: 1, 2, 4, 8, 16, 32, 64, 128.

Testing the Modem

When Fido ir first run, after initializing itself,

it will say:

Fido's Complete Operating Manual         Page INSTALL-32

Installing the Modem

- Waiting for a Call or Event

This means that Fido finished initializing the

modem, there is no incoming call, and that it is aiting for

an incoming call. This is the normal standby state of Fido.

When a call comes in, the modem (not Fido) answers

the call. (You can see the modem light the OH, Offhook,

lamp.) When the modem has completed the connection process,

it informs Fido of this with the Carrier Detect (CD) signal

(The modem lights the Carrier Detect (CD) lamp.) The /V

value is a bit mask used to isolate the CD signal from the

others.

When Carrier Detect is true, signaling an incoming

call, Fido displays:

- Incoming call

- Determining baud rate

At this point, Fido is waiting for the caller to hit

CR and/or space bar to determine the baud rate.

When the caller disconnects, the modem drops it's CD

signal, and Fido lowers Data Terminal Ready (DTR)

momentarily, raises it again, and goes back to the original

display:

- Waiting for a call or event

You must get Fido to perform this exact sequence of

events. If it does not, please refer to the Modem Problem

Determination section.

Custom Cabling

The idea here is to connect the cable such that you

can not only communicate to the modem, but also get to the

Carrier Detect and Data Terminal Ready signals to and from

it. If you already have MINITEL, PC-TALK, etc working with

the modem, thats a good start, but not enough. The Carrier

Detect line is not needed by programs such as MINITEL and

PC-TALK; Fido needs it to tell when a caller has made a

connection, or when they have hung up. Just because PC-TALK

or MINITEL work with the modem doesnt mean that Fido will.

The following are the pins that must be wired to an

IBM Async Card. You will have to determine how to connect

other computers using this as an example.

IBM PC          SMARTMODEM      USE

1               1               Ground,

2               2               Data, modem to PC

Fido's Complete Operating Manual         Page INSTALL-33

Installing the Modem

3               3               Data, PC to modem,

6               6               Carrier Detect

20              20              Data Terminal Ready

The other pins are not needed by Fido, though may be

needed by other comm. programs, and in any case can be left

in.

Carrier Detect: CD

CD is generated by the modem, and tells the computer

(Fido) that a call came in, and a connection made.

Conversly, Fido can tell when a caller hangs up or loses

connection, and knows to log that user off.

Data Terminal Ready: DTR

Fido uses this to disconnect users that have gone

over their limit, and to prevent incoming calls while Fido

is performing internal processing that may take a few

minutes.

DTR is also of extreme importance for FidoNet for

placing outgoing calls. Fido will not run properly if DTR is

not connected and working properly.

Fido's Complete Operating Manual         Page INSTALL-34

Commanding the modem: FIDOMDM.BBS

FIDOMDM.BBS is a special, optional, file that Fido

uses to command the modem. If it does not exist, then it is

not used. You need to use this file only if you have special

requirements. Fido will automatically initialize the modem

for you.

If FIDOMDM.BBS exists, the contents of it are output

line by line to the modem. The contents of the file are not

checked in any way; it is sent blindly.

After sending one line, Fido will wait up to three

seconds for and responses from the modem; these responses,

if any, are thrown away. The purpose is to avoid sending

commands to fast. If there is no response, Fido waits three

seconds and goes on to the next line.

FIDOMDM.BBS is processed after Fido initializes the

modem itself; this lets you override or modify the default

settings. For example, if you wanted to leave the

loudspeaker on, and have the modem answer on the second ring

instead of the first, for a Hayes type modem use:

ATS0=2

ATM1

Fido's Complete Operating Manual         Page INSTALL-35

Specific Computers and Modems

These are some notes on specific installations. If

your machine combination is not here, you will ahve to play

with the various settings of /V, /J and FIDOMDM.BBS

mentioned earlier.

Fido has only been tested on the machines listed

inside the front cover; it may or may not work on other

machines.

IBM Jr, PC, XT, AT and ALL CLONES

Use 128/V for all modem types. A 1 to 1 cable will

work with all modems.

DEC RAINBOW: ALL MODELS

SANYO 555

USe 16/V for all most types. I have very little

personal experience with these machines.

VICTOR 9000

Use 8/V.

OTRONA ATTACHE

Use 16/V. You will need to use the special Attache

MODEM cable; the PRINTER cable will not work without

modifications.

Fido's Complete Operating Manual         Page INSTALL-36

Specific Modems

The /J values are listed below. Please check for

notes on certain modem types.

1/J     Hayes

2/J     DEC DF03

3/J     Racal Vadic VA212

4/J     Novation/ IBM PC Junior

5/J     US Robotics

7/J     Prentice POPCOM

8/J     IBM PC Junior

9/J     Ventel MD212 with WECO option

10/J    GDC modem

11/J    Hayes 2400

12/J    US Robotics Courier

HINT:

The following is opinion only; it is not "fact". I

have had the chance to play with many modems while working

with Fido, so I'd thought I'd share some opinions.

This is the only plug you will find anywhere in this

manual for specific hardware. The differences in quality

between modems is great, and not always apparent to the

buyer. "Hayes compatibility" is a misnomer, there are very

few that truly are. Even some of Hayes' own products are not

"Hayes compatible".

For a 1200/300 baud modems, the Hayes is still by

far the best. It is expensive, but you cannot possibly go

wrong; it performs flawlessly, there is obviously lots of

software for it, and they are plentiful. Some of the clones

I have worked with personally, or have suffecient second

hand information to rely on.

Hayes clones: Prentice POPCOM, General Data Corp

modems are fine modems, though they are "supersets" of the

Hayes; you need to set further commands to make them

compatible. Once done, they seem to be fine. The US Robotics

212A and Password are fairly good; they have some bugs/side

effects problems, but Fido has software to compensate for

them. Fairly minor bugs, somewhat of an annoyance for BBS

use, but it is very inexpensive.

All Racal Vadic modems seem to be quite good. Only

one is compatible with Fido. They tend to have very wordy

"menus", which are nice for use with dumb terminals but

terrible for use with computer programs.

The DEC DF03 is a terrible modem, mostly because it

is a very ancient design. It dials only pulse. It is not a

smart modem.

Ventel seems to be a popular modem with the

"minicomputer" crowd, but it does not provide a Carrier

Detect signal, making it about as useful as a telephone

Fido's Complete Operating Manual         Page INSTALL-37

Specific Modems

without a ringer.

For 2400 baud, at this writing there seems to be

mainly Hayes, US Robotics, and Multitech. I have not used

the Multitech, but it is supposedly a good modem.

I do have both a Hayes and the US Robotics Courier;

the USR unit is *far* superior to the Hayes. The Hayes is

not "Hayes compatible", the USR is. The USR outsurpasses the

Hayes 2400 in all aspects. Except the cabinet is flimsy

plastic, oh well. I more or less reviewed the USR in

FidoNews (FidoNet Users' Newsletter) and will not repeat it

here. No contest, as far as I'm concerned; the supposed

handy features of the Hayes 2400 turn out to be useless

frills.

Please keep in mind that these are merely my

opinions (of which I have more than my share) and not

recommendations. If you are dissatisfied it is your own

fault.

Hayes SmartModem 1200

Modem Type:  1/J

Switch Settings (External Modem)

1 Up       Support DTR Lead

2 Down     Digit Result Codes

3 Down     Result codes sent

5 Up       Autoanswer first ring

6 Up       Supports Carrier Detect lead

8 Down     Recognize commands

Others as needed

Anchor Automation Signalman

Fido will not work with the Anchor Automation, as it

does not support the DTR control line. Fido cannot cause a

caller to hang up, nor can it disconnect during FidoNet mail

transfers. Since FidoNet would be unable to disconnect after

placing a call, with a remote possibility of an extremely

high phone bill, the decision has been made not to support

this modem.

Bizcomp Modem

This so called "Hayes compatible" is one of the

poorest modems ever made; it is barely compatible, and

unfortunately, cannot be used for Fido, nor many other comm.

type programs.

It will work OK at 1200 baud, but not at 300. For

some unknown reason, the modem transmits all characters at

1200 baud, even when connected at 300. While it sounds like

it simplifies things, by not having to switch baud rates, it

Fido's Complete Operating Manual         Page INSTALL-38

Specific Modems

also means that you must "throttle" data sent to it; you

cannot just send data at 1200 baud to the modem, when it is

leaving the modem at only 300 baud.

This has so many basic flaws that no attempt will be

made at making Fido accomodate it.

Qubie 1200 Modem

Another "Hayes compatible"that does not work. This

has many serious but subtle problems. The Carrier Detect

signal is "faked", and does not work. Other problems were

reported.  It probably will not work with Fido, unless Basic

Time revises the firmware.

US Robotics Modem

Modem Type: 5/J

Switch Settings:

RNG: Off

8HI: On

BAL: Off

DTR: On

The 212A and Password have a strange side effect: if

a partial command, or just garbage, is sent to the modem, it

will not answer the phone. The modem expects a CR after each

line, and will sit and wait until it gets one.

Fido fixes this by issueing commands to clear the

modem, hence it is not a problem. Make sure you select 5/J.

Ventel MD212A

All of the Ventel modems do not normally supply

Carrier Detect; this makes them absolutely useless for

Bulletin Board use. The MD212A, however, has an optional ROM

called "WECO" (dont ask me) that supplies the missing

signal. You will need to contact the factory for this

option.

Fido's Complete Operating Manual         Page INSTALL-39

Modem Problem Determination

SYMPTOM:

Fido says "Waiting for a call or event" but will not

answer the phone.

The modem is not in autoanswer mode. Did you use the

right /J setting? Is the modem on? Plugged in?

SYMPTOM:

Fido says "Waiting for a call or event", immediately

followed by "Incoming call, determining baud rate" when

there is no incoming call.

You have the modem installed incorrectly. The

problem is that Fido thinks that there is Carrier Detected

when there isnt. There are a number of possibilities.

You may have the /V value wrong. If the one

recommended one does not work, try the others. There are

eight possible values; 1,2,4,8,16,32,64,128.

You may have switch settings wrong: you must set any

switches needed to make the Carrier Detect and Data Terminal

Ready signals active. Fido cannot do this.

You may have a cable that does not have all the

signals needed; replace the cable.

SYMPTOM:

After a caller logs off, Fido rings the bell

continuously, displaying the message "CANNOT DISCONNECT THE

MODEM!".

Fido requires that it be able to cause a disconnect

by using the DTR line. The Anchor Automation modem does not

support DTR, and therefore cannot be used with Fido.

For modems with options switches, it may be merely a

switch setting to enable the use of the DTR line.

This can also be caused by a modem cable that does

not have all the connections made.

If you see this message, you can type Control-C to

abort it. You should also make sure the modem is

disconnected until you can repair this situation.

Fido's Complete Operating Manual         Page INSTALL-40

Install Index

"1" command............................. 11

-....................................... 25

/1 Set COM1............................. 29

/2 Set COM2............................. 29

/D Daily Time Limit..................... 28

/F First Time Users Time Limit.......... 28

/G Set FidoNet Attempts with Connect.... 29

/I Set multi-task environment........... 27,23

/J...................................... 39,27

/K K byte Download Limit................ 29

/L Normal User Time Limit............... 28

/M Private System....................... 28,19

/P Low Default User Privilege........... 28,18

/S Log On Time Limit.................... 28

/T Test Mode............................ 28,10

/U No File Transfer Display............. 29

/V Carrier Detect Mask Bit.............. 29,31

/Y Don't display pathnames.............. 27

1 command............................... 30

1.HLP................................... 30

128/V................................... 29

2 command............................... 30

2.HLP................................... 30

3 command............................... 30,21,18

3.HLP................................... 30

5 command............................... 30

5.HLP................................... 30

6 command............................... 30

6.HLP................................... 30

8 command............................... 30

8.HLP................................... 30

@....................................... 25,26

ANSWERS message area.................... 12

Basic subdirectories.................... 10

Batchfile............................... 27

BUFFERS = 50............................ 8,22

BULLETIN.BBS............................ 15

Bulletins............................... 15

C.HLP................................... 30

Carrier Detect.......................... 29,39,31,32

CD...................................... 31

CHGPRIV.BBS............................. 21

COM1.................................... 29

Fido's Complete Operating Manual         Page INSTALL-41

Install Index

COM2.................................... 29

Command line options.................... 27

Command Line Switch..................... 23

CONFIG.SYS.............................. 22,8

Control-C............................... 23

Custom Cabling.......................... 32

Data Terminal Ready..................... 32,39

Digits 1 to 9........................... 11

DIR.BBS................................. 9,27

DISGRACE................................ 18,21

Disk File error......................... 23

DISK FULL............................... 12

Diskettes............................... 22

Distribution diskette................... 10

Download................................ 24,22

DTR..................................... 32,31

EDTORIAL.BBS............................ 15

ENTERCMD.HLP............................ 30

ERRORLEVEL.............................. 22

EXTRA................................... 21

Fatal errors............................ 23

File Areas.............................. 6

File transfer information............... 29

FILEPRIV.BBS............................ 21

FILES = 20.............................. 22

Files Section........................... 24

FILES.BBS............................... 25,24

FILES.HLP............................... 30

First time callers...................... 28

G)oodbye................................ 12

HELP files.............................. 30

HELP path............................... 30

HINT:................................... 11,7,22,12,36,12,10

HINT:................................... 7,9

Insufficient Memory..................... 22

Interrupts.............................. 23

MAIL.HLP................................ 30

MAILPRIV.BBS............................ 21

MAIN.HLP................................ 30

MAINPRIV.BBS............................ 21

Memory disk............................. 22

Message and file areas.................. 2

Message Areas........................... 6

MKDIR................................... 8

Modem Problem Determination............. 32

Modem type.............................. 27

MQUERY.HLP.............................. 30

MSG.HLP................................. 30

MSGPRIV.BBS............................. 21

Fido's Complete Operating Manual         Page INSTALL-42

Install Index

New users............................... 28

NEWKEY.................................. 23

NEWUSER1.BBS............................ 16

NEWUSER2.BBS............................ 16

No time limit........................... 28

NODES.HLP............................... 30

NOPWD.BBS............................... 16,19

NORMAL.................................. 21

Offhook................................. 32

Other programs.......................... 23

Out of memory........................... 22

Password:............................... 10

Private message system.................. 28

Private system.......................... 19

PRIVEL.................................. 21

Privilege level......................... 28

Public access system.................... 16

Public system........................... 18

QNOPWD.BBS.............................. 16,19

Questionnaire........................... 19,16

READPRIV.BBS............................ 21

RUNBBS.BAT.............................. 22,19

Scheduler............................... 22

Screen blank programs................... 23

Seperate upload area.................... 25

Set File Download Area.................. 11

Set File Upload Area.................... 11

Set Message Area........................ 11

SIDE KICK............................... 23

Signon message.......................... 10

Subdirectories.......................... 2

SYSOP................................... 21,11

SYSTEM.BBS.............................. 10

Test mode............................... 28,10

Time limit per call..................... 28,28

TWIT.................................... 21

Upload and download..................... 24

USER.BBS................................ 10

Welcome messages........................ 10,2,15

WELCOME1.BBS............................ 15

WELCOME2.BBS............................ 15

XFERTYPE.HLP............................ 30

Y)ell................................... 15

YELL.BBS................................ 15

Your FIRST Name:........................ 10

Fido's Complete Operating Manual         Page INSTALL-43

Install Index

Your LAST Name:......................... 10