ET4000/W32 Windows 3.1 Video Driver Installation Instructions

** IMPORTANT **

The drivers named "ET4-W32" in the Windows Setup use the ET4000/W32

accelerator features. The remaining drivers supplied are the same as

used on the ET4000/AX.

This driver disk is designed to allow you to select or change the video

driver for Windows 3.1 using Windows Setup. Setup can be executed from

within Windows, or from DOS. PLEASE NOTE THAT THE DRIVER FILES ARE IN

COMPRESSED FORMAT AND MUST BE DECOMPRESSED.  The decompressed drivers can

only be accessed and executed by installing through Windows Setup.  Simply

copying the files into your \Windows\System subdirectory WILL NOT work and

may corrupt your Windows installation. Installing the drivers correctly is,

however, a simple process. It is accomplished by following these instructions:

Installation from Hard Disk

1) Having the drivers loaded on the hard disk will make it easier and faster

to change resolutions in the future. Make a subdirectory to hold the

driver files.

example: C:>md \w32drv

and switch to that subdirectory (C:>cd \w32drv)

2) Insert the drivers disk into the appropriate drive and type the following

command:

example: A:\win31\w32win

The drivers will be decompressed onto the w32drv subdirectory on drive C:

3) From your Windows directory, type SETUP and select DISPLAY to change the

video driver. Or, with Windows running, select Setup from the Program

Manager Main group. Refer to your Microsoft Windows User's Guide for

assistance in running Setup from inside Windows.

4) Select "Other disk..." to direct the program to the subdirectory holding

the new drivers. Note that once the drivers are installed, you should not

have to select "Other" again should you decide to change resolutions. Look

for the ET4000-based drivers in the Display selection menu to select the

display resolution you want.

5) When asked where the drivers will be found, state the following (deleting

what the program has displayed in the input window):

[drive:]\[directory]

example: C:>\w32drv

6) A standard Windows selection menu will appear with additional video

drivers from which to select.

7) Proceed normally through the remainder of the process. Check Microsoft

Windows installation procedures if necessary.

8) If you are having problems installing these drivers please contact your

board vendor for further assistance.(as your vendor may have done proprietary

enhancements that require custom drivers)

Also, the included are  drivers for 32K, 65K and 16M simultaneous colors.

If you attempt to run Microsoft Windows with a driver for which your

display board cannot support, a brief message will appear in the

upper left of your screen

"video driver unable to set desired mode"

before returning you back to the DOS prompt. If this occurs simply, run the

SETUP program again and choose a supported display mode.