Installing VGA Adapter Drivers for GEM Versions 3.0-3.1

The following instructions are applicable to users of GEM Versions 3.0

and 3.1, and the VGA Adapter.

The enclosed GEM driver files require a new GEMSETUP.TXT file to be

installed in place of that which is on GEM System Master Disk. The

drivers make full use of the extended graphics modes of the VGA

Adapter. Your VGA monitor must be capable of displaying these

resolutions in order to utilize this driver.

Be sure that your VGA monitor can display

800x600 and/or 1024x768-pixel resolution.

Choosing a higher resolution than your

monitor is capable of displaying will bring

about unsatisfactory results.

Before installing GEM, perform the following steps:

1) Make a DISKCOPY of GEM System Master Disk. This copy

will be used in place of the original during

installation.

Example:      C:>DISKCOPY A: B:    or

C:>DISKCOPY A: A:

2) Copy the appropriate setup file, GEMSETUP.V31 for

Version 3.1 or GEMSETUP.V30 for Version 3.0, from

your VGA Adapter Driver directory to the newly-

created copy of the GEM System Master Disk, as

GEMSETUP.TXT.

Examples:  For Version 3.0,

C:>COPY C:\GEM&VP\GEMSETUP.V30  A:GEMSETUP.TXT

For Version 3.1,

C:>COPY C:\GEM&VP\GEMSETUP.V31  A:GEMSETUP.TXT

After the GEM System Master Disk is copied and prepared with

the new GEMSETUP.TXT file, you can proceed with the

installation.

The installation of VGA extended mode drivers is a 2-step

process.

First, install GEM as per instructions, selecting

IBM 16-Color VGA for PS/2 (640x480) or Compatible

as your video driver. Complete the GEM installation instructions

normally. After completing installation, it is a good idea to

start GEM to ensure that it operates properly before doing the

next step.

Next, assuming that GEM worked properly with the IBM VGA driver

loaded, reinsert your System Master Disk and run GEMPREP again.

This time, choose to change your configuration. You want to

change the video adapter part of the configuration in order to

load the VGA Adapter.

Select from the following:

VGA Adapter (800x600) 16-Color Mode

VGA Adapter (1024x768) 16-Color Mode