OVERVIEW

There is a wide variety of monitors in use today with many iterations

of ET4000-based video adapters being used with them. Because of this,

a software utility is necessary to make the two components work

together by setting horizontal and vertical synchronization

frequencies, as well as video system clock rates. The screen that you

see on your monitor actually has to be re-projected repeatedly in

order for you to see it clearly. The rate at which each line is

displayed horizontally (left to right) is the horizontal scan rate.

This is measured in kilohertz, a unit of measurement that means one

thousand cycles per second. Since it takes longer to project each line

across a display than top to bottom, this frequency must be much

higher than vertical frequency, which is one cycle per second.

Therefore, vertical frequency, or refresh rate (the period of time to

re-project the entire screen image from top to bottom) is a more

critical measurement as the entire screen being projected is

susceptible to exhibiting flicker. The VMODE utility allows users to

adjust the rate at which the screen is displayed both in horizontal

and vertical terms. Additionally, the video system clock rate, or the

oscillator (clock) which sets the basic timing rate capabilities for

the video synchronization subsystem, is also adjustable to attain

compatibility with various monitor and video adapter designs. Because

the variance in these components is so diverse, preset menus to choose

synchronization rates are extremely difficult to construct and be

reliable. VMODE, then, is a manual approach to garner the best

compatibility between monitors and video adapter designs across a very

wide range of components.

VMODE SCAN RATES

The VMODE utility is provided to aid in eliminating flicker and other

video distortion exhibited when monitors cannot automatically

synchronize to frequencies outputted by video cards. By issuing one or

more VMODE commands, many monitors can accommodate different

frequencies and greatly improve the display. VMODE commands can be

issued from the DOS prompt, batched in the AUTOEXEC.BAT file, or

incorporated into separate batch files for starting applications that

require settings prior to loading.

example (WINN.BAT)

VMODE 35K

WIN

Additional modes are available to provide additional scan rates that

may improve synchronization with a variety of monitors. Normally, the

default scan rates are effective, but some monitors may require

different scan rates for the most satisfactory display results. By

setting these modes, frequencies are adjusted that affect displayed

graphic modes. The following modes are available via the VMODE

command.

example: VMODE x

VMODE   Modes           Vertical        Horizontal

x Value Affected        Refresh Rate    Frequency       Resolution

35K     29,2A,30        56Hz            35KHz           800x600

38K     29,2A,30        60Hz            38KHz           800x600

48K     29,30           72.7Hz          48.4KHz         800x600

45M     37i,38i         86.5Hz          35.5KHz         1024x768 *

65M     37n,38n         60.5Hz          49KHz           1024x768 **

* interlaced            ** non-interlaced

72HZ Modes

The following graphic (not text) modes can be displayed after setting

the vertical refresh rate to 72HZ. The 72HZ mode will provide a

clearer display with less flicker on some monitors. Set the VMODE

parameter to VMODE 72HZ. After this is done you can use VMODE to set

the display to any of the graphic modes listed below.

VMODE   Modes           Vertical        Horizontal

x Value Affected        Refresh Rate    Frequency       Resolution

72HZ    11              72.70Hz         38.70KHz        640x480*

72HZ    12              72.70Hz         38.70KHz        640x480*

72HZ    25              72.70Hz         38.70KHz        640x480*

72HZ    **2E            72.70Hz         38.70KHz        640x480*

* monitor must be capable of 72.70Hz vertical and 38.70KHz horizontal

frequency. ** requires minimum of 512kb display memory;

45/65 MHZ Modes (45m, 65m)

The following graphic (not text) modes can be displayed after setting

the system video clock rate to 45 or 65MHz. These modes will provide a

clearer display with less flicker on some monitors. Set the VMODE

parameter to VMODE 45m or 65m. After this is done you can use VMODE to

set the display to any of the graphic modes listed below.

VMODE   Modes           Vertical        Horizontal

x Value Affected        Refresh Rate    Frequency       Resolution

45m     37i,38i         86.96           35.52           1024x768

65m     37n,38n         60.58           48.95           1024x768

72/75/80 MHZ Modes (72m, 75m, 80m)

The following graphic (not text) modes can be displayed after setting

the system video clock rate to 72MHz. The 72MHz mode will provide a

clearer display with less flicker on some monitors. Set the VMODE

parameter to VMODE 72m. After this is done you can use VMODE to set

the display to any of the graphic modes listed below.

VMODE   Modes           Vertical        Horizontal

x Value Affected        Refresh Rate    Frequency       Resolution

72m     37              69.80           56.30           1024x768

72m     38              69.80           56.30           1024x768

75m     38n             69.10           56.48           1024x768

80m     38n             72.00           57.80           1024x768

80m     3Di             87.02           48.00           1280x1024

NOTE:  Choose modes/resolutions that are compatible with your video

board configuration and monitor capabilities.

Typing VMODE ? will display the full array of mode possibilities.

Users that are familiar with the characteristics of their monitors

will find these modes more useful. See Appendix D for definitions of

available modes. Note that choosing VMODE 60HZ will return the adapter

to normal operation.