*** Release Notes for HIT-IDE.SYS Ver.1.06d ***

1. General

HIT-IDE.SYS is the device driver for DOS CD-ROM Extensions that supports

ATAPI CD-ROM drives.

2. Command line switches of 'HIT-IDE.SYS'

(1) SYNTAX

DEVICE = [x:][\dir\]HIT-IDE.SYS /D:(device name) [/N:(number of drives)]

[/T:(drive type)] [/S:(base I/O address)] [/V]

(2) Meaning of switches

/D:(device name)		Specifies "Device Name".

/N:(number of drives)	Specifies number of drives to be supported by

this driver.  Value 1 to 8 are valid.

If this switch is omitted, the driver will

search connected drives.

/T:(drive type)		Specifies the first drive type to begin to

support.

Definition of drive types are as follows:

/T:   base I/O address    master/slave

---------------------------------------------

0     1F0h(primary port)  master

1     1F0h(primary port)  slave

2     170h(2nd port)      master

3     170h(2nd port)      slave

4     1E8h(3rd port)      master

5     1E8h(3rd port)      slave

6     168h(4th port)      master

7     168h(4th port)      slave

If this switch is omitted, it is the same as

/T:0.

/S:(base I/O address)	Change IRQ number to the specified base I/O

address.

base I/O           IRQ No.

address    without /S:    with /S:

(default)	  (alternate)

----------------------------------------------

170h		15		10

1E8h		12		11

168h		10		9

(1F0h)		(14)		(14)

/V			Display installation information.

/B			Use BIOS setting for data transfer.

3. Examples of installation

<Case 1> -- Using /N: and /T: switches together.  This setting

will reduce the loading time of the device driver.

DEVICE = HIT-IDE.SYS /D:MSCD001 /N:2 /T:2

-- Search beginning at /T:2 installing maximum 2 drives.

<Case 2> -- Default is automatic search for all drives connected.

DEVICE = HIT-IDE.SYS /D:MSCD001

-- Install all drives connected to primary, 2nd, 3rd & 4th.

<Case 3> -- Using /N: switch to limit the maximum number of drives.

DEVICE = HIT-IDE.SYS /D:MSCD001 /N:2

-- Install maximum of 2 drives while auto searching.

<Case 4> -- Using /T: switch to change the start of automatic search.

DEVICE = HIT-IDE.SYS /D:MSCD001 /T:2

-- Search beginning at /T:2 (2nd port and master).

<Case 5> -- Using /S: switch to use alternate IRQ numbers.

DEVICE = HIT-IDE.SYS /D:MSCD001 /S:170,168

-- Auto search all drives using alternate IRQ numbers

for 170 (IRQ 10) and for 168 (IRQ 9).   Use default

IRQ numbers for others (1F0 IRQ 14; 1E8 IRQ 12).

<Case 6> -- Using all switches together.

DEVICE = HIT-IDE.SYS /D:MSCD001 /N:1 /T:2 /S:170

-- Install only 1 drive starting search at 2nd port master.

Use alternate IRQ of 10 for 170.

4. History

Ver.1.06 --> 1.06a

- support multi-block read to improve CPU Utilization.

Ver.1.06a --> 1.06d

- improve auto-search sequence at booting.

- fixed bug of handling of some multi-session discs.

The above specifications are subject to change.

April 25, 1996

Hitachi, Ltd.