** NOTE: LAN Manager >REQUIRES< the subdirectory structure which is on this

diskette. If users want to copy the LAN Manager driver to another

disk, the subdirectory and file structure must be the same.

====================================================

=     CN100TX(B) PCI-BUS FAST ETHERNET ADAPTER     =

=     Driver Installation Procedure                =

= >>> LAN Manager Driver, OS/2 Version             =

====================================================

The driver for LAN Manager OS/2 version is in the \MSLANMAN.OS2 subdirectory

of this diskette.

1. Insert the driver floppy disk into drive A: or B:.

2. In the Group-Main window, click the OS/2 window or OS/2 Full Screen icon.

The full screen shell prompt will appear.

3. Change the directory from the root to the subdirectory, LANMAN.

4. Run the SETUP command.

5. Under "Configuration" choose Network Drivers. The user will

see the Workstation Configuration window.

6. Click the Add New Config button.

7. To select the driver for installation, click Other Driver under

Network Adapter Drivers.

8. Then assign the drive, A: or B:, without path. The driver will be

copied to the hard disk. Then, click the OK button.

9. In the window with the title "CNet CN100TX(B) Fast Ethernet Adapter" set

the network protocol to be NetBeui (or whatever is appropriate) and click

Ok.

10. Go back to the Workstation Configuration and confirm the protocols

and driver are correct.

11. Save the setting of the configuration in the Configuration Complete

window.

12. Check the PROTOCOL.INI file to have a correct line speed and

opearting mode. (Please refer to "AUTONEGOTIATION with the attached

device" section of the \README.1ST file for the line speed and operating

mode.)

13. Reboot.

###