1. About SUL321TD.Z file

The compressed image of the distribution diskette.

The DC21X4 distribution diskette contains all the

files required for installing the driver using

custom and netconfig, in particular:

- custom prep, init and remove scripts.

- netconfig init, reconfigure and remove scripts.

- netconfig info files

- Driver.o, the driver

- Space.c, the parameter file

- /usr/include/sys/d21x_space.h - header file, not

intended to be modified

2. Installing SUL321TD.Z driver:

To install the SCO Unix LLI Device Driver:

Copying the distribution files

------------------------------

a. Copy the file SUL321TD.Z to SUL321T.dd.Z on a SCO Unix machine.

The number in parentheses near the file's name (above) is the checksum

for the file. To verify, use:

sum -r SUL321T.dd.Z

and compare the results.

b. Uncompress SUL321T.dd.Z to SUL321T.dd, using:

uncompress SUL321T.dd

c. Copy the file to a diskette. To create a 3.5" floppy in drive 0, use:

dd if=SUL321T.dd of=/dev/rfd0135ds18

Mark the diskette:

DC21X4 PCI Ethernet Adapter LLI driver

Rel.03.21

d. Run custom. From the menu select:

Custom -> Install -> A New Product -> Entire Product

For SCO OpenServer (Everest), run the Software Manager. From the

menus select:

Software -> Install New -> From Host -> Media Floppy -> Full

e. Put the distribution diskette in the drive and select continue.

f. When prompted for distribution floppy 1, just press enter.

g. When custom has completed, remove the diskette and quit custom.

h. Run netconfig. Add the desired chain. The DC21X4 drivers are called:

d21x0, d21x1, d21x2 and d21x3. Remember that d21x0 must be configured

first, before any other driver is configured.

NOTE: For boards with more than one Digital Semiconductor 21X4X on

them, a driver must be installed for each active port on the

board. Thus, if a board has two separate ports, two separate

chains must be created in order to connect both ports to

networks. This is similar to installing two boards, each with

one port.

Not all ports must be active. If only one port is used,

it must be the first port (as it is found on the bus

by the PCI BIOS).

i. When the chain has been configured, select 'q' from the netconfig menu.

If you don't wish to modify the space.c file relink the kernel at the

prompt.

j. If you wish to change parameters in the space.c file, edit

/etc/conf/pack.d/d21x0/space.c and make the required changes.

The changes will apply to all the installed boards.

NOTE: If your system does not support 32-bit protected mode PCI

BIOS calls, you MUST modify the parameter XXX_HW_ACCESSES

to the PCI mechanism appropriate to your system (see space.c

for details).

k. To activate the adapter, you must reboot. When the system boots, you

should see the following message among the other configuration messages:

%d21x   0x2000-0x2200  5	   -	 type=DC21040/TP addr=12-34-56-78-9a-bc

or

%d21x   0x2000-0x2200  5	   -	 type=DC21140/100TX addr=12-34-56-78-9a-bc

with the appropriate I/O addresses, IRQ, serial media and IEEE address.

3.  Parameters

The space.c file contains the different parameters that can be modified.

They can be divided into several groups:

a. Port parameters. These include:

XXX_HW_ACCESSES

XXX_DC2104X_SIA_MODE

XXX_DC2114X_SERIAL_MODE

XXX_BUS_ARBITRATION

XXX_BURST_LENGTH

XXX_CACHE_ALIGNMENT

XXX_PASS_BAD_FRAMES

XXX_BACKOFF_CNTR

XXX_10MB_THRESHOLD

XXX_100MB_THRESHOLD

XXX_PARITY_ERROR_RESPONSE

XXX_LATENCY_TIMER

XXX_CRC_CALC

XXX_DC2114X_GPR_CTRL

XXX_DC2114X_GPR_DATA

XXX_TURBO

XXX_SNOOZE

b. Driver tuning parameters:

XXX_RX_BUFS_TO_POST

XXX_RX_PKTS_QUEUED_THRESH_HI

XXX_RX_PKTS_QUEUED_THRESH_LO

XXX_MAX_USRS

c. Driver diagnostics control:

XXX_DEBUG

The meaning of each is explained in detail in the space.c file, including

allowed values for each parameter. The values in space.c apply to ALL

the installed boards.

Space.c is NOT the place to modify the board's slot number line.

This must be modified in netconfig, by running the reconfiguration

script.

4. Errors and trouble shooting

a. Mismatch between PCI configuration and netconfig/space.c:

In order for the adapter to work correctly, the same values that are

used during the PCI configuration (e.g CFLT) must be used in the

space.c file. Mismatches may cause the adapter to malfunction, or even

hang the system.

Currently, the PCI setup takes precedence except for the value for the

latency timer, which is taken from the space.c file.

b. Wrong HW access method for PCI configuration space:

If the XXX_HW_ACCESSES parameter in space.c is set to a value not

supported by your system there will be problems at boot time. Set

this parameter to a value that is supported by your system.

By default, the parameter is set so the driver uses 32-bit protected

mode PCI BIOS calls.

c. Driver binds to wrong PCI board (network):

When multiple PCI boards are installed, the chains are bound to

the adapters in the order that the adapters are found on the PCI

bus by the PCI BIOS. Thus, d21x0 will bind to the first adapter

found by the PCI BIOS, d21x1 to the second etc. Removing or adding

boards without changing the configuration in netconfig can cause

a shift in the order that chains are bound. Also, mis-identifying

the order in which the adapters are found by the BIOS can cause

wrong bindings. This is especially true when using boards with

PCI-PCI bridges. The adapters on such boards are located on a PCI

bus with a number higher than 0 (bus #1, #2 etc.) and will be found

by the PCI BIOS after all the devices on bus zero.

5. Driver's error messages (when working in debug level > 0)

In "d21xi" the 'i' stands for a number between 0 and 3, indicating

the particular instance of the driver the message is reported by.

* "d21xi Could not intialize driver, PCI BIOS couldn't read/write configuration register"

This message will be printed in PCI board - if can't read or write the

configuration registers when using the PCI BIOS.

* "d21xi Could not intialize driver, found PCMC revision.1 with write-back cache mode"

This message will be printed in PCI board - if PCMC rev.1 chip was found

and it works with write-back cache mode (this mode can cause crash in the

driver).

* "d21xi Could not intialize driver, unsupported bus"

This message will be printed when unsupported bus (PCI is the supported

bus), found.

* "d21xi Could not intialize driver, unsupported PCI board - DC21040

revision.1"

This message will be printed in PCI board - if it is a board with Digital

Semiconductor 21040 revision 1 on it.

* "d21xi Could not intialize driver, unsupported board - DC21140 revision 1.0"

This message will be printed if the board has Digital Semiconductor

21140 revision 1 on it.

* "d21xi Could not intialize driver, unsupported Digital Semiconductor"

This message will be printed if the board doesn't have one of the following

Digital Semiconductor on it:

-DC21040

-DC21140

-DC21140A

-DC21041

-DC21142

* "d21xi Could not intialize driver, couldn't link interrupt handler to interrupt line"

This message will be printed if the driver couldn't link the interrupt

line to the interrupt handler.

* "d21xi Could not intialize driver, unsupported interrupt line was set by the PCI BIOS to CFIT"

This message will be printed in PCI board - if the value of the interrupt

line that was read from the CFLT register > 0xF

* "d21xi Could not intialize driver, unsupported selected serial media"

This message will be printed in boards with Digital Semiconductor 21041

or 21140 or 21140A or DC21142 - if unsupported serial mode was choose by

the user or read from the SROM (in case the user choose to use the SROM

value) for the serial interface of Digital Semiconductor 21041 or 21140

or 21140A or DC21142. And in boards with Digital Semiconductor 21040 -

if unsupported serial mode was choose by the user for the serial

interface of Digital Semiconductor 21040

* "d21xi [d21xinit] Could not initialize driver, autosense mode conflicts

with GPR values in space.c"

For DC21140/DC21140A/DC21142 based adapters, the selected serial mode is

AUTOSENSE and there are GPR values in space.c that override the defaults.

Either select serial mode explicitely, or set the GPR values in space.c

back to the default values (see space.c).

* "d21xi Could not intialize driver, unsupported DC21041 mode"

This message will be printed in boards with DC21041 on it - if the chip

was programmed to be in DC21040 mode (this mode is unsupported by the driver)

* "d21xi Could not intialize driver, PCI board - couldn't read board's address"

This message will be printed in PCI board (when reading IEEE address

using CSR9) if couldn't read board address.

* "d21xi Could not intialize driver, PCI board - wrong checksum of board's address"

This message will be printed in PCI board (when reading IEEE address

using CSR9) if wrong checksum of the IEEE address was read.

* "d21xi Could not intialize driver, PCI board - the upper 16 bits of the CBIO != 0"

This message will be printed in PCI board - if one of the upper 16 bits of

the CBIO register was set by the PCI BIOS

* "d21xi Could not intialize driver, SROM address is odd"

This message will be printed if the driver tried to read odd address in

the SROM.

* "d21xi Could not intialize driver, try to read beyond the SROM last address"

This message will be printed if the driver tried to read beyond the last

SROM address.

* "d21xi Could not intialize driver, can't read/write from/to the SROM"

This message will be printed if the driver couldn't read/write from/to

the SROM

* "d21xi Could not intialize driver, SROM's checksum wrong"

This message will be printed if the checksum of the SROM is wrong.

* "d21xi Could not intialize driver, didn't find board's info in the SROM"

This message will be printed if the board leaf-info in the SROM.

* "d21xi Could not intialize driver, read null address from the SROM"

This message will be printed if the null IEEE address was read from the

SROM.

* "d21xi Could not intialize driver, unsupported board"

This message will be printed if the driver found SROM on the board but

didn't find DC21140 or DC21041 or DC2114A or DC21142 chip.

* "d21xi Could not intialize driver, unsupported SROM version"

This message will be printed if unsupported SROM version was found by the

driver.

* "d21x ERROR, major number n, Ethernet controller configuration problem"

This message will be printed in the d21x_open routine when the driver

couldn't find a specific board with specific major number

* "d21xi [d21x_open] Ethernet controller not found"

This message will be printed in the d21x_open routine when a specific board

wasn't found by the D21Xinit routine and a request for opening a new stream

to the driver was requested.

* "d21xi [d21x_open] too many users are opened"

This message will be printed in the d21x_open routine when a request for

opening a new stream was given and there are too many streams open for

this driver

* "d21xi [d21x_close] Problems deleting the alias address"

This message will be printed in the d21x_close routine when closing a

stream that was opened to the driver and has set an alias address and the

driver can't delete the address and create a new address table to the

Digital Semiconductor 21X4 for some HW problems.

* "d21xi [d21x_close] Problems stopping adapter"

This message will be printed in the d21x_close routine when the last

user of the stream close the connection to the adapter and the driver try

to stop the adapter without success (due to PCI BIOS problems)

* "d21xi [d21x_srvr] putbq failed"

This message will be printed in the d21x_srvr routine when the driver

can't put a unitdata_req in the upper protocol's queue and can't return

it to its own queue (usually indicates a system problem).

* "d21xi Controller has transitioned to halted state

d21xi Halt ID halt_reason"

This message will be printed when the driver detects a HW error and is

moving to Halt state.

* "d21xi - wput : unknown stream message db_type"

This message will be printed in the d21x_wput routine when the driver is

given an unknown STREAM message.

* "d21xi Controller has transitioned to halted state!

Halt ID: TX Jabber Timer Expired"

or:

"d21xi Controller has transitioned to halted state!

Halt ID: Transmit Jabber Timer expiration interrupt"

This messages will be printed when a TX jabber timer expired when DC21X4

sends a packet. In this case the driver stop the adapter operation.

* "d21xi Controller has transitioned to halted state!

Halt ID: Couldn't use PCI BIOS"

This messages will be printed when a the driver failed to use the PCI BIOS

for read and write the configuration registers

* "d21xi Controller has transitioned to halted state!

Halt ID: Setupacket Timeout"

This messages will be printed when the driver fails to send setup packet

* "d21xi Controller has transitioned to halted state!

Halt ID: System Error occured"

This messages will be printed when the driver reads DC21X4 status

register and finds that the system error bit is set

* "d21xi Could not allocate an error_ack message block"

This message will be printed when the driver couldn't allocate an error ack

message (system resources problem).

* "d21xi [d21x_unbind] Bound protocol x not found in protocol table"

This message will be printed when the driver gets an unbind request with a

protocol value and there is no bound stream with this protocol value.

* "d21xi - Invalid MACIOC_SETMCA message"

This message will be printed when the driver gets an invalid MACIOC_SETMCA

message.

* "d21xi - Invalid MACIOC_DELMCA message"

This message will be printed when the driver gets an invalid MACIOC_DELMCA

message.

* "d21xi - Invalid MACIOC_CLRMCA message"

This message will be printed when the driver gets an invalid MACIOC_CLRMCA

message.

* "d21xi - Invalid MACIOC_GETMCA message"

This message will be printed when the driver gets an invalid MACIOC_GETMCA

message.

* "d21xi - Invalid MACIOC_GETADDR message"

This message will be printed when the driver gets an invalid MACIOC_GETADDR

message.

* "d21xi - Invalid MACIOC_SETADDR message"

This message will be printed when the driver gets an invalid MACIOC_SETADDR

message.

* "d21xi - Invalid MACIOC_PROMISC message"

This message will be printed when the driver gets an invalid MACIOC_PROMISC

message.

* "d21xi - Couldn't change driver's filter mode to promiscous"

This message will be printed when the driver gets a MACIOC_PROMISC but the

number of users that open the driver is > 1.

* "d21xi - Invalid MACIOC_GETRADDR message"

This message will be printed when the driver gets an invalid MACIOC_GETRADDR

message.

* "d21xi - Invalid MACIOC_GETMCSIZE message"

This message will be printed when the driver gets an invalid

MACIOC_GETMCSIZE message.

* "d21xi - Internal Error : Mismatch in transmit list pointers"

This message will be printed when something wrong the driver internals

transmit pointers.

* "d21xi - dl_unitdata_req with bad Ethernet header size"

This message will be printed when the driver gets a packet with length >

Ethernet packet length.

* "d21xi - dl_unitdata_req - stream is not bound"

This message will be printed when the driver gets packets to send from

an unbound protocol.

* "d21xi - dl_unitdata_req - DL unitdata request is invalid length!"

This message will be printed when the driver gets a packet to transmit

which isn't a legal Ethernet size.

* "d21xi - wput req : unknown; "

This message will be printed when an unknown wput request was sent from the

upper layer.

* "d21xi - wput ioctl : default, uknown ioctl; "

This message will be printed when an unknown ioctl message was sent from the

upper layer.

* "d21xi - Internal error in autodetect!"

This message will be printed when internal error was detected in media

autodetection

* "d21xi [d21xinit] could not initialize driver, unable to set phy's connection"

This message will be printed when the driver fails to set MII PHY connection

* "d21xi [d21xinit] could not initialize driver, unsupported admin control command"

This message will be printed when the driver try to set an unsupported admin

command to the MII PHY.

* "d21xi [d21xinit] could not initialize driver, can't read MII PHY register

This message will be printed when the driver try to read one of the MII PHY

regiters without a success.