NDIS Dirver for DEC Pathwork Installation

=============================================

This file contains the information about how to install NIC100.DOS

Driver (NDIS 2.0.1) into DEC Pathwork.

The NIC100 Adapter is assumed to have been installed in your computer

and physically connect to the network. And the "Pathworks for DOS" is

assumed to be installed on your hard disk.

Installation Porcedures:

a. Insert the DEC Pathworks Client Setup Disk #1 into yor floppy

driver (A or B).

b. Under DOS prompt, type "NETSETUP" <Enter>.

c. Unless you have an LK250 keyboard, Select "NO".

d. Select the DECNETT transport option and press <Enter>.

e. Insert the DNET TRNSPRT disk #1 into floppy driver.

f. Specify the destination driver letter (C or D).

g. Are you creating an initial workstation diskette? Select "NO".

h. Type the Node Name and Address (i.e. Node Number), Press <Enter>.

i. Type the Server Name and Address (i.i. Node Number), Press <Enter>.

j. Select "Ethernet" when prompt " Select the network adapter type".

k. Select "(Other NDIS)" when prompt "Select the network adapter".

l. Enter full path and driver name for NDIS driver.

A:\PATHWORK\NIC100.DOS	or  B:\PATHWORK\NIC100.DOS

m. If they are all accurate, select "WRITE KEY DISK" to start insta-

llation. Follow all prompts till completion.

n. At the configuration screen, press F10 to return DOS prompt.

o. Unsing text editor to edit PROTOCOL.INI at DECNET sub-directory and

CONFIG.SYS under root directory as following.

--- SAMPLE CONFIG.SYS file ---

:

:

DEVICE=C:\DECNET\PROTMAN.SYS /I:C:\DECNET

DEVICE=C:\DECNET\NIC100.DOS

:

:

--- Sample PROTOCOL.INI file ---

:

:

[IPX4MAC]

DRIVER=IPX$MAC

BINDINGS=NIC100.DOS

[DATALINK]

DRIVERNAME DLL$MAC

LG_BUFFERS=16

SM_BUFFERS=6

OUTSTANDING=32

HEURISTICS=0

BINDINGS=NIC100.DOS

NI_IRQ=3

[NIC100.DOS]

DRIVERNAME=NIC100$

IRQ=3

IOBASE=0X300