***********************************************

*  CNet ISA-Bus PowerNIC Driver Installation  *

*  > NetWare IPX Workstation                  *

***********************************************

The NetWare IPX workstation driver file is stored in the \IPX

subdirectory of the PowerNIC diskette.

1. At the DOS prompt, type the following command to execute the IPX

workstation driver --

[drive]:\IPX\IPX.COM

2. After executing the driver, run NETX.EXE.

3. The following example is to change IPX setting according to hardware

configuration.

a)  Display help screen.

[drive]:\IPX\IPX.COM /?

Novell IPX/SPX  v3.10 (911121)

(C) Copyright 1985, 1991 Novell Inc.  All Rights Reserved.

LAN Option: CNet PowerNIC CN200Eplus Series  v2.20 (960227)

Hardware Configuration: Auto Configuration

Usage: IPX [options]

valid options:

-I or /I                  Display version information

-D or /D                  Display hardware options

-O or /O<num>             Load using hardware option <num>

-C or /C=[path]<filename> Use an alternate configuration file

-? or /?                  Display this help screen

b)  Display hardware options.

[drive]:\IPX\IPX.COM /D

Novell IPX/SPX  v3.10 (911121)

(C) Copyright 1985, 1991 Novell Inc.  All Rights Reserved.

LAN Option: CNet PowerNIC CN200Eplus Series  v2.20 (960227)

Hardware options available:

*  0. Auto Configuration

1. IRQ = 3,      I/O Base = 300H, no DMA or RAM

2. IRQ = 2 or 9, I/O Base = 320H, no DMA or RAM

3. IRQ = 4,      I/O Base = 340H, no DMA or RAM

4. IRQ = 5,      I/O Base = 360H, no DMA or RAM

5. IRQ = 2 or 9, I/O Base = 300H, no DMA or RAM

6. IRQ = 3,      I/O Base = 240H, no DMA or RAM

7. IRQ = 5,      I/O Base = 280H, no DMA or RAM

8. IRQ = 4,      I/O Base = 2C0H, no DMA or RAM

9. IRQ = 4,      I/O Base = 300H, no DMA or RAM

10. IRQ = 5,      I/O Base = 320H, no DMA or RAM

11. IRQ =15,      I/O Base = 340H, no DMA or RAM

12. IRQ =15,      I/O Base = 360H, no DMA or RAM

13. IRQ = 5,      I/O Base = 300H, no DMA or RAM

14. IRQ =10,      I/O Base = 320H, no DMA or RAM

15. IRQ =11,      I/O Base = 340H, no DMA or RAM

16. IRQ =12,      I/O Base = 360H, no DMA or RAM

c)  Change IPX hardware options.

For example:

[drive]:\IPX\IPX.COM /O4