Packet	Driver

FTP PC/TCP Installation

==================================

This programs is complies  with the FTP PC/TCP Packet driver

Specifications. This packet driver allow you to run AE-200 and

ET-200 series Ethernet 16 bit Adapter with FTP PC/TCP software.

This driver included in the directory \PCTCP are contents as

below:

\PCTCP\AE200PD.COM    Packet Driver for 16 bit Ethernet Adapter

\PCTCP\INSTALL.DOC   This file descibes how to install the

packet driver for FTP PC/TCP

Procedure of installing the Packet Driver AE200PD.COM

--------------------------------------------------------------

1. Execute the >AE200PD.COM program by follows command line :

>AE200PD <SOFTWARE_INT> <IRQ> <IO_ADDR>

<SOFTWARE_INT> :  interrupt vector number, which can be

decimal, hexadecimal or octal format.

if:

decimal format -- from 96 to 128

hexadecimal ----- from 0x60 to 0x80

octal format ---- from 0140 to 0200

<IRQ> :	    interrupt setting. The setting can be selected

2,3,4,5,10,11 or 12 for AE-200 and ET-200 series

Ethernet 16 bit Adapter.

(The default setting is 3 on AE-200 and ET-200

series Adapter)

<IO_ADDR>  :     I/O Base Address setting on the AE-200 and ET-200

Ethernet 16 bit Adapter. The setting can be

selected 0x300, 0x320, 0x340, and 0x360 .

(The default setting is 0x300 on AE-200 and

ET-200 series Adapter)

2. Execute the \ETHDRV.EXE device program, which is provided

by FTP with PC/TCP software package together.

Now, your AE-200 or ET-200 series Adapter is work with FTP

PC/TCP.

NOTE: Except the FTP software's PC/TCP package, most of the

DOS version's TCP/IP software also can be run with

packet driver. Please refer to the software package

installation for packet driver install procedure.