There are two kinds of Remote Boot PROM used with NetWare networks.

The two types are referred to as follows:

. IPX Boot PROM

. ODI Boot PROM

It is important that you should know the type of Remote Boot PROM you

are using. Some procedures for setting up remote booting on your computer

are specific to the type of Remote Boot PROM you are using. For more

information about the Remote Boot PROM you are using, please refer to

your dealler.

*** IPX Boot PROM installation procedures ***

1. Make a bootable diskette, place that diskette into floppy driver A.

2. Copy the required files to that diskette, for example, "HIMEM.SYS",

"EMM386.EXE", "SETVER.EXE", "COMMAND.COM" ....

3. Copy "IPX.COM" and "NETX.COM" files to that diskette.

4. Create "AUTOEXEC.BAT" and "CONFIG.SYS" files.

5. Login NetWare as supervisor.

6. Type

"MAP G:=SYS:SYSTEM <Enter>"

"MAP F:=SYS:LOGIN <Enter>"

7. Change to the F drive, then type

"G:DOSGEN <Enter>"

8. Type

"RPLFIX NET$DOS.SYS <Enter>", if your DOS version is 5.0 or higher.

9. Copy "AUTOEXEC.BAT" file from the boot diskette to the LOGIN directory.

10. Turn on the diskless computer.

--- Sample CONFIG.SYS file ---

:

DEVICE=SETVER.EXE

LASTDRIVE=E

:

:

--- Sample AUTOEXEC.BAT file ---

:

IPX

NETX

F:LOGIN

:

:

*** ODI Boot PROM installation procedures ***

-- ODI Boot PROM for NetWare v3.x or NetWare v4.x --

1. Make a bootable diskette, place that diskette into floppy driver A.

2. Copy the required files to that diskette, for example, "HIMEM.SYS",

"EMM386.EXE", "COMMAND.COM" ....

3. Copy "LSL.COM", "NE2000.COM", "IPXODI.COM", "VLM.EXE" and all VLM

files to that diskette.

4. Create "AUTOEXEC.BAT", "CONFIG.SYS" and "NET.CFG" files.

5. Login NetWare as supervisor.

6. Type

"MAP G:=SYS:SYSTEM <Enter>"

"MAP F:=SYS:LOGIN <Enter>"

7. Change to the F drive, then type

"G:DOSGEN <Enter>"

8. Type

"RPLFIX NET$DOS.SYS <Enter>", if your DOS version is 5.0 or higher.

9. Copy "AUTOEXEC.BAT" file from the boot diskette to the LOGIN directory.

10. Turn on the diskless computer.

--- Sample CONFIG.SYS file ---

:

LASTDRIVE=Z

:

:

--- Sample AUTOEXEC.BAT file ---

:

LSL

NE2000

IPXODI

VLM

F:LOGIN

:

:

--- Sample NET.CFG file ---

:

LINK DRIVER NE2000

PORT 300

INT 3

FRAME ETHERNET_802.2

NETWARE DOS REQUESTER

FIRST NETWORK DRIVE = F

:

: