WESTERN DIGITAL

"PLUS"-FAMILY LAN ADAPTER BOARD DRIVERS

FOR USE WITH

NOVELL ADVANCED NetWare, SFT NetWare, AND ELS NetWare LEVEL II

v2.1 AND LATER VERSIONS

INSTALLATION GUIDE

This disk provides a single driver for the entire "PLUS"

(WD8003) family of Western Digital LAN adapter boards for

XT/AT and Micro Channel bus systems:

StarLAN:    StarCard PLUS  (WD8003S)

StarLink PLUS  (WD8003SH)

StarCard PLUS/A  (WD8003ST/A)

Ethernet:   EtherCard PLUS   (WD8003E, WD8003EBT, WD8003EB)

EtherCard PLUS/A  (WD8003ET/A, WD8003E/A)

Ethernet Twisted Pair:

EtherCard PLUS/TP (WD8003WT)

Included in this subdirectory are the following files:

INSTALL.DOC     This File.

RELEASE.DOC     Version number and specific

release information.

Included in the root directory are the actual driver files:

WD8003.LAN                   The LAN driver descriptor file.

WD8003AD.OBJ                 The Server A-Driver object file.

WD8003BD.OBJ                 The Server B-Driver object file.

WD8003CD.OBJ                 The Server C-Driver object file.

WD8003DD.OBJ                 The Server D-Driver object file.

WD8003SH.OBJ                 The Shell Driver object file.

NOTE

NetWare installation procedures require the driver files

listed above to be located in the root directory of a

floppy disk whose volume label is LAN_DRV_200.  If you

copy these files to another floppy disk, be sure that

you set the volume label of the new disk to LAN_DRV_200.

If you are using NetWare v2.12 or earlier that was purchased

from Western Digital, the required drivers are already

pre-installed.  Otherwise, refer to the NetWare Installation

manual and follow the instructions for loading a new LAN driver

from a floppy disk.

During software generation, you must choose "Load and Select

Item" from the "LAN Driver Options" menu.  At the point in the

procedures where you are directed to select the LAN drivers, the

WD8003 drivers will appear in the list of choices as follows:

WD Star/EtherCard PLUS

RAM BUFFER SIZE CONFIGURATION INFORMATION

For XT and AT bus boards, there are various configurations of

the drivers that you can select.  Some are designated for

adapter boards with 8 Kbyte RAM buffers, and some for boards

with 32 Kbyte RAM buffers.

The drivers configured for 32 Kbyte RAM buffers can be used

with any size RAM buffers.  This is becuase they will

automatically revert to using 8 Kbyte RAM if they find that

the adapter board only contains 8 Kbyte RAM or is configured

to only use 8 Kbyte RAM.

SERVER DRIVER CONFIGURATION

For XT/AT bus servers, choose one of the first 16 configuration

selections that has an appropriate interrupt, I/O Base, and RAM

address.

For Micro Channel bus servers, a Micro Channel configuration

must be selected; these are all described as "Micro Channel

Adapter Auto Configuration".  The usual configuration

parameters such as interrupt, I/O base address and RAM base

address have already been established by the system hardware

configuration procedure when you installed the adapter board

in your system.

NOTE

If your NetWare server contains more than one Micro Channel

LAN adapter board, be sure that you manually selected a

different Interrupt Request (IRQ) channel for each adapter

before running the Novell NETGEN procedure.  To do this,

you must use the manual configuration option of the Set

Configuration program on your PC system Reference Diskette

when you install the boards.

SHELL DRIVER CONFIGURATION

This Western Digital shell driver adapts to Micro Channel bus

boards even if a configuration for an XT/AT bus board was

selected.  The driver makes this adjustment based on the type

of adapter board that is actually installed in a workstation,

regardless of the configuration that you selected during the

NetWare shell generation.

Therefore, if you need a shell driver for both XT/AT and Micro

Channel bus workstations, you may generate one for the XT/AT bus

configuration that you need and use the same driver in your

Micro Channel bus workstations as well.

If an XT/AT bus workstation requires some combination of

interrupt, I/O Base and RAM address that is not offered in one

of the 16 configuration choices for XT/AT bus drivers, you can

use the NWPATCH program to create a different configuration.

This patch utility is available from Western Digital.